3GPP TSG RAN WG2 #110-e	R2-2006219

Electronic meeting, 1 – 12 Jun 2020

Source: 	Google

Title: 	Summary for Offline [020][NR15] UE cap IMS voice

Agenda Item:	5.4.3.1

Document for:	Discussion and decision

# Introduction

This document contains a list of TDocs covered in the following offline discussion:

[AT110e][020][NR15] UE cap IMS Voice (Google)

Scope: Treat R2-2005494, R2-2005499, R2-2005535, R2-2005540, R2-2005458, R2-2005459 (proponents are responsible to explain and drive)

Part 1: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

NR-DC CRs

R2-2005494	Introduction of IMS capabilities for NR-DC	Google Inc.	CR	Rel-15	38.306	15.9.0	0338	-	F	NR_newRAT-Core

R2-2005499	Introduction of IMS capabilities for NR-DC	Google Inc.	CR	Rel-16	38.306	16.0.0	0339	-	A	NR_newRAT-Core

R2-2005535	Introduction of IMS capability for NR-DC	Google Inc.	CR	Rel-15	38.331	15.9.0	1677	-	F	NR_newRAT-Core

R2-2005540	Introduction of IMS capability for NR-DC	Google Inc.	CR	Rel-16	38.331	16.0.0	1678	-	A	NR_newRAT-Core

NGEN-DC CRs

R2-2005458	Correction to IMS capabilities for NGEN-DC	Google Inc.	CR	Rel-15	36.306	15.8.0	1768	-	F	NR_newRAT-Core

R2-2005459	Correction to IMS capabilities for NGEN-DC	Google Inc.	CR	Rel-16	36.306	16.0.0	1769	-	A	NR_newRAT-Core.

Companies are invited to share their views.

# Discussion

## IMS voice over SCG bearer for NR-DC

For EN-DC and NGEN-DC, the UE can indicate support of IMS voice over SCG bearer by ims-VoiceOverNR-PDCP-SCG-Bearer-r15 and ims-VoNR-PDCP-SCG-NGENDC-r15 respectively. For NE-DC, the UE can indicate support of IMS voice over SCG bearer by voiceOverSCG-BearerEUTRA-5GC.

However, for NR-DC, there is no similar capabilty signaling for the UE to indicate support of IMS voice over SCG beaer. In other words, the UE is mandated to support IMS voice over SCG bearer. This mandatory requirement for NR-DC creates burdens in the UE implementation and IOT.

Q1: Do you agree to introduce a capability signalling for IMS voice over SCG bearer for NR-DC, as proposed in the NR-DC CRs (R2-2005494, R2-2005499, R2-2005535, R2-2005540)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Google | Yes | The UE should be able to indicate whether the IMS voice over SCG bearer is supported for NR-DC as EN-DC, NGEN-DC and NE-DC. |
| Lenovo | No | It is not clear to us what kind of serious UE implementation burden is created if UE supports VoNR and NR-DC. It should be noted that the situation in LTE is different when it was decided to introduce SCG-related UE capabilities for (NG)EN-DC considering the fact that new RAT and NR PDCP need to be supported by the LTE UE. |
| Huawei | No | We actually do not see much need to have this. The reason that why we have separate capability on (NG)EN-DC and NE-DC is mainly due to the different RAT leg. However for NR-DC, they are both using NR leg and we are not sure whether this is essential to add another capability specific for SCG. |
| Qualcomm Incorporated | Yes | We understand that IMS voice over SCG of NR-DC is not a popular option to be used in the field and foresee IOT availability issues. |
| vivo | Yes | Introducing a capability for IMS voice over SCG could allow more flexible UE implementation. |
| MediaTek | No | We also do not see an implementation problem to support voice over SCG, and as Huawei notes the legs are using the same RAT. So we don’t see a strong reason to have this capability. |
| Intel | No | We also agree with Huawei that for NR-DC, the RAT is the same and hence we do not see a real need for to introduce an additional capability bit. |
| Samsung | Yes | We don’t think there is real use case for IMS over SCG bearer in NR-DC. Then to relieve test/implementation burden, introducing the capability signalling is beneficial |
| Ericsson | Yes | We don’t have a strong view, if some UE vendors prefer a separation of this capability bit, we are fine with that. |
| Nokia |  | We agree with Lenovo that split of the capabilities for mixed RAT scenarios was more meaningful. Currently, we understand that voiceOverNR in pure NR cover both MCG and SCG. The Change would limit the existing capability bit to MCG bearer applicability only, while now it covers bith MCG and SCG. In our understanding this wasn’t a missing UE capability but proposal on different design. |
| OPPO | Yes | We also agree VoNR via SCG of NR-DC is not popular solution hence UE capability makes more sense |

[Summary of the discussion]

6 companies agree to introduce a capability signalling for IMS voice over SCG bearer for NR-DC and 5 companies don’t agree to do it. Given no absolute majority agree to introduce the capability, we will update the 38.306 CRs to remove the changes on the capability signalling for IMS voice over SCG bearer and there is no need to have 38.331 CRs.

## IMS voice over split bearer for NR-DC, NGEN-DC and NE-DC

It is described in section 4.3.1A in 36.306 [1] that “In this release of specification, IMS voice over split bearer is not supported for EN-DC”. For NR-DC, NGEN-DC and NE-DC, there is no such description. In other words, the UE is mandated to support IMS voice over SCG bearer for NR-DC, NGEN-DC and NE-DC. This mandatory requirement for NR-DC, NGEN-DC and NE-DC creates burdens in the UE implementation and IOT.

Q2: Do you agree the split bearer is not supported for NR-DC, as proposed in the NR-DC CRs above (R2-2005494, R2-2005499, R2-2005535, R2-2005540)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Google | Yes | We prefer to align all MR-DC cases with EN-DC. |
| Lenovo | Yes | It is our understanding that IMS voice is not supported for split bearer at all (i.e. in both LTE and NR). However, we agree that this has been not captured yet in NR specs. |
| Huawei | Yes | We agree that IMS voice over split bearer is not supported for all MR-DC options. |
| Qualcomm Incorporated | Yes |  |
| vivo | Yes | Split bearer is designed for heavy payload between different NBs but IMS voice is not a heavy throughput service. So in our understanding IMS voice is not supported for split bearer for any of the RAT combinations. |
| MediaTek | Slightly no | We understand the motivation not to support voice over a split bearer with different RATs, but as with the question above, we don’t see what the implementation problem is to support it for NR-DC. However, we acknowledge vivo’s point that it may not be a critical use case, and we can accept the majority view. |
| Intel | Yes | There is little motivation to support voice over split bearer. This can reduce unnecessary scenarios. |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Nokia | Yes | In our understanding this is not supported as for other scenarios, however, the specification doesn’t clarify the NR DC case. |
| OPPO | Yes |  |

[Summary of the discussion]:

10 companies agree that IMS voice over split bearer is not supported for NR-DC and 1 company slightly disagree it.

Q3: Do you agree the split bearer is not supported for NGEN-DC, as proposed in the NGEN-DC CRs above (R2-2005458, R2-2005459)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Google | Yes | We prefer to align all MR-DC cases with EN-DC. |
| Lenovo | Yes | Same comment as for Q2 above. |
| Huawei | Yes | See our comments above. |
| Qualcomm Incorporated | Yes |  |
| vivo | Yes |  |
| MediaTek | Yes | Here it makes sense to align to EN-DC. |
| Intel | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Nokia | Yes | Our expectation also has been the behaviour is aligned. |
| OPPO | Yes |  |

[Summary of the discussion]:

All companies agree that IMS voice over split bearer is not supported for NGEN-DC.

The change for the IMS voice over split bearer for NE-DC is missing in the CRs. We propose to discuss the NE-DC case in the discussion as well.

Q4: Do you agree that the split bearer is not supported for NE-DC?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Google | Yes | We prefer to align all MR-DC with EN-DC. If majority companies agree that the split bearer is not supported for NE-DC, we will update the NR-DC 38.306 CRs to capture the agreement. |
| Lenovo | Yes | Same comment as for Q2 above. |
| Huawei | Yes | See our comments above. |
| Qualcomm Incorporated | Yes |  |
| vivo | yes |  |
| MediaTek | Yes |  |
| Intel | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Nokia | yes | As commented above |
| OPPO | Yes |  |

[Summary of the discussion]:

All companies agree that IMS voice over split bearer is not supported for NE-DC.

## Other changes in NGEN-DC CRs (R2-2005458, R2-2005459)

Description for ims-VoNR-PDCP-SCG-NGENDC-r15 is missing in 36.306. The NGEN-DC CRs contain other changes to add description for ims-VoNR-PDCP-SCG-NGENDC-r15 and clarify the field ims-VoiceOverNR-PDCP-SCG-Bearer-r15 is for EN-DC.

Q5: Do you agree with the other changes in the NGEN-DC CRs?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Google | Yes |  |
| Lenovo | Yes | We agree on the changes, however there are some issues to fix: - Rel-15 CR R2-2005458: wrong spec version 15.9.0 on cover page. - For both Rel-15/16 CRs: we think that the CRs merely align 36.306 with 36.331. Therefore, “Consequences if not approved” can be revised by saying: TS 36.306 is not aligned with TS 36.331. Furthermore, on inter-operability aspect we can simply say “There are no inter-operability issues.” [Google] We are fine to say there are no inter-operability issues. Regarding “Consequences if not approved”, IMS voice over the split bearer for NGEN-DC is not specified in 36.331, the CR is not just to align with 36.331. - For both Rel-15/16 CRs: To be aligned with the descriptions in 36.331 we can add “RLC” for MCG/SCG bearer in the description of ims-VoiceOverNR-PDCP-MCG-Bearer-r15 and ms-VoiceOverNR-PDCP-SCG-Bearer-r15, i.e. say “MCG RLC bearer/SCG RLC bearer”. |
| Huawei | Yes | We are OK to add this, but we think “when configured with EN-DC” can be changed to “for EN-DC”. [Google] We are fine with the suggestion. |
| Qualcomm Incorporated | Yes | The description for ims-VoiceOverNR-PDCP-MCG-Bearer-r15 and ims-VoiceOverNR-PDCP-SCG-Bearer-r15 can further be aligned to those in 36.331. |
| vivo | Yes |  |
| MediaTek | Yes | Agree the wording should be fixed for alignment with 36.331. |
| Intel | Yes |  |
| Samsung | Yes |  |
| Ericsson | Yes |  |
| Nokia | Yes | Agree the wording should be fixed for aligned |
| OPPO | Yes |  |

[Summary of the discussion]:

All companies agree the intention of the other changes in the NGEN-DC CRs.

# Conclusion

Based on the discussion above, we propose the following

Proposal 1: Capture IMS voice over split bearer for NR-DC and NE-DC is not supported in the 38.306 CRs.

Proposal 2: Capture IMS voice over split bearer for NGEN-DC is not supported in the 36.306 CRs.

Proposal 3: Update the 36.306 CRs to align the wording with 36.331 for the other changes and take other preferred changes into account.

The updated 36.306 and 38.306 CRs are provided in R2-2006920, R2-2006921, R2-2006922 and R2-2006923.

# Reference

[1] 3GPP TS 36.306 v15.9.0.