3GPP TSG-RAN WG2 Meeting #110	R2-2006415

# Electronic, 1 – 12 June 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 1614 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the presence of ssb-perRACH-Occasion for the CSI-RS based CFRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE corporation, Sanechips, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-05-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the description in TS38.213, the indexing of the PRACH occasions indicated by ra-OccasionList is reset per PRACH association period. Thus, UE should be provided with the association between SSB and RACH occasions to derive the PRACH association period so that it can understand the indexing of the PRACH occasions indicated by ra-OccasionList when CSI-RS based CFRA is configured in RACH-ConfigDedicated, which cannot be achieved by the current RACH-ConfigDedicated because the field ssb-perRACH-Occasion is absent when the field resources in CFRA is set to csirs due to the following presence condition for ssb-perRACH-Occasion in RACH-ConfigDedicated. Conditional Presence; Explanation. SSB-CFRA; The field is mandatory present if the field resources in CFRA is set to ssb; otherwise it is absent.. Thus, it is suggested to change the presence condition of ssb-perRACH-Occasion and this field should be mandatory present when the field resources in CFRA is set to ssb or csirs. |
| Summary of change: |  | The conditional presence for ssb-perRACH-Occasion is changed from“SSB-CFRA” into “Mandatory” to indicate that this field should be mandatory present when the field resources in CFRA is set to ssb or csirs. Impact Analysis Impacted 5G architecture options: SA, NE-DC, NR-DC Impacted functionality: Random access Inter-operability: 1. If the network is implemented according to the CR and the UE is not, no inter-operability issue is foreseen. 2. If the UE is implemented according to the CR and the network is not, the field ssb-perRACH-Occasion is absent when the field resources in CFRA is set to csirs. UE is not able to decide the association pattern period and thus cannot understand the index of the PRACH occasions indicated by ra-OccasionList. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The field ssb-perRACH-Occasion is absent when the field resources in CFRA is set to csirs. UE is not able to decide the association pattern period and thus cannot understand the index of the PRACH occasions indicated by ra-OccasionList. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 6.3.2	Radio resource control information elements

#### –	RACH-ConfigDedicated

The IE RACH-ConfigDedicated is used to specify the dedicated random access parameters.

RACH-ConfigDedicated information element

-- ASN1START

-- TAG-RACH-CONFIGDEDICATED-START

RACH-ConfigDedicated ::=        SEQUENCE {

cfra                            CFRA                                                                    OPTIONAL, -- Need S

ra-Prioritization               RA-Prioritization                                                       OPTIONAL, -- Need N

...,

[[

rachConfigDedicatedIAB-r16      RACH-ConfigDedicated-IAB-v16xy                                          OPTIONAL, -- Need S

ra-PrioritizationTwoStep-r16    RA-Prioritization                                                       OPTIONAL, -- Need N

cfra-TwoStep-r16                CFRA-TwoStep-r16                                                        OPTIONAL  -- Need N

]]

}

CFRA ::=                    SEQUENCE {

occasions                       SEQUENCE {

rach-ConfigGeneric              RACH-ConfigGeneric,

ssb-perRACH-Occasion            ENUMERATED {oneEighth, oneFourth, oneHalf, one, two, four, eight, sixteen}

OPTIONAL  -- Cond Mandatory

}                                                                                                       OPTIONAL, -- Need S

resources                       CHOICE {

ssb                             SEQUENCE {

ssb-ResourceList                SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex        INTEGER (0..15)

},

csirs                           SEQUENCE {

csirs-ResourceList              SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS            RSRP-Range

}

},

...,

[[

totalNumberOfRA-Preambles INTEGER (1..63)                                                         OPTIONAL -- Cond Occasions

]]

}

CFRA-TwoStep-r16 ::=                    SEQUENCE {

occasionsTwoStepRA-r16                  SEQUENCE {

rach-ConfigGenericTwoStepRA-r16         RACH-ConfigGeneric,

ssb-PerRACH-OccasionTwoStepRA-r16       ENUMERATED {oneEighth, oneFourth, oneHalf, one,

two, four, eight, sixteen}                    OPTIONAL  -- Cond SSB-CFRA

}                                                                                                     OPTIONAL, -- Need S

msgA-CFRA-PUSCH-r16                     MsgA-PUSCH-Config-r16,

resourcesTwoStep-r16                    CHOICE {

ssb                                     SEQUENCE {

ssb-ResourceList                        SEQUENCE (SIZE(1..maxRA-SSB-Resources)) OF CFRA-SSB-Resource,

ra-ssb-OccasionMaskIndex                INTEGER (0..15)

},

csirs                                   SEQUENCE {

csirs-ResourceList                      SEQUENCE (SIZE(1..maxRA-CSIRS-Resources)) OF CFRA-CSIRS-Resource,

rsrp-ThresholdCSI-RS                    RSRP-Range

}

},

totalNumberOfTwoStepRA-Preambles-r16    INTEGER (1..62),

...

}

CFRA-SSB-Resource ::=           SEQUENCE {

ssb                             SSB-Index,

ra-PreambleIndex                INTEGER (0..63),

...

}

CFRA-CSIRS-Resource ::=         SEQUENCE {

csi-RS                          CSI-RS-Index,

ra-OccasionList                 SEQUENCE (SIZE(1..maxRA-OccasionsPerCSIRS)) OF INTEGER (0..maxRA-Occasions-1),

ra-PreambleIndex                INTEGER (0..63),

...

}

RACH-ConfigDedicated-IAB-v16xy ::=      SEQUENCE {

prach-ConfigurationPeriodScaling-r16    ENUMERATED {scf1,scf2,scf4,scf16,scf32,scf64},

prach-ConfigurationFrameOffset-r16      INTEGER (0..63),

prach-ConfigurationSOffset-r16          INTEGER (0..39)

}

-- TAG-RACH-CONFIGDEDICATED-STOP

-- ASN1STOP

Editor's note: Details on signalling the PRU for 2-step CFRA msg PUSCH is still TBD.

CFRA-CSIRS-Resource field descriptions

csi-RS The ID of a CSI-RS resource defined in the measurement object associated with this serving cell.

ra-OccasionList RA occasions that the UE shall use when performing CF-RA upon selecting the candidate beam identified by this CSI-RS. The network ensures that the RA occasion indexes provided herein are also configured by prach-ConfigurationIndex and msg1-FDM. Each RACH occasion is sequentially numbered, first, in increasing order of frequency resource indexes for frequency multiplexed PRACH occasions; second, in increasing order of time resource indexes for time multiplexed PRACH occasions within a PRACH slot and Third, in increasing order of indexes for PRACH slots.

ra-PreambleIndex The RA preamble index to use in the RA occasions associated with this CSI-RS.

CFRA field descriptions

occasions RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommon in the first active UL BWP.

ra-ssb-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGeneric Configuration of contention free random access occasions for CFRA. The UE shall ignore preambleReceivedTargetPower, preambleTransMax, powerRampingStep, ra-ResponseWindow signaled within this field and use the corresponding values provided in RACH-ConfigCommon.

ssb-perRACH-Occasion Number of SSBs per RACH occasion.

totalNumberOfRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for RA. The setting should be consistent with the setting of ssb-perRACH-Occasion, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

CFRA-SSB-Resource field descriptions

ra-PreambleIndex The preamble index that the UE shall use when performing CF-RA upon selecting the candidate beams identified by this SSB.

ssb The ID of an SSB transmitted by this serving cell.

CFRA-TwoStep field descriptions

msgA-CFRA-PUSCH PUSCH resource configuration(s) for msgA CFRA.

occasionsTwoStepRA RA occasions for contention free random access. If the field is absent, the UE uses the RA occasions configured in RACH-ConfigCommonTwoStepRA in the first active UL BWP.

ra-SSB-OccasionMaskIndex Explicitly signalled PRACH Mask Index for RA Resource selection in TS 38.321 [3]. The mask is valid for all SSB resources signalled in ssb-ResourceList.

rach-ConfigGenericTwoStepRA Configuration of contention free random access occasions for CFRA 2-step random access type. The UE shall ignore msgA-preambleReceivedTargetPower, preambleTransMax, msgA-powerRampingStep, msgB-ResponseWindow, msgA-TransMax signaled within this field and use the corresponding values provided in RACH-ConfigCommonTwoStepRA.

ssb-PerRACH-OccasionTwoStep Number of SSBs per RACH occasion for 2-step random access type.

totalNumberOfTwoStepRA-Preambles Total number of preambles used for contention free random access in the RACH resources defined in 2-step CFRA, excluding preambles used for other purposes (e.g. for SI request). If the field is absent but the field occasions is present, the UE may assume all the 64 preambles are for 2-step RA. The setting should be consistent with the setting of ssb-perRACH-OccasionTwoStep, if present, i.e. it should be a multiple of the number of SSBs per RACH occasion.

RACH-ConfigDedicated field descriptions

cfra Parameters for contention free random access to a given target cell. If this field and cfra-TwoStep are absent, the UE performs contention based random access.

cfra-TwoStep Parameters for contention free 2-step random access type to a given target cell. Network ensures that cfra and cfra-TwoStep are not configured at the same time.

rachConfigDedicatedIAB Prach configuration for the IAB-MT.

ra-prioritization Parameters which apply for prioritized random access procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

ra-PrioritizationTwoStep Parameters which apply for prioritized 2-step random access type procedure to a given target cell (see TS 38.321 [3], clause 5.1.1).

| Conditional Presence | Explanation |
| --- | --- |
| Mandatory | The field is mandatory present. |
| Occasions | The field is optionally present, Need S, if the field occasions is present, otherwise it is absent. |

End of change