3GPP TSG-RAN WG2 Meeting #110 electronic	R2-200xxxx

1 - 12 June, 2020

Agenda Item:	8.8

Source: 			Session Chair (Samsung)

Title: 		Report from session on LTE V2X and NR V2X

Document for:	Approval

- 4.3	V2X and Sidelink corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session.

- 4.3.0	In-principle agreed CRs

- 4.3.1	Other

R2-2005572	Correction on Uu and PC5 prioritization	ASUSTeK	CR	Rel-14	36.321	14.12.0	1471	1	F	LTE_V2X-Core	R2-2003642

R2-2005573	Correction on Uu and PC5 prioritization	ASUSTeK	CR	Rel-15	36.321	15.8.0	1470	1	A	LTE_eV2X-Core	R2-2003641

- 6.4	NR V2X

(5G_V2X_NRSL-Core; leading WG: RAN1; REL-16; started: Mar 19; target; June 20; WID: RP-200129; SR: RP-200431). Documents in this agenda item will be handled in a break out session

Time budget: 3 TU

- 6.4.1	General

Including incoming LSs, rapporteur inputs, etc. Contributions in this AI are reserved for WI rapporteur inputs and/or spec rapporteur inputs and do not count towards the tdoc limits.

R2-2004312	LS on the 3GPP work on the NR sidelink (S-200078; contact: VolksWagen)	5GAA WG4	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN, RAN1, RAN2

R2-2004314	LS on LTE V2X capabilities in NR V2X (R1-2002930; contact: Huawei)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004316	LS reply to RAN WG2 LS on NR V2X Security issues (S3-200820; contact: CATT)	SA3	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004336	Reply LS to RAN2 on Sidelink UE Information (R3-202831; contact: LGE)	RAN3	LS in	Rel-16	5G_V2X_NRSL	To:RAN2

R2-2004343	LS on HARQ parameters for Mode 1 (R1-2002848; contact: Ericsson)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004348	LS on sidelink HARQ operations (R1-2002985; contact: LGE)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004349	LS on sidelink CSI report (R1-2002986; contact: LGE)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004350	LS on NR V2X Slot number determination (R1-2002990; contact: CATT)	RAN1	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004374	Reply LS on Sidelink UE capability for (NG)EN-DC and NE-DC (R4-2005646; contact: CATT)	RAN4	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2005727	LS reply to RAN WG2 LS on the security related issues for NR SL (S3-201483; contact: CATT)	SA3	LS in	Rel-16	5G_V2X_NRSL-Core	To:RAN2

R2-2004576	(draft)Reply LS on sidelink HARQ operations	ZTE Corporation, Sanechips	LS out	Rel-16	5G_V2X_NRSL-Core	To:RAN1

R2-2004982	[draft]Reply LS on sidelink HARQ operations 	CATT	LS out	To:RAN1	Late

R2-2005036	LS on sidelink HARQ operations	Lenovo, Motorola Mobility	LS out	5G_V2X_NRSL-Core	To:RAN1

R2-2005229	[DRAFT] LS response to RAN1 on Sidelink HARQ operation	Intel Corporation	LS out	Rel-16	5G_V2X_NRSL-Core	To:RAN1

R2-2005299	Draft LS response on sidelink HARQ operations	vivo	LS out	To:RAN1

- 6.4.2	Control plane

- 6.4.2.1	RRC

Including [Post109bis-e][952][V2X], [Post109bis-e][953][V2X], [Post109bis-e][954][V2X], and RRC ASN.1 issues that require WI-specific discussion. For accepted RIL issues, the proponent company can provide a discussion doc with an annex TP (if needed). Contributions should be reserved for more complicated issued and minor issues are expected to be resolved in RRC email discussions without any tdoc. This agenda item will utilize a summary document (Huawei).

R2-2005711	Summary document of AI 6.4.2.1 -  RRC aspects	Huawei, HiSilicon	discussion	Late

Proposal 1: An SR configuration ID is specified in the SL-SRB configuration of each SCCH respectively. When the NW configures an SR configuration with the SR configuration ID associated with an SL-SRB, the SR configuration is used for that SL-SRB.

Proposal 1a: If Proposal 1 is not agreed, RAN2 agree that when SL-BSR is triggered by an SL-SRB, it can trigger SR transmission by using any SR configuration used for NR SL.

Proposal 2: RAN2 to confirm the following common understanding: for a PC5 RRC connection of NR SL unicast

- All SL-DRBs have the same security configuration on whether ciphering and/or integrity protection is enabled/configured;

- All security configurations on whether to enable/configure ciphering and integrity protection are exchanged in the upper layers via PC5-S signalling and then instructed to the AS, for SL-SRB carrying PC5 RRC signalling and for SL-DRBs.

Proposal 2a: For both SL-SRB carrying PC5 RRC signaling and SL-DRBs, no signaling exchange or operation in RRC is needed for the security related configurations to enable/configure ciphering and/or integrity protection.

Proposal 3: For H337, RAN2 to further discuss and conclude whether to move the SLRB addition/modification/release procedures out of current 5.8.9.1 and into a new subclause 5.8.9.X. If this is agreed, adopt the text proposal provided in [10].

Proposal 4: For H338, RAN2 to further discuss, for SLRB release procedures towards the SL DRB configured both by its NW and by its peer UE for bi-direction transmission, whether to:

- Keep the current spec style, i.e. UE releases the DRB only after both its NW and its peer UE inform the SL DRB release; or

- Change the current spec style, i.e. UE releases the DRB once either its NW or its peer UE inform the SL DRB release.

If there is no consensus or majority’s support for change, RAN2 keep the related texts in the current specification and do not pursue any changes.

Proposal 4a: If RAN2 agree to make change for Proposal 4, take the Related text proposals provided in R2-2002625 (OPPO) as the baseline.

R2-2005496	Summary of email discussion [952][V2X] RRC ASN.1 issues-38.331	Huawei, HiSilicon	discussion	Rel-16	5G_V2X_NRSL-Core	Late

Proposal 1: RAN2 agrees the left-over issues from last meeting from [Offline 701] are discussed during online session based on the summary document in R2-2005711, including H337, H338, H336 and H335.

Proposal 2: R2 agrees to update the RIL status as following in P2a-e, on which no company was against to the rapporteur’s Proposed Status.

Proposal 2a: Following RILs are agreed as “ConcAgree (WI-CR)”: B101, B102, E237, H331, E245, H333, H343, H339, H344, H345, H347, M116, H330, E208, V007, E267.

Proposal 2b: Following RILs are agreed as “ConcAgree (WI-CR)” with potential minor wording adjustment during the WI-CR review: E236, E214, E241, V023, H332, E246, E260, M117, H350, H351, H334, M113, M112

Proposal 2c: Following RILs are agreed as “ConcAgree (WI-CR)” conditionally, only if the original texts are not removed by [Post109bis-e][932][LTE/NR/ASN.1]; otherwise, they’ll be automatically “ConcReject”): E238, E248, E249, E250, S106, S107, S108, S109

Proposal 2d: Following RILs are agreed as “ConcAgree (WI-CR)” conditionally, if the multiple mode-1 pool is finally not supported by RAN1; otherwise, it is automatically “ConcReject)”: O306

Proposal 2e: Following RILs are agreed as “ConcReject”: E242, C403, CATT404, E262, B104, S117.

Proposal 3: Following RILs can be discussed by offline discussion (which are either with Status of DiscMeet or flagged by companies): CATT401, E211, E212, E261 (after see the Tdoc), CATT402(after see the Tdoc), E240, B103, , H352, E247, H340, H341, H342, H346, E210, O315, O312, H348, H349, M115, M114, O314, O313, V022, S116, O311 (after conclusion of MAC discussion).

Proposal 4: Following RILs are moved to ASN.1 review discussion, and covered by [Post109bis-e][932][LTE/NR/ASN.1], no discussion will be carried for them again in V2X session: O310, E047, E055, E057.

R2-2005495	Miscellaneous corrections to 38.331 for V2X	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1569	2	F	5G_V2X_NRSL-Core	R2-2004072	Late

R2-2005491	Corrections on V2X functionalities in TS 36.331	Huawei, Hisilicon	CR	Rel-16	36.331	16.0.0	4336	-	F	5G_V2X_NRSL-Core

R2-2004487	Summary of [Post109bis-e][954][V2X] SIB12 overhead reduction (OPPO)	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

Recommendation: No optimization on SIB12 size is introduced in Rel16.

R2-2004485	R2-20xxxxx_Introduction of segementation for SIB12	OPPO	CR	Rel-16	38.331	16.0.0	1607	-	F	5G_V2X_NRSL-Core

R2-2004486	R2-20xxxxx_Introduction of segementation for SIB28	OPPO	CR	Rel-16	36.331	16.0.0	4295	-	F	5G_V2X_NRSL-Core

R2-2004401	Left issues on RRC running CR [O311, O312, O315]	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004404	Correction on SL configuration procedure	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004525	Corrections to Interruption handling during RLF	Samsung Electronics Co., Ltd	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004577	Discussion on remaining issue related to RRC in NR V2X	ZTE Corporation, Sanechips	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004596	Remaining issues on RRC for NR V2X	Nokia, Nokia Shanghai Bell	discussion	Rel-16	5G_V2X_NRSL-Core	R2-2003312

R2-2004712	Size of sl-PSFCH-RB-Set in SIB12 [M117]	MediaTek Inc.	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004901	[C402] Correction on (Re)Selection of Synchronisation Reference	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004911	[C403] The Detail of Slot Number Determination in 38.331	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004935	[C401]New RRC connection establishment trigger	CATT	discussion	Late

R2-2004937	[C404]Issue on consistent zone configuration	CATT	discussion	Late

R2-2005131	[B103] TP for sidelink transmission during fast MCG link recovery	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2005132	[B104] TP for sidelinkUEinformation with fast MCG link recovery	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2005179	[E261] Miscellaneous corrections for NR V2X	Ericsson	draftCR	Rel-16	38.331	16.0.0	F	5G_V2X_NRSL-Core	Late

R2-2005180	[E212] Correction to addModList for SL measurements	Ericsson	draftCR	Rel-16	38.331	16.0.0	F	5G_V2X_NRSL-Core	Late

R2-2005293	Sidelink communication reception (RIL#V022)	vivo	discussion

R2-2005294	Align RRC and SA2 spec on sidelink SRB handling (RIL#V023)	vivo	discussion

R2-2005295	UE behavior upon detecting sidelink SRB integrity check failure (RIL#V024)	vivo	discussion

R2-2005310	Need codes in sl-RxPool [M114]	MediaTek Inc.	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005326	Corrections to SUI and RRCReconfigurationSidelink	InterDigital	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005327	Configuration of HARQ Enable for NR V2X	Interdigital	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005461	[H335] SR configuration for SL SRB	Huawei, HiSilicon	discussion

R2-2005462	[H336] Discussion on security policy related aspects for NR SL unicast	Huawei, HiSilicon	discussion

R2-2005463	[H352] Handling of integrity check failure in RRC for NR SL unicast	Huawei, HiSilicon	discussion

R2-2005530	Discussion on Interoperability of V2X UEs camped in different cells	Apple, InterDigital Inc.	discussion	Rel-16	5G_V2X_NRSL-Core	R2-2002808

R2-2005542	Remaining issues for NR SL preconfiguation parameters	Qualcomm Finland RFFE Oy	discussion	Rel-16	38.331

R2-2005544	Sidelink PDCP out of order delivery configuration	Samsung Electronics Co., Ltd	discussion	Rel-16	38.331	5G_V2X_NRSL-Core

R2-2005545	Configuration of remaining ROHC related parameters	Samsung Electronics Co., Ltd	discussion	Rel-16	38.331	5G_V2X_NRSL-Core

R2-2005546	Clarification of SLRB configuration procedures	Samsung Electronics Co., Ltd	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	R2-2003679

R2-2005615	Left issues on RRC for NR V2X	LG Electronics France	discussion	Rel-16	38.331	5G_V2X_NRSL-Core

Withdrawn:

R2-2004760	Introduction of Sidelink Counter Check Procedure	Apple	discussion	Rel-16	5G_V2X_NRSL-Core	Withdrawn

R2-2004899	[C402] Correction on (Re)Selection of Synchronisation Reference	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004919	[C401]New RRC connection establishment trigger	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004920	[C401]New RRC connection establishment trigger	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004921	[C401]New RRC connection establishment trigger	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004931	[C401]New RRC connection establishment trigger	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

R2-2004933	[C401]New RRC connection establishment trigger	CATT	discussion	Rel-16	38.331	Late

R2-2004934	[C401]New RRC connection establishment trigger	CATT	discussion	Rel-16	38.331	5G_V2X_NRSL-Core	Late

- 6.4.2.2	Others

Including [Post109bis-e][955][V2X], [Post109bis-e][956][V2X], and remaining other control plane issues (idle/inactive UE procedure, capabilities). Tdoc limitation: 1 tdoc for discussion with an annex TP (if needed) per specification. This agenda item will utilize summary documents (capability: OPPO, idle/inactive procedures: ZTE).

R2-2004402	Summary of [Post109bis#955] V2X UE capability issues (OPPO)	OPPO	report	Rel-16	5G_V2X_NRSL-Core	Late

Proposal 1	[Easy] For SL capability report on Uu-RRC, introduce RLC parameters: a) 12-bit SN length for UM, b) 18-bit SN for AM, and MAC parameter: multiple SR configuration.

Proposal 2	[FFS] FFS on the necessity of capability for Range-based HARQ feedback.

Proposal 3	[Easy] For SL capability report on Uu-RRC agreed in RAN2, they are per-UE capability.

Proposal 4	[Easy] For SL capability report on Uu-RRC agreed in RAN2, allow FDD/TDD differentiation only for a) Logical channel SR-delay timer, b) Multiple CGs and c) multiple SR configuration.

Proposal 5	[Easy] For SL capability report on Uu-RRC agreed in RAN2, no need for FR1/FR2 differentiation.

Proposal 6	[Easy] For SL capability report on Uu-RRC agreed in RAN2, conditionally (i.e., if UE supports NR sidelink) mandatory feature without capability signalling includes PDCP parameters: 1) 12-bit SN, 2) 18-bit SN, and RLC parameter: 1) 6-bit SN for UM, 2) 12-bit SN for AM;

Proposal 7	[FFS] RAN2 further discuss whether/how for UE to report the support of NR sidelink on Uu-RRC.

Proposal 8	[Easy] For SL capability report on Uu-RRC agreed in RAN2, optional feature with capability signaling includes RLC parameter: 1) 12-bit SN for UM, 2) 18-bit SN for AM; and MAC parameter: 1) LCP restriction, 2) Logical channel SR-delay timer, 3) Multiple CGs, 4) multiple SR configuration.

Proposal 9	[Easy] For SL capability report on PC5-RRC, introduce RLC parameters: a) 12-bit SN length for UM, b) 18-bit SN for AM.

Proposal 10	[Easy] For SL capability report on PC5-RRC agreed in RAN2, they are per-UE capability.

Proposal 11	[Easy] For SL capability report on PC5-RRC agreed in RAN2, no need for either FDD/TDD or FR1/FR2 differentiation.

Proposal 12	[Easy] For SL capability report on PC5-RRC agreed in RAN2, conditionally (i.e., if UE supports NR sidelink) mandatory feature without capability signalling includes PDCP parameters: 1) 12-bit SN, 2) 18-bit SN, and RLC parameter: 1) 6-bit SN for UM, 2) 12-bit SN for AM;

Proposal 13	[Easy] For SL capability report on PC5-RRC agreed in RAN2, optional feature with capability signaling includes PDCP parameter: out-of-order delivery, RLC parameter: 1) 12-bit SN for UM, 2) 18-bit SN for AM;

Proposal 14	[FFS] RAN2 further discuss how to define the RLC RTT for PC5 interface.

Proposal 15	[Easy] Maximum number of destinations is not considered in the definition of layer-2 buffer size.

Proposal 16	[Easy] RRC_CONNECTED UE reports the received SL capability (carrying RX UE capability received via UECapabilityInformationSidelink) via PC5-RRC to network using a container  within SidelinkUEInformationNR message.

Proposal 17	[Easy] RAN2 not pursue the timer to handle the failure case of UE capability transfer via sidelink.

Proposal 18	[Easy] RAN2 not purse signaling overhead optimization for capability transfer procedure for TX-UE forwarding peer-UE SL capability to network via Uu-RRC.

Proposal 19	[Easy] RAN2 not pursue signalling overhead optimization for capability transfer procedure via PC5-RRC.

R2-2004403	Summary of capability related Tdoc submitted to R2#109bis-E	OPPO	report	Rel-16	5G_V2X_NRSL-Core	Late

R2-2004578	Report of summary on NR V2X cell (re-)selection remaining issues	ZTE Corporation, Sanechips	discussion	Rel-16	5G_V2X_NRSL-Core	Late

Proposal 1: To follow LTE principle, for NR V2X, RRC_Connected UE can also perform cell reselection due to its sidelink service.

Recommendation 1: RAN2 is suggested to send LS towards CT1 to further check whether UE behaviour among the two options can be its implementation:

Alt 1: Perform a detach procedure and then perform PLMN selection triggered by V2X communication over PC5.

Alt  2:  Not initiate V2X communication over PC5.

Proposal 2: If a carrier doesn’t broadcast the V2X SIB but provide it by on-demand, then this frequency should be indicated by other frequency as anchor frequency in the V2X SIB. There will be no specification impact in TS 38.304, whether the definition of sl-NR-AnchorCarrierFreqList shall be modified in TS 38.331 should be further discussed.

Proposal 3: For a concerned frequency, If UE supports both NR and LTE SL, but UE’s camped cell can only provide one SL RAT configuration in the concerned frequency, then pre-configured SL resource on that frequency cannot be used on the other RAT if the UE is also in-coverage of that RAT.

Proposal 4: In the case UE supports both NR and LTE SL RAT, but UE’s camped cell can only provide one SL RAT configuration, then if the UE is also in coverage for the other RAT, then the UE may acquire the other SL RAT configuration by reading the broadcast V2X SIB on the concerned carrier. There will be no specification impact.

R2-2004579	(draft)Running CR on TS 38.304 for remaining NR V2X cell (re-)selection issues	ZTE Corporation, Sanechips	draftCR	Rel-16	38.304	16.0.0	5G_V2X_NRSL-Core	Late

R2-2004798	(draft)Running CR on TS 36.304 for remaining NR V2X cell (re-)selection issues	ZTE Corporation, Sanechips	draftCR	Rel-16	36.304	16.0.0	B	5G_V2X_NRSL-Core	Late

R2-2005721	Summary of NR V2X cell (re-)selection related contributions for RAN2 #110e	ZTE Corporation, Sanechips	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005075	Correction on NR sidelink description	Ericsson, Nokia	CR	Rel-16	38.300	16.1.0	0235	-	F	5G_V2X_NRSL-Core

R2-2005133	TP for 38.300 Conditional handover with sidelink	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2005465	Miscellaneous Stage-2 corrections for NR SL communication in TS 38.300	Huawei, HiSilicon	discussion

R2-2005480	Miscellaneous Stage-2 corrections for NR SL communication in TS 36.300 	Huawei, HiSilicon	discussion

R2-2005466	TP for final clean-up on RAN2 part in TR 37.985	Huawei, HiSilicon	pCR	Rel-16	37.985	1.3.0	5G_V2X_NRSL

R2-2004597	On the peer UE capability transfer in unicast sidelink	Nokia, Nokia Shanghai Bell	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004761	Discussion on SL Capability	Apple	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005044	TX resource pool configuration in mode1	Spreadtrum Communications	discussion

R2-2005076	Correction for reselection priority handling for V2X communications in 36.304	Ericsson, ZTE, Sanechips	CR	Rel-16	36.304	16.0.0	0797	-	F	5G_V2X_NRSL-Core

R2-2005077	Correction for reselection priority handling for V2X communications in 38.304	Ericsson, ZTE, Sanechips	CR	Rel-16	38.304	16.0.0	0170	-	F	5G_V2X_NRSL-Core

R2-2005127	Remaining issues of cell (re)selection for NR V2X	Lenovo, Motorola Mobility	discussion	Rel-16

R2-2005208	Remaining issue on groupcast RRC state transition and future p-t-M delivery	ITRI	discussion	5G_V2X_NRSL-Core

R2-2005296	Discussion on SL UE capability details	vivo	discussion

R2-2005547	Discussion for Sidelink UE Capability	Samsung Electronics Co., Ltd	discussion	Rel-16	38.306	5G_V2X_NRSL-Core

R2-2005587	Coverage status condition for NR sidelink communication transmission	Samsung Electronics Co., Ltd	discussion	Rel-16	5G_V2X_NRSL-Core

- 6.4.3	User plane

- 6.4.3.1	MAC

Including [Post109bis-e][957][V2X], [Post109bis-e][958][V2X], and remaining MAC issues. Tdoc limitation: 1 tdoc for discussion with an annex TP (if needed). This agenda item will utilize a summary document (LG).

R2-2005719	[Post109e#22] CR to 38.321 on Corrections to NR sidelink	LG Electronics Inc.	CR	Rel-16	38.321	16.0.0	0730	1	F	5G_V2X_NRSL-Core	R2-2003523

R2-2005720	Report of [Post109bis-e][957][V2X] MAC issues	LG Electronics Inc.	discussion

Recommendation C3: Assuming that a configuredGrantTimer for each HARQ process ID is agreed, UE (re-)starts or stops configuredGrantTimer for SL CG as specified for UL CG, and if configuredGrantTimer is not running, UE can perform new transmission in a CG resource for the corresponding HARQ Process ID.

Recommendation C4: cg-RetransmissionTimer is not applicable for SL transmissions regardless of whether configuredGrantTimer is used for SL CG.

Recommendation C5: configuredGrantTimer is introduced for SL CG operation in 38.321 as specified for UL CG.

Recommendation C6: If RAN2 agrees that a configuredGrantTimer is not used for SL CG, TX UE flushes a TB for the buffer of the Sidelink process associated to a HARQ Process ID before the next CG resource associated to the HARQ Process ID.

Recommendation Post-D2: If a TB has been transmitted with disabled SL HARQ feedback, the UE reports NACK to request further resources for blind retransmission and ACK otherwise.

Recommendation Pre-D3: RAN2 confirmed that a SL grant can be configured with PSFCH but without PUCCH for SL mode 1, as agreed in RAN1.

Recommendation D3: MAC can select either LCHs with FB disabled or LCHs with FB enabled for a SL grant configured with PSFCH but without PUCCH in SL LCP for both SL mode 1 and 2.

Recommendation Post-D5: MAC select only LCHs with FB disabled for a SL grant configured with neither PSFCH nor PUCCH in SL LCP for SL mode 2 as well as SL mode 1.

Recommendation Post-D7: If UE only has SL data on LCHs with FB enabled for a SL grant configured without PSFCH, the SL grant is skipped and so not used for transmission for SL mode 2 as well as SL mode 1.

Recommendation Post-E3: The following additional condition is needed for HARQ option1:

	The group size is greater than the number of candidate PSFCH resources associated with the selected PSSCH resource.

Recommendation E7: when TX UE enabled distance-based HARQ feedback by a SCI but RX UE’s location information is not available, RX UE sends no HARQ feedback.

Recommendation Pre-A3: a UE operating in Mode 1 triggers SR transmission if transmission of SL data over SCCH or STCH with the configured sidelink grant(s) cannot fulfil the latency bound associated to SL data.

Recommendation A3: Add one NOTE: a UE operating in Mode 1 may trigger SR transmission if transmission of a pending CSI report with the configured sidelink grant(s) cannot fulfil the latency bound associated to the CSI report.

Recommendation 1A: RAN2 will remove the related specification on prioritization between NR-UL and LTE SL from MAC specifications.

Recommendation 1B: RAN2 will specify the case that LTE SL transmission is prioritized while NR SL transmission is not prioritized, and apply the existing prioritization rules to the case.

Recommendation 1C: RAN2 will specify the case that LTE SL transmission is not prioritized while NR SL transmission is prioritized, and apply the existing prioritization rules to the case.

Recommendation 1D: Prioritization of UL transmission over SL transmission in 5.4.2.2 of TS 38.321 is changed as captured in Proposal 1C. Some editorial comments provided by companies can be further discussed during CR implementation.

Recommendation 1E: Additional UL/SL prioritization related to UL MAC CE is not considered in REL-16, except what has been already agreed or specified in 38.321.

Recommendation 2: add the following condition in clause 5.4.4 of TS 38.321 for SR.

3>	if the MAC entity is able to perform this SR transmission simultaneously with the transmission of the SL-SCH resource; or

Recommendation 3A: As in LTE DRX, if DRX is configured, the MAC entity may monitor the PDCCH for the MAC entity's SL-RNTI, SLCS-RNTI and SL Semi-Persistent Scheduling V-RNTI.

Recommendation 3B: RAN2 is requested to further discuss whether drx-RetransmissionTimerSL and drx-HARQ-RTT-TimerSL with the following definitions can be configured and drx-RetransmissionTimerSL is included in the Active Time.

Recommendation 3C: drx-RetransmissionTimerSL is included in the Active Time, if it is agreed.

Recommendation 3D: If drx-RetransmissionTimerSL and drx-HARQ-RTT-TimerSL are agreed, when to start/stop drx-RetransmissionTimerSL and drx-HARQ-RTT-TimerSL are specified and the above procedural texts can be considered as baseline for CR (noting that detailed wording for CR implementation can be further discussed)

Recommendation 3E: If drx-RetransmissionTimerSL and drx-HARQ-RTT-TimerSL are agreed, drx-RetransmissionTimerSL and drx-HARQ-RTT-TimerSL are added to ASN.1. (noting that detailed ASN.1 issues can be further discussed and led by RRC rapporteur)

Recommendation 4A: Upon MAC reset, the MAC entity sets the NDIs for all HARQ process IDs to the value 0 to receive the PDCCH for the MAC entity’s SL-RNTI and SLCS-RNTI in SL mode 1, as specified for UL HARQ.

Recommendation 4B: RAN2 does not specify the case that the MAC entity cancels triggered configured sidelink grant confirmation, upon MAC reset.

Recommendation 4C: SL specific partial MAC reset is introduced and specified in 38.321 and 38.331.

Recommendation 4D: Upon release of each PC5-RRC connection, UE RRC performs SL specific partial MAC reset.

Recommendation 4E: Upon MAC reset, the MAC entity flushes the soft buffers for all Sidelink processes for all TB(s) associated to the PC5-RRC connection.

Recommendation 4F: Upon MAC reset, the MAC entity cancels, if any, triggered Scheduling Request procedure only associated to the PC5-RRC connection (e.g. SR triggered by SL CSI Reporting).

Recommendation 4G: Upon MAC reset, the MAC entity cancels, if any, triggered Sidelink Buffer Status Reporting procedure only associated to the PC5-RRC connection.

Recommendation 5: The UE shall discard the received subPDU and any remaining subPDUs in the MAC PDU for unicast, when a MAC entity receives a MAC PDU on SL-SCH containing a Reserved LCID value.

Recommendation 6: if the SL BWP is deactivated, the MAC entity shall perform the following actions:

2>	not transmit PSBCH on the BWP, if configured;

2>	not transmit PSCCH on the BWP;

2>	not transmit SL-SCH on the BWP;

2>	not receive PSFCH on the BWP, if configured.

2>	not receive PSBCH on the BWP, if configured;

2>	not receive PSCCH on the BWP;

2>	not receive SL-SCH on the BWP;

2>	not transmit PSFCH on the BWP, if configured.

Recommendation 7A: When UE performs RACH on either SUL or NUL, if UE cannot simultaneously perform RACH and SL transmissions, UE prioritizes either UL transmission on SUL or SL transmission according to the existing prioritization rules. (i.e. RACH on SUL is prioritized over SL according to the existing rules.)

Recommendation 7B: When UE receives DCI switching to SUL or NUL, if UE cannot simultaneously perform UL and SL transmissions, UE performs switching as directed by the DCI but prioritizes either UL transmission on SUL or SL transmission according to the existing prioritization rules.

Recommendation 8: When RRC configures multiple resource pools, NR MAC performs TX resource pool (re)selection procedure.

Recommendation 8A: UE expects that PSFCH configuration is always present in at least one resource pool configuration in case that sl-HARQ-FeedbackEnabled of at least one SL LCH for the UE is set to enabled.

Recommendation 8B: RAN2 is requested to discuss whether the procedure is specified (based on LTE TX carrier (re)selection procedure) or left to UE implementation.

Recommendation 8C: If the procedure is specified, measured CBR result is used to select TX resource pool, as in LTE.

Recommendation 8D: If measured CBR result is used to select TX resource pool, the parameters inherited from LTE TX carrier (re)selection procedure, i.e. threshCBR-FreqReselection and threshCBR-FreqKeeping are introduced to 38.321 and 38.331, as in LTE, for NR TX resource pool (re)selection procedure.

Recommendation 8E: If the procedure is specified, HARQ feedback is used to select TX resource pool.

Recommendation 8F: If the procedure is specified, RAN2 is requested to discuss whether CR_limit is used to select TX resource pool, unlike in LTE V2X.

Recommendation 9: RAN2 expects that RAN1 will discuss whether ReTX resources of a MAC PDU are reserved neither right on nor after new TX resource of the next MAC PDU for a configured sidelink grant reserved for a particular Sidelink process.

Recommendation 10A: change the term ‘a configured sidelink grant’ for NR SL mode 2 in 38.321

Recommendation 10B: ‘a selected sidelink grant’ replaces ‘a configured sidelink grant’ for NR SL mode 2.

Recommendation 10C: For NR sidelink mode 1, only SL CG Type 1 and 2 create ‘a configured sidelink grant’ while dynamic SL grant creates ‘a dynamic grant’.

Recommendation 10D: Discuss whether definition of a sidelink grant needs to be clarified, if necessary, during CR implementation.

Recommendation 11A: The maximum number of transmitting Sidelink processes associated with each Sidelink HARQ Entity is ‘8’ in NR sidelink communication.

Recommendation 11B: For transmissions of multiple MAC PDUs in NR sidelink communication, the maximum number of transmitting Sidelink processes associated with each Sidelink HARQ Entity is ‘4’.

Recommendation 12: Any MAC CE is allowed to be mapped to CG Type 1 without any explicit configuration.

Recommendation 13A: mapping restriction between LCHs and SL configured grants in LCH configuration is introduced as specified in 38.331 and 38.321 for IIOT.

Recommendation 13B: Any MAC CE is allowed to be mapped to CG Type 2 without any explicit configuration.

Recommendation 14: add the following change in 38.321 for SL CSI reporting:

2>	if the MAC entity has SL resources allocated for new transmission and the SL resources can accommodate the SL CSI reporting MAC CE and its subheader as a result of logical channel prioritization:

3>	instruct the Multiplexing and Assembly procedure to generate a Sidelink CSI Reporting MAC CE as defined in clause 6.1.3.z;

Recommendation 15A: UE cannot initiate the Sidelink UE Information procedure to request the SR configuration used for Sidelink CSI reporting.

Recommendation 15B-A: UE cannot indicate the latency bound of the Sidelink CSI reporting to gNB, when it requests the corresponding SR configuration(s) in SidelinkUEInformation.

Recommendation 15B-B: RAN2 is requested to discuss whether UE can be configured with multiple SR configurations for CSI report and UE selects the SR configuration associated to the latency bound of the triggered CSI report.

Recommendation 16: gNB configure at least one SR configuration ID associated with the Sidelink CSI reporting. (i.e. the “zero” case is not supported).

Recommendation 17A: As specified in the current version of 38.321, when a SCI of interest is received, the receiving sidelink process performs SL-SCH reception procedure in 5.22.2.2.2. Thus, UE does not perform double checking Layer 1 ID to send NACK on PSFCH for unicast.

Recommendation 17B: As specified in the current version of 38.321, when a SCI of interest is received, the receiving sidelink process performs SL-SCH reception procedure in 5.22.2.2.2. Thus, UE does not perform double checking Layer 1 ID to send NACK on PSFCH for groupcast.

Recommendation 17C: As specified in the current version of 38.321, when a SCI of interest is received, the receiving sidelink process performs SL-SCH reception procedure in 5.22.2.2.2. Thus, UE does not perform double checking Layer 1 ID to send ACK on PSFCH for unicast.

Recommendation 17D: As specified in the current version of 38.321, when a SCI of interest is received, the receiving sidelink process performs SL-SCH reception procedure in 5.22.2.2.2. Thus, UE does not perform double checking Layer 1 ID to send ACK on PSFCH for groupcast.

Recommendation 18: Prioritization of SL-BCH over uplink transmission is not further discussed in RAN2.

Recommendation 19: Reallocate SL specific MAC CE(s) (at least Sidelink Configured Grant Confirmation) from LCID space to one-octet eLCID space. Further discuss whether to relocate Truncated Sidelink BSR and Sidelink BSR as well.

Recommendation 20: Use ‘sidelink resource allocation mode 1 and 2’ in 38.321 and 38.331 to be aligned with RAN1 specifications (noting that detailed wording for CR implementation can be further discussed)

R2-2005725	Summary of V2X MAC issues	LG Electronics Inc.	discussion 	Rel-16	5G_V2X_NRSL-Core

Rapporteur proposes to prioritize issue 3 on mixing blind and HARQ feedback issue during this e-meeting considering interaction with RAN1. Others are proposed to be postponed to next meetings, except some issues that can be discussed during CR implementation as proposed by rapporteur in the corresponding sections below.

R2-2004406	Left issues on MAC running CR	OPPO	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004520	Discussion on SL CSI report trigger	SHARP	discussion	5G_V2X_NRSL-Core

R2-2004580	Discussion on remaining issue related to NR V2X MAC	ZTE Corporation, Sanechips	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004751	Remaining issues for MAC	MediaTek Inc.	discussion	Rel-16

R2-2004759	Discussion on remaining issues on NR V2X MAC	Apple	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004889	Discussion on NR-V2X MAC left issues	Fujitsu	discussion	Rel-16	5G_V2X_NRSL-Core	R2-2002955

R2-2004981	Discussion on mixed blind and HARQ-based retransmissions	CATT	discussion	Late

R2-2004998	Remaining issues in MAC for NR sidelink	Nokia, Nokia Shanghai Bell	discussion	5G_V2X_NRSL-Core

R2-2005039	Remaining MAC Issues	Lenovo, Motorola Mobility, InterDigital Inc.	discussion	5G_V2X_NRSL-Core

R2-2005042	Remaining issues on MAC for NR V2X	Spreadtrum Communications	discussion

R2-2005043	Discussion on mixed blind and feedback-based HARQ retransmissions for NR sidelink	Spreadtrum Communications	discussion

R2-2005074	Discussion on MAC left issues	Ericsson	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005207	Groupcast HARQ feedback without location information 	Kyocera	discussion

R2-2005228	On mixing of blind and feedback based HARQ retransmissions	Intel Corporation	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005297	Remaining MAC issues	vivo	discussion

R2-2005325	Remaining Issues on HARQ for NR V2X	InterDigital, Apple, Lenovo, Motorola Mobility	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005492	Discussion on remaining MAC Open issues for 5G V2X with NR SL	Huawei, Hisilicon	discussion	5G_V2X_NRSL-Core

R2-2005515	Groupcast HARQ feedback from RX UE without location information	Futurewei	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005541	Remaining V2X MAC Issues	LG Electronics France	discussion	Rel-16	38.321

R2-2005564	Left NR V2X issues regarding SL MAC CE	ASUSTeK	discussion	Rel-16	38.331	5G_V2X_NRSL-Core

R2-2005575	Remaining MAC issues	Qualcomm Finland RFFE Oy	discussion	Rel-16	38.321

R2-2005705	Discussion on BSR prioritization issue	Beijing Xiaomi Software Tech	discussion

Withdrawn:

R2-2004941	Open issue on mixing blind and feedback-based HARQ retransmissions of a TB in the sidelink HARQ operations	CATT	discussion	Late

R2-2004968	Open issue on mixing blind and feedback-based HARQ retransmissions of a TB in the sidelink HARQ operations	CATT	discussion	Late

R2-2004980	Discussion on mixed blind and HARQ-based retransmissions	CATT	discussion	Late

- 6.4.3.2	Others

Including [Post109bis-e][959][V2X] and remaining other user plane issues (RLC, PDCP, and SDAP). Tdoc limitation: 1 tdoc for discussion with an annext TP (if needed) per specification. This agenda item will utilize summary documents (RLC: Ericsson, PDCP: CATT, SDAP: Vivo).

R2-2004888	38.323 CR for NR V2X	CATT	CR	Rel-16	38.323	16.0.0	0048	-	C	5G_V2X_NRSL-Core	Late

R2-2005724	Summary of PDCP remaining issues on NR V2X	CATT	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2005677	Summary of NR V2X SDAP related contribution	vivo	discussion	Late

R2-2004581	Discussion on the establishment-release of the Rx SDAP entity	ZTE Corporation, Sanechips	discussion	Rel-16	5G_V2X_NRSL-Core

R2-2004747	Remaining Issues on PDCP	CATT	discussion	Rel-16	Late

R2-2004881	 Draft LS on trigger of PDCP reestablishment	OPPO	LS out	Rel-16	5G_V2X_NRSL-Core	To:CT1

R2-2005045	Discussion on counter check procedure for NR sidelink	Spreadtrum Communications	discussion

R2-2005055	Remaining issues in PDCP for NR sidelink	Nokia, Nokia Shanghai Bell	discussion	5G_V2X_NRSL-Core

R2-2005298	Open issues on NR V2X SDAP	vivo	discussion

R2-2005343	Remaining issues for NR SL PDCP header format	Qualcomm Finland RFFE Oy	discussion	Rel-16	38.323

R2-2005464	Discussion on PDCP SN size for SL groupcast and broadcast in NR V2X	Huawei, MediaTek Inc.,HiSilicon	discussion

R2-2005548	Clarification of SL PDCP Operation	Samsung Electronics Co., Ltd	discussion	Rel-16	38.323	5G_V2X_NRSL-Core