3GPP TSG-RAN WG2 Meeting #110	R2-200xxxx

Electronic, 1 June– 12 June2020

Agenda Item:	6.20.1.3

Source: 	OPPO

Title:  	Email report of [AT110e][036][TEI16] TEI16 corrections (OPPO)

Document for:	Discussion and decision

# Introduction

This paper aims to trigger the email discussion for TEI 16 corrections.

- [AT110e][036][TEI16] TEI16 corrections (OPPO)

Scope: Treat R2-2004526, R2-2004527, R2-2005614, R2-2004388, R2-2004438, R2-2005429, R2-2004393 (proponents are responsible to explain and drive)

Part 1: Identify agreeable changes. Deadline: June 4, 0700 UTC.

Part 2: For agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

# Open issues for TEI16 corrections

Topic 1: MPS and MCS

R2-2004526	Corrections to PRACH prioritization procedure for MPS and MCS	Samsung Electronics Co., Ltd	CR	Rel-16	38.321	16.0.0	0705	1	F	TEI16	R2-2002560

R2-2004527	Corrections to PRACH prioritization procedure for MPS and MCS	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.0.0	1506	1	F	TEI16	R2-2002561

R2-2005614	CR to 38321 on RACH Prioritization for MPS and MCS	vivo	CR	Rel-16	38.321	16.0.0	0756	-	F	NR_newRAT-Core, TEI16

| Company | Comments |
| --- | --- |
| OPPO | We agree with the changes in CR: R2-2004526 and R2-2004527. For CR: R2-2005614, I wonder the intention is to expect the Access Identity(ies) is selected and provided to lower layers by the NAS? If so, I am not sure if there is a service point between NAS and MAC. I think the it makes sense that NAS provide the Access Identity(ies) to RRC, then RRC provide the Access Identity(ies) to MAC. So we cannot see the necessary of the change. |
| ZTE | We are fine with CR R2-2004526 and R2-2004527. For the R2-2005614, we share the similar view with OPPO. We also think the related information will be provided from NAS to RRC, and then from RRC to MAC. |
| Samsung | Agree with changes in R2-2004526 and R2-2004527 For CR R2-2005614 - Agree that current text needs to be updated. - Following phrase is used in RRC to refer to access identity 1/2: “if the UE is configured by upper layers with Access Identity”. So our suggestion is to replace “if one or more Access Identities has been explicitly provided by RRC” by “if the UE is configured by upper layers with Access Identity 1 or 2” |
| Ericsson | R2-2004526 and R2-2004527: Agree R2-2005614: Agree with the intention but prefer Samsung's updated proposal. |

Topic 2: SMTC2

R2-2004388	additional SSB-ToMeasure for smtc2-LP	OPPO, ZTE,CMCC	discussion	Rel-16	TEI16

| Company | Comments |
| --- | --- |
| OPPO | In R15, there is one SMTC configuration and one SSB-ToMeasure IE configured in SIB2/SIB4. It would be clear the SMTC with which SSB-ToMeasure IE is associated. In R16, however, it is not clear because there are two SMTC configurations configured in SIB2/SIB4 and only one SSB-ToMeasure IE configured. The new SMTC smtc2-LP will associate with a PCI list and the UE will perform SSB measurement for these cells indicated in PCI list based on SMTC smtc2-LP. So the set of SSBs to measure will be different. From UE power perspective, it is benefit to configure the additional SSB-ToMeasure for smtc2-LP. We agree with the proposals and changes. |
| ZTE | As one of the proponent companies, we support this. By introducing additional ssb-ToMeasure bitmap for smtc2-LP, the network can have more flexibility to deploy different SSB beams for sleeping cells and non-sleeping cells. |
| Samsung | Not essential. The main motivation for SMTC2 was to support different SSB periodicity in different cells on same frequency. SSBs transmitted in each cell on same frequency are same. |
| Ericsson | We agree with ZTE that this enhancement is motivated when there is a need to configure sleeper cells and “normal” cells with different beam configurations. Until now our understanding has been that only the SSB periodicity is increased when the cell goes to sleep, and the SSB configuration remains unchanged. Currently all cells on the same frequency share the same beam configuration. |

Topic 3: Voice fallback

R2-2004438	Correction on establishment cause value upon enhanced EPS voice fallback	Qualcomm Incorporated	CR	Rel-16	36.331	16.0.0	4236	1	F	TEI16	R2-2002581

| Company | Comments |
| --- | --- |
| OPPO | We agree with the changes in CR. |
| ZTE | We support the CR. |
| Samsung | Agree |
| Ericsson | We support the CR |

Topic 4: HO to EN-DC

R2-2005429	CR to 36.300 on support of inter-RAT HO from SA to EN-DC	Huawei, HiSilicon	CR	Rel-16	36.300	16.1.0	1286	-	F	TEI16

| Company | Comments |
| --- | --- |
| OPPO | We agree with the changes in CR. |
| ZTE | We support the CR. |
| Samsung | Agree |
| Ericsson | Agree |

Topic 5: NeedForGap

R2-2004393	Discussion on update of NeedForGap	OPPO	discussion	Rel-16	TEI16

We note that the need code for needForGapsConfigNR IE is “Need M” and this configuration will be maintained after next RRCReconfiguration and HO-CMD. The OtherConfig IE is included in RRCReconfgiuration message and RRCReconfgiuration is forwarded to the target node in HandoverPreparationInformation inter-node message.

It is not clear whether NeedForGap reporting, i.e. needForGapsInfoNR is also forwarded to the target node during HO.

For UE assistance information reporting, both UE assistance information configuration and UE assistance information reporting are forwarded to the target node during HO.

For the same reason, it makes sense that NeedForGap reporting, i.e. needForGapsInfoNR is also forwarded to the target node during HO.

Proposal 1: NeedForGap reporting, i.e. needForGapsInfoNR is forwarded to the target node during HO in HandoverPreparationInformation iner-node message.

The SCells release or addition via RRCReconfiguration will change the configured band combination, so the NeedForGap reporting will also be changed. The common understanding is that the needForGapsConfigNR will not always be included in RRCReconfiguraiton due to the need code for needForGapsConfigNR IE is “Need M”. So if needForGapsConfigNR is not included in RRCReconfiguraiton message, the UE will use the needForGapsConfigNR configured in prior RRCReconfiguration message or RRCResume message if configured.

If there is no change for the NeedForGap reporting, i.e. NeedForGapsInfoNR, it is not necessary for the UE to report the NeedForGapsInfoNR again in RRCReconfigurationComplete message and the network will consider the last NeedForGap reporting is valid.

We kindly asked RAN2 to confirm the following common understanding:

Proposal 2: RAN2 is kindly asked to confirm the following understandings:

- If needForGapsConfigNR is not included in RRCReconfiguraiton message, the UE will use the needForGapsConfigNR configured in prior RRCReconfiguration message or RRCResume message if configured.

- If NeedForGapsInfoNR is not included in RRCReconfigurationComplete message, the network will consider the last NeedForGap reporting is valid if received.

| Company | Do you agree with the above proposal 1 and 2? |
| --- | --- |
| OPPO | We agree with proposal 1 and 2. |
| ZTE | For proposal 1, we think it is only useful when the target gNB configures almost identical configuration as source side. E.g. SCells and physical layer configurations. Otherwise, the gap requirement might change after applying the handover command. We are not sure if handover without signification configuration update is normal case. But from network perspective, we see no harm to forward this information to target cell, so no strong opinion, could be acceptable. For proposal 2, we share the same understandings, and we think these are already captured in field description and text procedures. |
| Samsung | Agree |
| Ericsson | Proposal 1 looks fine but our understanding is that this is already supported. In the INM, section 11.2.3, we have the following sentence for the AS-Config. The source node shall include all fields necessary to reflect the AS configuration of the UE, except for the fields sourceSCG-NR-Config, sourceSCG-EUTRA-Config and sourceRB-SN-Config, which can be omitted in case the source MN did not receive the latest configuration from the source SN. For RRCReconfiguration included in the field rrcReconfiguration, ReconfigurationWithSync is included with only the mandatory subfields (e.g. newUE-Identity and t304) and ServingCellConfigCommon; Therefore, there is no need to signal this field explicitly. For proposal 2, in principle we agree with them but we disagree with any impact on the current specification. In fact, this need for gap IE is a setupRelease and therefore the network has the control on it. Therefore, okay with the proposal but not ok to have any impact on the specification. |

# Conclusions

# Reference