3GPP TSG-RAN2 Meeting #110 E-meeting	    R2-2005791

1- 12 Jun 2020

Agenda item:	6.15.1

Source	LG Electronics Inc.

Title	Report on RRC and UE capabilities aspects

Document for	Discussion and Decision

# Introduction

 [AT110e][101][CLI] RRC and UE capabilities aspects (LG)

Scope: Discuss the 38.331 and 38.306 CR on UE capabilities for CLI based on R2-2005695, R2-2005730 and R2-2004430. Also discuss the changes suggested by RAN1 in R2-2004347 and the new proposal in R2-2005309.

Initial intended outcome: summary of the offline discussion with e.g.:

- Set of proposals with full consensus, if any (agreeable over email)

- Set of proposals to discuss in the follow up conference call

Initial deadline (for companies' feedback):  Wednesday 2020-06-03 10:00 UTC

Initial deadline (for rapporteur's summary in R2-2005791):  Wednesday 2020-06-03 22:00 UTC

# Discussion

## TS38.331 CR

Two 38.331 CRs have been submitted to implement the UE features list for CLI in [1]:

| 17. NR_CLI_RIM | 17-1 | CLI-RSSI measurement | 1. Support CLI-RSSI measurement. The max number of resources across all CCs configured to measure RSSI simultaneously shall not exceed 64. 2. Maximum number of measurement resources configured for CLI-RSSI measurement | Per UE | TDD only | Candidate values for component 2 are {8, 16, 32, 64}. FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 17. NR_CLI_RIM | 17-2 | SRS-RSRP measurement | 1. Support SRS-RSRP measurement. The max number of SRS resources across all CCs configured to measure SRS-RSRP simultaneously shall not exceed 32. 2. Maximum number of measurement resources configured for SRS-RSRP measurement 3. Maximum number of measurement resources configured for SRS-RSRP measurement within a slot | Per UE | TDD only | Candidate values for component 2 are {4, 8, 16, 32}. Candidate values for component 3 are {2, 4, 8}. FFS: whether or how to handle licensed/unlicensed differentiation | Optional with capability signalling |

Option 1: In the first CR [2], above capabilities capture as follows:

The cli-RSSI-Meas-r16 and cli-SRS-RSRP-Meas-r16 are already defined in TS38.306 v16.0.0. To minimize the impact on current spec, new capabilities defined by RAN1 have been added separately (maxNumberCLI-RSSI-r16, maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16).

Please note that this option requires some restrictions in TS38.306, e.g. If UE sets maxNumberCLI-RSSI-r16, the UE shall set cli-RSSI-Meas-r16, to prevent UE from reporting maxNumberCLI-RSSI-r16 without cli-RSSI-Meas-r16.

Option 2: In [3], such restrictions are not needed because it is structurally impossible to report maxNumberCLI-RSSI-r16 without cli-RSSI-Meas-r16.

Companies are requested to provide input:

| Company | Preferred option | Comments |
| --- | --- | --- |
| Ericsson | 2 | Both options work, but from a specification quality point of view, option 2 has these benefits: - The relation between the parameters are clear already from ASN.1. I.e. one wouldn’t have to read 306 in parallel to 331 to understand the relation of these parameters. - 306 can be made simpler/less wordy by not require us to add the relation between the parameters in 306. As shown for example by the following question. - Since it will be impossible to implement it wrong with option 2 (since ASN.1 ensures correct behaviour of the UE) there is zero risk of misunderstanding. It is mentioned above "To minimize the impact on current spec". We don’t know if this is intended to say that option 1 brings less change to 306 compared to option 2? Either way, we don’t see why this would be the case. And even if that would be the case (which we haven't understood yet), we think that it is now (before ASN.1 freeze) that RAN2 can freely decide which option to take and to us option 1 has some drawbacks compared to option 2. |

## TS38.306 CR

The 38.306 CR [4] coincides with option 1 and include some restrictions as bellow:

cli-RSSI-Meas-r16 Indicates whether the UE can perform CLI RSSI measurements as specified in TS 38.215 [13] and supports periodical reporting and measurement event triggering as specified in TS 38.331 [9]. If the UE supports this feature, the UE needs to report maxNumberCLI-RSSI-r16.

cli-SRS-RSRP-Meas-r16 Indicates whether the UE can perform SRS RSRP measurements as specified in TS 38.215 [13] and supports periodical reporting and measurement event triggering based on SRS-RSRP as specified in TS 38.331 [9]. If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16.

If companies prefer to take the option 1, additional restriction needs to be added in TS38.306 as mentioned above. If option 2 is chosen, the restrictions in the current CR can be deleted.

Do you have any comments on TS38.306 CR?

| Company | Comments |
| --- | --- |
| Ericsson | If we would end up with option 1: "needs to" is not wording we usually use for UE requirements. It should have been "shall", we assume? |

## Subcarrier spacing for CLI-RSSI measurement

RAN1 sent an LS [5] and asked RAN2 to update the description of the parameter rssi-SCS-r16 in the IE MeasObjectCLI in TS38.331 as below:

rssi-scs-r16 Reference subcarrier spacing for CLI-RSSI measurement. Only the values 15, 30 kHz or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. UE performs CLI-RSSI measurement with the SCS of the active bandwidth part within the configured CLI-RSSI resource in the active BWP regardless of the reference SCS of the measurement resource.

In [3], Ericsson suggests the description slightly changed as below:

rssi-scs-r16 Reference subcarrier spacing for CLI-RSSI measurement. Only the values 15, 30 kHz or 60 kHz (FR1), and 60 or 120 kHz (FR2) are applicable. The UE performs CLI-RSSI measurements with the subcarrier spacing of the active bandwidth part within the configured CLI-RSSI resource in the active BWP, regardless of the reference subcarrier spacing of the measurement resource.

Question: Is it OK if the field description is updated as suggested by Ericsson?

| Company | Comments |
| --- | --- |
| LG | OK with the Ericsson`s suggestion. |
| Ericsson | Agree with LG |

## Impact of DRX on CLI SRS-RSRP measurement

In [6], Huawei analysed the impact of DRX on CLI SRS-RSRP measurement and proposed followings:

Proposal 1: Specify that the SRS used as CLI measurement resources are transmitted regardless of whether the MAC entity is in DRX active time or not when the DRX function is set up. Proposal 2: Introduce a method for the UE to distinguish SRSs for CLI measurement from other SRSs. Potential options are: - Option 1: define a new field in the SRS resource configuration IE (i.e., SRS-Resource) or the SRS resource set configuration IE (i.e., SRS-ResourceSet) indicating whether this SRS resource or SRS resource set is used for CLI measurement. - Option 2: define a new SRS resource set IE (e.g., SRS-CLIResourceSet-r16) configuring the SRS resources used as CLI measurement resources. - Option 3: the network explicitly notifies the UE which SRS resource(s) or SRS resource set(s) is used for CLI measurement.

Companies are requested to provide input on proposals in [6]:

| Company | Support? | Comments |
| --- | --- | --- |
| LG | No | Just an optimization and not essential to work. |
| Ericsson | No | Agree with LG |

# Conclusion

# Reference

[1] R2-2004358	LS on Rel-16 RAN1 UE features lists for NR (R1-2003072; contact: NTT DOCOMO, AT&T)	RAN1	LS in	Rel-16	NR_2step_RACH-Core, NR_unlic-Core, NR_IAB-Core, 5G_V2X_NRSL-Core, NR_L1enh_URLLC-Core, NR_IIOT-Core, NR_eMIMO-Core, NR_UE_pow_sav-Core, NR_pos-Core, NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core, TEI16, NR_CLI_RIM-Core	To:RAN2, RAN4

[2] R2-2005695 TS 38.331 CR	LG

[3] R2-2005730 TS 38.331 CR 	Ericsson

[4] R2-2004430 TS 38.306 CR 	Qualcomm

[5] R2-2004347

[6] R2-2005309	On the impact of DRX on CLI SRS-RSRP measurement	Huawei