3GPP TSG-RAN WG2 Meeting #110 electronic	draftR2-2005741

Online, June 1 – June 12 2020

Agenda item:	4.5.1

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary of Offline-201 - LTE Rel-15 TDD/FDD capability differentiation

Document for:	Discussion and Decision

# Introduction

The CRs in [1], [2] have raised the issue that Release-15 physical layer capabilities do not have the possibility to signal different values for TDD and FDD, even though the RAN1 feature list in [3] indicates that they should.

In this document we look at this issue and other related problems with TDD/FDD differentiation.

# Discussion

### 2.1 UE Capability field descriptions with FFS.

Currently there are a number of UE capabilities with FFS in the field descriptions, some as early as Rel-12.

| UE-EUTRA-Capability field descriptions | FDD/ TDD diff | Release | Suggestion |
| --- | --- | --- | --- |
| beamformed (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode, the UE capabilities concerning beamformed EBF/ FD-MIMO operation (class B) applicable for band combinations for which the concerned capabilities are not signalled. | TBD | R13 | Yes |
| channelMeasRestriction Indicates for a particular transmission mode whether the UE supports channel measurement restriction. | TBD | R13 | Yes |
| configN (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports non-precoded EBF/ FD-MIMO (class A) related configuration N for band combinations for which the concerned capabilities are not signalled. | TBD | R13 | Yes |
| crs-DiscoverySignalsMeas Indicates whether the UE supports CRS based discovery signals measurement, and PDSCH/EPDCCH RE mapping with zero power CSI-RS configured for discovery signals. | FFS | R12 | Yes |
| csi-ReportingAdvanced, csi-ReportingAdvancedMaxPorts (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of CSI-RS ports supported by the UE for advanced CSI reporting. The field csi-ReportingAdvanced indicates 32 CSI-RS ports whereas csi-ReportingAdvancedMaxPorts indicates 8, 12, 16, 20, 24 or 28 CSI-RS ports. The UE shall not include both csi-ReportingAdvanced and csi-ReportingAdvancedMaxPorts for a particular transmission mode. | FFS | R14 | Yes |
| csi-ReportingNP (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports CSI reporting on non-precoded CSI-RS with 20, 24, 28, or 32 antenna ports for band combinations for which the concerned capabilities are not signalled in MIMO-CA-ParametersPerBoBCPerTM, and the FD-MIMO processing capability condition as described in NOTE 8 is satisfied. | FFS | R14 | Yes |
| csi-RS-DiscoverySignalsMeas Indicates whether the UE supports CSI-RS based discovery signals measurement. If this field is included, the UE shall also include crs-DiscoverySignalsMeas. | FFS | R12 | Yes |
| densityReductionNP, densityReductionBF Indicates whether the UE supports CSI-RS density reduction with values 1, 1/2 and 1/3 for non-precoded CSI-RS and beamformed CSI-RS respectively. | FFS | R14 | Yes |
| discoverySignalsInDeactSCell Indicates whether the UE supports the behaviour on DL signals and physical channels when SCell is deactivated and discovery signals measurement is configured as specified in TS 36.211 [21], clause 6.11A. This field is included only if UE supports carrier aggregation and includes crs-DiscoverySignalsMeas. | FFS | R12 | Yes |
| dmrs-Enhancements (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode whether the UE supports DMRS enhancements for the indicated transmission mode. | TBD | R13 | “NO” |
| hybridCSI Indicates whether the UE supports hybrid CSI transmission as described in TS 36.213 [23]. | FFS | R14 | Yes |
| interferenceMeasRestriction Indicates whether the UE supports interference measurement restriction. | TBD | R13 | Yes |
| k-Max (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of NZP CSI RS resource configurations supported within a CSI process applicable for band combinations for which the concerned capabilities are not signalled. | TBD | R13 | Yes |
| n-MaxList (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the maximum number of NZP CSI RS ports supported within a CSI process applicable for band combinations for which the concerned capabilities are not signalled. For k-Max values exceeding 1, the UE shall include the field and signal k-Max minus 1 bits. The first bit indicates n-Max2, with value 0 indicating 8 and value 1 indicating 16. The second bit indicates n-Max3, with value 0 indicating 8 and value 1 indicating 16. The third bit indicates n-Max4, with value 0 indicating 8 and value 1 indicating 32. The fourth bit indicates n-Max5, with value 0 indicating 16 and value 1 indicating 32. The fifth bit indicates n-Max6, with value 0 indicating 16 and value 1 indicating 32. The sixt bit indicates n-Max7, with value 0 indicating 16 and value 1 indicating 32. The seventh bit indicates n-Max8, with value 0 indicating 16 and value 1 indicating 64. | TBD | R13 | Yes |
| nonPrecoded (in MIMO-UE-ParametersPerTM) Indicates for a particular transmission mode the UE capabilities concerning non-precoded EBF/ FD-MIMO operation (class A) for band combinations for which the concerned capabilities are not signalled in MIMO-CA-ParametersPerBoBCPerTM, and the FD-MIMO processing capability condition as described in NOTE 8 is satisfied. | TBD | R13 | Yes |
| semiOL Indicates whether the UE supports semi-open-loop transmission for the indicated transmission mode. | FFS | R14 | Yes |
| srs-Enhancements Indicates whether the UE supports SRS enhancements. | TBD | R13 | Yes |
| ul-dmrs-Enhancements Indicates whether the UE supports UL DMRS enhancements as defined in TS 36.211 [21], clause 6.10.3A. | FFS | R14 | Yes |
| zp-CSI-RS-AperiodicInfo Indicates whether the UE supports aperiodic ZP-CSI-RS transmission for the indicated transmission mode. | FFS | R14 | Yes |

In most cases these capabilities do have the possibility to signal different values for TDD and FDD so by default these FFS should be changed to “Yes”. The only one where we identified a preference as “no” is dmrs-Enhancements (in MIMO-UE-ParametersPerTM).

Q1: Do you agree the above capabilities should be changed as suggested in the appropriate releases (i.e. Rel-12, 13, 14 as appropriate)?

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments (including whether there is anything missing from the list) |
| --- | --- | --- |
|  | yes/no |  |
| Qualcomm |  | Thank you, Huawei for the above table listing all the inconsistencies in a comprehensible way! We think all of the above should be changed to “Yes”, including for dmrs-Enhancements-r13 for parameters that is allowed by signalling. (UE would not include XDD diff in case it is same anyway.) Changing from “TBD” to Yes does not introduce interoperability issues for the parameters already allowed by signalling, but TBD to NO is non-backward compatible for UEs which might have assumed they would signal differently (because the signalling allows it) which result in interoperability issues. |
| OPPO |  | For dmrs-Enhancements (in MIMO-UE-ParametersPerTM), we wonder if the following path implies the possibility to indicate different value for FDD/TDD? UE-EUTRA-CapabilityAddXDD-Mode-v1320PhyLayerParameters-v1320MIMO-UE-Parameters-r13MIMO-UE-ParametersPerTM-r13dmrs-Enhancements-r13. Sorry if any misunderstanding. For csi-ReportingAdvancedMaxPorts, since it relates to MIMO-UE-Parameters-v1470 discussed in Q2 below, so should be discussed in Q2 separately. |
| Lenovo | no | The status of the capabilities should be changed only case by case and not generally. And it should be well justified why is should be set to “Yes”, “No” or “-“. Furthermore, it is not clear to us why status of dmrs-Enhancements (in MIMO-UE-ParametersPerTM) should be set to “No”. |

### 2.2 Rel-14 ASN.1

PhyLayerParameters-v1430 has the TDD/FDD differentiation inside of UE-EUTRA-CapabilityAddXDD-Mode-v1430.

However, the following capabilities were added later:

PhyLayerParameters-v1450 ::=         SEQUENCE {

ce-SRS-EnhancementWithoutComb4-r14      ENUMERATED {supported}           OPTIONAL,

crs-LessDwPTS-r14                    ENUMERATED {supported}           OPTIONAL}

PhyLayerParameters-v1470 ::=         SEQUENCE {

mimo-UE-Parameters-v1470             MIMO-UE-Parameters-v1470     OPTIONAL,

srs-UpPTS-6sym-r14                   ENUMERATED {supported}           OPTIONAL

}

PhyLayerParameters-v14a0 ::=         SEQUENCE {

ssp10-TDD-Only-r14                   ENUMERATED {supported}           OPTIONAL

}

Most of these are TDD only capabilities, so no need to differentiate.

The MIMO capabilities introduced in v14.7.0 are applicable to FDD and TDD. There was no UE-EUTRA-CapabilityAddXDD-Mode-v1470 introduced at the time to allow different values to be signalled for TDD and FDD.

Q2: Do you agree the MIMO-UE-Parameters-v1470 needs to be introduced in UE-EUTRA-CapabilityAddXDD-Mode-v14e0 in a lateNonCriticalExtension in Rel-14?

Company views (to be completed during the meeting)

| Company | Do you agree (yes/no) | Comments (including whether there is anything else missing in Rel-15 ASN.1) |
| --- | --- | --- |
|  | yes/no |  |
| Qualcomm | No | The parameter added by MIMO-UE-Parameters-v1470 is csi-ReportingAdvancedMaxPorts. Since that can be signalled per UE (per TM) or per BoBC (per TM), the per BoBC would override per UE. So, no need of FDD/TDD diff. However, some editing on field description would be needed based on Q1 above. - Need to split field description for csi-ReportingAdvanced, csi-ReportingAdvancedMaxPorts (in MIMO-UE-ParametersPerTM) to two rows. - For the first parameter, change to Yes, and for the second one, change to ‘-‘. |
| Lenovo | no | We have the same comment as for Q1, i.e. it should be well justified why we need a FDD/TDD differentiation for csi-ReportingAdvancedMaxPorts-r14 in MIMO-UE-ParametersPerTM. |

Q3: Have you identified any other ASN.1 issue in Rel-14 or earlier related to FDD/TDD UE capability differentiation?

Company views (to be completed during the meeting)

| Company | (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |
| OPPO | Yes | The FDD/TDD indication is missing for zoneBasedPoolSelection-r14, for which we believe a “-” is OK since it is only for sidelink. |

### 2.3 Rel-15 ASN.1 and UE Capability field descriptions

According to the RAN1 feature list for Rel-15 in [3] there is a need to differentiate most of the physical layer capabilities for TDD and FDD, however this was never implemented in the ASN.1 signalling.

In order to avoid using late non-critical extension this needs to be done before freezing Rel-16.

Q4: Do you agree the PhyLayerParameters-v1530, v1540, v1550 need to be introduced in UE-EUTRA-CapabilityAddXDD-Mode-v15a0?

Company views (to be completed during the meeting)

| Company | (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |
| Qualcomm | Yes | Agree this should be done before ASN.1 freeze, even if each parametes in Q5 and later may need some discussion to finalize later. |
| OPPO | Yes | We are OK for this by assuming it is BC change. |
| Lenovo |  | This needs to be checked as the RAN1 LS is quite long and it is difficult to find the concerned capability in the feature list. For instance: - For dmrs-OverheadReduction-r15 (PhyLayerParameters-v1550) we agree as it is listed in feature list as entry 7-2. - For crs-IM-TM1-toTM9-OneRX-Port-v1540 and cch-IM-RefRecTypeA-OneRX-Port-v1540 (PhyLayerParameters-v1540) we are not sure as we can’t find them in the feature list. Are they related to CRS muting for BL UE (entry 9-4)? If yes, there is no FDD/TDD differentiation defined for them. - For many of the eMTC capabilities (related to RAN2/RAN4) the need for FDD/TDD differentiation is missing (see entries 9-13 to 9-22). |

If the PhyLayerParameters-v1530, v1540, v1550 need to be introduced in UE-EUTRA-CapabilityAddXDD-Mode-v15a0 then we also need to update the field descriptions for the related capabilities from “-“ to “Yes” or “No”.

According to the RAN1 feature list [3], most capabilities have a need for TDD/FDD differentiation, except for LAA and eV2X, and some HRLLC capabilities. We do need to check in detail for every capability, but in general the following table summarises per WI/feature.

| Feature | FDD/TDD diff |
| --- | --- |
| LAA | No |
| feComp | Yes |
| Aerials | Yes |
| eV2X | No |
| sTTI | Yes |
| HRLLC | Most should be “Yes” but for some it is “No”. Needs to be checked per individual capability. |
| 1024QAM | Yes |
| NB-IoT | This is already correct, and in a separate ASN.1 and TDD/FDD table. |
| eMTC | Yes |
| CA enhancement | Yes |
| CP latency reduction | Yes |
| SRS Antenna Switching | Yes |
| Advanced CSI | Yes |
| Alternative MCS Table | Yes |

Q5: Do you agree the field descriptions of parameters in PhyLayerParameters-v1530, v1540, v1550 need to be updated according to the RAN1 feature list in [3]?

Company views (to be completed during the meeting)

| Company | (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |
| Qualcomm | Yes |  |
| OPPO | Yes |  |
| Lenovo | Yes | For the capabilities which need FDD/TDD differentiation. |

Q6: Have you identified any other ASN.1 issue in Rel-15 related to FDD/TDD UE capability differentiation?

Company views (to be completed during the meeting)

| Company | (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |

### Need for LS

Assuming the above ASN.1 corrections are made, it is important for interoperability that the UE and network are implemented according to the CR, otherwise a UE implementing different capabilities for TDD/FDD for any specific feature will not be able to use those features. It is therefore recommended that an informative LS is sent to at least RAN, RAN1.

Q7: Do you agree that an LS is needed to RAN and RAN1 to inform agreement of these CRs and explain interoperability?

Company views (to be completed during the meeting)

| Company | (yes/no) | Comments |
| --- | --- | --- |
|  | yes/no |  |
| Qualcomm | Depends | Depends on what final changes we end up concluding. If no interoperability is broken, then probably not needed. |
| Lenovo | yes | We can inform RAN1 (cc: RAN, RAN4) that RAN2 will implement the missing FDD/TDD differentiation acc. to the RAN1 feature list. |

# Conclusion

[To be updated after offline]

# References

- R2-2005083	“Correction to the LTE Rel-15 TDD/FDD capability differentiation”	Huawei, HiSilicon

- R2-2005084	“Correction to the LTE Rel-15 TDD/FDD capability differentiation”	Huawei, HiSilicon

- R2-1813308  	“LS on Rel-15 LTE UE feature list”	RAN1