3GPP TSG-RAN WG2 #110b-e	R2-200xxxx

E-meeting, June 2020

Agenda Item:	x.x.x.x

Source:	OPPO

Title:	Summary of open issue for V2X capability (OPPO)

Document for:	Discussion, Decision

# Introduction

This is for the discussion of open issues for V2X capability.

- [AT110-e][702][V2X] 38.331/38.306 CR (OPPO)

Capture the agreements related to NR L2 capabilities, procedures and LTE L1 and L2 capabilities (for NR controlled LTE SL communication) in 38.331 (in R2-2005953) / 38.306 (in R2-2005954). It can also include some discussion points to implement CRs (R2-2005955). This offline discussion scope may be updated. Deadline is 6/8 10:00am (UTC)

# Phase-I Discussion

# Impact to 38-spec for NR-Uu controlling NR-PC5

One left issue is the support of MR-DC scenario, which relates to the following FFS-proposal from R2-2004403

Proposal 3	[FFS] RAN2 further discuss the support of NR/LTE PC5 band combination(s) per Uu band combination for (NG)EN/NE/NR-DC scenario.

- On the one hand, since from RAN2 perspective, the functionality specified in R16 can already support MN-controlled sidelink since R16, to follow the release independent spirit of band combination, it is straightforward to define the sidelink band combination for MR-DC scenario (including NR-DC) already in R16, limited to MN-controlled case. Otherwise, after RAN4 define any PC5 BC for MR-DC scenario in R17, R16 UE would fail to report it in capability, i.e., the spirit of release independent cannot be fulfilled.

- On the other hand, there is some further difference between (NG)EN/NE-DC and NR-DC

  - For the former one, not specifying SL BC in R16 means that we do not need to work on the UE-MRDC-Capability container;

  - For the latter one, not specifying SL BC in R16 does not remove our work on the UE-NR-Capability, which is anyway needed for NR SA scenario (please note that the difference between a NR BC supporting CA only and supporting CA + DC is just the presence of ca-ParametersNRDC).

So two separate questions are provided for (NG)EN/NE-DC and NR-DC.

Q2.1-1: Do you agree to introduce PC5 BC for (NG)EN/NE-DC scenario, limited to MN-controlled case?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | No | It is still not clear to us how much one would need such cases. However, purely from signalling point of view, if this is not critical to have, we prefer to not specify anything for the MR-DC container – since it basically reduces the amount of signalling needed for this and complexity. |
| MediaTek | No | Agree with Ericsson. Considering the limited time it seems safest not to take on the additional work on the UE-MRDC-Capability container, and the RAN4 LS was explicit that these band combinations do not need to be defined in Rel-16. |
| Samsung |  | No strong view but in some sense this should be release independent we are fine to consider the scenario. |
| ZTE | No | Since “RAN4 will not define RF requirements for MR-DC + LTE/NR PC5 in Rel-16 specification. LTE/NR PC5 band combination(s) per Uu band combination need not to be introduced for MR-DC scenario in Rel-16.”, RAN2 shall not introduce PC5 BC for (NG)EN/NE-DC scenario and NR-DC scenario. |
| CATT |  | No strong view but slightly prefer no, since it can reduces the amount of signaling and complexity. |
| Huawei, HiSilicon | No | Since RAN4 has already concluded not to support any forms of joint “MCG+SCG+PC5” band combination in this release, we in RAN2 do not need to dive into further details on the capability signaling design details for support such cases either. Also, it seems that even if the UE reports a MCG+PC5 BC for an MR-DC case, we wonder whether it is of any use, because the RAN still does know the SCG capability the UE can support in this case, and thus is still unable to figure out the proper configurations to support UE’s communication concurrently on MCG, SCG and NR/LTE PC5. There should be no problem, if we introduce the UE capabilities for such simultaneous MR-DC+PC5 communication when in later releases RAN4 really supports such cases. |
| Nokia | No | From our point of view, it makes no sense at this point in time, that RAN2 aims to define all possible multiRAT scenarios all in one go without waiting for RAN4 definition (i.e. define CA-ParametersEUTRA, CA-ParametersNR, CA-ParametersNRDC, MR-DC-Parameters) |
| Intel | No | We also share the view with other companies that it is not clear from signaling perspective if this scenario is critical to support |
| Apple | No | Same view as Ericsson |
| Qualcomm | No | Agree with Ericsson |
| vivo | No | Same view as Ericsson. |

- [Easy] RAN2 not introduce PC5 BC for (NG)EN/NE-DC scenario.

Q2.1-2: Do you agree to introduce PC5 BC for NR-DC scenario, limited to MN-controlled case?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | Since we anyway need to introduce PC5 support for NR BC in UE-NR-Capability. |
| Ericsson | Yes | This is indeed correct and our understanding is that this could be solved by just adding one parameter. |
| MediaTek | Yes |  |
| Samsung | Yes |  |
| LG | Yes |  |
| ZTE | No | Since “RAN4 will not define RF requirements for MR-DC + LTE/NR PC5 in Rel-16 specification. LTE/NR PC5 band combination(s) per Uu band combination need not to be introduced for MR-DC scenario in Rel-16.”, RAN2 shall not introduce PC5 BC for (NG)EN/NE-DC scenario and NR-DC scenario. |
| CATT | Yes |  |
| Huawei, HiSilicon | No | See out comments for Q2.1-1. |
| Nokia | Yes |  |
| Intel | No | As per RAN4 agreement, we are not sure if this is scenario shall be supported. So, we agree with ZTE |
| Apple | No | Agree with ZTE. |
| Qualcomm | Yes |  |
| vivo | No | Agree with ZTE, the motivation for this scenario may need to be further clarified. |

Considering for a same NR BC, it may support both CA and DC, rapporteur suggest to solve this issue by discussing method to indicate whether the PC5 BC is supported for DC or not, which is anyway needed even if one does not want to have NR-DC support in Rel-16.

- [FFS] Introduce parameter to indicate a PC5 BC can be supported for a Uu BC when configured as NR-DC or not.

In RAN2#110-E, we agree that

4:	For SL capability report on Uu-RRC agreed in RAN2, allow FDD/TDD differentiation only for a) Logical channel SR-delay timer, and c) multiple SR configuration.

10:	For SL capability report on PC5-RRC agreed in RAN2, no need for either FDD/TDD or FR1/FR2 differentiation.

As discussed online in RAN2#110-E, there is a need to clarify the target carrier (Uu or PC5) when indicate the ‘FDD/TDD diff’ column for a capability. For Uu interface, 38.306 provides an Annex, to clarify the target cell for TDD-FDD CA.

Annex A.1:	TDD/FDD differentiation of capabilities in TDD-FDD CA

Q2.1-3a: For NR SL capability of Logical channel SR-delay timer and multiple SR configuration, how should we indicate the ‘FDD/TDD diff’ in 38.306?

- Option-1: Indicate it as ‘Yes’

- Option-2: Other

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 | We can follow the agreement. |
| Ericsson | 1 | Follow the agreement. |
| MediaTek | 1 |  |
| Samsung | 1 | These two are applied on Uu band which controls sidelink operation. |
| LG | 1 |  |
| ZTE | 1 |  |
| CATT | 1 |  |
| Huawei, Hisilicon | 1 | Since that Logical channel SR-delay timer and multiple SR configuration are related to Uu band, difference would exist when SR is applied to Uu band for TDD and FDD. Therefore, we can indicate the ‘FDD/TDD diff’ of these features as yes as in Uu. |
| Nokia | 1 |  |
| Intel | 1 |  |
| Apple | 1 |  |
| vivo | 1 |  |

- [Easy] For NR SL capability of Logical channel SR-delay timer and multiple SR configuration, indicate it as Yes for ‘FDD/TDD diff’ in 38.306.

Q2.1-3b: For NR SL capability except Logical channel SR-delay timer and multiple SR configuration, how should we indicate the ‘FDD/TDD diff’ in 38.306?

- Option-1: Indicate it as ‘No’

- Option-2: Indicate it as ‘TDD-only’;

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 1 | No strong view, we can keep the agreement as it is. |
| Ericsson | 1 but | If the capability has no meaning for FDD, then option 2 should be selected. However, if it has a meaning for FDD as well, then option 1. Could be maybe good to clarify this aspect before. |
| MediaTek | 1 | We understand that it is meaningful for (Uu) FDD and TDD. |
| Samsung | 1 | Except these two, no difference is expected for FDD and TDD on Uu band. |
| LG | 1 |  |
| ZTE | 1 |  |
| CATT | 1 |  |
| Huawei, Hisilicon | No strong view, | Generally, value ‘No’ denotes that there would be no difference when the feature applied to FDD and TDD, but band provided for NR SL is mainly for TDD as discussed online. Maybe it is more straightforward to indicate ‘TDD-only’ in this case. |
| Nokia | 2 | Actually there are 3 possibilities: 1: same for FDD and TDD, 2: FDD-only, 3: TDD-only. So we agree with Ericsson: should be clarified. |
| Intel | 1 |  |
| Apple | 1 |  |
| vivo | 1 |  |

Considering TDD-only may also lead to the confusion that whether the ‘TDD’ is for PC5 carrier or Uu carrier, rapporteur suggest to follow majority view.

- [Easy] For NR SL capability except Logical channel SR-delay timer and multiple SR configuration, indicate it as No for ‘FDD/TDD diff’ in 38.306.

In LTE spec, the FDD/TDD differentiation has 3 attributives, i.e., Yes/No/-

NOTE 3:	The column 'FDD/TDD diff' indicates if the UE is allowed to signal a different value for FDD and TDD when the capability applies to both FDD and TDD modes. '-' is used when the capability applies to one mode only, 'No' is used for dual mode capabilities where a common value is signalled for both modes, and 'Yes' is used for dual mode capabilities where a separate value is signalled for each mode. Common capabilities and FDD capabilities are reported in the fields of UE-Capability-NB except field tdd-UE-Capability. TDD capabilities are reported in tdd-UE-Capability.

Q2.1-3c For the LTE SL capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, do you agree to align with the conclusion from Q2.1-3b?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | In LTE, the SL related capability are all indicate as “-”, for which the intention is the same as we agreed the ‘FDD/TDD diff’ as ‘No’ in RAN2#110-E for NR SL capability. |
| Ericsson | Yes | It is okay to follow the LTE SL principle when introducing this capabilities in NR spec. |
| MediaTek |  | Should it say “the conclusion from Q2.1-3b”? We have the impression that these should be captured as “no differentiation” like the previous question. |
| Samsung | See comment | If there is any LTE V2X capability which is different for FDD and TDD, we are fine to follow Q2.1-3a. |
| LG | Yes |  |
| ZTE | Yes |  |
| CATT | Yes |  |
| Huawei, Hisilicon | No strong view, | Fine to follow the majority |
| Nokia | Yes |  |
| Intel | No strong view | Ok to follow the majority |
| Apple | Yes |  |
| vivo | Yes |  |

- [Easy] For LTE SL capability, indicate it as No for ‘FDD/TDD diff’ in 38.306.

Q2.1-3d: For LTE/NR SL capability, do you agree to introduce an Annex in TS 38.306, similar to Annex A.1, to explain the target carrier of the ‘FDD/TDD diff’?

- Yes;

- No;

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | As discussed during online, it is good to clarify. |
| Ericsson | Yes | It is still not clear to us, at the moment, which capabilities may be ambiguous in this case, maybe the ones with FDD/TDD diff discussed above, but in any case, yes we added clarification for 38.306 for such cases and we can add as well for sidelink. We should of course use the same structure as in Annex A.1, but we assume that this is the intention. |
| MediaTek | Yes | This seems necessary and we agree to use Annex A.1 as a model. We understand that the capabilities with FDD/TDD differentiation should be disambiguated. |
| Samsung | See comment | We are not sure what else could be included in Annex A.1. In general it is fine to define as Annex A.1 if needed. |
| LG | Yes |  |
| ZTE | Yes | Similar as Uu, it is better to introduce such as Annex for sidelink. |
| CATT | Yes |  |
| Huawei, Hisilicon | Yes | An Annex is helpful to explain differentiation of SL capabilities more clearly. |
| Nokia | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| vivo | Yes |  |

- [Easy] For LTE/NR SL capability, introduce an Annex in TS 38.306, similar to Annex A.1, to explain the target carrier of the ‘FDD/TDD diff’.

# Impact to 38-spec for NR-Uu controlling LTE-PC5

Furthermore, due to the LS from RAN1 in R2-2004314

RAN1 is preparing Rel-16 NR UE feature lists. In NR V2X, for support of NR Uu controlling LTE PC5, RAN1 considers that the contents of UE-EUTRA-Capability for LTE-V2X sidelink is sufficient. RAN2 is requested to define the necessary NR signalling.

So RAN2 has to decide whether the LTE-V2X capability has to be introduced into 38.331.

| Index | Capability Name | Definition |
| --- | --- | --- |
| 1 | v2x-SupportedTxBandCombListPerBC-r14, v2x-SupportedRxBandCombListPerBC-r14 | This field indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedTxBandCombinationList or v2x-SupportedRxBandCombinationList on which the UE supports simultaneous transmission and reception of EUTRA and V2X sidelink communication respectively. |
| 2 | v2x-SupportedBandCombinationList-r14 | This field indicates the bands on which the UE supports V2X sidelink communication, as defined in TS 23.285 [29] and specified in TS 36.331 [5]. If a UE supports V2X sidelink communication, the UE shall support a maximum number of 8 sidelink processes associated with the Sidelink HARQ Entity for the transmission of V2X sidelink communication on SL-SCH. |
| 3 | zoneBasedPoolSelection-r14 | This parameter indicates whether the UE supports zone based transmission resource pool selection for V2X sidelink communication. |
| 4 | ue-AutonomousWithFullSensing-r14 | This parameter indicates whether the UE supports transmitting PSCCH/PSSCH using UE autonomous resource selection mode with full sensing (i.e., continuous channel monitoring) for V2X sidelink communication and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [6]. |
| 5 | ue-AutonomousWithPartialSensing-r14 | This parameters indicates whether the UE supports transmitting PSCCH/PSSCH using UE autonomous resource selection mode with partial sensing (i.e., channel monitoring in a limited set of subframes) for V2X sidelink communication and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [6]. |
| 6 | slss-TxRx-r14 | This parameter indicates whether the UE supports SLSS/PSBCH transmission and reception in UE autonomous resource selection mode and eNB scheduled mode for V2X sidelink communication. |
| 7 | sl-CongestionControl-r14 | This parameter indicates whether the UE supports Channel Busy Ratio measurement and reporting of Channel Busy Ratio measurement to eNB for V2X sidelink communication. |
| 8 | v2x-TxWithShortResvInterval-r14 | This parameter indicates whether the UE supports 20 ms and 50 ms resource reservation periods for UE autonomous resource selection and eNB scheduled resource allocation for V2X sidelink communication. |
| 9 | v2x-numberTxRxTiming-r14 | This parameter indicates the number of multiple reference TX/RX timings counted over all the configured sidelink carriers for V2X sidelink communication. |
| 10 | v2x-nonAdjacentPSCCH-PSSCH-r14 | This parameter indicates whether the UE supports transmission and reception in the configuration of non-adjacent PSCCH and PSSCH for V2X sidelink communication. |
| 11 | slss-SupportedTxFreq-r15 | This parameter indicates whether the UE supports the SLSS transmission on single carrier or on multiple carriers in the case of sidelink carrier aggregation. |
| 12 | sl-64QAM-Tx-r15 | This parameter indicates whether the UE supports 64QAM for the transmission of V2X sidelink communication. |
| 13 | sl-TxDiversity-r15 | This parameter indicates whether the UE supports transmit diversity for V2X sidelink communication. See TS 36.101 [6]. |
| 14 | sl-64QAM-Rx-r15 | This parameter indicates whether the UE supports 64QAM for the reception of V2X sidelink communication. It is mandatory to support 64QAM for the reception of V2X sidelink communication for UEs which are supporting Rel-15 V2X sidelink communication as specified in TS 36.331 [5]. |
| 15 | sl-RateMatchingTBSScaling-r15 | This parameter indicates whether the UE supports rate matching and TBS scaling of V2X sidelink communication. It is mandatory to support rate matching and TBS scaling of V2X sidelink communication for UEs which are supporting Rel-15 V2X sidelink communication as specified in TS 36.331 [5]. |
| 16 | sl-LowT2min-r15 | This parameter indicates whether the UE supports 10ms as minimum value of T2 for resource selection of V2X sidelink communication. It is mandatory to support 10ms as minimum value of T2 of V2X sidelink communication for UEs which are supporting Rel-15 V2X sidelink communication as specified in TS 36.331 [5]. |
| 17 | v2x-SensingReportingMode3-r15 | This parameter indicates whether the UE supports sensing measurements and reporting of measurement results in eNB scheduled mode for V2X sidelink communication. |
| 18 | ue-CategorySidelinkEUTRA | Define reception and transmission capabilities for V2X sidelink communication. |
| 19 | v2x-HighReception-r14 | This parameter indicates whether the UE supports reception of 20 PSCCH in a subframe and decoding of 136 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication. |
| 20 | v2x-eNB-Scheduled-r14 | This parameter indicates whether the UE supports transmitting PSCCH/PSSCH using dynamic scheduling, SPS in eNB scheduled mode for V2X sidelink communication, reporting SPS assistance information and the UE supports maximum transmit power associated with Power class 3 V2X UE, see TS 36.101 [6] in a band. |
| 21 | v2x-HighPower-r14 | This parameter indicates whether the UE supports maximum transmit power associated with Power class 2 V2X UE for V2X sidelink transmission in a band, see TS 36.101 [6]. |
| 22 | v2x-EnhancedHighReception-r15 | This parameter indicates whether the UE supports reception of 30 PSCCH in a subframe and decoding of 204 RBs per subframe counting both PSCCH and PSSCH in a band for V2X sidelink communication. |
| 23 | bandwidthClassTxSidelinkEUTRA, bandwidthClassRxSidelinkEUTRA | The bandwidth class for V2X sidelink transmission and reception supported by the UE as defined in TS 36.101 [14], Table 5.6G.1-3. The UE explicitly includes all the supported bandwidth class combinations for V2X sidelink transmission or reception in the band combination signalling. Support for one bandwidth class does not implicitly indicate support for another bandwidth class. |

Q2.2-1: Do you agree that any of the following LTE-V2X capability has to be introduced into 38.331 for NR-Uu controlling LTE-PC5 scenario, as indicated in LS R2-2004314?

| Company | Option from 1-23 above | Comments (Reason for any IE that should be omitted/modified) |
| --- | --- | --- |
| OPPO | All, with change on 20 | For 1, it should be modified, since it should be for NR BC instead of EUTRA BC. For 20, v2x-eNB-Scheduled-r14 should be modified, since it should be for gNB. |
| Ericsson | All | Ok with the change proposed by Oppo. |
| MediaTek | All | Agree with OPPO. |
| Samsung | All | Okay with the change by OPPO |
| LG | All | Agree with OPPO |
| ZTE | All | Change the terms to NR accordingly. |
| CATT | All | Agree with OPPO |
| Huawei, Hisilicon | All, with minor changes | Some revisions may be needed to the descriptions of the above features: - For the definition of feature 1, it should be revised to “for a particular band combination of NR, …”, ‘the UE supports simultaneous transmission and reception of NR Uu and V2X sidelink communication respectively’. - For the definition of feature 6, it should be ‘and gNB scheduled mode for V2X sidelink communication’ - For the definition of feature 20, the scenario of NR-Uu controlling LTE-PC5, it is the gNB that is provide SL resources/configuration for mode-3, thus it should be defined like v2x-gNB-Scheduled. |
| Nokia | All | We do not agree with the formulation of the question, since R2-2004314 does not indicate anything at all – the RAN1 LS just requests RAN2 to specify the signaling. |
| Intel | All | Agree with OPPO |
| Apple | All |  |
| Qualcomm | All |  |
| vivo | all | Agree with OPPO about the modification. |

Since majority view is to rely on container-based method, we can check the detailed wording on field description in CR implementation.

- [Easy] LTE-V2X capability is introduced into 38.331 for NR-Uu controlling LTE-PC5 scenario, detailed rewording can be solved in CR implementation.

Q2.2-2: For the LTE-V2X capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, how should it be captured (note that example of option-1/2 is provided in the draft-CR):

Option-1: capture as explicit bits, i.e., basically to copy the bits from 36-spec to 38-spec;

Option-2: use the container-based method;

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 2 | We see no technical reason that option-2 is not feasible, which however apparently reduce the specification effort as shown in the running CR, especially considering when later we introduce new UE capability for LTE/NR PC5. |
| Ericsson | 1 | Our view is that it is better to capture this explicitly since there should be no dependencies between NR and LTE capabilities in NR capability container. It does not seem to be much extra work since we anyway have to have a clear view on which capabilities should be applicable in 38-spec, as also asked in the previous questions. A further reason for going for Option 1 is that we do not want to break the principle that that NR is not mandated to understand LTE capabilities and vice-versa. |
| MediaTek | 2 | We think parallel copies of the capability should be avoided; someday, someone will make a mistake copying an addition from one to the other. We don’t think this really violates the principle that NR is not mandated to understand LTE capabilities; these are the UE’s capabilities for something that the NR node is controlling, so they are best seen as capabilities for operation under NR even though they originated in an LTE spec. |
| Samsung | 2 | WE think container method is fine. |
| LG | 2 |  |
| ZTE | 2 | Similar as previous agreements for crass-RAT operations, container-based method shall be used as much as possible if there is no technical reason. |
| CATT | 2 with comments | We agree using container method can reduce the specification effort. But we wonder if using container method, how to modify the IE as proposed by OPPO in Q2.2-1‎, e.g., to modify “v2x-eNB-Scheduled-r14” into gNB. |
| Huawei, Hisilicon | 2 | If option 1 is adopted, the NR module in the UE needs to deal with the LTE V2X capabilities, which is complex from implementation perspective especially for supportedBandCombinationListSidelinkEUTRA-r16. On the contrary, in option 2 the LTE V2X capabilities are just handled by the LTE module in the UE. However, note that as shown in the corresponding 38.331 CR provided by the Rapp, some descriptions/explanations may be needed on the following parameters. |
| Nokia | 2 | Container-based capability transfer is preferable |
| Intel | 2 | We prefer the container based approach, which is also inline with the gNB signalling to be provided to UE over NR Uu for LTE SL operation (as was agreed earlier) |
| Apple | 2 | Agree with OPPO |
| Qualcomm | 2 | Agree with OPPO |
| vivo | 2 |  |

- [Easy] For the LTE-V2X capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, it is captured using container-based method.

Q2.2-3: For the LTE-V2X capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, is there any capability that needs FR1/FR2 differentiation (which was not defined in LTE spec)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | No | We can indicate “No” for ‘FR1/FR2 Diff’ column for all LTE SL capabilities, and anyway we can further solve that later if any issue identified. |
| Ericsson | No but | In principle we are fine but we are not sure whether it can be easily solved for parameters that are not anyway within a structure that allows FR1/FR2 diff (assuming there is one), but see no reason anyway to differ from LTE. |
| MediaTek | No | We don’t identify anything where differentiation is needed. |
| Samsung | No | At this stage, we do not see any difference for FR1/FR2 in LTE-V2X capability. |
| LG | No |  |
| ZTE | No |  |
| CATT | No |  |
| Huawei, Hisilicon | No with comments | As LTE SL is at Band 47 only, we have not seen necessity to indicate FR1/FR2 differentiation for LTE SL capabilities. But If the question implies that LTE V2X is allow to use licensed band, we think FR1/FR2 differentiation is required for LTE SL capabilities. |
| Nokia | No | As of now there are no FR1/FR2 diff LTE-PC5 capability existing in 38.306. |
| Intel | No | None at this stage |
| Apple | No |  |
| Qualcomm | No |  |
| vivo | No |  |

- [Easy] For the LTE-V2X capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, no need for FR1/FR2 differentiation.

# Impact to 36-spec for LTE-Uu controlling NR-PC5

RAN2#110-E has decided the following L2 capability for NR-PC5.

| Parameter | Detailed component |
| --- | --- |
| RLC parameter | 1) 12-bit SN for UM |
|  | 2) 18-bit SN for AM |
| MAC parameter | 3) LCP restriction |
|  | 4) Logical channel SR-delay timer |
|  | 5) Multiple CGs |
|  | 6) multiple SR configuration (e.g., up to 8 for sidelink triggered SR) |

Q2.3-1: Do you agree that any of the NR-V2X capability above has to be introduced into 36.331 for LTE-Uu controlling NR-PC5 scenario?

| Company | Option from 1-23 above | Comments (Reason for any IE that should be omitted/modified) |
| --- | --- | --- |
| OPPO | Only 1/2/3/5 | 4/6 are for SR, which is not useful for LTE-Uu controlling NR-PC5 scenario. |
| Ericsson | Only 1/2/3/5 |  |
| MediaTek | 1/2/3/5 |  |
| Samsung | 1/2/3/5 |  |
| LG | 1/2/3/5 |  |
| ZTE | 1/2/3/5 |  |
| CATT | 1/2/3/5 | Agree with OPPO |
| Huawei, Hisilicon | 1),2),3),5) | As SR for SL is not supported in the case of LTE-Uu controlling NR-PC5, there is no necessity to introduce NR SL capability for 4) and 6). |
| Nokia | 1/2/3/5 |  |
| Intel | 1, 2, 3, 5 | Same view as Huawei |
| Apple | 1,2,3,5 |  |
| Qualcomm | Only 1/2/3/5 |  |
| vivo | 1/2/3/5 |  |

- [Easy] NR-V2X capability above (RLC parameter: 12-bit SN for UM, 18-bit SN for AM; MAC parameter: LCP restriction and Multiple CG) has to be introduced into 36.331 for LTE-Uu controlling NR-PC5 scenario, detailed rewording can be solved in CR implementation.

Q2.3-2: For the NR PC5 capability that has to be introduced into 36-spec for LTE-Uu controlling NR-PC5 scenario, how should it be captured (note that example of option-1/2 is provided in the draft-CR):

Option-1: capture as explicit bits, i.e., basically to copy the bits from 36-spec to 38-spec;

Option-2: use the container-based method;

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | 2 | We see no technical reason that option-2 is not feasible, which however apparently reduce the specification effort as shown in the running CR, especially considering when later we introduce new UE capability for LTE/NR PC5. |
| Ericsson | 1 | Our view is that it is better to capture this explicitly since there should be no dependencies between NR and LTE capabilities in NR capability container. It does not seem to be much extra work since we anyway have to have a clear view on which capabilities should be applicable in 38-spec, as also asked in the previous questions. A further reason for going for Option 1 is that we do not want to break the principle that that NR is not mandated to understand LTE capabilities and vice-versa. |
| MediaTek | 2 | Same comments as for LTE PC5 capability in 38.xxx. |
| Samsung | 2 | Container based is fine. |
| LG | 2 |  |
| ZTE | 2 | Similar as previous agreements for crass-RAT operations, container-based method shall be used as much as possible if there is no technical reason. |
| CATT | 2 |  |
| Huawei, Hisilicon | 2 | If option 1 is adopted, the LTE module in the UE needs to deal with the NR V2X capacities, which is complex from implementation perspective especially for v2x-SupportedBandCombinationListNR-r16. On the contrary, in option 2 the NR V2X capacities are just handled by the NR module in the UE. In addition, container-based method (i.e. option 2) is more suitable if the related V2X capabilities need to be modified in the future. However, note that in the corresponding daft 36.331 CR provided by OPPO, option 2 needs descriptions/explanations on the V2X capability parameters i.e. option 2 in the draft CR is not complete. |
| Nokia | 2 | Same as for NR-Uu controlling LTE-PC5 |
| Intel | 2 | Same view as for NR Uu controlling LTE PC5 |
| Apple | 2 | Same view as the case of NR Uu control LTE SL |
| Qualcomm | 2 |  |
| vivo | 2 |  |

- [Easy] For NR PC5 capability that has to be introduced into 36-spec for LTE-Uu controlling NR-PC5 scenario, it is captured using container-based method.

# Phase-II Discussion

# NR-PC5 RAN1 feature list

# 15-4/15-15

| 15-4 | Synchronization sources for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 |
| --- | --- | --- | --- |
| 15-15 | eNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports eNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 |

Q3.1.1-1: Do you agree that 15-15 only need to be reported to eNB, and thus only need to be captured in LTE spec?

- Yes

- No

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | Yes | 15-15 component-2/3 is for scenario where eNB is deployed. |

# 15-7/15-9

| 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can be scheduled over NR Uu by DCI format 3_1 for LTE sidelink mode 3 transmission.. 2) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3 | UE supports LTE V2X sidelink |
| --- | --- | --- | --- |
| 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can be configured over NR Uu for LTE sidelink mode 4 operation | UE supports LTE V2X sidelink |

Q3.1.2-1: With 15-7 and 15-9 being introduced, do you agree that the v2x-eNB-Scheduled-r14 (as the outcome of Q2.2-1) does not need to be captured into NR spec (for the NR-Uu controlling LTE-PC5 scenario), i.e., different from the other LTE sidelink capability which should be captured in NR spec?

- yes

- no

| Company | Option | Comments |
| --- | --- | --- |
| OPPO | yes |  |

# 15-16

| 15-16 | Simultaneous transmission of uplink and sidelink | 1) UE supports simultaneous transmission of NR uplink and NR sidelink (in different bands) in a band combination for which the UE indicated simultaneous sidelink and uplink support in a band combination. | At least one of 15-2 and 15-3 |
| --- | --- | --- | --- |

Q3.1.3-1: For 15-16, should it be captured separately for simultaneous transmission and reception, as we did in LTE?

- Yes

- No

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | As we did in LTE, i.e., v2x-SupportedTxBandCombListPerBC, v2x-SupportedRxBandCombListPerBC Indicates, for a particular band combination of EUTRA, the supported band combination list among v2x-SupportedBandCombinationList on which the UE supports simultaneous transmission or reception of EUTRA and V2X sidelink communication respectively. The first bit refers to the first entry of v2x-SupportedBandCombinationList, with value 1 indicating V2X sidelink transmission/reception is supported. |

# Other

According to agreement

- 2a Option 1: To skip the UE feature group entry if the FFS is in any column other than mandatory/optional column for any component in that feature group

- For the CRs that RAN2 is working on separately from the mega CR, the above agreement also applies i.e. 2a Option 1: To skip the UE feature group entry if the FFS is in any column other than mandatory/optional column for any component in that feature group. The CR rapporteur to take responsibility for this.

Q3.1.4-1: Is there anything in RAN1 feature list that is immature to be captured?

| Company | Comments |
| --- | --- |
| OPPO | According to the agreement above, FG15-1, 15-2, 15-3 15-5, 15-6, 15-11, 15-14, 15-18, 15-19, 15-22, 15-23, 15-24 should be skipped. The note related to “38.101-1 Table 5.2E-1”, which is dependent on RAN4 reply, should be skipped. Definition on basic feature group, which is a general question and thus may need conclusion from main room discussion on how to capture that, should be skipped, The NR-PC5 RAN1/4 capability for LTE-Uu controlling NR-PC5 scenario is FFS in R2-2006096, so should be skipped. |

# Left issues from Phase-I discussion

As discussed in Q2.1-2, we may need to have a parameter to indicate UE supports the PC5 BC when the NR Uu BC is configured as NR-DC or not, then the left issue is what the granularity of this parameter is.

Q3.2-1: For the parameter which indicate UE supports the PC5 BC when the NR Uu BC is configured as NR-DC or not, what the granularity of this parameter is:

Option-1: Per-UE;

Option-2: Per NR Uu BC;

Option-3: Per PC5 BC;

Option-4: Per NR Uu BC and Per PC5 BC;

| Company | Option | Comments |
| --- | --- | --- |

Q3.2-2: On PC5-RRC, do you agree to introduce a parameter to indicate the release, i.e., to mimic Uu AccessStratumRelease?

- Yes

- No

| Company | Yes/No | Comments |
| --- | --- | --- |
| OPPO | Yes | It is necessary considering PC5 unicast will evolve in Rel-17 and later. |

# Conclusion

We have the following proposals:

Proposal 1	[Easy] RAN2 not introduce PC5 BC for (NG)EN/NE-DC scenario.

Proposal 2	[FFS] Introduce parameter to indicate a PC5 BC can be supported for a Uu BC when configured as NR-DC or not.

Proposal 3	[Easy] For NR SL capability of Logical channel SR-delay timer and multiple SR configuration, indicate it as Yes for ‘FDD/TDD diff’ in 38.306.

Proposal 4	[Easy] For NR SL capability except Logical channel SR-delay timer and multiple SR configuration, indicate it as No for ‘FDD/TDD diff’ in 38.306.

Proposal 5	[Easy] For LTE SL capability, indicate it as No for ‘FDD/TDD diff’ in 38.306.

Proposal 6	[Easy] For LTE/NR SL capability, introduce an Annex in TS 38.306, similar to Annex A.1, to explain the target carrier of the ‘FDD/TDD diff’.

Proposal 7	[Easy] LTE-V2X capability is introduced into 38.331 for NR-Uu controlling LTE-PC5 scenario, detailed rewording can be solved in CR implementation.

Proposal 8	[Easy] For the LTE-V2X capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, it is captured using container-based method.

Proposal 9	[Easy] For the LTE-V2X capability that has to be introduced into 38-spec for NR-Uu controlling LTE-PC5 scenario, no need for FR1/FR2 differentiation.

Proposal 10	[Easy] NR-V2X capability above (RLC parameter: 12-bit SN for UM, 18-bit SN for AM; MAC parameter: LCP restriction and Multiple CG) has to be introduced into 36.331 for LTE-Uu controlling NR-PC5 scenario, detailed rewording can be solved in CR implementation.

Proposal 11	[Easy] For NR PC5 capability that has to be introduced into 36-spec for LTE-Uu controlling NR-PC5 scenario, it is captured using container-based method.

# Reference

- R2-2002638	Summary of [Post109e#20] V2X Remaining UE capability issues (OPPO)	OPPO	report	Rel-16	5G_V2X_NRSL-Core	Late

- R2-2002639	Summary of capability related Tdoc submitted to R2#109bis-E	OPPO	report	Late