3GPP TSG RAN WG2 Meeting #110-e		R2-200xxxx

1 – 12 June 2020

Title:	LS on system support for WUS

Response to:	R2-2004317/S2-2003217 Reply LS on assistance indication for WUS

Release:	Release 15

Work Item:	NB_IOTenh3-Core, LTE_eMTC5-Core

Source:	RAN2

To:	SA2, RAN3

Cc:

Contact person:	Mungal Dhanda

mdhanda (at) qti (dot) qualcomm (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 has discussed changes to its specification to implement system support for WUS as advised in the SA2 LS in R2-2004317/S2-2003217. RAN2 has already notified SA2/RAN3 of the issue resulting from handover/reestablishment in R2-2005939.

RAN2 has identified an additional issue with the solution described in R2-2004317/S2-2003217. The scenario is as follows:

- UE that supports WUS reselects to another cell and initiates RRC connection establishment. This cell also supports WUS.

- After eNB receives RRCConnectionSetupComplete message (MSG5), eNB is unable to establish S1 connection (e.g. due to core network congestion). As eNB cannot send RRCReject message because the RRC connection has been established, the eNB sends RRCConnectionRelease message. But there is no cause in RRCConnectionRelease message to indicate eNB was unable to setup S1 connection.

- Upon reception of RRCConnectionRelease message, UE would return to idle and expect to receive WUS from this eNB because it is in the ‘last used cell’. But the eNB will not page UE with WUS because, according to MME, this cell was not the last cell where S1 connection was released/suspended.

Issue: While UE remains camped on this cell it is now not reachable by the eNB till the UE sends subsequent data to the network (i.e. MO data, MO signalling). This issue can also happen with 5GC.

Due to this and the earlier issue identified in R2-2005939, RAN2 has postponed the CRs.

# 2	Actions

To 3GPP SA2, RAN3

ACTION: RAN2 respectfully asks SA2 and RAN3 to provide feedback on the issue identified above.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #111e	17 – 28 August 2020

TSG RAN WG2 Meeting #112e	2 – 13 November 2020