3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2006573

Online, 17 – 28 August 2020

Agenda Item:	8.7.4

Source: 	MediaTek Inc.

Title:  	Initiation of relaying operation

Document for:	Discussion and decision

# Introduction

As discussed in legacy LTE ProSe operation (Rel-13), the relaying initiation includes both relay discovery procedures and Relay UE Selection/Reselection procedures, as a precondition for Sidelink Relay operation.

In general, Relay discovery and Relay UE Selection/Reselection occur sequentially. The Remote UE may obtain multiple ProSe UE-to-network relay UE candidates that fulfills the discovery criteria and then a selection needs to be made to select one of them.

# Relay Discovery

## Relay Discovery Discussed at SA2

In legacy LTE ProSe, there are two types of ProSe Direct Discovery: open and restricted. Open discovery is the case where there is no explicit permission that is needed from the UE being discovered, whereas restricted discovery only takes place with explicit permission from the UE that is being discovered. Both types of ProSe Direct Discovery can be supported at 5G ProSe Direct Discovery. Besides, there are two models for 5G ProSe Direct Discovery inherited from LTE ProSe (as defined in TS 23.303): Model A and Model B (as depicted in Figure 1 and 2 below).

Figure 1: Model A based Discovery.                                     Figure 2: Model B based Discovery.

During the study within 5G ProSe, SA2 discussed the procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity as 5G ProSe Direct Discovery. The ProSe direct discovery procedure is used for a UE to discover or be discovered by other UE(s) in proximity over the PC5 interface. The UE can discover other UE(s) with interested application(s) and/or interested group(s) using the ProSe direct discovery procedure. The solutions discussed by SA2 strive to achieve the common direct discovery procedure for discovering a 5G Prose enabled UE, 5G Prose UE-to-Network Relay and a 5G Prose UE-to-UE Relay. SA2 studied both in coverage case and out of coverage case for 5G ProSe Direct Discovery. Network is assumed to be able to control the ProSe direct discovery procedure when the UE is in coverage.

As captured by TR23.752 [1], the following table lists some candidate solutions for Relay discovery and overview analysis is provided.

| #Solution at TR23.752 | Title of the solution | Overview |
| --- | --- | --- |
| #1 | Restricted direct discovery | Reuse legacy mechanism to include both Model A and Model B based discovery procedure |
| #2 | V2X based solution for ProSe direct discovery | Direct Communication Request is used to broadcast or unicast the discovery message |
| #3 | Solution for ProSe 5G Direct Discovery using PC5 communication channel | Discovery message as MAC payload is Multiplexed with PC5 communication supporting both Model A and Model B based discovery procedure |
| #8 | UE-to-UE Relay Selection Without Relay Discovery | Direct Communication Request is used as the discovery message for UE-to-UE Relay case |
| #9 | Connection establishment via UE-to-UE Layer-2 Relay | Establish the relaying link without Relay discovery procedure for UE-to-UE Relay case |

RAN WGs needs to consider the transmission scheme of the discovery message based on the conclusion that will be made at SA2 for ProSe discovery solution. For example, specific to solution #1, RAN WGs (i.e. RAN1/RAN2) needs to provide the size limitation on the discovery message (i.e. PC5-S message). However, this issue can be left to normative phase.

Observation 1: The discussion on the size limitation on the Relay discovery message needs to be provided by RAN WGs to SA2 but this issue can be left to normative phase.

In addition, solution #8 suggests a UE-to-UE Relay selection without explicit Relay discovery procedure. If solution #8 is selected, the corresponding relaying initiation procedure skips Relay discovery procedure.

## Initiation of Relay Discovery

The initiation of Relay discovery procedures is used for the selection of a Relay UE in the case of both in-coverage Remote UEs and out-of-coverage Remote UEs. For both cases, the Relay discovery procedure can be either initiated by the Relay UE or by the Remote UE, which is subject to the PC5-S procedures.

We can take solution 1/3 as discussed in TR23.752 as the example. In the case of Model A based discovery, the Relay UE can announce the discovery messages (i.e. Announcement message) in broadcast manner at pre-defined discovery intervals and the Remote UEs that are interested in Relay service can monitor them and read them. In case of Model B based discovery, only restricted discovery type is supported. The Remote UE can transmits a request (i.e. Solicitation message) containing certain information about Relay Service information. Then all Relay UEs in proximity can respond to this message (i.e. sending Response message) if it can provide the corresponding Relay Service as requested by Remote UE.

Looking back to Rel-13 LTE ProSe UE to Network Relay operation, the initiation of Relay discovery procedures is governed by upper layer (i.e. PC5-S). The related discussion at SA2 also suggests the PC5-S based Relay discovery operation, since PC5-S has a better position to interact with the application layer. Then we need to confirm the working assumption from RAN2 perspective. Note that there is no relay discovery procedure e.g. for UE-to-UE relay case, according to the candidate solutions as discussed at SA2.

Proposal 1: The initiation of Relay discovery procedures if required is managed by upper layer (i.e. PC5-S) and transparent to AS layer.

## Basic Principle of Relay Discovery

During the discussion for Rel-13 LTE ProSe UE to Network Relay, the transmission of Relay discovery message reuses the same mechanism for Rel-12 D2D discovery.  The PC5-D protocol stack is used to assemble the PC5-S Relay discovery messages. And MAC PDU of Relay discovery message is carried by specific physical channel PSDCH. The communication mode of Rel-13 LTE ProSe UE to Network Relay discovery is sidelink discovery based.

Figure 4: Protocol stack for Relay Discovery message

As for NR Sidelink, there is no definition for sidelink discovery based communication mode. PC5-S Relay discovery message should be based on sidelink communication. Then the same protocol stack of PC5 sidelink communication signalling as depicted in Figure 4 (i.e. PC5-S/PDCP/RLC/MAC/PHY) should be applied to Relay discovery message. Relay discovery message is carried by general physical channel PSCCH/PSSCH.

Proposal 2: The protocol stack PC5-S/PDCP/RLC/MAC/PHY is used to transmit PC5-S Relay discovery message over PC5.

Traditionally, the UE (Relay) discovery message over PC5 is resolved at PC5-S layer, which means the UE receiving the relay related discovery message can only identify the UE after resolving the PC5-S message during discovery stage. Reusing PSCCH/PSSCH to carry the discovery message provides the possibility to add UE identity or Relay service into the physical layer (within SCI) and/or MAC header.

For example, in the context of Model A based discovery, the Relay UE can announce the discovery messages piggybacking the Relay Service Code (corresponding to the supported Relay service he can provide) into the MAC header of MAC PDU carrying the Discovery message (i.e. Announcement message). The receiving Remote UE can do packet filtering based on the Relay Service Code at MAC layer when he decodes the MAC PDU. This can enable Remote UE to do early packet filtering and to identify the right Relay UE in a more efficient manner comparing to traditional handling.

Overall, it would be beneficial to support AS layer based packet filtering for Relay Discovery message to enable early identification of Relay UE. Then we provide the proposal as below:

Proposal 3: Support AS layer based packet filtering for PC5-S Relay discovery message over PC5 for NR Sidelink Relay.

## AS Criterion for Relay Discovery

According to the Remote UE behaviour as specified for Rel-13 LTE ProSe UE to Network Relay, remote UE can decide when to start monitoring for Relay discovery. Remote UE checks if his Uu link signal quality is lower than the threshold broadcasted by the eNB before transmiting Relay discovery solicitation message, in order to communicate with Relay UE. The Remote UE in RRC_CONNECTED mode indicates to eNB that it is a Remote UE and wants to participate in Relay discovery and communication. Remote UE stops using Relay communication resources when his Uu link RSRP goes above the broadcasted threshold.

The abovementioned two threshold approach can be reused for NR Sidelink UE-to-Network Relay. From Relay Discovery perspective, only the first threshold is used. But when the Remote UE is out of coverage, the threshold used to determine the start of Relay discovery should be preconfigured.

Proposal 4a: Support Uu link signal quality threshold based approach for remote UE to determine the start of Relay discovery for the case of NR Sidelink UE-to-Network Relay.

Specific to UE-to-UE Relay, the Uu link signal quality based threshold may be not used. The UE-to-UE Relay based operation can occur even if there is good coverage for both remote UE and Relay UE. For example the communication between two member cars can be forwarded by the head node in a platooning.

Proposal 4b: The Uu link signal quality threshold based approach is not used for remote UE to determine the start of Relay discovery for the case of NR Sidelink UE-to-UE Relay.

## Resource Allocation for Relay Discovery

With regards to Rel-13 LTE ProSe UE to Network Relay, both transmission resources and reception resources can be provided by Base Station to the UE. Dedicate Resource Discovery pools (for both TX and RX) were defined for Rel-13 LTE ProSe UE to Network Relay. Those pools can also be shared with other public safety usage.

As far as NR Sidelink (Rel-16), there is no discovery pool concept defined. In order to support NR sidelink Relay oriented discovery operation, it is necessary to introduce discovery resource pool specific to NR sidelink UE discovery. Alternatively, specific communication pool for UE (relay) discovery purpose can be defined for NR sidelink, since the discovery message is based on sidelink communication channel (PSCCH/PSSCH).

Proposal 5a: Specific discovery resource pool is defined for purpose of Relay discovery (for both TX and RX).

The configuration of the abovementioned discovery pool follows NR Rel-16 V2X legacy procedure: SIB/dedicated signaling/pre-configuration. Both mode 1 and mode 2 based resource pool configuration should be allowed. A desirable scenario is that the TX discovery pool configured to TX UE is aligned with the RX discovery pool configured to RX UE. But we believe the alignment can be ensured by network implementation.

One additional issue that was discussed at Rel-13 LTE ProSe UE to Network Relay is the interval of discovery message transmission. As discussed above, the initiation of Relay discovery message is controlled by upper layer, but the radio resource pool is configured by AS layer. The transmission of a particular upper discovery message may be restricted by the resource pattern as configured and defined by TX discovery pool. For example, if a Relay discovery message is initiated by upper layer but there is no radio resource for the transmission, that Relay discovery message may be dropped or postponed.

Proposal 5b: The transmission of Relay discovery message depends on the availability of transmission resources defined by the configured discovery resource pool.

# Relay Selection/Reselection

## Scenarios for Relay Selection/Reselection

There are various scenarios where the Relay UE selection procedure needs to be performed.The support of NR sidelink relay operation requires the functionality that UE needs to select a proper Relay UE when multiple Relay UEs are available. When the Remote UE moves his link from cellular to PC5, a Relay UE needs to be selected before the cellular link quality dropped in order to enable necessary path switch  procedure (from Uu to PC5) to ensure service continuity. Relay UE selection procedure can also be triggered when Remote UE is out of cellular coverage and Remote UE wishes to establish relay connection with the network (i.e. UE-to-Network case) or another UE (i.e. UE-to-UE case). During the sidelink relay operation, there is a need for Remote UE to dynamically reselect the best Relay UE to keep or improve the relay link quality.

Based on the discussion above, overall, there are four cases where the Remote UE needs to trigger Relay (Re)Selection for corresponding purpose:

| Scenario list | Description | Purpose | Categorization |
| --- | --- | --- | --- |
| Scenario-A | Remote UE and Relay UE are in cellular network coverage | Switch from Uu to PC5 (Relay Selection) | UE-to-Network Relay |
| Scenario-B | Remote UE is out of coverage and Relay UE is in coverage | Establishment of UE-to-Network Relay link (Relay Selection) | UE-to-Network Relay |
| Scenario-C | Both Initiating Remote UE and Peer Remote UE are out of coverage | Establishment of UE-to-UE Relay link (Relay Selection) | UE-to-UE Relay |
| Scenario-D | Remote UE is connecting with a Relay UE | Reselect the best Relay link (Relay Reselection) | UE-to-Network Relay and UE-to-UE Relay |

Proposal 6: Consider the following scenarios for Relay (Re)Selection:

Scenario-A: Remote UE and Relay UE are in cellular network coverage (Relay Selection, UE-to-Network Relay)

Scenario-B: Remote UE is out of coverage and Relay UE is in coverage (Relay Selection, UE-to-Network Relay)

Scenario-C: Both Initiating Remote UE and Peer Remote UE are out of coverage (Relay Selection, UE-to-UE Relay)

Scenario-D: Remote UE is connecting with a Relay UE (Relay Reselection, UE-to-Network Relay and UE-to-UE Relay)

## Criterion for Relay Selection/Reselection

For in coverage remote UEs, when the Uu link quality becomes poor, the measurement results of neighbouring cells are not good enough for a potential handover. In this case, remote UE performs the relay discovery procedure and tries to select a relay UE to connect to the network (for UE-to-Network relay case). For an out of coverage remote UE, when it wants to connect to the network or other UE and no suitable cell can be found, the remote UE may turn to a Relay UE. In this case, it first performs the relay discovery procedure and tries to select a relay UE for proceeding communication.

During the discussion of LTE ProSe UE-to-Network Relay at Rel-13, the PC5 radio link quality was agreed for relay UE selection/re-selection from RAN perspective. The need of adopting Uu link quality of Relay UE was discussed for relay UE selection/re-selection but it was not agreed. It should be up to relay UE to ensure the Uu link quality with the Base Station.  There were also additional aspects discussed:

- Relay UE load level, Uu load level

- Relay Battery status

- Connectivity details provided by the Relay UE (e.g. PLMN identity)

However, it was concluded that Remote UE selects relay purely based on PC5 radio link quality i.e. no additional factors are considered at AS layer. In general, from radio perspective, the mechanism adopted by Rel-13 LTE ProSe UE-to-Network Relay selection/re-selection can be reused for NR sidelink Relay: Select the UE-to-Network Relay which has the best link quality on PC5.

In case of UE-to-UE relay, it is obviously no need to consider the Uu link quality as the concerned the link quality is only PC5. So then the same criterion for UE-to-Network Relay (Re)Selection may apply.

Apart from the criterion discussed for Relay (Re)Selection at AS layer, there should be additional consideration at upper layer. For example, the upper layer criterion (subject to the decision of CT1) may include service based filtering.  A Relay UE can be considered suitable in terms of radio criteria if the PC5 link quality exceeds configured threshold. The Remote UE selects the Relay UE, which satisfies higher layer criterion and has best PC5 link quality among all suitable Relay UEs.

Proposal 7a: Reuse legacy mechanism as baseline: select the Relay UE which has the best link quality on PC5 and satisfies higher layer criterion for both UE-to-Network relay case and UE-to-UE relay case.

In Rel-13 LTE ProSe UE-to-Network Relay discussion, there was a discussion on who should make the final decision on Relay (re)selection: AS layer or upper layer. The conclusion is no need to specify but to relay on network implementation on the interaction between the layers for Relay (re)selection. Instead, different criterions were specified at different layers. It seems there is no block issue to reuse the same principle here for NR sidelink relay.

Proposal 7b: Reuse legacy mechanism as baseline: for both UE-to-Network relay case and UE-to-UE relay case, AS/upper layer separately specifies the criterion for Relay (re)selection and leave the interaction between the layers to network implementation.

On top of the baseline of the criterion for Relay (re)selection, additional metrics can be considered for NR sidelink relay. It should be noted that the relay UE knows its level of loading relative to the capacity of its resource pool. Then if the relay UE can broadcast his load level to Remote UE. The Relay UE Selection/Reselection can be performed by Remote UE based on a combination by PC5 link signal quality and Relay load level.

From upper layer perspective, we may consider the Relaying QoS capability for Relay UE including its supportable throughput over both Uu and PC5 (restriction may be over Uu) and the latency performance of relay link (i.e. due to relay, additional latency is introduced compared to Uu and relay UE should ensure that the final end-to-end latency should not exceed allowed PDB). Then Remote UE should always select a SL relay UE which can satisfy both throughput requirement and latency requirement. One thing to be considered is if the Remote UE’s service request (in terms of QoS req) is clear enough at the early stage of Relay Discovery. It may be clear when there is Uu link between Remote UE and network via direct path and Remote UE aims to switch the link PC5.

Proposal 7c: Discuss additional metrics for selection/reselection of both UE-to-Network relay and UE-to-UE relay.

## Measurements for Relay Selection/Reselection

The radio measurements at PC5 can be measured based on the DM-RS of PSCCH/PSSCH, as there is no specific physical channel for Sidelink discovery message. The details of physical layer aspects can be coordinated with RAN1. At least RSRP can be measured for purpose of Relay (Re)Selection. The RSRQ based measurement can be also considered.

Sidelink radio measurements is done at physical layer, and then measurement averaging can be performed when the UE is identified in terms of high-layer identity (e.g. Layer 2 Relay UE ID).

Proposal 8: The radio measurements at PC5 for purpose of Relay (Re)Selection can be the RSRP measured on the DM-RS of PSCCH/PSSCH. Coordinate with RAN1/RAN4.

## Triggering for Relay Reselection

As specified for Rel-13 LTE ProSe UE-to-Network Relay, there are two triggers for Remote UE to initiate the Relay reselection. The first one is that the PC5 signal strength of current Relay UE is below the configured signal strength threshold. The second one is that Remote UE receives a layer-2 link release message (upper layer message) from the Relay UE. The first trigger is obvious. The second trigger indicates the link release request sourced from application layer.

We think these two triggering conditions for Rel-13 LTE ProSe UE-to-Network Relay can be both reused for NR sidelink relay (UE-to-Network Relay case and UE-to-UE Relay case).

Proposal 9: Reuse legacy mechanism: Triggering for Relay reselection can be (1) low PC5 signal strength of current Relay UE; (2) link release from current Relay UE. Additional triggering can be discussed further.

# Conclusion

This document promulgated the following proposals:

Observation 1: The discussion on the size limitation on the Relay discovery message needs to be provided by RAN WGs to SA2 but this issue can be left to normative phase.

Proposal 1: The initiation of Relay discovery procedures if required is managed by upper layer (i.e. PC5-S) and transparent to AS layer.

Proposal 2: The protocol stack PC5-S/PDCP/RLC/MAC/PHY is used to transmit PC5-S Relay discovery message over PC5.

Proposal 3: Support AS layer based packet filtering for PC5-S Relay discovery message over PC5 for NR Sidelink Relay.

Proposal 4a: Support Uu link signal quality threshold based approach for remote UE to determine the start of Relay discovery for the case of NR Sidelink UE-to-Network Relay.

Proposal 4b: The Uu link signal quality threshold based approach is not used for remote UE to determine the start of Relay discovery for the case of NR Sidelink UE-to-UE Relay.

Proposal 5a: Specific discovery resource pool is defined for purpose of Relay discovery (for both TX and RX).

Proposal 5b: The transmission of Relay discovery message depends on the availability of transmission resources defined by the configured discovery resource pool.

Proposal 6: Consider the following scenarios for Relay (Re)Selection:

Scenario-A: Remote UE and Relay UE are in cellular network coverage (Relay Selection, UE-to-Network Relay)

Scenario-B: Remote UE is out of coverage and Relay UE is in coverage (Relay Selection, UE-to-Network Relay)

Scenario-C: Both Initiating Remote UE and Peer Remote UE are out of coverage (Relay Selection, UE-to-UE Relay)

Scenario-D: Remote UE is connecting with a Relay UE (Relay Reselection, UE-to-Network Relay and UE-to-UE Relay)

Proposal 7a: Reuse legacy mechanism as baseline: select the Relay UE which has the best link quality on PC5 and satisfies higher layer criterion for both UE-to-Network relay case and UE-to-UE relay case.

Proposal 7b: Reuse legacy mechanism as baseline: for both UE-to-Network relay case and UE-to-UE relay case, AS/upper layer separately specifies the criterion for Relay (re)selection and leave the interaction between the layers to network implementation.

Proposal 7c: Discuss additional metrics for selection/reselection of both UE-to-Network relay and UE-to-UE relay.

Proposal 8: The radio measurements at PC5 for purpose of Relay (Re)Selection can be the RSRP measured on the DM-RS of PSCCH/PSSCH. Coordinate with RAN1/RAN4.

Proposal 9: Reuse legacy mechanism: Triggering for Relay reselection can be (1) low PC5 signal strength of current Relay UE; (2) link release from current Relay UE. Additional triggering can be discussed further.

# References

[1]	3GPP Draft TR 23.752

[2]	3GPP TS36.300

[3]	RP-193253 SID NR sidelink relay

[4]	3GPP TS23.303