3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007890

Online, August 17th - 28th, 2020

Agenda Item:	6.16

Source:	MediaTek Inc.

Title:	(Re)start condition of drx-shortCycleTimer for secondary DRX

Document for:     Discussion and Decision

# Introduction

The CRs of secondary DRX have been approved in RP#88. In this paper, we discuss the (re)start condition of drx-shortCycleTimer.

# Discussion

According to current MAC spec [1], when UE receives a DRX command MAC CE on DRX group 1, both DRX group 1 and DRX group 2 will use short DRX cycle, but only DRX group 1 will (re)started drx-ShortCycle timer. Then, for the UE, it’s unclear what value of drx-ShortCycle timer DRX group 2 should apply for.  To be specific,

- Case 1: If DRX group 2 applies short DRX cycle before UE receives the DRX command MAC CE

  - According to current spec, DRX group 2 continues to use current drx-ShortCycle timer after UE receiving DRX command MAC CE on DRX group 1.

- Case 2: If DRX group 2 applies long DRX cycle before UE receives the DRX command MAC CE (with ambiguity)

  - It’s unclear what value of drx-ShortCycle timer DRX group 2 should apply for after UE receiving DRX command MAC CE on DRX group 1

Observation 1: It's unclear what value of drx-ShortCycle timer a DRX group should apply if this DRX group uses long DRX cycle and DRX command MAC CE is received on the other DRX group.

Proposal 1: RAN2 clarify the operation of drx-ShortCyle timer for one DRX group when UE receives DRX command MAC CE on the other DRX group.

In our understanding, current spec intends to allow the two DRX groups to maintain their own drx-ShortCyle timer separately. Therefore, case 1 works fine and we should address case 2 only. It means when one DRX group receives DRX command MAC CE, if the other DRX group use long DRX cycle, the other DRX group should (re)start its drx-ShortCyle timer; otherwise, the other DRX group continue to use its current value of drx-ShortCyle timer.

Proposal 2: If UE receives DRX command MAC CE on one DRX group, the other DRX group (re)start its drx-ShortCyle timer if it applied long DRX cycle before; otherwise, the other DRX group continues using current value of drx-ShortCyle timer.

## 5.7 Discontinuous Reception (DRX) For each DRX group, the MAC entity shall: ------------<ignore irrelevent part>--------------- 1> if a DRX Command MAC CE is received: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer for this DRX Group in the first symbol after the end of DRX Command MAC CE reception; 3> use the Short DRX Cycle for both DRX groups. 2> else: 3> use the Long DRX cycle for both DRX groups.

# Conclusion

Observation 1: It's unclear what value of drx-ShortCycle timer a DRX group should apply if this DRX group uses long DRX cycle and DRX command MAC CE is received on the other DRX group.

Proposal 1: RAN2 clarify the operation of drx-ShortCyle timer for one DRX group when UE receives DRX command MAC CE on the other DRX group.

Proposal 2: If UE receives DRX command MAC CE on one DRX group, the other DRX group (re)start its drx-ShortCyle timer if it applied long DRX cycle before; otherwise, the other DRX group continues using current value of drx-ShortCyle timer.

# Reference

- TS 38.321 v16.1.0

# Appendix: Exemplary TP

## 5.7 Discontinuous Reception (DRX) ------------<ignore irrelevent part>--------------- For each DRX group, the MAC entity shall: ------------<ignore irrelevent part>--------------- 1> if a DRX Command MAC CE is received: 2> if the Short DRX cycle is configured: 3> start or restart drx-ShortCycleTimer for this DRX Group in the first symbol after the end of DRX Command MAC CE reception; 3> if the long DRX cycle was used for the other DRX group: 4> start or restart drx-ShortCycleTimer for the other DRX Group in the first symbol after the end of DRX Command MAC CE reception; 3> use the Short DRX Cycle for both DRX groups. 2> else: 3> use the Long DRX cycle for both DRX groups.