- 3GPP TSG-RAN WG2#111-e	Draft-R2-2008212

- Electronic meeting, August 17 - 28, 2020

Title:		[Draft] LS Reply on SA WG2 assumptions on architecture aspects for using satellite access in 5G

Reply to:	R2-2006530_S2-2004688

Release:	Release 17

Work Item:	FS_5GSAT_ARCH, NR_NTN_solutions-Core

Source:	Qualcomm Inc. [To be RAN2]

To:		SA2

Cc:		RAN3, CT1

Contact person:	Bharat Shrestha

bshresth@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN2 thanks SA2 for the LS on SA WG2 assumptions on architecture aspects for using satellite access in 5G.

RAN2 discussed the question in the LS and would like to provide the following feedback.

SA2 question:

- 	SA2 has discussed whether cells, as core network sees them, are earth-moving or earth-fixed. Currently in terrestrial networks a Cell ID determined and provided by RAN is used to represent UE location in different services and systems (e.g. to route emergency calls to a suitable PSAP). For satellite coverage e.g. with moving beams, would the Cell ID received by CN still correspond to an earth-fixed area?

-	In case of earth-fixed cells (as seen from the CN), how the RAN node connected to 5GCN (e.g. CU or gNB) maps the radio component of the cell that can potentially have moving coverage (e.g. in case of NGSO satellite) to earth-fixed cells represented by CGI would be up to RAN WGs to decide.

RAN2 feedback:

- 	RAN2 has identified that radio cells for NTN can be permanently fixed, temporarily fixed or continuously moving. Existing cell ID of a moving radio cell corresponds to different earth area over time. RAN2 thinks any potential solution to mapping cell ID to earth areapotential  is in the scope of RAN3. RAN2 assumes RAN3 will discuss the issue of mapping radio cell to earth-fixed cell to be seen by CN.

# 2	Actions

To SA2:

ACTION: 	RAN2 respectfully requests SA2 to take the above information into account and provide feedback if any.

# 3	Dates of next TSG RAN WG2 meetings

TSG-RAN WG2#112e			Nov 2 – 13, 2020	Online meeting