3GPP TSG RAN WG2 Meeting #111-e                                     R2-2008231

E-Meeting, August 17th – 27th, 2020

Title:	[Draft] Reply LS on early UE capability retrieval for eMTC

Reply to:	Reply LS on early UE capability retrieval for eMTC (S2-2004446/R2-2006166)

Release:	Release 16

Work Item:	LTE_eMTC5-core

Source:	Qualcomm Inc [To be RAN2]

To:	SA2

CC:                          RAN3, CT1

Contact person:	Prasad Kadiri

pkadiri@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

# 1	Overall description

RAN2 thanks SA2 for their reply LS confirming that SA2 has not identified any technical issues to support early UE capability retrieval for eMTC UEs for EPS from system architecture perspective.

RAN2 discussed the concerns raised by SA2 for 5GS and would like to provide the responses as follows:

SA2 LS indicated:

- Concerns to enable use of truncated 5G-S-TMSI for all UEs accessing ng-eNBs connected to 5GC as this would reduce the available AMF Set ID number space, AMF Pointer number space and TMSI number space (due to truncation) for all UEs.

RAN2 response:

RAN2 has a potential solution to identify in Msg3 to include InitialUE-Identity-5GC which enables ng-eNB to differentiate between BL UEs and non-BL UEs in CE mode.

RAN2 thinks the other two concerns are out of RAN2 scope and up to SA2 to address.

Therefore, RAN2 respectfully asks SA2 to take above response into consideration.

# 2	Actions

ACTION to SA2:

RAN2 respectfully asks SA2 to take above response into consideration.

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #112e			 Nov 2020, E-Meeting

TSG RAN WG2 Meeting #113			 March 2021