3GPP TSG RAN WG2#111-e	R2-2006509

Online meeting, 17th - 28th August, 2020

3GPP TSG RAN WG1 Meeting #101-e	R1-2005078

e-Meeting, May 25th – June 5th, 2020

Title:	LS on Intra-UE Prioritization for data with different priorities

Release:	Release 16

Work Item:	NR_IIOT-Core

Source:	TSG RAN WG1

To:	TSG RAN WG2

Cc:

Contact Person:

#### Name:	Lihui Wang

E-mail Address:	wanglihui@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN1 discussed intra-UE prioritization for data with different priorities and achieved following agreement and conclusion:

Agreement

- For collision handling between CG and CG with different priorities,

  - If MAC delivers two MAC PDUs, it is up to UE implementation to make sure that the low priority CG PUSCH transmission can be cancelled before the start of the high priority CG PUSCH.

Conclusion

There is no consensus in RAN1 for the support of the following:

- high priority DG cancel the transmission of low priority CG in the physical layer

- high priority CG cancel the transmission of low priority DG in the physical layer

No further discussion for Rel.16.

2. Actions:

To RAN2 group.

ACTION: 	RAN1 respectfully asks RAN2 to take above information into account. It is up to RAN2 whether to design the solutions that can accommodate current PHY behaviour to support the intra-UE prioritization for DG and CG with different priorities (one possible solution would be that MAC layer only provides one MAC PDU with the high priority to PHY layer).

3. Date of Next TSG-RAN WG1 Meetings:

3GPPRAN1#102-e		17 Aug - 28 Aug 2020	e-Meeting

3GPPRAN1#103-e		26 Oct – 14 Nov 2020	e-Meeting