3GPP TSG RAN WG2#111-e	R2-2008615

Online meeting, 17th - 28th August, 2020

3GPP TSG RAN WG1 Meeting #102-e	R1-2007197

e-Meeting, August 17th – 28th, 2020.

# Title:	Reply LS on remaining eMIMO issues

# Response to:	LS on detail MIMO MAC CE operations (R2-2005811)

# Release:	Rel-16

# Work Item:	NR_eMIMO-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Jinhyun Park

E-mail Address:		jhyun85.park@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments: 	-

1. Overall Description:

RAN1 would like to provide replies on the following questions from RAN2 in R2-2005811:

Question 1: Could RAN1 specify how the TCI state codepoints are mapped when the number of codepoints in the TCI field of DCI format 1_2 is less than the maximum number of codepoints in the TCI field of DCI format 1_1 taking into account that the MAC CE may signal TCI states corresponding to maximum amount of DCI codepoints.?

Answer to Q1: RAN1 will specify the mapping for TCI codepoints of DCI format 1_2, of which details are under discussion.

Question 2: Is RAN2 understanding correct that the intended functionality of the Serving cell set based SRS spatial relation indication MAC CE is to only indicate the SRS spatial relations for SRS resource(s) which are applied for all configured serving cell set? (i.e. no need to activate/deactivate transmission of SRS resources(s) for all configured cells in the serving cell set)

Answer to Q2. RAN1 has the same understanding with RAN2.

Question 3: Is RAN2 understanding correct that the Serving cell set based SRS spatial relation indication MAC CE can be used to indicate the SRS spatial relations in SUL configuration?

Answer to Q3. From RAN1 perspective, spatial relation is only applicable to FR2 irrespective of NUL or SUL. Currently no SUL carriers are defined for FR2 by RAN4. Therefore, the network cannot at this point configure this feature for SUL.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG-RAN WG1 Meeting #103-e 	October 26 – November 13, 2020	e-meeting

TSG-RAN WG1 Meeting #104 	March 1 – 5, 2021			Athens, Greece