- 3GPP TSG RAN WG2 Meeting #112-e      	                           R2-200xxxx
E-Conference, 2nd – 13th Nov. 2020

Agenda item:	5.4.1.1

Source:	Qualcomm Incorporated

Title:	Email discussion summary of [005][NR15] RRC Conn Control I

WID/SID:	NR_newRAT-Core – Release 15

Document for:	Discussion and Decision

# Introduction

It is summary for phase 2 of below email discussion:

- [AT112-e][005][NR15] RRC Conn Control I (Qualcomm)

Treat R2-2008715, R2-2009183, R2-2009184, R2-2009185, R2-2010563, R2-2010665, R2-2010666, R2-2009355, R2-2009356, R2-2009844, R2-2009845, R2-2010530, R2-2010531, R2-2010557, R2-2010558

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

Summary of phase 1 discussion can be found in [1]. In Phase-2 discussion, we discuss the remaining issues list in summary proposals:

- For Rel-15/Rel-16 CRs on HARQ-ACK spatial bundling and CBG

Proposal 5: Continue to discuss whether to pursue the spec change “UE cannot be configured with both spatial bundling and codeBlockGroupTransmission within the same cell group” in phase 2. Opponent can show the specific RAN1 spec to revert it.

- For Rel-15/Rel-16 CRs on p-Max in FR2

Proposal 5: Pursue Rel-15/Rel-16 CR R2-2010530/R2-2010531 with down-selection between the following two ways in phase 2:

- Alt-1: “The Network does not configure p-Max for a carrier frequency in FR2”

- Alt-2: “if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]”.

- For Rel-15/Rel-16 CRs on SCells for overheating Mitigation

Proposal 8: Continue to discuss whether and how to clarify that the Network may also deactivate the active downlink / uplink SCells to alleviate the overheating upon reception of UAI.

For rapporteur to prepare report, companies are requested to provide input by Tuesday 2020-11-10 13:00 UTC.

# Discussion

## HARQ-ACK spatial bundling and CBG

According to phase 1 discussion, the controversial parts:

- Whether 38.213 has covered the change?

- If the change is agreed to be captured in spec, whether it is applied to Rel-16

Rapporteur suggest to agree the Apple’s suggestion as baseline to continue discuss in phase 2 (i.e. “UE cannot be configured with both spatial bundling and codeBlockGroupTransmission within the same cell group”). Opponent can show the RAN1 spec part in phase 2 to revert this suggestion.

Rapporteur checked 38.213, and list related part below:

Type-1 codebook (section 9.1.2.1)

A UE determines o0ACK,  {o1ACK,...,oOACK−1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a HARQ-ACK codebook for transmission in a PUCCH according to the following pseudo-code. In the following pseudo-code, if the UE does not receive a transport block or a CBG, due to the UE not detecting a corresponding DCI format 1_0 or DCI format 1_1, the UE generates a NACK value for the transport block or the CBG. The cardinality of the set MA,c defines a total number Mc of occasions for PDSCH reception or SPS PDSCH release for serving cell c corresponding to the HARQ-ACK information bits.

Set c=0 – serving cell index: lower indexes correspond to lower RRC indexes of corresponding cell

Set j=0- HARQ-ACK information bit index

Set NcellsDL to the number of serving cells configured by higher layers for the UE

while c<NcellsDL

omit

elseif harq-ACK-SpatialBundlingPUCCH is provided, and the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c,

ojACK = binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks of this cell - if the UE receives one transport block, the UE assumes ACK for the second transport block;

j=j+1;

elseif PDSCH-CodeBlockGroupTransmission is provided, and NHARQ−ACK,cCBG/TB,max CBGs are indicated by maxCodeBlockGroupsPerTransportBlock for serving cell c,

Set nCBG=0- CBG index

while nCBG<NHARQ−ACK,cCBG/TB,max

oj+nCBGACK = HARQ-ACK information bit corresponding to CBG nCBG of the first transport block;

if the UE is configured by maxNrofCodeWordsScheduledByDCI with reception of two transport blocks for the active DL BWP of serving cell c

oj+nCBG+NHARQ−ACK,cCBG/TB,maxACK = HARQ-ACK information bit corresponding to CBG nCBG of the second transport block;

end if

nCBG=nCBG+1;

end while

j=j+NTB,cDL⋅NNARQ−ACK,cCBG/TBmax, where NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for the active DL BWP of serving cell c;

omit

end while

Thus, it seems TS 38.213 indeed has specified the UE behavior when both spatial bundling and codeBlockGroupTransmission are configured for Type-1 codebook, i.e. when both harq-ACK-SpatialBundlingPUCCH and PDSCH-CodeBlockGroupTransmission are configured, spec enters the highlighted part which seems to state the UE procedure looks following spatial bundling.

Observation 1: TS 38.213 seems to have specified the UE behaviour when both spatial bundling and codeBlockGroupTransmission are configured for Type-1 codebook.

Type-2 codebook (section 9.1.3.1)

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for NcellsDL,CBG serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,CBG=NcellsDL

the UE determines the o0ACK,  {o1ACK,...,oOACK−1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for SPS PDSCH release, SPS PDSCH reception, and for TB-based PDSCH receptions scheduled by DCI formats 1_0 on the NcellsDL,CBG serving cells and by DCI formats 1_0 and DCI formats 1_1 on the NcellsDL,TB serving cells

-	NcellsDL is replaced by NcellsDL,CBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,CBG serving cells for CBG-based PDSCH receptions scheduled by DCI format 1_1, and

-	Instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,CBG serving cells, the UE generates NHARQ−ACK,maxCBG/TB,max HARQ-ACK information bits, where NHARQ−ACK,maxCBG/TB,max is the maximum value of NTB,cDL⋅NHARQ−ACK,cCBG/TB,max across all NcellsDL,CBG serving cells and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c it is NTB,cDL⋅NHARQ−ACK,cCBG/TB,max<NHARQ−ACK,maxCBG/TB,max, the UE generates NACK for the last NHARQ−ACK,maxCBG/TB,max−NTB,cDL⋅NHARQ−ACK,cCBG/TB,max HARQ-ACK information bits for serving cell c

-	The pseudo-code operation when harq-ACK-SpatialBundlingPUCCH is provided is not applicable

Thus, TS 38.213 seems to have specified the both configuration of spatial bundling and codeBlockGroupTransmission is not applicable for Type-2 codebook.

Observation 2: TS 38.213 seems to have specified the both configuration of spatial bundling and codeBlockGroupTransmission is not applicable for Type-2 codebook.

Then based on Observation 1 and Observation 2, it is Rapporteur’s understanding that it seems to be already covered by TS 38.213. Thus, Rapporteur suggest not to pursue this CR

Q1: Do you agree not to pursue the clarification on HARQ-ACK spatial bundling and CBG (R2-2009184/R2-2009185)? considering Observation 1 and Observation 2:

- Observation 1: TS 38.213 seems to have specified the UE behaviour when both spatial bundling and codeBlockGroupTransmission are configured for Type-1 codebook.

- Observation 2: TS 38.213 seems to have specified the both configuration of spatial bundling and codeBlockGroupTransmission is not applicable for Type-2 codebook.

| Company | Yes/No | Comments (If you answer “No”, please indicate which parts TS 38.213 has not covered) |
| --- | --- | --- |
| CATT | No | Indeed we agree the key point is now to confirm that in 213 the R1 spec the case when spatial bundling and codeBlockGroupTransmission are both configured is not clearly specified. Taking observation 1 as an example => The part of 213 spec cited above does not specify the case when CBG and spatial bundling are configured at the same time. For example, in 9.1.2.1 you have two “elseif”, which does not work for the case when both are configured. So observation 1 is not correct in our understanding and we would encourage companies to check again. |

## p-Max in FR2

Rapporteur think both options (NW doesn’t configure vs UE ignore) were well discussed in phase-1 discussion. And majority prefer to clarify from NW side:

- 6 companies prefer to clarify from NW side: Apple, Huawei, Nokia, MediaTek, Samsung, LG

- Even Proponent (DCM) also said “We are fine to remove the second sentence.”

As Rapporteur, we are also confused how Network can configure p-Max in FR2 given RAN4 doesn’t define any requirement. Since two companies requested to reconsider this case with below justification:

- For system information it is specified that the UE shall ignore the IE in this release when configured:

“If absent the UE applies the maximum power according to TS 38.101-1 [15] in case of an FR1 cell or TS 38.101-2 [39] in case of an FR2 cell. In this release of the specification, if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]”.

- We assume this is with the intention that p-Max for FR2 may be specified in later releases. We have preference the UE ignores the field when configured. Otherwise the NW would be required to take the release of the UE into account when p-max for FR2 is added in a later release.

Rapporteur think all companies agree to clarify in spec but have difference preference on wording. Considering which wording results in similar behaviour, Rapporteur would suggest to decide based on majority view in phase-2:

Q2: On clarification on p-Max in FR2, which wording do you prefer to capture in spec?

- Alt-1: “The Network does not configure p-Max for a carrier frequency in FR2”

- Alt-2: “if p-Max is present on a carrier frequency in FR2, the UE shall ignore the field and applies the maximum power according to TS 38.101-2 [39]”.

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT |  | we are fine with either way if there is majority’s support. |

## Deactivation of SCells for Overheating Mitigation

During phase-1 discussion, 7 companies disagree the change while 3 companies (Qualcomm, Apple and Samsung) support it. However, after reviewing company input, Rapporteur think it seems to be common understanding that upon reception of UAI, the Network may deactivate the active downlink / uplink SCells to alleviate the overheating, besides deconfiguration. It seems the confusion results from the CR drafted in NBC manner, but it is not Proponent’s intention. Thus, Rapporteur suggest to continue to discuss whether and how to clarify it in phase 2.

To make progress, Rapporteur would like to ask 3 more questions:

Q3: Do you agree that it’s up to the network implementation when to consider the overheating UAI provided by the UE?

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes | It is network implementation. |

Q4: Do you agree, if network decided to consider the overheating UAI, it can either de-configure or deactivate an active CC (or CCs)?

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes | It is network implementation. It may or may not base on ue report. |

Q5: If you agree Q4, do you agree it is captured in spec?

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | No | Current spec seems ok, nothing broken. |

# Contact Information

| Company | Email |
| --- | --- |

# Conclusion

# References

[1] R2-200xxxx, Phase-1 email discussion summary of [005][NR15] RRC Conn Control I