3GPP TSG RAN WG2 #112-e	draft R2-20xxxxx

Online, 02 – 13 Nov 2020

Source: 	Huawei, HiSilicon

Title: 	[draft] Summary for Offline [025][R4 NR16] CSI-RS for Mobility

Agenda Item:	6.15

Document for:	Discussion and decision

# Introduction

This document is a summary of the following offline discussion:

- [AT112-e][025][R4 NR16] CSI-RS for Mobility (Huawei)

Treat R2-2008749, R2-2010585, R2-2010586, R2-2009775, R2-2009776, R2-2009777, R2-2009365,

Intended outcome: Determine agreeable parts. For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

The discussion is triggered by an LS from RAN4 (R2-2008749).

# Discussion

RAN1 has already discussed this issue in the first week of RAN1 #103-e and reached the following agreements:

- Agreements: - Ask RAN2/4 to introduce a new UE capability for the increased maximum number of configurable CSI-RS resources per MO for L3 mobility in the reply LS – lated draft LS (R1-2009369) is approved, with final LS in R1-2009444 – Daewon (Intel) - RAN1 assumes following design for the new UE capability signaling - Name: increasedNumberofCSIRSPerMO-r16 - Component description: Indicates support of up to 192 CSI-RS resource for L3 mobility configuration per measurement object configured with associatedSSB - Prerequisite FG: 1-4 - Reporting type: Per UE - TDD/FDD differentiation: No - FR1/FR2 differentiation: Yes - Mandatory/optional: Optional with capability signaling - RAN1 believes up to 96 CSI-RS resource (for L3 mobility) configuration per cell and up to 192 CSI-RS resource configuration per MO (as per RAN4 request) should satisfy request from RAN4. The draft 38.214 CR is approved, with final CR in R1-2009448 (38.214, CR0133) Summary in R1-2009331

According to the agreed CR (R1-2009448), the related texts in 38.214 clause 5.1.6.1.3 have been updated to:

A UE configured with the higher layer parameters CSI-RS-Resource-Mobility may expect to be configured - with no more than 96 CSI-RS resources per higher layer parameter MeasObjectNR for UEs not supporting [increasedNumberofCSIRSPerMO-r16] when all CSI-RS resources configured by the same higher layer parameter MeasObjectNR have been configured with associatedSSB, or, - with no more than 192 CSI-RS resources per higher layer parameter MeasObjectNR for UEs supporting [increasedNumberofCSIRSPerMO-r16] when all CSI-RS resources configured by the same higher layer parameter MeasObjectNR have been configured with associatedSSB, or, - with no more than 64 CSI-RS resources per higher layer parameter MeasObjectNR when all CSI-RS resources have been configured without associatedSSB or when only some of the CSI-RS resources have been configured with associatedSSB by the same higher layer parameter MeasObjectNR - For frequency range 1 the associatedSSB is optionally present for each CSI-RS resource - For frequency range 2 the associatedSSB is either present for all configured CSI-RS resources or not present for any configured CSI-RS resource per higher layer parameter MeasObjectNR.

RAN2 needs to figure out the impacts on RAN2 spec based on RAN4 LS and RAN1 agreements.

Q1: Do you agree to add a UE capability as per RAN1 agreements?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | Yes |  |

  - RAN1 believes up to 96 CSI-RS resource (for L3 mobility) configuration per cell and up to 192 CSI-RS resource configuration per MO (as per RAN4 request) should satisfy request from RAN4.

Currently, RAN2 spec already allows configuring a maximum of 96 CSI-RS resources per cell and at most 96 cells per MO, therefore the RAN4 request has been satisfied.

Q2: Do you agree that there’s no need to modify the RAN2 signalling related to CSI-RS resources (for L3 mobility) configuration and reporting?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | Yes |  |

Besides, it is worth noting that the current field description of csi-rs-ResourceList-Mobility refers to the RAN1 spec. However, when consulting the RAN1 spec, one may open 38.214 v16.3.0 (2020-09), i.e. not including the latest revisions.

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB (see TS 38.214 [19], clause 5.1.6.1.3).

To make sure the UE capability is considered, we propose to also add the reference to the new UE capability.

CSI-RS-CellMobility field descriptions

csi-rs-ResourceList-Mobility List of CSI-RS resources for mobility. The maximum number of CSI-RS resources that can be configured per measObjectNR depends on the configuration of associatedSSB (see TS 38.214 [19], clause 5.1.6.1.3) and on the support of increasedNumberofCSIRSPerMO (see TS 38.306 [26], clause 4.2.9).

Q3: Do you agree with the above change?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | Yes |  |

R2-2009775 points out that the value range of UE capability maxNumberCSI-RS-RRM-RS-SINR should also be extended to support 192:

maxNumberCSI-RS-RRM-RS-SINR             ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL

As per the definition in TS 38.306, this capability concerns the number of CSI-RSs per single slot:

maxNumberCSI-RS-RRM-RS-SINR

Defines the maximum number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot. If UE supports any of csi-RSRP-AndRSRQ-MeasWithSSB, csi-RSRP-AndRSRQ-MeasWithoutSSB, and csi-SINR-Meas, UE shall report this capability.

Q4: Do you think the capability maxNumber-CSI-RRM-RS-SINR should support a value of 192?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | No | Since this capability is per slot, it is not related to the RAN4 LS. We don’t see the need to extend this, unless there is some UE vendor eager to implement this. Another concern is, if the UE is implemented according to the change but the network is not, and the UE reports 192 for this capability, the network cannot decode this UE capability. |

Q5: Do you think RAN2 need to send a reply LS?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Huawei | No | We think RAN4 has done their job. They can refer to the latest RAN2/RAN1 spec if they are interested in the detailed design. |

Q6: For any other change related to the RAN4 LS that is not mentioned by the rapporteur but considered necessary by you, please list it here.

| Company | Comments |
| --- | --- |

# Conclusion

To be added.