3GPP TSG-RAN WG2 #112-e	R2-20xxxxx

E-meeting, 02 – 13 November 2020

Agenda item:	8.4.2

Source: 	Samsung (rapporteur)

Title: 	Discussion summary - [AT112-e][030][eIAB]

Document for:	Discussion and decision

# Introduction

This tdoc contains a collection of proposed solutions for various issues identified in R2-2009073 and in the company submissions to AI 8.4.2 of RAN2#112-e.

More specifically, this document is produced to capture the following discussion:

- [AT112-e][030][eIAB] Fairness Latency Congestion (Samsung)

Scope: A) Confirm easy agreeable proposals captured in R2-2009073 (short deadline), make modifications to the proposals if needed for final agreement.

B) From R2-2009073 and input contributions below put applicable solution proposals on the table, with a short principal solution description, how the solution is intended to help and possibly comments on complexity, if applicable. In case there are many solutions, initial focus could be on promising and widely proposed/supported solutions. Further discussion and decision making is expected on-line week 2.

Intended outcome: Report

Deadline: Ready Nov 11 (for on-line discussion Nov 11), Intermediate deadlines by Rapporteur.

This document focuses on the yellow-highlighted bit (i.e. aspects under B)); the discussion under A) will be carried out separately.

# Phase-1: collecting details on proposals

## Fairness

Based on various issues identified in R2-2009073, and the company submissions to AI 8.4.2 of RAN2#112-e, main solutions to ensuring fairness focused on enhancements to IAB node decision-making and were centred around the following 3 pillars:

- Share with IAB nodes the number of bearers in a channel

- Include bearer ID in the BAP header

- Provide enhanced scheduling assistance info/cost function

The companies are invited to fill in the Table below, by providing their own solutions, which can be based on above. Enough detail needs to be provided so that in Phase-2 we can assess the solutions.

If an existing entry already captures your solution, or contains as-is a solution you support, you can just add your company name (in brackets) in that row. If you think your solution is a variant of an existing entry, you may wish to number your entry to show the link (e.g. existing solution is no.3  your solution becomes no.3a).

Table 1 – focus on fairness

| Solution ID | Company | Short solution description | How does this solution address fairness? | Comments on complexity |
| --- | --- | --- | --- | --- |
| 1 | LG, Apple, Ericsson, Samsung, Sony, Futurewei, Kyocera | The IAB-donor CU can configure/allocate LCH priority differently depending on the location of each UE. For this, LCH priority can be increased to support finer topology-wide fairness. | The IAB-donor CU can figure out how to satisfy QoS for each UE’s service all over the path. To be specific, even though UE1 (one hop distance), UE2 (two hops distance), and UE3 (three hops distance) want to receive same service, i.e., same QoS, the QoS is related to LCH (logical channel) priority on each hop and the IAB-donor CU can configure/allocate LCH priority differently depending on the location of each UE. | Complexity is low because this solution uses the current configuration as much as possible. Increasing LCH priority would be straightforward and specification impact is marginal. |
| 2 | AT&T, Apple | Allow exchange of BH scheduling benefit metric directly between child and parent nodes via BAP which can be used to provide downstream radio conditions to the schedulers of higher-up IAB nodes or donor nodes | The scheduler at the parent-IAB node may not have all the information needed to make appropriate scheduling decisions for different bearers/RLC channels across multiple child-IAB nodes (especially the closer to the donor the parent node is). A benefit metric from child-IAB nodes to parent-IAB nodes can allow the parent-IAB node to provide appropriate weighting of bearers as well (e.g. for a PF scheduler). In addition, such a benefit metric could incorporate link quality across multiple hops for a given route. | Complexity would be roughly analogous to downstream RLF indication from Rel-16. Need to define BAP control PDUs with one or more fields which allow the introduction of a BH scheduling benefit metric. |
| 3 | Intel, Samsung, Kyocera | QoS-based load balancing for balancing traffic load among BH RLC channels holding UE bearers with similar QoS profile. | It can help to solve unfairness caused by unbalanced workload among BH RLC channels. Fairness between different BH RLC channels with similar QoS requirement can be also beneficial to reduce congestion and reach similar latency for UEs with same QoS requirement | Low complexity, Mobility load balancing for SON can be used as baseline, only need to modify the load related information report to donor CU per BH RLC channel, where load related information has been defined in TS38.300 for SON load balancing. |
| 3a | vivo | For DL, the fairness issue is left to implementation. For UL, the CU can simply reconfigure prioritisedBitRate of the BH RLC channel accordingly when the IAB-donor-CU multiplexes a DRB onto or removes a DRB from the BH RLC channel. | CU is aware of the number of DRBs and the required QoS of each BH RLC channel, it should be feasible for CU to reconfigure the prioritisedBitRate of the BH RLC channel to meet the fairness requirements, whenever the IAB-donor-CU multiplexes a DRB onto or removes a DRB from the BH RLC channel. | Specification impact is minor. Only needs to enable the reconfiguration of prioritisedBitRate once the status of the BH RLC channel has been changed. |
| 4 | Intel, ETRI, Apple, Samsung | Include bearer ID and hop count of the routing path in the BAP header | It can provide latency reference to intermediate IAB node, in order to take the UE bearer with higher hop number into higher priority for fairness scheduling. Hence bring end user same QoS experience regardless of the hop number. | Low complexity, introduce a new BAP header format |
| 5 | Intel | Introduce a fairness enforcer between IAB node DU and the MT | Fairness scheduling depends on LCH priority, it is also possible multiple UE bearers map to the same LCH. Fairness enforcer before the DU submits data it has received to the MT, can help to reduce impact of unfair data volume and hop number to fairness scheduling | IAB DU need a new function entity as fairness enforcer. |
| 6 | Qualcomm | Configure on the IAB nodes the number of bearers in the BH RLC CH. This number could be N=1 to indicate 1-to-1 bearer mapping. It is up to the CU if this number represents a time-average, e.g. to reduce signaling updates to frequent changes of bearers. | BH RLC CHs may aggregate more than one bearer as opposed to access RLC CHs. BH RLC CHs may further aggregate different numbers of bearers. Per QoS/priority, those BH RLC CHs that aggregate more bearers should obtain more resources to ensure fairness. This number of aggregated bearers should be known to the scheduler. | Minimal. The CU configures a value related to the actual or average number of bearers per BH RLC CH. |
| 7 | ZTE, Samsung | keep IAB node informed of the QoS information of QoS flows/UE DRBs mapped to the BH RLC channel. | Once IAB-DU know the QoS info of QoS flows/DRBs aggregated to this BH RLC channel, IAB-DU could allocates more radio resources for child IAB-MT which has BH RLC channel aggregated with multiple QoS flows/DRBs. | In current F1AP specification, the QoS information of both QoS flows and UE DRB is delivered from CU to DU. We can reuse this design for BH RLC channel. |
| 4a | ZTE | Include UE bearer ID in the BAP header | IAB node may determine which UE DRB the data packet belongs. Then IAB node may schedule the data packets of BH RLC channel taking into account the QoS profile of each UE DRB. | Introduce new BAP header format |
| 1a/2a/4b or 8 [whichever is simpler for rapporteur to adopt] | Apple, Samsung | Include per hop latency and per hop packet loss at each intermediate IAB-DU which is to be provided to the CU. | Adjust Requested 5QI PDB and requested 5QI loss rate (mechanism FFS) to reflect the True PDB and True loss rate due to multi-hop introduced latency and loss rates so that flows with the lowest residual PDB can be prioritized. Perform packet discard if residual PDB <= 0. | Introduce new BAP header format. Re-adjusting of PDB to reflect residual PDB should be done in a way to not cause additional delays. |
| 9 | Fujitsu | Different level of fairness can be enforced by introducing a new logical channel variable (similar to Bj) for LCP procedure enhancement at IAB-MT. It may be based on one of the two solutions provided by rapporteur: - Share with IAB nodes the number of bearers in a channel - Include bearer ID in the BAP header | When N:1 mapping is used, it allows IAB-MT to do UE bearer level or BH RLC channel level buffering and prioritization, thus provide UE bearer level or BH RLC channel level fairness, respectively. | Low impact on the LCP procedure. Please refer to R2-2009006 for details. |
| 10 | Ericsson | Include on the BAP header information to retrieve the amount of UE DRBs conveyed in a certain BAP packet. | The IAB node can determine the amount of UE DRBs to be served at any given time, on the basis of the information included in the BAP header of the various packets buffered at the IAB node and that are waiting to be scheduled. In this way, the IAB node can properly weight the various BH RLC channel depending on the BAP packets available for transmission. | Extend the BAP header to carry information related to the UE DRBs conveyed in a packet. |
| 11 | Futurewei | IAB donor CU-UP derives QoS performance for flow based on information reported by access IAB node (e.g. using NR User Plane Protocol). IAB donor achieved QoS of flow, and provides feedback to IAB nodes that process the QoS flow to assist scheduler (e.g. increase/decrease flow priority, throughput, or latency) | The IAB donor evaluates achieved performance of QoS flow, and fairness among different flows. IAB donor provides feedback to IAB node along routing path of flow to increase of decrease resources to flow in order to achieve desired QoS. IAB nodes do not need to understand specific about network topology, such as number of hops, etc. | Leverage existing capabilities of NR User Plane Protocol to evaluate achieved QoS per flow. Enhancements are not precluded. Feedback provided to IAB nodes via enhancements to NR UP protocol , CP enhancements, and/or extension to BAP header |

## Latency

Based on various issues identified in R2-2009073, and the company submissions to AI 8.4.2 of RAN2#112-e, main solutions to ensuring fairness focused on the following issues and related solutions:

- Enhancements to IAB node decision-making

  - Providing IAB node with topology info for incoming packets (e.g. number of remaining hops)

  - Providing IAB node with QoS info (e.g. CU shares info; BAP header contains QoS info)

  - Include bearer ID in the BAP header

  - Support packet discard at intermediate nodes due to PDB restrictions

  - Enhancements to pre-BSR

  - Increasing the number of LCGs on BH links

- Allowing local re-routing for purposes other than RLF

- Enhancements to CU configuration capabilities

  - Providing Donor-CU with info on e.g. link status (RLF/congestion)

The companies are invited to fill in the Table below, by providing their own solutions, which can be based on above. Enough detail needs to be provided so that in Phase-2 we can assess the solutions.

If a previous entry already captures your solution, you can just add your company name in that row. If you think your solution is a variant of an existing entry, you may wish to number your entry to show the link (e.g. existing solution is no.3  your solution becomes no.3a).

Table 2 – focus on latency

| Solution ID | Company | Short solution description | How does this solution address latency? | Comments on complexity |
| --- | --- | --- | --- | --- |
| 1 | CATT, Fujitsu, Sony | IAB-node is provided with topology information by donor CU, e.g., the number of remaining hops in the upstream or downstream. | Topology-wide fairness requires that UE experience the same QoS for the same service regardless of the number of hops between the UE and the Donor CU. UE with more hops should be prior scheduled | Specification impact is minor. The information of remaining hops can be configured by donor CU. The details can be further studied. |
| 1a | Intel, ETRI, Apple, Samsung, Kyocera | Include hop count in the BAP header | Only remaining hop number known at intermediate IAB node is not enough. Other than the issue mentioned in solution 1, for upstream, it is also possible that different UE has same remaining hop number, but total hop number is different. In this case, only consider the remaining hop may not be useful. The UE with higher total hop number should be scheduled with priority due to long latency it has experienced. | Low Complexity. New BAP header format is required. |
| 2 | CATT, ETRI, Samsung | Donor CU provides the one-hop PDB to IAB-node. IAB-node can discard the expired packets due to the provided one-hop PDB limitation. | Discarding the expired packets at intermediate IAB-node can improve the radio resource efficiency. It also has benefits for congestion mitigation | Specification impact is minor. The one-hop PDB to IAB-node is provided by donor CU. |
| 2a | Vivo, Samsung, Apple | The remaining PDB of the packet is indicated to each IAB-node | The IAB-node is able to prioritize the packet that is about to run out of the PDB, and accordingly shorten the latency. | Might impact the format of the current BAP PDU, as the BAP header seems to be the most appropriate role for carrying the information. |
| 2b | Ericsson, Samsung, Apple | Include on the BAP header information related to the “remaining PDB” at the IAB node of the previous hop. | The IAB node becomes aware of the PDB left by the scheduler of the IAB node of the previous hop. Hence, such IAB node can adjust its own scheduling decisions on the basis of the PDB left at the previous hop. An IAB node can combine this information received from the previous hop with the PDB configured by the CU for the BH RLC channel and for the BAP destination of the packet (as per solution ID 11). In this way, the IAB node can figure out the total PDB to be fulfilled for a given packet associated to a certain BH RLC channel and intended to a certain BAP destination, by taking into account both the CU configuration (as per solution ID 11) and the PDB left from the previous hop. | Extend the BAP header to carry information related to the PDB left at the previous hop. |
| 3 | CATT, vivo, Fujitsu, Ericsson, Huawei, Kyocera | The number of LCG for IAB-MT can be increased. The detail of LCG number can be further studied. | If the number of LCG is increased for IAB-MT, the scheduling in parent nodes will be more refined based on extension LCGs. | It should be specified new BSR and pre-emptive BSR formats regarding to extension LCGs. |
| 4 | LG, Samsung, Huawei, Kyocera | Buffer size calculation for Pre-emptive BSR is specified. | Buffer size calculation for pre-emptive BSR is left to implementation in Rel-16. However, since there is no aligned principle for buffer size calculation, some IAB nodes may report the pre-emptive BSR with a larger buffer size information than the expected data volume to be arrived, in order to reduce UL transmission latency and enhance UL transmission efficiency | Complexity and specification impact would be low. |
| 5 | Qualcomm | Donor CU to provide one-hop PDB also for access link (between access IAB-DU and UE) to IAB-node per access DRB. This is necessary since the total PDB needs to be sliced up into individual-hop PDBs. - Note: PDB for BH RLC CH is already defined per hop in F1-AP. - Note that determination of one-hop PDB is subject to donor CU’s implementation, e.g. depending on hop counts and/or radio quality etc., to achieve various objectives including fairness. | In Rel16, for access link, end-to-end PDB (between UPF and UE) and CN_PDB (between UPF and donor CU) are provided to an IAB-node per access DRB, which is not sufficient for the IAB-node to determine the one-hop latency bound at the access link, because - latency between donor CU and IAB-node-DU, including possible multiple hops in between, is unknown to the IAB-node. One-hop PDB is subject to dynamic change due to topology adaptation. | Very low. It may be sufficient to add description of problem and reinterpretation of PDB value provided to access RLC channel. |
| 6 | Qualcomm, ZTE | Donor CU provides additional Discard PDB with a more relaxed value than one-hop PDB to intermediate and access IAB-nodes for discard of expired packets. - Note one-hop PDB can be used for scheduling priority among channels with different PDB, - E.g. Donor CU may set: One-hop PDB= total_wireless_PDB/hop_count; while discard PDB= total_wireless_PDB. | Discarding the expired packets at intermediate and access IAB-nodes can improve the radio resource efficiency and relieve congestion. Separate PDBs shall be provides for an IAB-node, one for discarding expired packets, and one for determination of scheduling priority, because the latter one with a more strict value can be done in a best-effort manner. | Low complexity because an additional discard PDB is provided to IAB-node per BH RLC in F1-AP. Whether discard PDB for access link shall reuse (PDB-CN_PDB) or be provided with new signaling can be further discussed. |
| 7 | Qualcomm, Apple | Clarify definition of PDB provided for BH_RLC channel in F1-AP: - Option 1:. latency between BAP layers of gNB-DU and child IAB-MT. Option 2: latency over the airlink between gNB-DU and IAB-MT. | Without clear definition, it is not clear whether IAB-node shall take into account stack processing time when applying the provided PDB into scheduling decision. | Low complexity because PDB for BH_RLC is clarified with more detailed information in F1-AP. |
| 8 | Vivo, Samsung, Kyocera | IAB-donor-CU is provided with the buffer/link status of IAB-nodes so that the IAB-donor-CU can update the routing configuration timely. | By considering the congestion status of each IAB-node within the topology, the IAB-donor-CU is able to reconfigure the most appropriate path (with less congestion risk, higher resource efficiency) for the transmission of the packets, which solves the latency issue from the first place. | Some new RRC or F1-C signaling might be introduced to convey the information between IAB-donor-CU and IAB-node. |
| 9 | ZTE, Apple, Samsung, Kyocera | IAB node measure and report the one hop latency to donor CU. | Upon receiving the one hop latency per BH RLC channel, donor CU could estimated the latency for different routing paths and then (re)-configure appropriate routing path for each UE DRB. | IAB node should be configured measure and report the one hop latency. |
| 10 | Fujitsu, Sony, Samsung, Huawei, Futurwei, Kyocera | Allowing local re-routing for purposes other than RLF | Local re-routing can be for the purpose of load balance, to avoid long delay in one path. | Triggers of local re-routing need to be specified. Candidate routing paths can be configured by CU. |
| 11 | Ericsson | IAB donor CU provides the IAB node with a PDB value per BH RLC channel per BAP destination. | Currently, the CU can provide each IAB node with a PDB per BH RLC channel However, it is not possible for the CU to indicate a PDB per BH RLC channel per BAP destination. Hence, the IAB node will treat in the same way all the packets associated to the same BH RLC channel, even though they are intended to different BAP destinations. The solution would make possible for the CU to configure different PDBs for different BAP destinations of a given BH RLC channel. As a consequence, the IAB node scheduler can be aware of the latency requirements of packets intended to different destinations, even though they are conveyed in the same BH RLC channel. | Very low complexity. Just need to extend the current F1 PDB configuration which is “per BH RLC channel”, to a PDB configuration “per BH RLC channel per BAP destination.” |

## Congestion

Based on various issues identified in R2-2009073, and the company submissions to AI 8.4.2 of RAN2#112-e, main solutions to ensuring fairness focused on the following issues and related solutions:

- Enhancements to DL HbH congestion control (e.g. support forwarding of feedback to ancestor node; agreeing new triggering conditions e.g. reception of feedback from child node)

- Whether to introduce UL HbH flow control (what would be the benefits etc.)

- Allowing local re-routing for purposes other than RLF

- Introduce new types of RLF indication on the backhaul

The companies are invited to fill in the Table below, by providing their own solutions, which can be based on above. Enough detail needs to be provided so that in Phase-2 we can assess the solutions.

If a previous entry already captures your solution, you can just add your company name in that row. If you think your solution is a variant of an existing entry, you may wish to number your entry to show the link (e.g. existing solution is no.3  your solution becomes no.3a).

Table 3 – focus on congestion

| Solution ID | Company | Short solution description | How does this solution address congestion? | Comments on complexity |
| --- | --- | --- | --- | --- |
| 1 | CATT, ETRI, Samsung | When parent IAB-node receives a DL HbH flow control message from child IAB-node, the parent IAB-node forwards the DL HbH flow control message to ancestor IAB-node. | It can reduce the data transmission from ancestor IAB-node to parent IAB-node to avoid the long-term congestion. | We should discuss when the parent IAB-node forwards the DL HbH flow control message to ancestor IAB-node. |
| 1a | Intel, Apple, Samsung | Hop-by-hop flow control feedback is triggered by “receipt of flow control feedback from child node” | An IAB node who receives flow control feedback can inform congestion to ancestor nodes (i.e., parent node and parents of parent nodes) in advance (before it itself experience congestion) and avoid series IAB node congestion, it can also help to reduce packet transmission from the source of the data. | Low complexity comparing with solution 1, add a trigger condition simple to achieve. |
| 1b | Samsung | Like in Solution 1, when parent IAB-node receives a DL HbH flow control message from child IAB-node, the parent IAB-node forwards the DL HbH flow control message to ancestor IAB-node. However, the information is not just forwarded, but potentially grouped together/processed (e.g. combined/average buffer status of several child IAB-nodes). It is also possible to simplify notifications even when they are not grouped – e.g. report a number indicating one of a handful levels of congestion. | Similar to solution 1, it can reduce the data transmission from ancestor IAB-node to parent IAB-node to avoid the long-term congestion. | Potentially added complexity or at least added standards work since new feedback format may be needed. However, it reduces overhead, potentially significantly. |
| 1c | Samsung | Like in solution 1a, but use additional triggering conditions, e.g. only trigger flow control feedback to ancestor node if child node’s buffers occupancy is above a certain threshold. | Similar to solution 1a. | Potentially added complexity over 1a (although standards load here is minimal since only triggering conditions are being added, not necessarily new formats). However, it reduces overhead, potentially significantly. |
| 2 | CATT, Intel, ETRI, Fujitsu, Huawei, Futurewei | Parent IAB-node should indicate the UL HbH flow control message to child IAB-node. | Child IAB-node can reduce the UL grant of descendant IAB-node. It can mitigate the congestion of child IAB-node. If child IAB-node is in DC case, it will mitigate the congested link rather than both two links. Suspending upstream data after receiving flow control message, also can help to reduce packet drop. | A new UL HbH flow control message should be designed. |
| 3 | CATT, LG, Intel, ETRI, Fujitsu, Ericsson, Samsung, Sony, Huawei, Apple, Futurewei, Kyocera | Upon receiving HbH Flow control message, the local re-routing can be performed to mitigate the congestion. | In DL, the parent node can select another link for data transmission. It will not cause the congestion of parent IAB-node. Local re-routing can mitigate the long-term congestion. | The R16 re-routing mechanism due to RLF can be re-used. The mapping table for re-routing is controlled by donor CU. |
| 4 | CATT, LG, Intel, vivo, ZTE, Apple, Ericsson, Huawei, Kyocera | ‎‎Introduce two new RLF notifications, such as “BH recovering indication” and “BH recovered indication”. The detail behaviours can be further discussed. This issue has been covered by another e-mail discussion. | When the IAB-node receives “BH recovering indication”, the IAB-node may reduce or stop the upstream transmission or try to find an alternative path to replace the one in problem. When the IAB-node receives “BH recovered indication”, the IAB-node may recover the upstream transmission. “BH recovering indication” that is triggered upon detection of BH RLF and “BH recovered indication” that is triggered upon recovery from BH RLF | Design two new RLF notifications, such as “BH recovering indication” and “BH recovered indication”. |
| 5 | Intel, vivo, Apple | Introduce a flow control leaving feedback when buffer falls back under threshold | It can avoid IAB node sending flow control feedback multiple times due to long-term congestion or when buffer falling back under buffer threshold. | Low complexity. A new flow control feedback from congested IAB node to its parent node |
| 6 | Intel | Introduce a congestion indication to child node of congested IAB node | This can help to avoid long-term packet loss at child node of congested IAB node. Child node of congested IAB node can switch to a new parent node (by donor-CU/local rerouting/ RRC re-establishment). | Low complexity. A new message from congested node to its child nodes. |
| 7 | vivo | Enhancements on the format design of control feedback can be investigated further to improve the size efficiency. For example, to report the flow control feedback per BH RLC CH group, instead of per BH RLC CH. | This might not be able to alleviate the DL congestion situation, but the large size of feedback message (for UL) is not resource efficient. Considering the number of BH RLC channel/routing IDs that could be configured for an IAB node, the size reduction of FC message could be promising. | Specification impact is minor. To specify a new format of the flow control feedback message. |
| 8 | ZTE, Samsung, Sony, Kyocera | IAB node sends the congestion report to donor CU. | When congestion is detected, IAB node may send the congestion report to donor CU,which could then update the routing path configuration for DL/UL traffic to alleviate the congestion. | IAB node may be configured to report the congestion status to donor CU. |
| 9 | Nokia, Nokia Shanghai Bell, Huawei, Futurewei | Hop-by-hop flow control for upstream traffic. | An IAB node in DC with one congested UL cell group can send feedback regarding Routing IDs or BH RLC channels mapped to that cell group. From parent to child nodes successively, this feedback can propagate all the way to the traffic sources, i.e. access IAB nodes scheduling UEs in uplink. | The feedback indications of Rel-16 BAP flow control can be made applicable to Rel-17 flow control for upstream traffic. To avoid impact on IAB MT MAC mechanisms like BSR and LCP, flow-control feedback from a parent node can be used by the IAB DU of the child node to determine how much uplink data (e.g. per Routing ID) it will further pass to the co-located IAB MT. |
| 10 | Samsung | Enhancements to polling for flow control feedback: consider conditions or “triggers” for polling | By way of example, polling could be triggered by change in transmission rate from the ancestor node of the parent node [so that parent node can anticipate changes in its own buffer occupancy and its egress rate, and ask for feedback from downstream node(s)], change in number of child nodes and UEs attaching to the node [similar reasoning as in the previous example wrt the benefits], and the reconfiguration by the CU of the routing table [this could anticipate increase in load of certain downstream links, at the expense of others]. | Complexity is low – this simply introduces certain rules for polling. This may help with the overhead as well. |

# Phase-2: collecting views on solutions proposed

In this section, based on input received in Phase-1, the discussion rapporteur has grouped similar/overlapping solutions, for each of the 3 main topics (fairness [F] / latency [L] / congestion [C]). The companies are asked to a) express their support for specific solutions, and b) comment on the grouping and whether it includes all solutions / addresses all issues.

## Fairness

Please note that the RAN2 specification impact of solutions 1, 3a, 5 and 11 (Section 2.1) is unclear to the rapporteur – while the way solutions would work is clear, what is unclear is what if anything RAN2 would need to do to enable these solutions. In other words, to the discussion rapporteur these solutions appear to be an implementation matter (or in case of solution 11 – a RAN3 matter). If the proponents disagree, they are invited to provide a further explanation into Table 5.

The remaining solutions from Section 2.1 are grouped according to the following, and captured in Table 4 for companies to express their support with a ‘X’ in the relevant field. Further comments can be made in Table 5.

F1: IAB nodes are configured with additional information by the CU

F1-1: Related to the number of bearers in a specific BH RLC channel (e.g. actual number, average number) [Based on solution 6 in Table 1]

F1-2: Related to QoS of bearers in a specific BH RLC channel [7]

F1-3: Related to LCP procedure at IAB nodes – introduce new logical channel variable to allow UE bearer-level buffering and prioritization [9]

F2: Additional information is added in the BAP header

F2-1: Bearer ID [4a]

F2-2: Bearer ID and hop count of the specific path [4]

F2-3: Number of UE DRBs in a specific BAP packet [10]

F3: Additional signaling exchange is introduced between parent and child nodes

F3-1: Downstream radio conditions are shared with the parent node(s) [2]

F4: Additional signaling is introduced from IAB nodes to the CU

F4-1: Related to load information per BH RLC channel [3]

F4-2: Related to per-hop latency and per-hop packet loss on individual links [8]

Table 4

| Company | F1-1 | F1-2 | F1-3 | F2-1 | F2-2 | F2-3 | F3-1 | F4-1 | F4-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple |  |  | X |  |  | X |  |  | X |
| Kyocera |  |  |  |  |  |  |  | X | X |
| Intel | X |  |  | X | X |  | X | X |  |

Table 5

| Company | Additional comments |
| --- | --- |
| Apple | For F1-1 and 1-2 kind of solutions it is not clear how the implementations will work in the case of N:1 and 1:1 RLC mappings. While hop count is a good metric, a moderately high number of hops is not always an indication of unfairness. Consider a scenario where we have 3 unloaded hops to donor compared to one heavily loaded hop with considerable delay using an alternate path. Hop count, therefore, has to be included with other metrics to get a clearer picture of what and where the true bottle neck is present causing the fairness (or latency or congestion) issue at hand. The issue with “load” as a metric is the definition of load itself. Is it due to the number of Connected UEs, # of resources in UL or in DL or is there any other definition and agreeing to such a definition will waste a lot of RAN2 time. Instead we feel directly looking at the impact causing metrics like PDB, latency and/or loss rate would yield a much better result for all the different criteria mentioned in this discussion. |
| Kyocera | Regarding Solution 1 in Table 1, we agree with the rapporteur that it’s up to IAB-DU implementation for DL. We think, however, for UL there need some standard solution since it may affect to e.g., LCP in IAB-MT. |
| vivo | We tend to agree that the fairness issue could be resolved by implementation, as indicated by the rapporteur that our solution (3a) might have no impact on RAN2. Whenever a DRB is established, the BH RLC channel(s) will be initiated top-down by IAB-donor-CU, the IAB-donor-CU is aware of the QoS requirements of every DRB and can configure the prioritisedBitRate per BH RLC channel accordingly to meet the demands. Suppose another DRB with same/similar QoS requirements to the previous DRB is going to be established, and note that this is actually the case of N:1 mapping (no issue arises for the 1:1 mapping case), the ancestor BH RLC channels can be proportionally re-configured to a larger prioritisedBitRate as per the number of DRBs mapped onto the same BH RLC channel so that to ensure the fairness across different BH RLC channels. In summary, we’d like to focus on the discussions of latency and congestion instead. |
| Intel | [5], According to TS38.321, fairness scheduling is based on in order to achieve fairness among UE bearers, in order to avoid a UE bearer with huge data amount occupying more resource of certain logical channel, a fairness enforcer can be introduced to IAB-DU. This cannot be done by implementation, as it may affect the mapping between UE bearers and logical channels. |

## Latency

Please note that the RAN2 specification impact of solutions 2 and 7 (Section 2.2) is unclear to the rapporteur – while the way solutions would work is clear, what is unclear is what if anything RAN2 would need to do to enable these solutions. In other words, to the discussion rapporteur these solutions appear to be an implementation matter (or in case of solution 7 – a RAN3 matter). If the proponents disagree, they are invited to provide a further explanation into Table 7.

The remaining solutions from Section 2.2 are grouped according to the following, and captured in Table 6 for companies to express their support with a ‘X’ in the relevant field. Further comments can be made in Table 7.

L1: IAB nodes are configured with additional information by the CU

L1-1: Related to the number of remaining hops in the upstream or downstream [Based on solution 1 in Table 2]

L1-2: Related to the enhanced PDB-derived values (e.g. “remaining PDB”, “discard PDB”) of a specific BAP packet [6]

L1-3: Related to one-hop PDB and configured for the access node [5]

L1-4: Related to PDB value per BH RLC channel per destination [11]

L2: Additional information is added in the BAP header

L2-1: Related to the total hop count per path in the BAP header [1a]

L2-2: Related to the remaining PDB (as determined by the previous-hop scheduler) of a specific BAP packet [2a, 2b]

L3: Additional signaling is introduced from IAB nodes to the CU

L3-1: Buffer/link status of IAB-nodes is shared with the CU [8]

L3-2: Latency measurements for individual hops per BH RLC channel are shared with the CU [9]

L4: New behaviour or feature is specified for the IAB nodes

L4-1: Buffer size calculations for pre-emptive BSR are specified [4]

L4-2: Local re-routing is allowed for purposes other than RLF (e.g. based on delay on outgoing link) [10]

L4-3: The number of LCGs for IAB-MT is increased [3]

Table 6

| Company | L1-1 | L1-2 | L1-3 | L1-4 | L2-1 | L2-2 | L3-1 | L3-2 | L4-1 | L4-2 | L4-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple |  | X |  | X |  |  | X | X | X | X |  |
| Kyocera |  |  |  |  | X |  | X | X | X | X | X |
| vivo |  |  |  |  |  | X | X |  | X |  | X |
| Intel |  |  |  |  | X |  |  |  |  | X |  |

Table 7

| Company | Additional comments |
| --- | --- |
| Apple | L1-4 is a better compromise for L1-2 in terms of implementation complexities. However, if any packet discard mechanisms are implemented at the intermediate nodes, L1-4 could lead to higher number of discards than L1-2. L3-2 falls into the category of L1-4 from our view, just that the metric used is different. L1-2 howver provides better accuracy in cases of incoming packets from multiple parent nodes to an intermediate child node. For L2-1, similar to comments in Table 5, the number of hops might not always be a best indicator for each of the three metrics of fairness, latency and congestion. L3-1 and L3-2 are complementary solutions and L3-1 may be linked to L4-1. A request to the rapporteur to possibly have an even more generic table for the solutions as they seem to be very specific at this point and we are unsure if these can bring us any closer to consensus. |
| vivo | L3-1 is irrelevant to L4-1, where the former solution aims to solve the congestion issue by updating the routing configuration constantly with the assistance information offered by IAB-nodes, and the latter solution is designed to overcome the calculation ambiguities of the expected data. The increased number of LCG for IAB-MT provides a finer scheduling scheme, which is preferred from our perspective. |

## Congestion

The solutions from Section 2.3 are grouped according to the following, and captured in Table 8 for companies to express their support with a ‘X’ in the relevant field. Further comments can be made in Table 9.

C1: Enhancements to DL HbH flow control

C1-1: HbH flow control feedback is forwarded to upstream (ancestor) nodes [Based on solution 1 in Table 3]

C1-2: HbH flow control feedback is processed and shared with upstream (ancestor) nodes (e.g. combined/average status of several child nodes, simplified indication of status of links) [1b]

C1-3: Trigger HbH flow control feedback when HbH flow control feedback is received from child node [1a]

C1-4: Trigger HbH flow control feedback when HbH flow control feedback is received from child node and certain additional conditions on the content of this feedback are met (e.g. buffer occupancy is above a certain threshold) [1c]

C1-5: Enhance format design of DL HbH flow control messages (e.g. different granularity – report per BH RLC channel group) [7]

C1-6: Introduce polling triggers (e.g. transmission from the ancestor node changes) [10]

C1-7: Enhance flow control feedback mechanism – avoid sending multiple reports for long-term congestion [5]

C2: Introduction of UL HbH flow control

C2-1: Parent node indicates to child node information on upstream links [2, 6, 9]

C3: Additional signaling exchange is introduced between parent and child nodes

C3-1: New RLF notifications are introduced from parent to child node [4]

C4: Additional signaling is introduced from IAB nodes to the CU

C4-1: IAB node sends a congestion report to the CU [8]

C5: New behaviour or feature is specified for the IAB nodes

C5-1: Local re-routing is performed based on reception and content of HbH flow control messages [3]

Table 8

| Company | C1-1 | C1-2 | C1-3 | C1-4 | C1-5 | C1-6 | C1-7 | C2-1 | C3-1 | C4-1 | C5-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Apple |  |  |  |  |  | X |  | X | X | X |  |
| Kyocera |  |  |  |  |  |  |  |  | X | X | X |
| vivo |  |  |  |  | X |  | X |  | X | X |  |
| Intel |  | X |  | X |  | X | X | X | X |  | X |

Table 9

| Company | Additional comments |
| --- | --- |
| Apple | Similar comments as in other tables. We feel there should first be an agreement on the terms “load” and “congestion” before some of these solutions are discussed though we have indicated our preference. This as mentioned earlier can potentially waste a lot of our time in RAN2. One potential way forward is for those definitions to be left up to implementations while ensuring QoS is maintained irrespective of hops ? We request the rapporteur to make the individual company proposals generic and conceptual while keeping the problem statements specific. |
| vivo | We are wondering whether C4-1 can be discussed together with L3-1, as some similarities seem to arise between these two solutions, which we believe that are both proposed with the intention of updating the routing configuration constantly to avoid potential congestion/latency issue. |
| Intel | Solution [6] in congestion, the intention of the “congestion indication” is for downstream FC enhancement, where the long-term congested parent node can send congestion indication to its child node (in downstream), so that child node can avoid resource waste due to congestion (packet loss) at its parent node, and allow it switch to a new parent node. Hence, this solution should be listed as C1-8, rather than C2-1. Besides, the boundary between C1-1/C1-2/C1-3/C1-4 is not quite clear, from our understanding, these three options are supplement with each other. The flow control feedback received from child node as proposed in C1-2 is also based on child nodes’s buffer occupancy is above a certain threshold (child node follows Rel-16 trigger condition). Hence, we don’t see any difference between C1-2 and C1-4. C1-7: Maybe it’s good to change as following in order to provide more details so that companies can understand what is potential solution during review? C1-7: Enhance flow control feedback mechanism – avoid sending multiple reports for long-term congestion (e.g. introduce a flow control leaving feedback when buffer falls back under threshold) [5] |