3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Electronic, 02nd – 13th November 2020

Agenda item:	x.x.x

Source: 	Huawei

Title: 	Summary of [AT112-e][036][MBS] SA2 LS on MBS

Document for:	Discussion and Decision

# Introduction

This document is for the following offline discussion.

- [AT112-e][036][MBS] SA2 LS on MBS (Huawei)

Scope: Treat R2-2008755, and related contributions. While not overlapping with already done email discussions, collect comments and reply proposals for the questions asked by SA2 and identify easy agreements / options with some support.

Intended outcome: Report, to be treated on-line Friday Nov 6

Deadline: Nov 6

The relevant contributions submitted to RAN2#112-e meeting on the SA2 reply LS include:

R2-2008755	LS on RAN impact of FS_5MBS Study (S2-2006044; contact: Huawei)	SA2	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, RAN, RAN2, RAN3

R2-2008751	Reply LS on RAN impact of FS_5MBS Study (RP-202086; contact: Huawei)	RAN	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, SA2	Cc:RAN2, RAN3

R2-2008768	Reply LS on RAN impact of FS_5MBS Study (SP-200884; contact: Huawei)	SA	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:RAN, SA2	Cc:RAN2, RAN3

R2-2009335	Discussion on SA2 LS on RAN impact of FS_5MBS Study	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

R2-2009336	Draft reply LS to SA2 on RAN impact of FS_5MBS Study	Huawei, HiSilicon	LS out	Rel-17	NR_MBS-Core	To:SA, SA2, RAN3	Cc:RAN

R2-2009822	draft_Reply LS on RAN impact of FS_5MBS Study	ZTE, Sanechips	LS out	Rel-17	To:SA2, RAN3

R2-2009954	SA2 questions about RRC state transitions for multicast	Ericsson	discussion	Rel-17	NR_MBS-Core

R2-2009196	MBS L2 Architecture, user plane and control plane	Intel Corporation	discussion	Rel-17	NR_MBS-Core

R2-2008791	Discussion on Requirement and Architecture of MBS	CATT	discussion	Rel-17	NR_MBS-Core

# Discussion

- 2.1 Issues on RRC states for Multicast reception

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: - UE within a multicast MBS session shall stay in CM-CONNECTED state, - UE can receive data of a multicast MBS session also while in CM-IDLE state. …

First, it should be noted that the proposals are about the multicast solution (i.e. not for the broadcast solution) defined in SA2.

The multicast solution is characterized by:

- a multicast MBS session; and

- the UE joining procedure in 3GPP networks.

So the 3GPP network is aware of which UEs are receiving the MBS service and will establish the multicast MBS sessions towards only the serving gNBs of these UEs.

The broadcast solution is characterized by:

- a broadcast MBS session; and

- No UE joining procedure defined in 3GPP networks (there can be UE joining procedures in the application layer).

So it is the application server to decide the broadcast area and indicate to the 3GPP network to broadcast the content in these areas, i.e. indicate the relevant gNBs/cells. In this sense, the LTE solutions including MBSFN and SC-PTM can be all seen as broadcast solutions from 3GPP perspective.

The email discussion in Post111-e906 MBS Idle mode support has discussed the RRC states for “multicast services”, but companies may have different understandings on the terminology of “multicast services”, e.g. whether it is from application layer point of view or from 3GPP point of view, and whether the broadcast solution defined by 3GPP can be used for the “multicast service”, e.g. services with low reliability requirement.

Therefore, in this offline email, we would like to further collect views from companies to clarify this point.

Question 0: Do you agree with the understanding that the terminologies of multicast MBS session and broadcast MBS session are defined from 3GPP network point of view, and it is NOT directly mapped to “multicast services” or “broadcast services” in application layer, e.g. a groupcast service in application layer can be mapped to a broadcast session in 3GPP networks?

| Company | RRC state(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | An example is that in LTE, the group call for public safety, which is a multicast service in application layer, can be delivered via LTE SC-PTM which is a broadcast solution defined in 3GPP. |

Question 1: In the multicast solution (in contrast to the broadcast solution), in which RRC state(s) should the UE be able to receive multicast MBS session data?

| Company | RRC state(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | RRC_CONNECTED | The concept of the “multicast solution” is very similar to the Unicast solution, in the sense that the 3GPP network is fully aware of which UEs are receiving the services, and establishes the unicast PDU session or multicast MBS session only towards these UEs. Different from the broadcast solution which is cell-oriented, the multicast/unicast solutions are all UE-oriented. Therefore, we would like to reuse as much as possible the unicast concept for the multicast solution, i.e. the reception of the service is only in RRC_CONNECTED state. |

- 2.2 Issues on RRC states when no data ongoing for multicast MBS session

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: … - UEs can transition into CM-IDLE while no multicast MBS data are transmitted. …

Even if the UE is only allowed to receive multicast data in RRC_CONNECTED state, there can be the case that there is no data ongoing for the multicast MBS session. The question is whether the UE is allowed to be released to RRC_IDLE or RRC_INACTIVE states in case of no traffic.

Q2: When there is no data ongoing for a multicast MBS session, in which RRC state(s) do you think the UE is allowed to be?

| Company | RRC state(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | RRC_CONNECTED/RRC_INACTIVE (preferred), or RRC_IDLE | By reusing the concept of the unicast solution, when there is no traffic a multicast MBS session, the network is allowed to release the UE to RRC_INACTIVE or RRC_IDLE based on implementation, or even keep the UE in RRC_CONNECTED. The desired RRC state mostly depends on the service requirement, e.g. how long is the latency required for the UE to receive the service when the data arrives again. |

- 2.3 Issues on trigger indication to establish resources

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: … - Some solutions propose that 5G CN may trigger notification to CM-IDLE and/or CM-CONNECTED mode UEs (e.g. paging CM-IDLE mode UEs) for establishing transmission resources for an multicast MBS session when data of an multicast MBS session are ready to be delivered. …

There can be different issues in the description from different perspectives. RAN2 should discuss how to trigger a RRC_IDLE or RRC_INACTIVE UE to establish RRC connection for multicast reception, in case the UE is allowed to be released to RRC_IDLE/INACTIVE when no data ongoing.

It is proposed to reuse the legacy CN paging or RAN paging to trigger the RRC_IDLE or INACTIVE UE respectively to establish RRC connection for multicast reception.

Q3: Do you agree that as the baseline the legacy CN paging or RAN paging can be reused to trigger the RRC_IDLE or INACTIVE UE respectively to establish RRC connection for multicast reception?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | By reusing the concept of the unicast solution, when there is no traffic a multicast MBS session, the network is allowed to release the UE to RRC_INACTIVE or RRC_IDLE. If the UE was released to RRC_INACTIVE, similar to unicast PDU sessions, the multicast MBS session should be kept at RAN, and when there is data coming for this multicast MBS session, RAN can trigger RAN paging to trigger the UE to establish the RRC connection for multicast reception. If the UE was released to RRC_IDLE, similar to unicast PDU sessions, the multicast MBS session should be released at RAN, and when there is data coming for this MBS service, CN can trigger CN paging to trigger the UE to establish the RRC connection for multicast reception. |

- 2.4 Issues on activation/deactivation of multicast MBS session

The related description and questions in the SA2 LS are:

- There are different proposals how to handle the CM-IDLE/CM-CONNECTED state transitions: … - Some solutions propose that the multicast MBS session can be deactivated by the network while no multicast MBS data are transmitted to save power. - Some solutions propose that the network can activate the multicast MBS session and trigger notification to UEs when multicast MBS data are transmitted again. …

It is rapporteur’s understanding that the activation/deactivation of a multicast MBS session is only a solution from core network’s perspective, but from RAN perspectice RAN may still see the multicast MBS session as established or released. In this sense, it would be completely up to SA2 to decide on this feature. Companies are invited to comment on the following question:

Q4: Do you agree that activation/deactivation of a multicast MBS session is not within the RAN2 scope?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | RAN may decide about the UE’s RRC state based on MBS session data activity as for unicast session as we clarified in our answer to Q2. |

- 2.5 Issues on data forwarding

The related description and questions in the SA2 LS are:

- Some Xn/N2 handover solutions in the SA2 study are documented in the TR. - Some solutions consider to have temporary MBS data forwarding from S-RAN to the T-RAN, to address potential data loss or duplication in case of a UE moving to a T-RAN supporting 5MBS. - Some solutions have left forwarding FFS and would appreciate RAN feedback on possibilities for forwarding at Xn/N2 handovers with considerations of minimization of data loss, data duplication and complexity. …

As we have discussed the issue of data forwarding in the email discussion [Post111-e][905][MBS] Connected Mode Mobility with Service Continuity, rapporteur suggest to just wait for the conclusion of that email discussion summary.

Q5: Are you ok to reply to SA2 on this issue of data forwarding based on the conclusion of email discussion [Post111-e][905][MBS] Connected Mode Mobility with Service Continuity?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

- 2.6 Issues on local MBS service

The related description and questions in the SA2 LS are:

- Some Xn/N2 handover solutions in the SA2 study are documented in the TR. … - Some solutions introduce HO for local MBS service that can only transmit data in a certain area, which has impact on RAN for service area restriction. …

It is rapporteur’s understanding that it is up to SA2 to decide on the support of local MBS service, and RAN2 may discuss based on SA2’s progress.

Q6: Do you agree that it is up to SA2 to decide on the support of local MBS service, and RAN2 may discuss based on SA2’s progress?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

- 2.7 Issues on broadcast

The related description and questions in the SA2 LS are:

- SA2 is debating whether broadcast (i.e. without the network’s awareness about UEs receiving broadcast contents and for other use cases than the ones excluded already for Rel-17) should be further down-scoped in Rel-17 for remaining broadcast requirement in the SID. Some companies have provided solutions on broadcast (which are documented in the TR). SA2 would like to ask SA, RAN, RAN2 and RAN3 for feedback on broadcast support in Rel-17.

RANP in RAN#89e has concluded that broadcast is within the scope of NR MBS in Rel-17. Therefore, RAN2 can reply that RAN2 will continue the work of broadcast based on the RAN decision.

Q7: Do you agree to reply to SA2 that based on the RAN decision RAN2 will continue the work on the broadcast support?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |

- 2.8 Issues on assistance information for PTP/PTM switch

The related description and questions in the SA2 LS are:

- Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching. SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any.

Some proposed assistance information have been summarized as below:

- Information#1: assistance information from 5GC to help gNB to decide which delivery method for specific MBS or UE [6];

- Information#2: “information of MBS services/groups subscribed by the UE, e.g. TMGI [8];

- Information#3: session start time [8].

- Information#4: QOS requirements from CN [9]

Q8: what assistance information from 5GC to RAN above do you support for PTP/PTM switch?

| Company | Answer | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | #2, #4 | The choice of the delivery method depends mainly on the QoS requirements of the service and the radio condtions of the UE. Hence, only #2 and #4 are needed from CN and the rest of the information is available directly in the gNB (e.g. based on UE measurements or feedback). |

# Conclusion

# Reference

- R2-2008755	LS on RAN impact of FS_5MBS Study (S2-2006044; contact: Huawei)	SA2	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, RAN, RAN2, RAN3

- R2-2008751	Reply LS on RAN impact of FS_5MBS Study (RP-202086; contact: Huawei)	RAN	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:SA, SA2	Cc:RAN2, RAN3

- R2-2008768	Reply LS on RAN impact of FS_5MBS Study (SP-200884; contact: Huawei)	SA	LS in	Rel-17	FS_5MBS, NR_MBS-Core	To:RAN, SA2	Cc:RAN2, RAN3

- R2-2009335	Discussion on SA2 LS on RAN impact of FS_5MBS Study	Huawei, HiSilicon	discussion	Rel-17	NR_MBS-Core

- R2-2009336	Draft reply LS to SA2 on RAN impact of FS_5MBS Study	Huawei, HiSilicon	LS out	Rel-17	NR_MBS-Core	To:SA, SA2, RAN3	Cc:RAN

- R2-2009822	draft_Reply LS on RAN impact of FS_5MBS Study	ZTE, Sanechips	LS out	Rel-17	To:SA2, RAN3

- R2-2009954	SA2 questions about RRC state transitions for multicast	Ericsson	discussion	Rel-17	NR_MBS-Core

- R2-2009196	MBS L2 Architecture, user plane and control plane	Intel Corporation	discussion	Rel-17	NR_MBS-Core

- R2-2008791	Discussion on Requirement and Architecture of MBS	CATT	discussion	Rel-17	NR_MBS-Core