3GPP TSG-RAN2 Meeting #112 Electronic 	R2- 200xxxx

- 2nd November – 13th November 2020

Agenda Item:	6.13.1

Source: 	Samsung

Title:  	Offline 101 - MAC corrections for eMIMO

Document for:	Discussion and decision

# Introduction

- [AT112][101][eMIMO] MAC corrections (Samsung)

Scope: Discuss the CRs in AI 6.13.1

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes / merges with other CRs (with an indication of

the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Tuesday 2020-11-03 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2010760):  Tuesday 2020-11-03 09:00 UTC

CRs listed as "can be agreed as is" in R2-2010760 and not challenged until Tuesday 2020-11-03 13:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

# BFR Procedure

R2-2009098    Correction to parameter list for beam failure recovery procedure		Samsung

R2-2009904    Miscellaneous on 38.321 for BFR and BFR MAC CE          ZTE Corporation, Sanechips      design	ZTE Corporation, Sanechips

R2-2010494     Correction to bitmap length determination in MAC CEs for BFR   		Fujitsu

Common changes in R2-2009098 and R2-2009904 related to SCell beam failure recovery.

## 5.17	Beam Failure Detection and Recovery procedure

:

RRC configures the following parameters in the BeamFailureRecoveryConfig, BeamFailureRecoverySCellConfig and the RadioLinkMonitoringConfig for the Beam Failure Detection and Recovery procedure:

-	beamFailureInstanceMaxCount for the beam failure detection;

-	beamFailureDetectionTimer for the beam failure detection;

-	beamFailureRecoveryTimer for the SpCell beam failure recovery procedure;

-	rsrp-ThresholdSSB: an RSRP threshold for the SpCell beam failure recovery;

-	rsrp-ThresholdBFR: an RSRP threshold for the SCell beam failure recovery;

Q1: Do you agree with the above changes?

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |

Additional changes in R2-2009098 related to beam failure recovery.

## 5.17	Beam Failure Detection and Recovery procedure

:

-	ssb-perRACH-Occasion: ssb-perRACH-Occasion for the SpCell beam failure recovery using contention-free Random Access Preamble;

-	ra-ResponseWindow: the time window to monitor response(s) for the SpCell beam failure recovery using contention-free Random Access Preamble;

-	prach-ConfigurationIndex: prach-ConfigurationIndex for the SpCell beam failure recovery using contention-free Random Access Preamble;

-	ra-ssb-OccasionMaskIndex: ra-ssb-OccasionMaskIndex for the SpCell beam failure recovery using contention-free Random Access Preamble;

-	ra-OccasionList: ra-OccasionList for the SpCell beam failure recovery using contention-free Random Access Preamble;

- 	candidateBeamRSList: list of candidate beams for SpCell beam failure recovery;

- 	candidateBeamRSSCellList: list of candidate beams for SCell beam failure recovery.

Q2: Do you agree with the above changes?

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |

Correction to title of Figure 6.1.3.23-1 and Figure 6.1.3.23-2

Option 1 (as in R2-2010494):

Figure 6.1.3.23-1: BFR and Truncated BFR MAC CE with the highest ServCellIndex of this MAC entity's SCell for which beam failure is detected  is less than 8

Figure 6.1.3.23-2: BFR and Truncated BFR MAC CE with the highest ServCellIndex of this MAC entity's SCell for which beam failure is detected  is equal to or higher than 8

Option 2 (as in R2-2009904):

Figure 6.1.3.23-1: BFR and Truncated BFR MAC CE with one octet Ci field

Figure 6.1.3.23-2: BFR and Truncated BFR MAC CE with 4 octets Ci field

Q3: Which option do you prefer for correcting the description of Figure 6.1.3.23-1 and Figure 6.1.3.23-2?

| Company | Option 1 /Option 2/ other, if other specify | Detailed Comments |
| --- | --- | --- |

Others

According to R2-2010494,

" In 5.17, it is captured that “Beam failure is detected by counting beam failure instance indication from the lower layers to the MAC entity” and “The MAC entity shall for each Serving Cell configured for beam failure detection…”. It seems that “Beam failure is detected” means that the MAC entity counts beam failure instance indication. It incurs a misunderstanding that “SCell configured with beam failure detection” and “SCell for which beam failure is detected” are same. Based on this, e.g., the field Ci of the MAC CEs for BFR is set to 1 when the MAC entity counts beam failure instance indication. "

Following change is proposed:

## 5.17	Beam Failure Detection and Recovery procedure

The MAC entity may be configured by RRC per Serving Cell with a beam failure recovery procedure which is used for indicating to the serving gNB of a new SSB or CSI-RS when beam failure is detected on the serving SSB(s)/CSI-RS(s). If beamFailureRecoveryConfig is reconfigured by upper layers during an ongoing Random Access procedure for beam failure recovery for SpCell, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure using the new configuration.

Q4: Do you agree with the above change?

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |

# BFR trigger point / BFR MAC CE generation

R2-2009796    Clarification on the BFR trigger upon candidate search     Nokia, Nokia Shanghai Bell, Ericsson, ZTE

R2-2010009    Correction on BFR MAC CE generation     Qualcomm Incorporated, Samsung

The issue was discussed in RAN2 #111e. Due to lack of consensus on TP, the issue was postponed. R2-2010009 includes the TP which was discussed in last meeting. R2-2009796 proposes an alternate way to handle the issue.

Table 1 below summarizes the key differences between these two CRs.

Table 1

|  | R2-2009796 | R2-2010009 |
| --- | --- | --- |
| A. Indication of SCell as failed in BFR MAC CE | UE will not indicate SCell as failed in BFR MAC CE until candidate beam evaluation is completed. | UE may or may not indicate SCell as failed in BFR MAC CE until candidate beam evaluation is completed. Proposed Note uses phrase ‘need not’ as shown below: “ When the MAC entity has triggered BFR for an SCell and not cancelled and is in the process of evaluating the candidate beams according to the requirements as specified in TS 38.133 [11], it need not report the SCell as failed in a BFR MAC CE or a Truncated BFR MAC CE;” |
| B. SR trigger | SR will not be triggered if there is only one failed SCell and candidate beam evaluation is ongoing for this failed SCell | If there is only one failed SCell and candidate beam evaluation is ongoing for this failed SCell, - SR will be triggered if sufficient UL-SCH resources are not available |
| C. Impact to RAN4 | Impacts RAN4 specification. - Spec needs to be updated to specify that candidate beam evaluation is performed upon beam failure detection and not upon BFR trigger from MAC. May be MAC should also be updated as follows: 2> if BFI_COUNTER >= beamFailureInstanceMaxCount: 3> if the Serving Cell is SCell: 4> consider the beam failure as detected for this serving cell; 4> trigger a BFR for this Serving Cell upon completing evaluation of the candidate beams according to the requirements as specified in TS 38.133 [11]; 3> else: 4> initiate a Random Access procedure (see clause 5.1) on the SpCell. | No impact |

Q5: Do you agree with the analysis in Table 1 with respect to ' Indication of SCell as failed in BFR MAC CE'?

| Company | Agree / Disagree | Detailed Comments |
| --- | --- | --- |

Q6: Do you agree with the analysis in Table 1 with respect to 'SR Trigger'?

| Company | Agree / Disagree | Detailed Comments |
| --- | --- | --- |

Q7: Do you agree with the analysis in Table 1 with respect to 'RAN4 impact'?

| Company | Agree / Disagree | Detailed Comments |
| --- | --- | --- |

# multi-CC simultaneous TCI activation with multi-TRP/panel transmission

R2-2010013	Discussion on Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE on multiple CC case	Qualcomm Incorporated

R2-2010014	Correction on Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE	Qualcomm Incorporated	CR	38.321

R2-2010628	Multi-CC simultaneous TCI activation with multi-TRP/panel transmission	Ericsson	discussion

R2-2010634	Reply LS on multi-CC simultaneous TCI activation with multi-TRP/panel transmission	Ericsson	LS out	Rel-16	NR_eMIMO-Core	To:RAN1

R2-2010637	Correction for CC list operation for TCI state update MAC CE	Ericsson, Samsung	CR		38.321

RAN1 asked whether/how to support the feature of multi-CC simultaneous TCI activation for PDSCH with the two features specified for multi-TRP/panel transmission in the LS [1].

In the case of single DCI based multi-TRP/panel transmission, companies think that clarification from RAN2 is needed on whether/how this can be operated together with the feature of multi-CC simultaneous TCI activation. Switching between single DCI based multi-TRP/panel and single TRP/panel is controlled by MAC-CE, not by RRC. From RRC configuration perspective, therefore, it seems possible to include a CC/BWP being operated with single DCI based multi-TRP/panel in the CC list, and it seems that the new MAC-CE introduced for supporting single DCI based multi-TRP/panel, i.e. Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE, applies to one specific CC/BWP even in the case when the CC/BWP is included in the CC list by RRC configuration. Question: Is the following understanding is correct? - By current RAN2 specification, it is not precluded to enable single DCI based multi-TRP/panel operation in one or more CC(s)/BWP(s) included in simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 by using Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE.

RAN2 made the following conclusion in RAN2 #109e agreements,

- If the CC indicated in the MAC CE is configured as part of a CC-list, this MAC CE applies to all the CCs in the CC list; otherwise, the MAC CE applies to single CC.

- Multiple TRP case is not considered for MAC CEs regarding multiple CCs/BWPs, i.e. TCI States Activation/Deactivation for UE-specific PDSCH MAC CE and TCI State Indication for UE-specific PDCCH MAC CE.

For the second agreement above (i.e. yellow highlight), the understanding from [2][3] and [4][5][6] is different. [2][3] mentioned that RAN2 already precluded the sPDDCH mTRP operation with simultaneous multiple CCs/BWPs but [4][5][6] mentioned it was not it was not discussed in detail if this applies to sPDCCH mTRP as well as mPDCCH mTRP case.

Q8: What UE operation should be described in Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE?

- Option 1: If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, this MAC CE shall apply to the indicated Serving Cell only.

- Option 2: If the indicated Serving Cell is configured as part of a simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, this MAC CE applies to all the Serving Cells configured in the set simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2, respectively;

| Company | Preference | Detailed Comments |
| --- | --- | --- |

# Enhanced PUCCH Spatial Relation Activation Deactivation MAC CE

R2-2009903	38.321 Correction on  Enhanced PUCCH Spatial Relation ActivationDeactivation MAC CE	ZTE Corporation, Sanechips	CR		38.321

For the current Enhanced PUCCH Spatial Relation Activation/Deactivation MAC CE, this CR [7] includes below issues:

- The mapping relationship between PUCCH resource ID and the corresponding Spatial relation info is missed.

- In RRC specification, the value range of the PUCCH-SpatialRelationInfoID is [1, 64], however, the current description of Spatial Relation info ID field only can indicate the value range from [0, 63].

Q9: Do companies agree with this CR?

| Company | Agree as is; Agree with changes; Disagree | Detailed Comments |
| --- | --- | --- |

# Conclusions

TBD

# References

[1] R2-2008609, LS on multi-CC simultaneous TCI activation with multi-TRP/panel transmission, RAN1.

[2] R2-2010013, Discussion on Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE on multiple CC case	Qualcomm Incorporated.

[3] R2-2010014, Correction on Enhanced TCI States Activation/Deactivation for UE-specific PDSCH MAC CE	Qualcomm Incorporated	CR	38.321.

[4] R2-2010628, Multi-CC simultaneous TCI activation with multi-TRP/panel transmission	Ericsson	discussion.

[5] R2-2010634, Reply LS on multi-CC simultaneous TCI activation with multi-TRP/panel transmission	Ericsson	LS out	To:RAN1.

[6] R2-2010637, Correction for CC list operation for TCI state update MAC CE	Ericsson, Samsung	CR		38.321.

[7] R2-2009903, 38.321 Correction on  Enhanced PUCCH Spatial Relation ActivationDeactivation MAC CE	ZTE Corporation, Sanechips	CR		38.321.

# Annex: contact person(s) for each participating company

| Company | Name | Email address |
| --- | --- | --- |
| Samsung | Anil Agiwal | anilag@samsung.com |
| Samsung | Seungri Jin | seungri.jin@samsung.com |