- 3GPP TSG RAN WG2 Meeting #112-e      	                           R2-200xxxx
E-Conference, 2nd – 13th Nov. 2020

Agenda item:	8.7.3.1

Source:	Qualcomm Incorporated

Title:	Email discussion summary of [612][NR17] Open issues on L3 relay (Qualcomm)

WID/SID:	FS_NR_SL_relay – Release 17

Document for:	Discussion and Decision

# Introduction

It is summary report for below email discussion:

- [AT112-e][612][Relay] Open issues on L3 relay (Qualcomm)

Scope: Discuss the remaining open issues on L3 relay architecture, including:

- NAS transport

- Overhead

- QoS

- RRC states

- Remaining open items in the current TR

Intended outcome: Summary in R2-2010871

Deadline:  Wednesday 2020-11-11 1200 UTC

Based on the scoping, this email discussion intends to address remaining issues on L3 relay including the issues raised in company contributions (e.g. NAS, overhead, QoS...) and open items in RAN2 TR. As rapporteur, we try to reflect all companies’ views expressed in their contributions, which include:

[1] R2-2008962, Discussion on remaining issues of L3 relay, Qualcomm Incorporated

[2] R2-2009033, Discussion on Remaining issues on L3 relay, ZTE Corporation, Sanechips

[3] R2-2009124	, Overhead in N3IWF based L3 relaying architecture, MediaTek Inc.

[4] R2-2009720	, Discussion on L3 UE-to-NW relay architecture, Ericsson

[5] R2-2010345	, NAS transmission and QoS management in L3 U2N relay, Huawei, HiSilicon

[6] R2-2008778, Left issues on QoS, Security and L2/3 comparison, OPPO

Please note that we don’t cover service continuity of L3 relay because it is covered in email discussion#621.

For rapporteur to have enough time drafting summary report, we would like to have the following two phases:

- Phase 1: collect companies’ view, by Tuesday 2020-11-10 12:00 UTC

- Phase 2: rapporteur will share summary report based on input of phase 1 for review, by Monday 2020-11-11 12:00 UTC

# Discussion

## NAS transport with N3IWF

First, to avoid confusion, please note that this discussion is only for the N3IWF option of the L3 Relay.

It is related to below Editor Note on security in TR 38.836:

Editor note: RAN2 will evaluate any impact in RAN2 scope from these solutions.

In [1][2][5], one specific issue is discussed: whether SL-SRB/Uu-DRB or SL-DRB/Uu-DRB is used to transfer remote UE’s NAS message. And if SL-DRB/Uu-DRB is used, how to differentiate NAS and UP traffic?

[1][2][5] discussed this issue. Their key views are summarized below:

|  | SRB or DRB for NAS? | If DRB, how to differentiate NAS and UP traffic? |
| --- | --- | --- |
| [1] R2-2008962 | DRB | - NAS sent over Uu/PC5 DRB as it is sent via IPSec tunnel. It is similar to existing LTE/NR IMS signaling - Differentiation between UP and NAS is achieved by assigning different 5QI for NAS and UP traffic and mapping to different DRB - It is similar to existing LTE/NR IMS (e.g. DRB with QCI-5 for IMS signaling and DRB with QCI-1 for voice). |
| [2] R2-2009033 | DRB | - NAS sent over Uu/PC5 DRB as it is non-reasonable for relay UE to transmit NAS signaling and UP traffic of remote UE indiscriminately - Differentiation between UP and NAS is achieved by applying a new traffic filter for N3IWF introduced by SA2 (solution#45 in 23.752) |
| [5] R2-2010345 | No clear preference | No solution is proposed |

Based on company input, Rapporteur think majority view is to use PC5/Uu DRB. Actually, Rapporteur fail to understand how PC5/Uu SRB can work because all the traffics are sent as PDU session for untrusted non-3GPP access. It is same as current N3IWF solution for untrusted non-3GPP access. Then, with regarding how to differentiate between UP and NAS sent over PC5/Uu DRB, Rapporteur think it is not an essential issue because NAS message is usually sent over SRB2 which has the same priority as that one of the DRB. If further handling is required, Rapporteur think that both solutions in [1] and [2] can work, which both have no RAN2 impact. And it is up to the Network implementation either one or both to use. Thus, there is no need to down-select between them.

In summary, Rapporteur think the majority view can be summarized as:

- Remote UE’s NAS is sent over PC5/Uu-DRB.

- Differentiation between UP and NAS can be achieved by assigning different 5QI for NAS and UP traffic and mapping to different DRB, and/or applying higher layer solution (e.g. a new traffic filter for N3IWF introduced by SA2 in solution#45 in 23.752).

- There is no identified RAN2 impact.

Q1: On how to send Remote UE’s NAS in L3 U2N relay with N3IWF, do you agree:

- Remote UE’s NAS is sent over PC5/Uu-DRB;

- Differentiation between UP and NAS can be achieved by assigning different 5QI/PQI for NAS and UP traffic and mapping to different DRB, and/or applying higher layer solution (e.g. a new traffic filter for N3IWF introduced by SA2 in solution#45 in 23.752);

- There is no identified RAN2 impact

| Company | Yes / No? | Comments (if you answer “No”, please list identified RAN2 impacts) |
| --- | --- | --- |
| MediaTek | Yes | We wonder if the differentiation between UP and NAS by assigning different 5QI/PQI to the corresponding DRBs (which is lower than SRB) is sufficient to ensure the QoS requirement of NAS message, since there is no QoS definition for NAS message. |
| OPPO | Yes | As for the QoS definition of NAS message, we can leave it to SA2 |
| Samsung | Yes | Agree with OPPO. |
| Futurewei | No | 5QI/PQI is defined for user plane data, associated with a PDU session. NAS signaling belongs to control plane, and is not associated with any PDU session. Solution#45 in 23.752 uses “"UE or Network Requested PDU Session Modification” to set/modify QoS parameters, which only apply to user plane data, not to control plane signaling, particularly not to NAS signaling. |
| Spreadtrum | Yes |  |
| vivo | Yes | Whether 5QI/PQI can be defined to NAS message may be left for SA2 but anyway we understand if PC5/Uu-DRBs is used to carry remote UE’s NAS, they should be configured at least to turn on integrity protection function. Regarding Futurewei’s comments, confirmation may be needed from SA2. |
| CATT | Yes | We share the same view with OPPO. |
| Nokia | Yes | We can leave to SA2/CT1 the specification of QoS parameters for NAS. As NAS is robust (e.g. it may work non-3GPP access networks as well), no special QoS requirements (e.g. very low latency) are expected. |
| Lenovo | Yes | The QoS for NAS will be the same as the QoS used by the UE to establish the IPsec connection with the N3IWF. Whether a specific QoS is needed is up to SA2. |
| Apple | No | We do not agree with 2) “Differentiation between UP and NAS can be achieved by assigning different 5QI/PQI for NAS and UP traffic and mapping to different DRB, and/or applying higher layer solution (e.g. a new traffic filter for N3IWF introduced by SA2 in solution#45 in 23.752)”; Actually, differentiation between UP and NAS is not visible to AS layer for Layer3 U2N solutions (in either PC5 DRB or Uu DRB), that is why there is no RAN2 impact. RAN2 shall not endorse or comment on SA2 solutions, this can be left to SA2. |
| Convida | No | Agree with OPPO and Apple. There is no RAN2 impact. RAN2 shall not endorse or comment on SA2 solutions, this can be left to SA2. |
| ZTE | Yes with comment | NAS signaling will be forwarded with a specific SPI allocated by N3IWF. From relay UE’s perspective, it can only recognize different traffic by SPI. In consequence, a new traffic filter is sufficient to ensure the QoS of remote UE’s NAS traffic. And how traffic filter applied on remote UE’s NAS signaling is up to SA2’s decision. However, it seems that the new added traffic filter can only be applied on uplink. Because, traffic filter is served for relay UE’s Uu PDU session. We think a we need to send an LS to SA2 to check whether solution#45 is only applied on uplink. |
| Qualcomm | Yes | For Futurewei’s comment, we don’t see extra spec impacts besides SA2 may need to specify QoS definition in N3WIF solution which should be discussed in SA2. Actually, whether to define QoS only impacts priority of NAS. Please Note that NAS signaling can also be signaled via SRB2, whose priority is similar to DRB. We are fine to send LS to SA2 on the QoS definition part For Apple, OPPO and Convida’s comments, we are fine to remove 2) to address your concern. According to current input, it is common understanding that no RAN2 impact is identified for NAS transport in N3IWF solution. Then, we think a way-forward is that: - RAN2 conclude that Remote UE’s NAS is sent over PC5/Uu-DRB in L3 relay N3IWF solution and no RAN2 impact is identified. - Include the above conclusion in the status report LS to SA2 (discussed in email discussion [601]). SA2 can decide whether to do further work (e.g. define QoS requirement for NAS signaling). For ZTE’s comment, we think you can propose directly in SA2. As far as we see, companies in RAN2 don’t want to comment on SA2 solution. |
| Huawei | Yes to 1) No to 2) 3) | We are fine with 1), i.e. remote UE’s NAS messages can be delivered via PC5 DRB. For 2), we are not sure if the differentiation between UP and NAS can/will be done by CN assigning different 5QI/PQI, however we also agree this is in SA2 scope, no need to capture in RAN2, and we are also wondering if we need to send LS to SA2 to check on this. For 3), based on what solution to be adopted by SA2 (the solution in 2) or others), there might be RAN2 specification impact, .e.g. if NAS is not treated by QoS flow in remote UE AS layer, AS will need to be aware it is NAS, and the DRB configuration will not include SDAP config. |
| Intel | Yes | We think it is in SA2 realm to support the solution for differentiation of NAS and UP by defining separate QoS or other method. |
| LG | Yes | We think the QoS of NAS message is a realm of SA2. |
| Ericsson | Yes | We are generally fine with all the three statements and we also agree with the way forward proposed by QC. |
| Fraunhofer | Yes | Agree with the way-forward as proposed by the rapporteur |
| Philips | Yes |  |
| InterDigital | No | Agree with comments from OPPO and Apple. |

Another issue was raised in [5]: how a remote UE to obtain PC5 bearer configuration for NAS message transmission? Specifically, the UE needs to obtain QoS flow to DRB mapping and also the corresponding DRB configuration. [5] think that it seems there is no related QoS information defined for NAS message in SA2, so that the AS mechanism of obtaining DRB configuration may need to be adjusted for NAS transmission correspondingly.

For this issue, Rapporteur also think it is not an essential issue. The same QoS flow to SL-DRB mapping framework specified in Rel-16 V2X can be reused with a fixed DRB mapping for NAS over PC5/Uu-DRB. It is also similar to existing LTE/NR IMS mechanism, where IMS signaling is sent with specific QoS (e.g. QCI-5). In Rapporteur understanding, the only delta may be that SA2 can specify a QoS requirement for NAS signaling, which anyhow doesn’t have RAN2 impact.

Q2: For the issue how a remote UE to obtain PC5 bearer configuration for NAS, do you agree that the same QoS flow to SL-DRB mapping framework in Rel-16 V2X can be reused with a fixed DRB mapping for NAS signaling (i.e. there is no RAN2 impact) ?

| Company | Yes / No? | Comments (if you answer “No”, please list identified detailed impacts) |
| --- | --- | --- |
| MediaTek | Yes with comment | Specific DRB should be defined by RAN2 like SRB. |
| OPPO | Yes | Once QoS requirement for NAS is defined by SA2, RAN2 can just following exsiting framework i.e. no RAN2 impact is foreseen |
| Samsung | Yes | Agree with OPPO. |
| Futurewei | No | QoS flow is defined only for user plane data. NAS signaling is in control plane. IMS signaling, mentioned by the rapporteur, is carried as application data to 3GPP system, and in 3GPP system’s user plane. Hence, there is no QoS flow corresponding to NAS signaling, and the current QoS flow to DRB mapping in RAN can’t be applied to map NAS signaling to SL/Uu-DRB. |
| Spreadtrum | Yes |  |
| vivo | Yes | A fixed NAS signaling to DRB (or SRB, if DRB cannot be used according to SA2) mapping is simpler. |
| CATT | Yes | Agree with OPPO. |
| Nokia | Yes |  |
| Lenovo | Yes | Agree with OPPO |
| Apple | No | remote UE AS layer SHALL treat this as same as other upper layer UP traffic with similar QoS, we do not agree to have a specific fixed PC5 DRB mapping for *end-to-end NAS signaling* in RRC configurations for SLRB. If such a mapping is needed, then there is RAN2 impact. |
| Convida | Yes | Agree with OPPO. |
| ZTE | Yes | According to 23.752, NAS signaling of Remote UE is encapsulated by TCP protocol, as shown in following, which means from PC5 AS layer’s perspective of remote UE, NAS signaling is IP service data. In consequence, legacy mapping mechanism can be reused: IP->PC5 QoS flow -> SL-DRB . |
| Qualcomm | Yes | For Apple’s comment, we are fine to remove “a fixed DRB mapping for NAS signaling”. As indicated in our response of Q1, we agree with OPPO that QoS requirement for NAS can be defined by SA2. And it is common understanding that there is no RAN2 impact identified. Considering we have addressed QoS issue in Q1, we suggest to change the conclusion to: RAN2 conclude that the QoS flow to SL-DRB mapping framework in Rel-16 V2X can be reused for NAS signaling, and no RAN2 impacts are identified. |
| Huawei | No | It depend on the assumption related to Q1 2). So better to wait for SA2’s feedback before we make conclusion. Assuming SA2 will specify a QoS requirement for NAS signaling, then the existing mapping between QoS to SLRB can be used, otherwise there would be AS impact like Apple commented. And if SA2 will not specify QoS requirement for NAS, then AS needs to be aware of NAS messages, and map them into a SRB or DRB. |
| Intel | Yes | Agree with OPPO’s view |
| LG | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Philips | Yes | Agree with OPPO |
| InterDigital | Yes | With the proposed change by QC, we agree. |

Summary of Q1 and Q2

Most companies agree:

- Remote UE’s NAS is sent over PC5/Uu-DRB.

- For its RAN2 impacts:

  - if SA2 define a QoS requirement for NAS signalling, the legacy PC5 QoS to SL-DRB configuration framework in Rel-16 V2X can be reused for remote UE to configure SL-DRB for NAS signalling.

  - Otherwise (i.e. if SA2 will not define a QoS requirement for NAS signalling), RAN2 can study it in WI phase.

Because not much RAN2 spec change is expected even if SA2 doesn’t define QoS requirement for NAS. Thus, Rapporteur think it is not an essential issue, and so RAN2 can make conclusion that it can be supported without RAN2 impacts. This conclusion can be included in the status report LS to SA2 (discussed in email discussion [601]). SA2 can decide whether to do further work (e.g. define QoS requirement for NAS signaling). Because most company agree that RAN2 should not endorse or comment on SA2 solutions, Rapporteur intend to avoid comment/suggestion on SA2 solution.

Thus, Rapporteur suggest below proposal:

Proposal 1: In L3 relay N3IWF solution (solution#23 in TR 23.752), RAN2 understanding is that remote UE’s NAS is sent over PC5/Uu-DRB and it can be supported without RAN2 impacts. It can be included in the status report LS to SA2 (discussed in email discussion [601]).

Proposal 2: If any AS impact of NAS transport in solution#23 is identified by SA2, RAN2 can further discuss it in WI phase.

## IP header overhead with N3IWF

It is also related to the same Editor Note on security in TR 38.836:

Editor note: RAN2 will evaluate any impact in RAN2 scope from these solutions.

The protocol stacks of L3 relay with N3IWF are shown in Figure.1 and Figure.2. The issue is that some company had concern on the overhead due to extra IP header needed, e.g. IPsec (in tunnel mode with the Encapsulating Security Payload protocol), Inner IP, and GRE.

Figure 1: user plane protocol stack of L3 U2N relay with N3IWF

Figure 2. Control plane protocol stacks for N3IWF router based L3 UE-to-Network Relay

[1][3][4] discussed this issue. Their key views are summarized below:

| [1] R2-2008962 | Outer IP header on each hop can be compressed by ROHC "ESP/IP profile” and leave the discussion on the overhead issue caused by inner IP header of IPSec to SA2. |
| --- | --- |
| [3] R2-2009124 | - RoHC profile 0x0103 supports ESP/IP, meaning that the 28-byte IPsec and ESP headers could be reduced to a few bytes. However, the ICV is not compressible (it depends on the payload contents), and the remaining headers from Inner IP and GRE are inside the encrypted payload and cannot be compressed. - Send an LS to SA2 indicating that RAN2 have studied the user-plane overhead in the N3IWF based L3 relaying architecture and discerned an overhead of >32 bytes per packet, representing a variable overhead that can exceed 50% for small-packet services (in particular, VoNR would be expected to see high overhead). |
| [4] R2-2009720 | - Overhead caused by N3IWF is limited for service with large packet size. - The capacity decrease is inevident in typical PS scenarios with a mix of voice and video services. |

To make progress, Rapporteur provided an analysis on these contributions:

- Both [1] and [3] mentioned outer IP header on each hop can be compressed by ROHC “ESP/IP profile”. However, ROHC may not work for inner IP header because it is not visible to AS.

- Both [3] and [4] provide analysis on the impacts of overhead on different services.

For 1), Rapporteur think RAN2 can conclude that that outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact should be evaluated by SA2. However, the exact number of overhead (e.g. 32 byte) depends on IP version, which SA2 can further study.

For 2), Rapporteur think that RAN2 should first conclude whether need to notify SA2. And if RAN2 agree to notify SA2, then RAN2 doesn’t need to provide this analysis because SA2 can determine what type of traffic would be supported over the link. And RAN2 don’t need to emphasize on a particular use case and place the number like 50% or 32 byte there.

Q3: For the IP header issue of L3 U2N relay with N3IWF, do you agree RAN2 can conclude that outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact should be evaluated by SA2?

| Company | Yes / No? | Comments |
| --- | --- | --- |
| MediaTek | Yes with comments | (1) We conclude that even with RoHC applied, each packet acquires 32 bytes of incompressible headers/trailers, in addition to the result of compressing the IPsec and ESP headers. This overhead occurs both on the Uu and PC5 interfaces, since the involved layers are end-to-end between the N3IWF and the remote UE. (2) Although SA2 did not send an LS requesting any action from RAN2, it seems clear from their note in TR that they would benefit from information from RAN2 on the overhead of the N3IWF design. (3) The ability of RAN2 to do a detailed analysis is limited (CT1 should be involved if a really thorough analysis is expected), but given the foregoing observations, it seems possible for RAN2 to send a LS to SA2/CT1 to present a general picture. (4)We are not sure if SA2 will continue their study on the issue. |
| OPPO | Yes | Apart from ROHC, maybe we can also indicate that around 32 BYTE overhead per hop is possible as indicated by Mediatek |
| Samsung | Yes | Not sure that we need to quantify the overhead on behalf of SA2 (this is in reference to proposal from MediaTek and OPPO). A statement that the impact should be evaluated by SA2 should suffice. |
| Futurewei | Yes |  |
| Spreadtrum | Yes |  |
| vivo | Yes | Agree with Samsung that we could just indicate the inner IP header can’t be compressed by the AS layer, and leave the overhead analysis to SA2, after all the actual extend of overhead is dependent on service type. |
| CATT | Yes with comments | - We don’t think SA2 and CT1 will evaluate this impact; - While N3IWF is involved, the increase of header cost is acceptable since N3IWF had been studied before and adopted by 3GPP specification. |
| Nokia | Yes | We agree with Samsung that RAN2 should indicate that inner IP header cannot be compressed by AS. Then we can leave to SA2 to investigate the issue. |
| Lenovo | Yes | Agree with Samsung |
| Apple | Yes with comment | It seems the N3IWF header size (ROHC compressed or not) needs to be further detailed, so that SA2 can make an informed decision . |
| Convida | Yes | Need to indicate to SA2 that the inner IP header can’t be compressed by the AS layer |
| ZTE | Yes |  |
| Qualcomm | Yes | We agree with Samsung that we don’t need to quantify the overhead on behalf of SA2. So, we can keep the current wording (maybe “should” can be replaced by “could” to soften tone), i.e. For the IP header issue of L3 U2N relay with N3IWF, RAN2 conclude that outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact could be evaluated by SA2 |
| Huawei | Yes with comments | Share the similar views as MediaTek and Apple, as indicated in SA2’s TR, RAN2’s evaluation on overhead of N3IWF solution seems to be expected by SA2. We can at least indicate that RAN2 see “around 32 BYTE overhead per hop is possible” as OPPO suggested. |
| Intel | Yes |  |
| Ericsson | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

Q4: If Q3 is agreed, do you agree to notify SA2 about this agreement in the status report LS to be sent to SA2 (discussed in email discussion [601])?

| Company | Yes / No? | Comments |
| --- | --- | --- |
| MediaTek | Yes with comments | We agree to coordinate with SA2 to indicate our analysis but we need to avoid a lengthy LS |
| OPPO | Yes |  |
| Samsung | Yes with comments | Please see our comments to Q3. |
| Futurewei | Yes |  |
| Spreadtrum | Yes |  |
| vivo | Yes |  |
| CATT | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Apple | Yes with comments | Please see Q3. |
| Convida | Yes |  |
| ZTE | Yes |  |
| Qualcomm | Yes | We agree with MediaTek that RAN2 should avoid a lengthy LS to SA2. We can only include the conclusion of Q3 in the status report LS to SA2, and SA2 can decide what to do by themselves. |
| Huawei | Yes with comments | More details on the inner IP overhead could be provided as well if we inform SA2 on IP header issue of L3 U2N relay with N3IWF. |
| Intel | Yes |  |
| LG | Yes |  |
| Ericsson | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

Summary of Q3 and Q4

Most companies agree:

- Outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact could be evaluated by SA2.

- The above conclusion can be included in the status report LS to be sent to SA2

Some companies prefer to include some quantitative overhead analysis. However, the number depends on IP version, and how 32byte overhead is calculated is not fully discussed in RAN2. If SA2 will further evaluate it, Proponent can also bring the quantitative analysis in SA2. Thus, Rapporteur think it is not necessary to include it.

Thus, Rapporteur suggest below proposal:

Proposal 3: For the IP header overhead of L3 U2N relay with N3IWF, RAN2 conclude that outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact could be evaluated by SA2.

Proposal 4: Include conclusion of IP header overhead of L3 U2N relay with N3IWF in the status report LS to be sent to SA2 (discussed in email discussion [601]).

## L3 U2N QoS

It is related to the Editor Notes on QoS in TR 38.836:

Editor note: RAN2 can discuss AS impacts related to SA2 specified QoS solutions.

Editor note: RAN2 further discuss whether it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation.

Basically, companies ([1][2][5][6]) identified the AS impact is PC5 bearer mapping based QoS requirement (legacy PQI or mapping between PQI and 5QI). Their key views are summarized below:

| [1] R2-2008962 | - For Solution#25, no new AS procedure is required because gNB and relay can independently reconfigure Uu/PC5 bearers via legacy AS procedures. - For Solution#24, we also think no new AS procedure is required. According to TR 23.752, relay can obtain a mapping between PQI and 5QI from core network (SMF/PCF). With such QoS mapping, we think it is up to relay UE implementation to calculate the updated QoS profile. And then relay can accordingly enforce E2E QoS via legacy PC5 RRC procedure including SLRB reconfiguration and the bearer mapping. |
| --- | --- |
| [2] R2-2009033 | - For L3 U2N relay without N3IWF - For uplink data transmission of remote UE, bearer mapping can be realized by relay UE using its uplink QoS rules. - For downlink data transmission of remote UE, bearer mapping between Uu traffic and PC5 traffic shall be considered. - For L3 U2N relay with N3IWF - The SPI of downlink traffic needs to be taken into consideration for downlink bearer mapping. |
| [5] R2-2010345 | RAN2 to further discuss the forward compatibility for multi-hop of solution #24 and solution #25 captured in SA2 TR 23.752 for L3 relay QoS management |
| [6] R2-2008778 | Remove the 2 editor notes on QoS of L3 U2N relay, and conclude there is no AS impact due to SA2 QoS solution, for which legacy PC5-RRC procedure is sufficient. |

From above 4 contributions, we identified two questions from RAN2 perspective:

- Whether legacy PC5-RRC procedure is sufficient for SA2 L3 U2N relay solution#24 and solution#25

- Whether need to consider forward compatibility for multi-hop of SA2 solution#24 and solution#25

For 1), we think [6] provided useful information on SA2 L3 U2N relay QoS solution#24 and solution#25:

For solution#24, according to TR 23.752 V0.5.0, the related impact is for PC5-S layer, i.e., the handling of QoS parameter.

6.24.3	Impacts on services, entities and interfaces

The solution has impacts in the following entities:

SMF:

-	SMF optionally supports modifying the PDB for a QoS Flow serving the Remote UE based on either PCC rules or pre-configuration.

UE:

-	5G ProSe UE-to-Network Relay supports the mapping of Uu flow level QoS parameters to PC5 QoS parameters, including the mapping of 5QIs to PQIs, based on configuration.

-	5G ProSe UE-to-Network Relay modifies the PQI of the PC5 link to match the QFI of the derived QoS rule.

-	Remote UE supports to decide the PC5 part QoS parameters based on the E2E QoS parameters.

PCF:

-	supports to decide the Uu part QoS parameters based on the E2E QoS parameters.

For solution#25, according to TR 23.752 V0.5.0, similarly, the related impact is for PC5-S layer, i.e., the transfer of QoS parameter. For the L2 link modification procedure, it is also PC5-S related.

6.25.4	Impacts on services, entities and interfaces

PCF:

-	PCF generates PCC rules (for QoS control on Uu) and the PC5 QoS parameters (for QoS control on PC5).

SMF:

-	Provides the PC5 QoS parameters to UE-to-Network Relay during PDU session modification procedure.

UE-to-Network Relay:

-	UE-to-Network Relay modify the Layer-2 link based on the PC5 QoS parameters received from CN.

-	Forwards the E2E QoS requirement received from remote UE to CN.

Remote UE:

-	Sends the E2E QoS requirement to UE-to-Network Relay.

Hence, rapporteur think the above highlighted SA2 spec should be able to address the concern from [2] and [5]. Hence, RAN2 can conclude that there is no AS impact for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure is sufficient, and the two Editor Notes can be removed.

Q5: Do you agree that there is no AS impact for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure is sufficient, so that the following two Editor Notes can be removed?

Editor note: RAN2 can discuss AS impacts related to SA2 specified QoS solutions.

Editor note: RAN2 further discuss whether it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation.

| Company | Yes / No? | Comments |
| --- | --- | --- |
| MediaTek | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Futurewei | Yes? with comments | The intention of the question seems to be only about impact on the PC5-RRC procedure of SLRB reconfiguration. There is impact on AS QoS control of the remote UE, e.g., reflective QoS/DRB mapping and control of UE-AMBR are not possible. |
| Spreadtrum | Yes |  |
| vivo | Yes | We understand the question is to confirm no additional work in AS to support E2E QoS is needed, thus removing above editor’s note is fine to us. |
| CATT | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Apple | No | If the intention is to say “it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation”, then we do not agree. So far, RAN2 has not make any conclusion about this is sufficient or not. The only thing is that the solutions discussed has no RAN2 impact. |
| Convida | Yes | No RAN2 impact for SA2 QoS solution#24 and #25. But do not agree “it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation”. |
| ZTE | Yes with comments | In solution#24, after receiving the QoS rules and flow level parameters, the Relay UE determines the corresponding PC5 QoS Flows to establish and the mapping between the Uu QoS Flows and the PC5 QoS Flows. In solution#25, PCF generates PCC rules (for QoS control on Uu) and the PC5 QoS parameters (for QoS control on PC5). It seems that both solution#24 and 25 create a mapping between Uu QoS flow and PC5 QoS flow. However, downlink data may not include QFI in the packet header in Uu interface, which means even if relay UE knows the mapping between Uu QoS flow and PC5 QoS flow, it still doesn’t know which PC5 QoS flow or DRB can be used to forward downlink packet. In consequence, we think an LS needs to be send to SA2 to check whether solution#24 and 25 create a mapping between Uu QoS flow and PC5 QoS flow, and whether they need QFI always be included in downlink packet? If prior is YES and latter is NO, AS solution may needs to be introduced to forward the downlink data. |
| Qualcomm | Yes | For Futurewei’s comments, reflective QoS/DRB mapping was agreed not to be supported by NR V2X, and correspondingly is not supported in sidelink relay. Control of UE-AMBR is SA2 expertise For Apple’s comments, the intention of question is to say whether RAN2 impacts are identified (i.e. we can rely on legacy PC5 RRC procedure). So, we can re-word the RAN2 conclusion to: There is no AS impact identified for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure is sufficient can be reused. For ZTE’s comment, we think you can propose directly in SA2. As far as we see, companies in RAN2 have common understanding that no RAN2 impacts are identified and don’t want to comment on SA2 solution. |
| Huawei | Yes with comments | Share the same view as Apple and Convida, so far we may only confirm there is no RAN2 impact for solution #24 and #25. We are fine with the wording proposed by Qualcomm. |
| Intel | Yes |  |
| LG | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes with comments. | We are fine with the wording proposed by QC. We wonder, however, whether to simply remove the second editors note, or whether such study needs to continue in RAN2 and/or SA2. |

Summary of Q5

Majority companies agree:

- No AS impact is identified for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure can be reused

- Two Editor Notes related to L3 U2N relay QoS in TR 38.836 can be removed

Thus, Rapporteur suggest below proposal:

Proposal 5: No AS impact is identified for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure can be reused. Remove the following 2 Editor Notes in Section 4.6.2 of TR 38.836:

Editor note: RAN2 can discuss AS impacts related to SA2 specified QoS solutions.

Editor note: RAN2 further discuss whether it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation.

In [5], it proposed to further discuss the forward compatibility for multi-hop of solution #24 and solution #25 captured in SA2 TR 23.752 for L3 relay QoS management. Specifically, [5] think it is feasible to inform the number of hops to PCF/SMF and PCF/SMF is also able to derive QoS profile of each hop. However, except the first hop relay UE, the other relay UEs have no connection to the PCF/SMF. Then how could these relay UEs get the QoS profile?

Rapporteur is confused why this solution is proposed in RAN2:

- Multi-hop relay is not in Rel-17 SI scope. Forward compatibility can be discussed only if time allows. We are not sure why [5] think it is an essential issue

- How intermediate hop UE gets the QoS profile is a SA2 issue

- RAN2#111-e has made agreement that RAN2 don’t intent to study QoS enhancement for L3 U2N relay

- Proposal 8: RAN2 don’t intend to study QoS enhancement for L3 UE-to-NW relay to SA2 (e.g. whether gNB can perform PDB split). RAN2 can discuss AS impacts related to SA2 specified QoS solutions.

Thus, Rapporteur suggest that RAN2 don’t discuss this issue and encourage proponent to propose it in SA2.

Q6: Do you agree that Rapporteur’s recommendation?

Recommendation: for L3 relay QoS management, RAN don’t discuss the forward compatibility for multi-hop of solution #24 and solution #25 captured in SA2 TR 23.752. Proponent can propose it in SA2.

| Company | Yes / No? | Comments (if you answer “No”, please justify why it is essential to be discussed in RAN2) |
| --- | --- | --- |
| MediaTek | Yes | Maybe this can be clarified in the TR |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Spreadtrum | Yes |  |
| vivo | Yes | QoS parameters distribution for L3 multi-hop relay should be discussed in SA2, may be in the later release. |
| CATT | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Apple | yes |  |
| Convida | Yes |  |
| ZTE | Yes |  |
| Qualcomm | Yes | We agree with MediaTek to have one RAN2 agreement that: For L3 relay QoS management, RAN2 don’t intend to study the forward compatibility solution. |
| Huawei | Yes | We are fine with the wording proposed by Qualcomm. |
| Intel | Yes |  |
| LG | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

Summary of Q6

Rapporteur suggest below proposal:

Proposal 6: For L3 relay QoS management, RAN2 don’t intend to study the forward compatibility solution for multi-hop support.

## RRC states

In last RAN2 meeting, it was agreed:

Proposal 16: For L3 UE to NW relay, a relay UE must be in RRC_CONNECTED to perform relaying of data.

Revised Proposal 15: For L3 UE to NW relay, the Uu RRC state of the relay UE and remote UE can change when connected via PC5.  Both relay UE and remote UE can perform relay discovery in any RRC state.  A remote UE can perform relay discovery while OOC.

In this RAN2 meeting, there are no contributions to discuss RRC states of L3 relay. Hence, Rapporteur list our understanding:

- There is no restriction on relay and remote UE’s RRC state combination. Because remote UE is invisible to gNB in relay path, remote and relay like two sidelink UEs in Rel-16 NR V2X, and same principle as LTE Prose relay is reused. In addition, similar to LTE Prose relay, both remote UE and relay can be in RRC_CONNECTED because there is only one connection terminated in gNB.

- Both relay and remote UE can be in RRC_INACTIVE state.

Q7: For RRC state of L3 U2N relay, do you agree:

- Both relay and remote UE can be in RRC_INACTIVE state.

- There is no restriction on relay and remote UE’s RRC state combination.

| Company | Yes / No? | Comments |
| --- | --- | --- |
| MediaTek | Yes | The RRC states of Relay UE and Remote UE is not relevant and procedure is independent. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Futurewei | No | When a remote UE is in RRC-Connected state, its serving cell has active configurations on the remote UE for data transmission, i.e., SDAP/PDCP/RLC/MAC configurations for the data path (QoS flow to DRB to logical channel to LCP to SR, etc). The remote UE can’t just ignore its serving cell’s configuration, and autonomously move data transmission to L3 U2N relay. The remote UE should also continue measurement reporting as configured, and a HO may be triggered by the remote UE’s serving cell. Hence, it is doubtful that a remote UE can still stay in RRC-Connected state in its serving cell, after L3 U2N relay is established. Similar issues exist for remote UE in RRC-Inactive state, e.g., on the triggering of RRC resume procedure at data arrival. |
| Spreadtrum | Yes |  |
| vivo | Yes with comments | From our point of view, relay UE in RRC_CONNECTED and remote UE in RRC_IDLE can be a baseline, which means that there are only on-going relayed data from remote UE to NW via a relay UE. Before or when a relay link establishes, the relay UE can be in RRC_INACTIVE or RRC_IDLE. When a remote UE communicates with NW via a relay UE, its Uu state may be RRC_CONNECTED or RRC_INACTIVE, which means that there may be two types of services, on-going indirect services and on-going/suspended direct services. Details can be left to WI phase. |
| Nokia | Yes | The Remote UE has no RRC state in the gNB of the Relay UE related to the Relay connection. Whether the Remote UE can be in RRC Connected for its own Uu connection when it is connected via a Relay is up-to UE implementation/capabilities (e.g. dual radio implementation). Therefore, there is no need to introduce restrictions in the specification. |
| Lenovo, MotM | Yes |  |
| Apple | Agree with 1), not sure about 2) | We think the correct phrase for point 2 should be: “there is no specific RRC states defined for remote UE using a L3 U2N relay, i.e., remote UE RRC state is independent of its U2N relay operation and/or the RRC states of relay UE” |
| Convida | Yes | Agree with Apple |
| ZTE | Yes |  |
| Qualcomm | Yes | For Futurewei’s comments, we agree with Nokia/vivo that remote UE can be in CONNECTED state for its own Uu connection when it is connected via a relay. And it was specified in 36.331: #### ==================== #### 5.10.11.2 AS-conditions for relay related sidelink communication transmission by sidelink remote UE A UE capable of sidelink remote UE operation shall inform upper layers whether it is configured with radio resources that can be used for relay related sidelink communication transmission if the following conditions are met: 1> if the UE is out of coverage; and is preconfigured with SL-Preconfiguration including discTxPoolList and preconfigRelay; 1> else if in RRC_IDLE; and if the UE's serving cell is suitable as defined in TS 36.304 [4]; and if SystemInformationBlockType18 includes commTxPoolNormalCommon and commTxAllowRelayCommon; and if SystemInformationBlockType19 includes discConfigRelay; and if the sidelink remote UE threshold conditions as specified in 5.10.11.5 are met; 1> else if in RRC_CONNECTED; and if the UE is configured with commTxResources; and the UE is configured with commTxAllowRelayDedicated set to true; ================ For Apple/Convida’s comments, we think it makes sense. We can change conclusion to: - Both relay and remote UE can be in RRC_INACTIVE state. - Remote UE RRC state is independent of its U2N relay operation and/or the RRC states of relay UE. |
| Huawei | Yes to 1), No to 2) | We understand the intention, but as Futurewei and Apple commented, the wording of 2) maybe a little bit misleading. We still have the limitation on the RRC states as “Proposal 16: For L3 UE to NW relay, a relay UE must be in RRC_CONNECTED to perform relaying of data.”. And we feel it is enough to capture 1) on top of the previous meeting agreements of “Proposal 16” and “revised Proposal 15”. So we prefer to only capture 1). |
| Intel | Yes |  |
| LG | Yes | We think it’s hard to define the Remote UE’s RRC state in L3 indirect connection. The Remote UE’s RRC state can be defined in only direct connection. So, there is no restriction to the Remote UE’s sate in L3 relay. |
| Ericsson | Yes | We want to echo Nokia/QC/Vivo comments and we are fine with the latest proposals by QC. |
| Fraunhofer | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

Summary of Q7

Majority companies agree:

- Both relay and remote UE can be in RRC_INACTIVE state.

- Remote UE RRC state is independent of its U2N relay operation and/or the RRC states of relay UE. Huawei seem to have concern on the wording. Rapporteur think we can add “Same as LTE Prose relay” to address their concern.

Rapporteur suggest below proposal:

Proposal 7: Both relay and remote UE can be in RRC_INACTIVE state.

Proposal 8: Same as LTE Prose relay, Remote UE RRC state is independent of its U2N relay operation and/or the RRC states of relay UE.

## Security of L3 U2N relay

Current TR 38.836 has the following FFSs / Editor’s Notes on the requirement of Security in L3 U2N relay:

Editor note: whether the SA2 captured solutions can satisfy the security requirement depends on SA3.

SA3 has sent reply LS to SA2 [6] to indicate that E2E security solution of L3 U2N relay (i.e. solution#23) is feasible to meet end-to-end security requirements for the remote UE:

SA3 did a preliminary analysis of the UE-to-Network Relay, Layer-3 Relay detailed in solution #6 and #23 vs Layer-2 Relay detailed in solution #7, against the key issues and potential security requirements agreed in Version 0.1.1 of the SA3 TR 33.847 and concluded both solution#23 and solution #7 are feasible to meet end-to-end security requirements for the Remote UE.

Thus, [1][4][6] proposed to remove the Editor Notes:

| [1] R2-2008962 | Proposal 3: In TS 38.836, capture “SA3 concluded solution#23 with N3IWF is feasible to meet end-to-end security requirements for the remote UE”, and remove the following FFS: “Editor note: whether the SA2 captured solutions can satisfy the security requirement depends on SA3.” |
| --- | --- |
| [4] R2-2009720 | - RAN2 to capture in TR 38.836 the following: - N3IWF based L3 UE to NW relay supports end-to-end traffic confidentiality and/or IP address preservation; |
| [6] R2-2008778 | RAN2 confirm for U2N Relay, there is no RAN2 impact for L3 solution if relying on IPSec and for L2 solution if relying on PDCP layer, and in TR 38.836 section 4.6.3, remove the note of “whether other security solution is introduced depends on SA2.” and “RAN2 will evaluate any impact in RAN2 scope from these solutions”. |

Rapporteur think the LS from SA3 is quite clear, and so RAN2 can conclude and capture it in TR.

Q8: For security of L3 U2U relay, do you agree:

- Capture “As solution#23 of TR 23.752 with N3IWF is feasible to meet end-to-end security requirements, no AS level enhancements related to security are needed for this solution”, and remove the FFS “Editor note: whether the SA2 captured solutions can satisfy the security requirement depends on SA3.”

- Remove the note of “whether other security solution is introduced depends on SA2.” and “RAN2 will evaluate any impact in RAN2 scope from these solutions”. [6]

| Company | Yes / No? | Comments (if you answer “No”, please indicate your justifications) |
| --- | --- | --- |
| MediaTek | Yes for first part of 1 | We assume the question should be casted to L3 UE-to-NW Relay We assume SA3 concluded the security issue for N3IWF based L3 UE to NW relay, but not for IP based L3 UE to NW relay. |
| OPPO | Yes with conclusion on solution#23 with comment | SA3 doesn’t address security issue of solution#6 which is taken as baseline for L3 relay. Hence we need keep the note “Editor note: whether the SA2 captured solutions can satisfy the security requirement depends on SA3”. We are fine to remove 2nd and 3rd note in section 4.6.3 |
| Samsung | Yes |  |
| Futurewei | Yes with comments | The first editor’s note should still be kept. |
| Spreadtrum | Yes with comments | The editor’s note in the first bullet should be kept. |
| vivo | Yes |  |
| CATT | Yes |  |
| Nokia | Yes | Our understanding is that this question is for U2N relay |
| Lenovo | Yes | Agree with MediaTek |
| Apple | Yes |  |
| Convida | Yes with comments | The first editor’s note should still be kept. |
| ZTE | Yes |  |
| Qualcomm | Yes | OK to keep 1st editor’s note according to MediaTek’s comments, i.e. RAN2 TR keeps below EN: “Editor note: whether the SA2 captured solutions can satisfy the security requirement depends on SA3.” |
| Huawei | Yes with comments | Share the similar views as above companies that the first editor’s note should still be kept. |
| Intel | Yes for the first part of 1 | We prefer to keep the note as SA3 analysis is preliminary; we also wonder if we need to check with SA3 from RAN2 point of view whether the IP router based L3 U2N relay without IPSec/N3IWF can satisfy end-to-end security for Remote UE and we need to capture their response as part of our evaluation. |
| LG | Yes |  |
| Ericsson | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

Summary of Q8

Companies’ opinions are basically aligned. Thus, Rapporteur suggest below proposal:

Proposal 9: For security of L3 U2N relay:

- Capture “As solution#23 of TR 23.752 with N3IWF is feasible to meet end-to-end security requirements, no AS level enhancements related to security are needed for this solution” in Section 4.6.3 of TR 38.836.

- Remove the below Editor notes in Section 4.6.3 of TR 38.836

“Editor note: whether other security solution is introduced depends on SA2.”

“Editor note: RAN2 will evaluate any impact in RAN2 scope from these solutions”.

## Control plane procedure of L3 U2N relay

Current TR 38.836 has the following 2 FFSs / Editor’s Notes on Security of L3 U2N relay:

Editor note: FFS if there is RAN2 impact to support the related control plane procedures.

Editor note: RAN2 will further consider procedures with RAN2 impact.

In [1], it proposed that there is no AS impact identified beyond E2E QoS enforcement as discussed in QoS, and thereby the two Editor Notes can be removed:

- For step 4, further capture that “relay can enforce E2E QoS via legacy PC5 RRC procedure based on the mapping between PQI and 5QI, or PC5 QoS parameter obtained from core network”.

- Remove the following two Editor Notes:

“Editor note: FFS if there is RAN2 impact to support the related control plane procedures.

Editor note: RAN2 will further consider procedures with RAN2 impact.”

- Further AS impacts (if any) can be discussed in WI phase

Q9: For control plane procedure of L3 U2N relay, do you agree:

- For step 4, further capture that “relay can enforce E2E QoS via legacy PC5 RRC procedure based on the mapping between PQI and 5QI, or PC5 QoS parameter obtained from core network”.

- Remove the following two Editor Notes:

“Editor note: FFS if there is RAN2 impact to support the related control plane procedures.

“Editor note: RAN2 will further consider procedures with RAN2 impact.”

- Further AS impacts (if any) can be discussed in WI phase

| Company | Yes / No? | Comments (if you answer “No”, please indicate your justifications and additional identified RAN2 impacts) |
| --- | --- | --- |
| MediaTek | Yes with 2/3 | The added bullet one may be not essential for AS layer. |
| OPPO | Yes |  |
| Samsung | Yes with 2/3 | Same reasoning as MediaTek. |
| Futurewei | No | Based on Solutions 24 & 25 in TR 23.752, E2E QoS can only be enforced after step 6. Figure 4.6-4 in TR 38.836 needs to be updated with Figure 6.25.3-1 of TR 23.752 to address E2E QoS issue. |
| Spreadtrum | Yes for 2/3 | We have the same view with MediaTek. |
| vivo | Yes | For 1), we are fine to capture it or leave it to be discussed later after SA2 QoS solution is decided. Futurewei’s suggestion is also acceptable. |
| CATT | Yes |  |
| Nokia | Yes |  |
| CATT | Yes |  |
| Apple | Only for 2/3 | We do not agree with bullet 1. |
| Convida | No with 1 Yes with 2/3 | Agree with Futurewei |
| ZTE | Yes with 1 No with 2/3 | We think how to perform downlink data forwarding is not clear. In R16 sidelink, UE derives the PC5 QoS rules by itself according to QoS characteristic received from V2X application. In legacy LTE relay, mapping between QCI of EPS bearer and PPPP are used to realize downlink bear mapping. When it comes to NR sidelink relay, without PC5 QoS characteristic, relay UE can not derive PC5 QoS rules. After receiving downlink remote UE’s packet, relay UE does not know which PC5 QoS flow or DRB can be used to carry the downlink packet. In consequence, we prefer to keep 2 and capture this issue in the TR. |
| Qualcomm | Yes for 2/3 | OK to remove bullet 1 in rapporteur proposal. For ZTE’s comment, we think in QoS solution#24, relay can obtain mapping of 5QIs to PQIs from CN, right? Then it is up to relay implementation how to perform downlink data forwarding based on QoS mapping. |
| Huawei | No to 1), Yes to 2/3 |  |
| Intel | Yes |  |
| LG | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes for 2/3 only. |  |

Summary of Q9

Companies’ opinions are basically aligned. Thus, Rapporteur suggest below proposal:

Proposal 10: For control plane procedure of L3 U2N relay, remove the following two Editor Notes in Section 4.6.5 of TR 38.836. Further AS impacts (if any) can be discussed in WI phase.

“Editor note: FFS if there is RAN2 impact to support the related control plane procedures.

“Editor note: RAN2 will further consider procedures with RAN2 impact.”

## L3 U2U relay

[1] [2] [6] discussed how to capture L3 U2U relay in RAN2 TR. Their key views are summarized below:

| [1] R2-2008962 | SA2 TR 23.752 [4] has captured two L3 U2U solutions which are transparent to AS layer: - Solution#10 based on IP routing - Solution#32 based on IPv6 link-local addresses RAN2 confirm that L3 U2U solutions (i.e. Solution#10 and Solution#32 in TR 23.287) has no RAN2 impacts and leave the design to SA2. RAN2 capture the conclusion in RAN2 TR 38.836 |
| --- | --- |
| [2] R2-2009033 | - Capture the QoS solution#31 for L3 U2U relay of 23.752 in TR, and leave the details to SA2. - Security protection of L3 U2U relay can be realized by legacy sidelink method, IPSec or application implementation, those solutions have no RAN2 impacts and the choice of these solutions us up to SA2’s solution. |
| [6] R2-2008778 | In TR 38.836 section 5.6.3, add an editor-note for L3 U2U relay for the security section, i.e., “RAN2 needs to consider SA3 input.” |

Rapporteur think [1][2][6] basically are aligned that SA2 solutions of L3 U2U relay had no RAN2 impact and leave the design to SA2/SA3. Rapporteur would like to confirm whether it is accepted by majority company

Q10: For L3 U2U relay, do you agree that SA2 solutions of L3 U2U relay (Solution#10/ Solution#31/ Solution#32) have no RAN2 impact and leave the design to SA2?

| Company | Yes / No? | Comments (if you answer “No”, please list identified detailed impacts) |
| --- | --- | --- |
| MediaTek | Yes | The handling of L3 and L2 should be aligned for U2U relay procedure. For L2 U2U Relay, the corresponding SA2 solutions within TR23.752 have no RAN2 impact and the design is left to SA2. |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Futurewei | Yes with comments | L3 U2U study can be down-prioritized, and left to SA2 for now. It’d be more accurate to just say “no RAN2 impact is identified”. |
| Spreadtrum | Yes |  |
| vivo | Yes with comments | At this SI stage, no RAN2 impact has been detected. In the normative design, whether RAN2 works is needed can be further discussed in WI phase. |
| CATT | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Apple | Yes |  |
| Convida | Yes |  |
| ZTE | Yes |  |
| Qualcomm | Yes | OK to change to: No RAN2 impacts of SA2 solutions of L3 U2U relay (Solution#10/ Solution#31/ Solution#32) have no RAN2 impact are identified and leave the design to SA2 |
| Huawei | Yes | Fine with the wording proposed by Qualcomm. |
| Intel | Yes |  |
| LG | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

[1] [2] [6] have specific proposals on how to capture the conclusion in RAN2 TR. Rapporteur think there is no confliction of their proposals, and thereby intend to ask company whether it is acceptable:

Q11: For L3 U2U relay, do you agree the below proposals on how to capture conclusion in RAN2 TR:

- For QoS, capture that “The solution captured in SA2 TR 23.752 (solution#31) have no RAN2 impacts and the design is in the scope of SA2” [2]

- For Security:

  - Capture that “security protection of L3 U2U relay can be realized by legacy sidelink method, IPSec or application implementation, those solutions have no RAN2 impacts and the choice of these solutions us up to SA2’s solution” [2];

  - Capture an editor-note for L3 U2U security: “whether the SA2 captured solutions can satisfy the security requirement depends on SA3.” [3]

- For control plane procedure, capture “The solutions captured in SA2 TR 23.752 (e.g. solution#10 and solution#32) have no RAN2 impacts and the design is in the scope of SA2” [1]

| Company | Yes / No? | Comments (if you answer “No”, please indicate your justifications) |
| --- | --- | --- |
| MediaTek | Yes |  |
| OPPO | Yes |  |
| Samsung | Yes |  |
| Futurewei | Yes with the intention | It’d be more accurate to just say “no RAN2 impact is identified”, as RAN2 haven’t really spent time on L3 U2U relay. |
| Spreadtrum | Yes |  |
| vivo | Yes with comments | Suggestion on rewording as following: security protection of L3 U2U relay can be realized by legacy sidelink method, IPSec or application implementation, those end-to-end solutions have no RAN2 impacts and the choice of these solutions us up to SA2’s solution |
| CATT | Yes |  |
| Nokia | Yes |  |
| Lenovo | Yes |  |
| Apple | Yes |  |
| Convida | Yes |  |
| ZTE | Yes |  |
| Qualcomm | Yes | OK to use wording “no RAN2 impacts are identified”, i.e. “No RAN2 impacts of the solution captured in SA2 TR 23.752 (solution#31) are identified and the design is in the scope of SA2” “No RAN2 impacts of the solutions captured in SA2 TR 23.752 (e.g. solution#10 and solution#32) are identified and the design is in the scope of SA2” |
| Huawei | No to 2 a), Yes to others | For 2 a), we think we could say “the security solutions for L3 U2U are up to SA2 and SA3’s decision.” |
| Intel | Yes | Minor edit: those solutions have no RAN2 impacts and the choice of these solutions is up to SA2’s solution” [2]; |
| LG | Yes |  |
| Ericsson | Yes |  |
| Fraunhofer | Yes |  |
| Philips | Yes |  |
| InterDigital | Yes |  |

Summary of Q10 amd Q11

Companies’ opinions are basically aligned. Thus, Rapporteur suggest below proposal:

Proposal 11: No RAN2 impact of SA2 solutions on L3 U2U relay (Solution#10/ Solution#31/ Solution#32) is identified and the design is in the scope of SA2

Proposal 12: For L3 U2U relay, capture below conclusion in TR 38.836:

- In Section 5.6.2, capture that “No RAN2 impact of the solution captured in SA2 TR 23.752 (solution#31) is identified and the design is in the scope of SA2”

- In Section 5.6.3:

  - Capture that “security protection of L3 U2U relay is in the scope of SA2 and SA3. No RAN2 impact is identified”;

  - Capture an Editor-note: “whether the SA2 captured solutions can satisfy the security requirement depends on SA3.”

- In Section 5.6.4, capture “No RAN2 impact of the solutions captured in SA2 TR 23.752 (e.g. solution#10 and solution#32) is identified and the design is in the scope of SA2”

# Conclusion

Proposal 1: In L3 relay N3IWF solution (solution#23 in TR 23.752), RAN2 understanding is that remote UE’s NAS is sent over PC5/Uu-DRB and it can be supported without RAN2 impacts. Include it in the status report LS to SA2 (discussed in email discussion [601]).

Proposal 2: If any AS impact of NAS transport in solution#23 is identified by SA2, RAN2 can further discuss it in WI phase.

Proposal 3: For the IP header overhead of L3 U2N relay with N3IWF, RAN2 conclude that outer IP header on each hop can be compressed by ROHC "ESP/IP profile”, but the inner IP header can’t be compressed by the AS layer, whose impact could be evaluated by SA2.

Proposal 4: Include conclusion of IP header overhead of L3 U2N relay with N3IWF in the status report LS to be sent to SA2 (discussed in email discussion [601]).

Proposal 5: No AS impact is identified for SA2 QoS solution#24 and #25, for which legacy PC5-RRC procedure can be reused. Remove the following 2 Editor Notes in Section 4.6.2 of TR 38.836:

Editor note: RAN2 can discuss AS impacts related to SA2 specified QoS solutions.

Editor note: RAN2 further discuss whether it is sufficient to enforce E2E QoS via legacy PC5 RRC reconfiguration of SLRB and resource allocation.

Proposal 6: For L3 relay QoS management, RAN2 don’t intend to study the forward compatibility solution for multi-hop support.

Proposal 7: Both relay and remote UE can be in RRC_INACTIVE state.

Proposal 8: Same as LTE Prose relay, Remote UE RRC state is independent of its U2N relay operation and/or the RRC states of relay UE.

Proposal 9: For security of L3 U2N relay:

- Capture “As solution#23 of TR 23.752 with N3IWF is feasible to meet end-to-end security requirements, no AS level enhancements related to security are needed for this solution” in Section 4.6.3 of TR 38.836.

- Remove the below Editor notes in Section 4.6.3 of TR 38.836

“Editor note: whether other security solution is introduced depends on SA2.”

“Editor note: RAN2 will evaluate any impact in RAN2 scope from these solutions”.

Proposal 10: For control plane procedure of L3 U2N relay, remove the following two Editor Notes in Section 4.6.5 of TR 38.836. Further AS impacts (if any) can be discussed in WI phase.

“Editor note: FFS if there is RAN2 impact to support the related control plane procedures.

“Editor note: RAN2 will further consider procedures with RAN2 impact.”

Proposal 11: No RAN2 impact of SA2 solutions on L3 U2U relay (Solution#10/ Solution#31/ Solution#32) is identified and the design is in the scope of SA2

Proposal 12: For L3 U2U relay, capture below conclusions in TR 38.836:

- In Section 5.6.2, capture that “No RAN2 impact of the solution captured in SA2 TR 23.752 (solution#31) is identified and the design is in the scope of SA2”

- In Section 5.6.3:

  - Capture that “security protection of L3 U2U relay is in the scope of SA2 and SA3. No RAN2 impact is identified”;

  - Capture an Editor-note: “whether the SA2 captured solutions can satisfy the security requirement depends on SA3.”

- In Section 5.6.4, capture “No RAN2 impact of the solutions captured in SA2 TR 23.752 (e.g. solution#10 and solution#32) is identified and the design is in the scope of SA2”

# References

[1] R1-2007044, [102-e-LS-AI5-04] Discussion on NR SRS carrier switching

# Contact Information

| Company | Email |
| --- | --- |
| Qualcomm (Peng Cheng) | chengp@qti.qualcomm.comm |
| Futurewei (Hao Bi) | Hao.bi@futurewei.com |
| vivo(Jing) | liangjing@vivo.com |
| Convida (Zhuo Chen) | Chen.Zhuo@convidawireless.com |
| Huawei (Rui Wang) | wangrui46@huawei.com |
| Intel (Ansab Ali) | ansab.ali@intel.com |
| Ericsson (Tony) | antonino.orsino@ericsson.com |