3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

2 – 13 November 2020

Agenda item:	6.4.3

Source: 	Huawei (Rapporteur)

Title: 	Summary of [AT112-e][714][V2X] MAC reset CR (Huawei)

Document for:	Discussion and decision

# Introduction

This is the trigger of the following offline discussion:

Proposal 2: Discuss R2-2009217, R2-2010186, and R2-2010309 as corrections to MAC reset CR.

- Agreed.

- [AT112-e][714][V2X] MAC reset CR (Huawei)

Discuss the contributions listed in the above proposal 2 and prepare agreeable CR in R2-2010951 (discussion summary in R2-2010952 if needed). CR will be agreed by email. Deadline is 12:00pm 11/12/2020 (UTC).

Companies are requested to provide their views on the issues listed in this document.

# Remaining issues on Uu MAC reset

#### Issue 1: Initialize SBj for each logical channel to zero

In R2-2010309, it is proposed to initialize SBj for each logical channel to zero upon Uu MAC reset. Similar as initializing Bj, rapporteur think initialize SBj is needed especially for mode 1. If after MAC reset, Uu scheduling is not possible, then it makes no sense to continue to maintain the SBj.

Question 1:	Do companies agree to initialize SBj for each logical channel to zero upon Uu MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | See comment | Considering Uu MAC re-set implies cell change, i.e., mode-1 scheduler relocation, SBj may be re-initialized at network side, it may be motivated to reinitialize SBj at UE side as well. But it is not the case for mode-2 at least. It may not be an critical issue anyway, we are fine with it if limited to mode-1. |
| Ericsson (Min) | Yes | Support to initialize SBj to zero for Mode 1. |
| CATT | Yes | We share the same view as the rapporteur. |
| Intel | Yes | We are fine to initialize it to zero for mode1. |
| Apple | Yes |  |
| InterDigital | Yes |  |

#### Issue 2: Cancel triggered sidelink CSI reporting procedure

In R2-2010309 and R2-2009217, it is proposed to cancel if any, triggered sidelink CSI reporting procedure upon Uu MAC reset.

Question 2:	Do companies agree to cancel the triggered sidelink CSI reporting procedure upon Uu MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | We assume the proponents are considering the new cell may not provide grant to the UE to carry CSI report (which is limited to mode-1), but since the CSI report timer would cancel it finally anyway, to cancel it proactively as proposed seems not necessary, i.e., it is still possible that a new SR procedure at new cell can be used to apply for a SL grant in the new cell. |
| Ericsson (Min) | Yes |  |
| CATT | No | Share the same view as OPPO. |
| Intel | No | No strong view on this, but we slightly prefer to keep the existing text since as OPPO mentioned, the CSI report timer would eventually expire anyway |
| Apple | No | As OPPO explained, not cancelling it will still work. So, we slightly prefer to keep it as it is. |
| InterDigital | Yes | We think a cleaner solution is to cancel the triggered CSI, rather than relying on the timer to expire. The UE could then trigger a new CSI request if needed after the MAC reset. |

#### Issue 3: Reset all numConsecutiveDTXs

In R2-2010309 and R2-2009217, it is proposed to reset all numConsecutiveDTXs upon Uu MAC reset.

Question 3:	Do companies agree to reset all numConsecutiveDTXs upon Uu MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | numConsecutiveDTXs Is used for PSFCH monitoring, i.e., even if the UE fails to get a SL grant for PSSCH from new cell in time, it would not affect PSFCH monitoring (i.e., no need for PSFCH monitoring if no PSSCH transmission), so our view is the Uu-MAC reset does not require the proposal here. |
| Ericsson (Min) | Yes |  |
| CATT | No | numConsecutiveDTXs is a count which is used to judge HARQ-based Sidelink RLF. Upon Uu MAC reset, the SL re-transmission associated with the HARQ process is still executed. Thus, the numConsecutiveDTXs still needs to be maintained to judge the SL RLF for the PC5 RRC connection. |
| Intel | No | Agree with CATT |
| Apple | No | Agree with CATT |
| InterDigital | No | SL RLF counting and Uu MAC reset are not related, so we see no need to reset the counter at MAC reset. |

# Remaining issues on sidelink MAC reset

#### Issue 4: Initialize SBj for each logical channel associated to the PC5-RRC connection to zero

In R2-2010319, it is proposed to initialize SBj for each logical channel associated to the PC5-RRC connection to zero upon sidelink MAC reset.

Question 4: Do companies agree to initialize SBj for each logical channel associated to the PC5-RRC connection to zero upon sidelink MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson (Min) | Yes |  |
| CATT | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |

#### Issue 5: Cancel triggered sidelink CSI reporting procedure associated to the PC5-RRC connection

In R2-2010309 and R2-2009217, it is proposed to cancel if any, triggered sidelink CSI reporting procedure associated to the PC5-RRC connection upon sidelink MAC reset.

Question 5:	Do companies agree to cancel the triggered sidelink CSI reporting procedure associated to the PC5-RRC connection upon sidelink MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | Yes |  |
| Ericsson (Min) | Yes |  |
| CATT | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |

#### Issue 6: Stop all timers associated to the PC5-RRC connection

In R2-2010309 and R2-2010186, it is proposed to stot all timers if running associated to the PC5-RRC connection upon sidelink MAC reset.

Question 6:	Do companies agree to stop (if running) all timers associated to the PC5-RRC connection upon sidelink MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | See comment | Besides the sl-CSI-ReportTimer, is there any other timers as “timers associated to the PC5-RRC connection”? If No, whether the operation in Q5 is sufficient already? |
| Ericsson (Min) | See comment | Agree with OPPO. Q5 should be sufficient. |
| CATT | See comment | Same comment as OPPO. |
| InterDigital | No | Q5 is sufficient. |

#### Issue 7: Reset the numConsecutiveDTX associated to the PC5-RRC connection

In R2-2010309 and R2-2009217, it is proposed to reset the numConsecutiveDTX associated to the PC5-RRC connection upon sidelink MAC reset.

Question 7:	Do companies agree to reset the numConsecutiveDTX associated to the PC5-RRC connection upon sidelink MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| Ericsson (Min) | Yes |  |
| OPPO | Yes |  |
| CATT | Yes |  |
| Intel | Yes |  |
| Apple | Yes |  |
| InterDigital | Yes |  |

#### Issue 8: Set the NDIs for all HARQ process IDs to the value 0 associated to the PC5-RRC connection

In R2-2010186, it is proposed to set the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1 associated to the PC5-RRC connection upon sidelink MAC reset.

Question 8:	Do companies agree to set the NDIs for all HARQ process IDs to the value 0 associated to the PC5-RRC connection upon sidelink MAC reset?

- Yes.

- No (Please clarify the reason).

| Company | Yes/No | Comment |
| --- | --- | --- |
| OPPO | No | In case of sidelink MAC reset due to RLF or upper layer triggered PC5 release, this is not needed obviously. Or if Sidelink specific MAC re-set is triggered by counter-part TX-UE, network of RX-UE may not be aware of the operation, so that the operation related to Uu interface should be affected by the sidelink specific MAC re-set. |
| Ericsson (Min) | Yes |  |
| CATT | No | Agree with OPPO. |
| Apple | No |  |
| InterDigital | No |  |

# Reference

[1] R2-2009217 Correction to sidelink MAC reset, Ericsson

[2] R2-2010186 Correction to sidelink specific MAC reset, Samsung Electronics

[3] R2-2010309 Correction on the MAC reset, Huawei, Hisilicon