3GPP TSG RAN WG2#112-e	R2-2008717

Online meeting, 2nd - 13th November, 2020

3GPP TSG-RAN WG1 #102-e	R1-2007420

e-Meeting, August 17th – August 28th, 2020

# Title:	LS response on power sharing for LTE mobility enhancements

# Response to:	R1-2005211(R2-2006377)

# Release:	Rel-16

# Work Item:	LTE_feMob-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name: Claes Tidestav

Tel. Number: +46702672120

E-mail Address: claes.tidestav@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN1 would like to thank RAN2 for the LS on power sharing for LTE mobility enhancements.

RAN1 has discussed the LS and reached the following agreement:

In the LS, RAN2 asked the following questions:

Answer to Q1: RAN1 concluded that the LTE DC uplink power sharing mechanism (i.e., mode 1 and mode 2) is applicable for inter band LTE DAPS HO, i.e., for some inter-frequency cases.

Answer to Q2: An IE with the same fields as PowerCoordinationInfo-r12 (up to RAN2 whether to reuse PowerCoordinationInfo-r12 or define a new IE) is introduced to configure UL power sharing for LTE DAPS. The IE would contain the fields (up to RAN2 to change field names):

- p-MeNB-r12:		INTEGER (1..16): power sharing parameter for target eNB.

- p-SeNB-r12:		INTEGER (1..16): power sharing parameter for source eNB.

- powerControlMode-r12:	Indicates the power sharing mode

In addition, optional UE capabilities for the support of UL power sharing for LTE DAPS are required:

- Support of power sharing for sync DAPS.

- Support of power sharing for async DAPS.

A UE that supports LTE DAPS HO optionally supports power sharing via these capabilities.

2. Actions to RAN2:

RAN1 respectfully asks RAN2 to take the above information into consideration

3. Date of next TSG RAN WG1 Meetings:

TSG RAN WG1 Meeting #103-e	26th October – 13th November, 2020, online.

TSG RAN WG1 Meeting #104	1st – 5th March, 2021		GR