3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-2101048

Elbonia, 25 January – 5 February 2021	Revision of R2-2009981

Agenda item:	8.10.2.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on 2-Step RACH adaptation in NTN

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2#111e, the following was agreed regarding the inclusion of 2-step RACH in Rel-17 NTN [1]:

Agreements:

- Both 2-step and 4-step RACH are supported in Rel-17 NTN. FFS enhancements to RACH to accommodate the NTN environment.

If both 2-step and 4-step RACH are supported in one NTN cell, how to select RA type should be further studied on top of legacy RSRP threshold, to accommodate the NTN environment. It was agreed in RAN2-112e meeting that, the enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH should be studied.

Agreements via email - offline 103:

2. At least the following are FFS in Rel-17 NTN:

- Report UE-calculated TA in e.g. msg3/msg5/msgA

- Enhancements to RSRP-based selection mechanism of 2-step vs. 4-step RACH

- LCP impact caused by disabling HARQ UL retransmission

In this paper, we discuss 2-step RACH adaptation in NTN, including the 2-step and 4-step RACH RA type selection as well as MsgA PUSCH coverage enhancement.

# 2	Discussion

2.1			General

Support 2-step RACH procedure in NTN has been agreed in RAN2#111e meeting as one solution to reduce latency. In current specification, UE will select 2-step RACH if measured RSRP is above the threshold. E.g. the selection of 2-step RACH or 4-step RACH based on RSRP since higher channel quality and higher SINR may be desired for transmission of MsgA PUSCH for 2-step RACH which is specified in TS38.321 as below:

“1>else if the BWP selected for random access procedure is configured with both 2-step and 4-step RA type random access resources and the RSRP of the downlink pathloss reference is above RSRP_THRESHOLD_RA_TYPE_SELECTION; Or …………. 2> set the RA_TYPE to 2-stepRA;”

Based on the above principle, in cellular network, the UE in the cell center with higher RSRP will have more opportunity to select the 2-step RACH. However, in NTN deployments, since the near-far effect is not obvious or less pronounced compared to TN [2], all UEs may have similar range of RSRP levels (path loss in combination with shadowing and LOS/NLOS). Therefore, the RSRP criteria for RA type selection may lead to suboptimal situations where only a the UEs very close to the satellite beam centre would use 2-step RACH.

Figure 1: A sketch of near-far effect in different scenarios: (a) Terrestrial Network; (b) NTN [TR38.821]

Observation 1: RSRP based 2-Step selection criteria may be a suboptimal criterion in NTN, due to no obvious near-far effect in NTN.

In NTN, large differential delay could be experienced by two UEs within the same cell. The UE in the coverage center (e.g. nadir) and UE in coverage edge (e.g. small elevation angle) may have different transmission delay from UE to satellite. Let’s take the LEO satellite at 600km altitude as the example, the maximum delay is 6.44 ms for the UE with 10° elevation angle (e.g, α in figure 2), the minimum delay is 2 ms for the UE at nadir point as indicated in TR 38.811 Table 5.3.4.1-1.

Figure 2: The delay and distance between a satellite and a ground terminal

The motivation of 2-step RACH over 4-step RACH is that there is one round trip cycle between sending MsgA and receiving MsgB, instead of two round trip cycles between sending Msg1 and receiving Msg4, which leads to reduced latency for fast access and uplink scheduling. The longer the round trip delay, the more latency reduction can be achieved with 2-step RACH. Based on current criteria to select between 2-step RACH and 4-step RACH, the cell edge user which have large transmission latency have lower opportunity to use 2-step RACH since the RSRP is lower (maybe not much per observation 1), but these cell edge user should be given higher priority to use 2-step RACH to reduce the large latency. Therefore, it seems reasonable to select cell/beam edge UE to use 2-step RACH to reduce long latency with new criteria. However, the channel conditions for the beam edge UE still need to be considered. At lower elevation angles (see Figure 2) the probability of NLOS and deeper shadowing increases, hence the RSRP levels might be lower. This increases the likelihood of 2-step RACH failures, and lead to wasted radio resources.

Observation 2: UE located at cell edge can reduce latency more than UE at cell center by using 2-step RACH if UEs have similar coverage, considering no obvious near-far effect in NTN.

For the criteria to identify UE with long transmission delay at cell edge, it can be based on the location of the UE and the location of the serving satellite beam, which result in different transmission latency as well as distance between UE and satellite. For example, if UE estimated transmission latency is above a threshold configured by gNB, UE should select 2-step RACH to reduce latency, provided the measured RSRP level is above a certain threshold.

Furthermore, service QoS requirement (e.g. delay) may be quite different from different type of NTN UEs which is up to the upper layer application requirement. 4-step RACH should be selected for delay-tolerant service while 2-step for time-critical service. The UE can select RA type based on its QoS requirement and the threshold configured by gNB.

Proposal 1: RAN2 to discuss new criteria for 2-step RA selection, such as UE’s QoS (e.g. latency) requirements and/or the transmission delay/distance between UE and serving satellite in combination with an NTN specific RSRP threshold criteria.

2.2			Intra-satellite vs. inter-satellite

The optimal RA selection should also consider the intra-satellite or inter-satellite scenario.  For intra-satellite scenarios the source and target cells are generated from the same satellite, and therefore the propagation delays and radio conditions on the two Uu interfaces are likely to be the same. In intra-satellite scenarios, a 2-step RACH procedure is more suited because the UE does not need to re-adjust its timing advance and, additionally the transmission of MsgA is more likely to succeed. For inter-satellite scenarios, the source and target cells are generated by different satellites (possibly different feeder links) and therefore the propagation delays and radio conditions on the two Uu interfaces are in general very different. Therefore, it may be advantageous to use 4-step RACH for this scenario.

For example, in RRC_CONNECTED mode, the intra/inter-satellite hand-over cases can be identified simply by using the NR cell ID (PCI , GCI). The source gNB can determine if the measurement reports from the UE corresponds to a cell from the same satellite or different satellite. The UE then can be instructed to perform 2-step RACH if it is an intra-satellite handover; else the default configured 4-step RACH is to be used by UE.

Proposal 2: RAN2 to discuss the 2-step and 4-step RACH selection mechanism on the intra-satellite or inter-satellite scenarios.

2.3			MsgA PUSCH coverage

In RAN2-111e meeting, RAN2 also agreed to study UL scheduling enhancement that UE carry BSR in MsgA PUSCH, to reduce UL scheduling latency.

Agreements via email - from offline 107: 1. At least the following methods to enhance UL scheduling are further studied in NTN: configured grant and BSR over 2-step RACH. (other solutions to enhance UL scheduling are not precluded)

In NTN SI, two other potential enhancements for MsgA were discussed and captured in TR 38.821 as below.

- UE may include assistance information (i.e. value of TA applied by UE) in MsgA PUSCH payload.

In 7.2.1.1.2 2-Step RACH Procedure …for UE with location information to perform initial timing advance…, while in 2-step random access procedure, UE can include some assistance information in the PUSCH payload for network to know the value of TA applied by UE.

- UE may include assistance information (i.e. SFN index) in MsgA PUSCH payload.

Possible solution of problem of ambiguity on preamble reception at the network side: For the case when 2-step RACH is used, assistance information, e.g., SFN index can be included in MsgA to help network link the received preamble to the corresponding RO.

Although the above described assistance information in the PUSCH (e.g. SFN index, BSR, or value of TA applied by UE) may benefit 2-step RACH, it really adds much more payload to MsgA PUSCH. For example, the minimum Rel-16 NR MsgA PUSCH payload size is 56/72 bits with content which includes the same information that can be conveyed in Msg3 for 4-step RACH, however, for new assistance information, it seems the payload size is quite large. e.g.

- BSR (if long BSR information is agreed): (8+8*LCG number) bits, with maximum 72 bits when LCG#=8.

- Absolute TA value: TBD bits to be addressed by RAN2. (12 bits for 2ms in legacy NR, but now NTN UE need to support up to 541ms)

- Assistance SFN information: 10 bits

To avoid 2-step RACH fallback to 4-step RACH, both the MsgA PRACH and MsgA PUSCH should be decoded successfully by gNB. Otherwise, the network would order the UE to fallback to 4-step RA type by transmitting a required grant for transmission Msg3, which would therefore not reduce the delay in RACH. The increased MsgA PUSCH payload may reduce the MsgA PUSCH coverage while it has no impact to MsgA PRACH coverage. Therefore, for a correct 2-step RACH operation it is important to improve MsgA PUSCH coverage to keep both channels having similar link budget/coverage.

Observation 3: Regardless of the increased MsgA PUSCH payload, in order to reduce the RACH delay, the MsgA PUSCH coverage should be kept similar to the MsgA PRACH to avoid the UE falling back from 2-step RACH to 4-step RACH.

In order to achieve similar link budgets (coverage) for MsgA PUSCH and MsgA PRACH with increased MsgA PUSCH payloads, one approach for gNB is to assign more resource elements (e.g. increase the amount of PRBs assigned) to the UE for MsgA PUSCH transmission following MsgA PRACH preamble, which makes it possible to select robust MCS and support larger TBS in one shot PUSCH transmission. The side effect of high PRB reservation in a single TTI is that, it may restrict the scheduling flexibility for network as less PRBs remain available for data transmission in this TTI. Especially from resource management point view, where the reserved MsgA PUSCH PRBs are used only when RACH is triggered,  the increased number of reserved PRBs reduces the overall capacity of the system.

Another approach to achieve similar link budgets (coverage), is to support MsgA PUSCH retransmissions as for legacy PUSCH, e.g. following a PRACH preamble, there are multiple PUSCH transmissions with same or different RV version, with soft combination gain of multiple transmissions thus reduce PRB reservation for each PUSCH transmission.

Proposal 3: To support the increased MsgA PUSCH payload in NTN, MsgA PUSCH retransmissions may need to be considered in 2-step RACH.

# 3	Conclusion

Based on the discussion, the following observations and proposals have been made.

Observation 1: RSRP based 2-Step selection criteria may be a suboptimal criterion in NTN, due to no obvious near-far effect in NTN.

Observation 2: UE located at cell edge can reduce latency more than UE at cell center by using 2-step RACH if UEs have similar coverage, considering no obvious near-far effect in NTN.

Observation 3: Regardless of the increased MsgA PUSCH payload, in order to reduce the RACH delay, the MsgA PUSCH coverage should be kept similar to the MsgA PRACH to avoid the UE falling back from 2-step RACH to 4-step RACH.

Proposal 1: RAN2 to discuss new criteria for 2-step RA selection, such as UE’s QoS (e.g. latency) requirements and/or the transmission delay/distance between UE and serving satellite in combination with an NTN specific RSRP threshold criteria.

Proposal 2: RAN2 to discuss the 2-step and 4-step RACH selection mechanism on the intra-satellite or inter-satellite scenarios.

Proposal 3: To support the increased MsgA PUSCH payload in NTN, MsgA PUSCH retransmissions may need to be considered in 2-step RACH.

# 4	Reference

- R2-2008122 – “Report from Break-out session on R16 eMIMO, CLI, PRN, RACS and R17 NTN and REDCAP” – RAN2 Vice Chairman (ZTE Corperation)

- 3GPP TR 38.821 V16.0.0, Solutions for NR to support non-terrestrial networks (NTN), December 2019.