3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2102016

Online, January 25th - February 5th, 2021

Agenda item:	8.10.3.3

Source:	Ericsson

Title:	Report of [AT113-e][106][NTN] CHO aspects (Ericsson)

Document for:	Discussion & Decision

# 1Introduction

New trigger conditions were agreed for CHO in RAN2#112-e meeting and this email discussion is to understand further the needed procedures and other details when specifying the CHO for NTN:

- [AT113-e][106][NTN] CHO aspects (Ericsson)

Scope: Discuss CHO aspects based on the proposals in R2-2100346 (P1~P10), R2-2101197, R2-2101708, R2-2100383, R2-2100744 and R2-2101129

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Monday 2021-02-01 17:00 UTC

Initial deadline (for rapporteur's summary in R2-2102016../../../../../../../Data/3GPP/archive/RAN2/RAN2%23112/Tdocs/R2-2010761.zip): Monday 2021-02-01 23:00 UTC

Proposals marked "for agreement" in R2-2102016 not challenged until Tuesday 2020-02-02 11:00 UTC will be declared as agreed by the session chair. For the rest the discussion will continue online.

# 2 Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| Nokia | Jedrzej Stanczak | jedrzej.stanczak[at]nokia.com |
| Sony | Vivek Sharma | Vivek.sharma@sony.com |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |
| InterDigital | Dylan Watts | Dylan.Watts@interdigital.com |
| BT | Salva Diaz | salva.diazsendra@bt.com |
| ZTE | Yuan Gao | gao.yuan66@zte.com.cn |
| Apple | Sarma Vangala | svangala@apple.com |
| Qualcomm | Bharat Shrestha | bshrestha@qti.qualcomm.com |
| Rakuten Mobile | Awn Muhammad | Awn.muhammad@Rakuten.com |
| Lenovo | Min Xu | xumin13@lenovo.com |
| OPPO | Haitao Li | lihaitao@oppo.com |
| CMCC | Yuzhen Liu | liuyuzhen@chinamobile.com |
| Xiaomi | Yi Xiong | xiongyi3@xiaomi.com |
| Spreadtrum | Qufang Huang | Qufang.huang@unisoc.com |
| Thales | Camille Bui | Camille.bui@thalesaleniaspace.com |
| CATT | Sidong Li | lisidong@catt.cn |
| Samsung | Nishith Tripathi | nishith.t@samsung.com |

# 3 Background

Agreements related to CHO from RAN2#102:

Agreements:

- RAN2 to consider the case where gNB is co-located at the GW with higher priority.

- RAN2 will continue working with the assumption that service link switch implies L3 mobility (meaning that at least in case the SSBs are on the same sync raster point the PCIs need to be different). Check if an LS to RAN1 asking for feasibility of having same PCI as well can be agreed

Agreements

- Reconfiguration with sync is the baseline for connected mode mobility in NTN (the use of legacy RLF and re-establishment mechanism are not excluded)

- The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline for NTN CHO.

3.	NTN specific CHO execution condition can be further discussed.

4.	The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

5.	Legacy SSB periods (as in TN) shall be supported in NTN

Agreements via email - offline 105:

- Time or timer based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario.  FFS on how to configure the time or timer based CHO triggering event. Also FFS how to consider the feeder/service link switch timing.

- DAPS HO for NTN is de-prioritized in this release.

- Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario. FFS on how to configure the location based CHO triggering event. FFS if location based CHO triggering event only (not in combination with other events) can also be considered.

- The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

# 4 Discussion for adopting A4 event for NTN CHO

The existing CHO events A3 and A5 can serve as signal strength/quality measurement event for NTN CHO. In R2-2100346 it is proposed that it should also be considered if the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event. As the RSRP/RSRQ levels of different NTN cells may be similar and/or unpredictable w.r.t. location or time, which may possibly be configured together with RSRP/RSRQ events, it is seen beneficial to have possibility to have even that compares to absolute threshold rather than event that compares relative RSRP/RSRQ of two cells.

Q4.1) Do companies agree that the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Ericsson | Yes | Proponent of the proposal |
| Nokia | Yes | The NTN scenario justifies the introduction of A4 as a triggering event for CHO due to the likely small difference in RSRP/RSRQ levels between the serving and the neighbour. |
| Sony | Yes | We are fine to discuss additional trigger for CHO in NTN and think that event A4 is reasonable. |
| MediaTek | Yes |  |
| InterDigital | Yes | This would be a natural triggering event when combining measurement with another NTN-specific trigger (e.g. time + A4 or location + A4). |
| Huawei, HiSilicon | Yes |  |
| BT | No | The radio conditions experienced by UE connected to satellites are different from UEs connected into TN. For that reason, current mechanisms shouldn’t apply directly. A minor margin may have a severe impact in the network performance. |
| ZTE | Ok to discuss | We understand that A4+time/location event might be useful in NTN but would prefer to focus on what we have agreed earlier first, i.e. A3/A5 + time/location event. |
| Apple | Maybe | Depending on how the other triggers have to be propagated to the UE. |
| Qualcomm | Yes | For now we see even A4 would be sufficient when used together this time-based or location-based condition. |
| Rakuten Mobile | Yes |  |
| Lenovo | Yes | A4 can be used in combination with location or time. |
| OPPO | No | We don’t see clear benefit of A4 event for RSRP measurement, compared to existing A3/A5 event. |
| CMCC | Yes | It is fine to discuss A4 for CHO. |
| Xiaomi | Yes | Time or timer and location based CHO triggering event, in combination with the existing R16 CHO measurement (CondEvent A3 and CondEvent A5) based event should be a priority. Introducing A4 event can be used as an additional enhancement. Since the near-far effect is not obvious in NTN, we should study A4 event, in combined with time or timer and location based CHO triggering event. |
| Spreadtrum | Yes | A4 can be applied in combination with location. |
| Thales | Yes | We agree that A4 event should be defined as a CHO event. However, in NTN, the absolute value of RSRP/RSRQ doesn’t vary much across the area of cells in the satellite coverage. Hence it is not easy to define an absolute value threshold that is applicable for all cells. For this reason, we recommend that the A4 event considers the relative RSRP/RSRQ variation to detect the boundary between two neighbour cells. If the difference is higher than a threshold, than the A4 event is triggered. |
| CATT | Yes | We agree that the legacy event is the baseline of NTN, and we think it can be used together with time-based and location-based info. |
| Samsung | Yes- Pl. see Comments | In principle, we agree that neighbor cell RSRP would be very useful in combination with another trigger. In our view, a standalone A4 should not be used for CHO and traditional handover in an NTN due to its inadequacy. Up to this, RAN2 has identified following combination triggers. (A) Neighbor Cell RSRP AND time. (B) Neighbor Cell RSRP and location-based trigger. For (A), we suggest a combination trigger such as “Neighbor cell RSRP > Threshold” AND “timeSinceLastHandover > TimeThreshold.” This can be viewed as the combination of the A4 related signal measurement trigger and the time/timer trigger. For (B), we suggest a combination trigger such as “Neighbor cell RSRP > Threshold” AND “distanceFromServingCellCenter > distanceThreshold,” where distanceFromServingCellCenter is the distance of the UE from the center of the serving cell. We observe that different triggers would be suitable for Earth-fixed beams, quasi-Earth-fixed beams, and Earth-moving beams. For example, the combination trigger of neighbor RSRP and distance from the serving cell center would be quite useful for Earth-fixed beams and Earth-moving beams. Similarly, the combination trigger of neighbor RSRP and time/timer is quite useful for quasi-Earth-fixed beams and Earth-moving beams. Additionally, we note that multiple simultaneously-configured trigger conditions may be necessary to increase robustness and reliability of handover. We observe that the combination triggers are useful for all these scenarios: (i) traditional handover- early indication and regular indication, (ii) CHO measurement events, (iii) CHO execution conditions, and (iv) cell reselection. Since there would be multiple combination triggers, we suggest that RAN2 not define multiple new events for an NTN. RAN2 can perhaps define one new NTN event and define multiple trigger combinations inside a single event to support different scenarios. |

Yes 15,

no 2

maybe/ok to discuss 2

Clear majority supports supporting A4 event for NTN CHO

Proposal 1 support A4 event for NTN CHO

# 5 Discussion for time/timer trigger definition

For the time/timer based event, it should be discussed how the feeder/service link switch scenario is supported. For example, is it possible to support with one time/timer based event the case that current serving cell ceases to serve the area of the cell. There were three different proposals presented in R2-2100346, R2-2101197 and R2-2101708. It seems beneficial to try to understand the use and pros and cons for each proposal, as well as the implications for defining the time/timer event per serving cell versus per candidate target cell, or if both are needed. When event is defined per serving cell, UE needs to evaluate or monitor it “per UE”, whereas when event is defined per candidate target cell UE needs to evaluate it per monitored candidate target cell.

In R2-2100346, and the time/timer event is proposed to be defined as absolute time without TTT or hysteresis. As an option UTC time was mentioned though the absolute time can also be represented as e.g. frame number.

In R2-2101708, in Proposal 3 also an absolute time is proposed.

The difference between these two definitions for the absolute time seem to be that in R2-2101708, UE is assume to check the other associated condition for the CHO and if that condition is fulfilled by the absolute time, the CHO triggers. In R2-2100346 some options were provided but main thinking was that after the absolute time, when the associated other event triggers, UE executes the CHO.

In R2-2101708, in Proposal 2 a timer is suggested such that upon expiry of the timer UE checks if the other condition is fulfilled or not and decides on CHO. That is, if the other event had been fulfilled during the timer, the CHO triggers.

In R2-2101197, the thinking is similar that a time range is provided and if the other event is fulfilled during the time range, given by two absolute times or one absolute time and duration, the CHO triggers.

If it beneficial to understand use cases for the CHO to be able to discuss how the time related event should be defined and if other time related information is needed.

When feeder/service link switch happens, from serving cell point of view, at some point, the serving cell ceases to serve the area. Thus, connected mode UEs need to be handed over to another cell before this happens or otherwise the connection is dropped. For this, it might be beneficial to be able to configure the UE with a time by which the HO needs to happen. This time is not per candidate target cell but per serving cell. Actually, if UE is configured with timer/absolute time/time range related only to candidate target cell, the UE may unnecessarily delay the CHO and the serving cell may disappear before the UE has executed the HO.

The other issue that was identified during the SI was that it is beneficial to distribute when the HO to the target cell happens. For this, a UE specific, can also be seen as serving cell specific time can be given that is the first possible time when UE may consider HO. This time can be also given per candidate target cell to accommodate when that cell starts to be available.

Additionally, UE may be given further information on candidate target cell about when it is no longer available.

All the above may also be used for the moving beam LEO although setting the time values may be more challenging in practice.

Question 5.1

First question is related to whether UE is assumed to be configured with information when serving cell is going to stop serving the area. This can be given in CHO configuration (FFS where), or it can be given in elsewhere in the serving cell configuration. This means that it could be possible to either configure CHO with only this time, or to indicate in the configuration that by this time a HO needs to be executed. The use case is that network is assumed to have configured CHO towards the cells that are going to take over the same.

Q5.1) Do companies see beneficial that UE is assumed to be configured with information when serving cell is going to stop serving the area?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Ericsson | Yes | For conditional HO where network may not after configuration control when UE performs the HO it is beneficial for the network to give early a HO command, CHO, that network knows that by this time or at this time UE performs the HO to the upcoming cell. Use case example: Earth fixed NTN cell has appeard to serve area and has UEs connected to it. Network may configure UEs with CHO where some candidate target cells are real neighbor cells where UE may move to due to UE’s own mobility. These CHO configurations may even be without any time event. One candidate target cell is with CHO with potentially information about when that target cell becomes available and information when serving cell will disappear(or when serving cell wants that UE to make HO to the replacing cell). This option is like timed regular HO where network just decides HO for a UE but now it can be timed in advance(and different UEs may be timed differently to spread the load), hence it use be the CHO structure. |
| Nokia | In some cases | The discussion on that (in another thread) is not yet concluded and so far, based on the feedback collected before RAN2#113, slight majority was assuming such information is not needed. If A4 is considered for NTN CHO then serving cell’s availability may not be of utmost importance, but rather the timer should say when the new/CHO candidate cell is expected to be available. The information about when serving cell is going to stop serving the area could be more doable in Earth-fixed deployments. And anyway, it is likely not a part of this discussion (i.e. not a part of CHO config, but could be included in SI or ephemeris). |
| Sony | Yes |  |
| MediaTek | Not Always | The time a serving cell serves a UE will change depending on UE’s location within the beam footprint. As such measurement based events are more suitable for mobility. Timing information only makes sense for cases where the serving cell is known to change (e.g. feeder link switch or change of earth-fixed beam in LEO). |
| InterDigital | In some cases | For earth fixed cell and feeder-link switch when the time a cell changes is uniform for all UEs served by the cell this would be beneficial. For earth moving cell the time a cell appears/disappears is UE-specific and based on relative position between the UE and cell center. In this case a location or distance-based trigger (e.g. UE-cell center distance threshold) may be more suitable. |
| Huawei, HiSilicon | Partially Yes | We have agreed to introduce timer/time based CHO, and if this timer is related to “when serving cell is going to stop serving the area” is not so important. The exact value can be provided by network for each UE, e.g. for accommodating all UEs during a short handover period in feeder link switching scenario. So it’s not necessary to restrict the timer length is equal to service time, the value can be determined by network. |
| BT |  | It seems that more discussion is needed on this before we conclude it is beneficial or not. |
| ZTE | Not always | We understand the time range during which the candidate target cell is available would be more important to UE in CHO decision as it is better for UE to perform CHO to an available cell before the serving cell expires. Also for earth moving cell, the expire time of a cell would be different for UE in different areas. |
| Apple | Not always | From our understanding, the time and timer based CHOs aren’t necessarily needed to be related to the residual cell serving time. We definitely need to discuss this further. Based on the question Q5.1, the benfits of this parameter are limited. |
| Qualcomm | No | The time in time-based condition should imply when the target cell is going to appear, and the CHO command should be assumed valid. We are not sure why this is needed because if the target cell disappears, UE will meet the leaving condition of CHO, e.g. in even A4. |
| Rakuten Mobile | Not Always | Its not useful for Earth moving satellites as the “time” would depend on UE location in the cell. Useful for earth fixed Cells for feeder switch etc. |
| Lenovo | Not always | Time(r)-based execution condition for CHO can somehow reflect the time when serving cell is going to stop serving the area. In this case additional information is not necessary. |
| OPPO | No | For CHO, we should focus more on the time when UE can execute CHO. For serving cell stopping serving, it can be addressed by serving cell measurements. |
| CMCC | Not Always | For the timer/time based CHO, the information when serving cell is going to stop serving the area may be redundant. |
| Xiaomi | Not always | When time or timer based triggering event is configured to UE by network, UE can trigger CHO based on the time or timer information in the event. Although the information about “when serving cell is going to stop serving the area” can be used to set the value of the time or timer in the CHO trigger condition, the question about how to set the value can be solved by network implementation. In other word, the configuration of the time or timer based CHO event doesn’t have to be related to the service time information of the serving cell. |
| Spreadtrum | Partly Yes | It is reasonable at least for the fixed beam case. |
| Thales | Yes, but not for all scenarios | Information on when a cell is going to stop serving the area and information about the upcoming cells (PCIs, time or timer) should be provided to the UE. But, the triggering conditions work differently for each scenario (earth fixed, quasi fixed, moving cells). So we propose to separately evaluate CHO trigger condition and measurement for each type of cell. The time/timing triggering condition works for fixed UE. However, it may not be sufficient for moving UE because the HO best occurrence may also depend on UE’s speed and direction in this case. Timing/timer base triggering condition works in the following scenarios: - Both feeder link switch and service link switch due to satellite (resp. HAPS) switch-over in (quasi) earth fixed cell scenario - Only feeder link switch in earth moving cell scenario. In earth moving cell scenario, the ephemeris and timing information may not enough for the UE to determine when cells are active over a given area. . FFS if additional information about the cell geographical pattern are needed. |
| CATT | Not always | Case by case, we understand this timer is beneficial for earth fixed cell and feeder-link switch cases. Especially for hard feeder link switch, to let the UEs know when the serving cell will stop the service could avoid unnecessary RLF monitor before the target cell is activated. For the normal service link handover, it seems not necessary. The UEs should access to the target cell(s) as soon as possible when the target cell is available, e.g. in case of A4 condition is fulfilled. |
| Samsung | Yes/No: Pl. see Comments. | Since the connected mode UEs need to be distributed in time to avoid peak signaling load and peak random access channel loading, the absolute time on its own would be less useful. The gNB can provide a UE-specific timer threshold for quasi-Earth-fixed beams and Earth-moving beams. The gNB can distribute the UEs in time by choosing different UE-specific timers “TimeThreshold” in the example below. Here is a combination trigger that combines neighbor cell RSRP and timer: [(Neighbor cell RSRP > Threshold) AND (timeSinceLastHandover > TimeThreshold)]. |

Based on the responses it is not clear if the UE should be explicitly configured with a time when serving cell stops serving an area.

Question 5.2

Second question is related to whether UE is assumed to be configured with information when candidate target cell is going to start and or stop serving the area. This may be given in the CHO/HO configuration and it can be discussed further if and how this information translates into rules if and when CHO is executed. This information might also serve assisting information in case UE is moving and e.g. considering two target candidate serving cells where one of them is about to leave soon.

Q5.2) Do companies see beneficial that UE is assumed to be configured with information when candidate target cell is going to start and/or stop serving the area?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Ericsson | Yes | Especially information on when a cellcandidate target cell will start serving the area is useful. Optionally it could also have information when a given target cell is going to stop serving that area as that can also be used at UE when selecting target, e.g. in case UE itself is moving. |
| Nokia | Yes | That should be the primary focus of the timer-related CHO discussion. |
| Sony | Yes | We think multiple cells information e.g. when the corresponding target cell becomes available, the related timing information can be included in CHO command together with DRB setup message. |
| MediaTek | Not Always | Response is similar to Question 5.1 |
| InterDigital | Yes | For the scenarios listed in our Q5.1 comment this would be useful. Agree that when a target cell would start serving the area is most important information. |
| Huawei, HiSilicon | Partially Yes | Similar comments as on Q5.1. Anyway it’s a timer/time configured by network, and it’s not necessary to restrict it must be related to “when candidate target cell is going to start and/or stop serving the area.” |
| BT |  | It seems that more discussion is needed on this before we conclude it is beneficial or not. |
| ZTE | Yes | If A3/A5+time is considered for NTN CHO, the time range showing availability of the candidate target cell would be useful. For example, when A3+time range is configured, the a3-Offset can be configured as a smaller value compared to the case when only A3 is configured for CHO. In this case, the A3 event is relaxed a little bit and CHO can be triggered earlier with both the A3 and time range showing availability of the candidate target cell taken into consideration. |
| Apple | Not always | Please see response in Q5.1 |
| Qualcomm | No | See our response in Q5.1. |
| Rakuten Mobile | Yes | Yes, this is beneficial for earth fixed cells. |
| Lenovo | Not always | Time(r)-based execution condition for CHO can somehow reflect the time when candidate target cell is going to start and/or stop serving the area. In this case additional information is not necessary. |
| OPPO | No | For CHO, what matters is the timing for CHO execution. In our understanding, this is up to network, e.g. to balance the CHO execution load, and may not necessarily be related to the timing when target cell is going to start serving the area. |
| CMCC | Not Always | Similar to Q5.1, the mentioned information may not be needed for the timer/time based CHO. |
| Xiaomi | Not always | Similar to the comment in Q5.1, the configuration of the time or timer based CHO event doesn’t have to be related to the service time information of candidate target cell. |
| Spreadtrum | Partly yes | It is reasonable at least for the fixed beam case. |
| Thales | Partly yes | This information should be provided for UEs for triggering CHO in the following scenarios. - Earth fixed cells during satellite switch-over and feeder link switch-over - Earth moving cells during feeder link switch-over only The UE shall trigger CHO to connect to the upcoming cell with the new PCI. This could help to control the number of CHO over time. |
| CATT | Partially Yes | It seems beneficial to use the start time of the serving cell for several cases, but the stop time of the target cells may not be needed. |
| Samsung | No | In practice, it would be difficult to define “what physical area” a candidate cell will serve, because there are no “fixed-Earth cells” for quasi-Earth-fixed beams and Earth-moving beams. It is not practical to relate a cell’s coverage area to an exact Earth-fixed cell, especially when there are cell or beam overlaps. It is better to let the gNB configure a good candidate list and a time threshold. |

Based on the responses it is not clear if the UE should be explicitly configured with a time when target candidate cell starts or stops serving an area.

Question 5.3

The next set of questions are related to how time trigger should be understood in CHO configuration and what options should be supported.

Q5.3) Should it be possible to configure UE with time(see Q5.4, Q5.5 for details) which is per serving cell and upon which UE makes HO? That is, the time is per “RRCReconfig->conditionalReconfig”. If yes, should the specification support option that this be configured without RSRP/Q related event? (That is, specification is not restricted to mandate related RSRP/Q event)

| Company | Yes/No | Comments (reasoning if limitation is suggested) |
| --- | --- | --- |
| Ericsson | Yes to both | For the below use case it is needed and it is needed to be able to configure it without RSRP/RSRQ event. This is for UEs conneting, e.g. from idle mode, to a cell that wants to make timed HO without waiting RSRP/RSRQ event to be fulfilled, possible for replacing cell case. Use case example copied from above: Earth fixed NTN cell has appeard to serve area and has UEs connected to it. Network may configure UEs with CHO where some candidate target cells are real neighbor cells where UE may move to due to UE’s own mobility. These CHO configurations may even be without any time event. One candidate target cell is with CHO with potentially information about when that target cell becomes available and information when serving cell will disappear(or when serving cell wants that UE to make HO to the replacing cell). This option is like timed regular HO where network just decides HO for a UE but now it can be timed in advance(and different UEs may be timed differently to spread the load), hence it use be the CHO structure. |
| Nokia | No | The RSRP/RSRQ event should be also met before the UE executes any CHO configuration associated with a timer. |
| Sony | Yes for both |  |
| MediaTek | Yes, but | However, this must be configured with RSRP/RSRQ event, as there is no point in attempting a handover without finding a cell, even if the timer is configured. Hence, it must be a joint configuration. |
| InterDigital | Yes for both | Satisfying both and RSRP/RSRQ + time trigger would be ideal, but we see benefits to having a purely time-based CHO trigger as a “fallback”-like event. As mentioned by the rapporteur this would be useful to prevent UE from delaying execution of CHO so long that serving cell disappears, or to spread out UEs in time to prevent RACH collision as mentioned in SI. Another options would be to configure time + low threshold A4 event, but this seems unnecessarily complicated. |
| Huawei, HiSilicon | Yes | Time alone CHO could be an option. If network ensure the RSRP/Q can be met, a time alone CHO can be configured by network. And we are also fine with the combination of time and RSRP/Q as a CHO trigger. |
| BT | Yes but | This requires an accurate UE position. |
| ZTE | No | As mentioned above, we understand it is better to provide the valid time range of each candidate target cell and the RSRP/RSRQ/SINR event configured together should be fulfilled when CHO is triggered. |
| Apple | Yes but | As some of the othes have suggested, this should be a joint configuration. We have provided some analysis in R2-2100882 on why individual triggers might not work for all possible configurations. |
| Qualcomm | No | Time is when target cell appears, i.e., when UE should start evaluating CHO for the target cell (not make HO). Also, the time configuration should be configured with event like A4. |
| Rakuten | Yes to both |  |
| Lenovo | Yes to both | Both “Time OR RSRP/Q” and “Time AND RSRP/Q” can be considered for NTN. |
| OPPO | Yes but | We have agreed to take RSRP/RSRQ event as baseline, and we think to consider time/timer, it should be a joint configuration. |
| CMCC | Both are ok |  |
| Xiaomi | Yes to both | We support that the time can be configured per “RRCReconfig->conditionalReconfig” and be configured without RSRP/Q related event, which is suitable for the scenario of feeder/service link switch. In the scenario, the timing of feeder/service link switch can be predicted by network based on ephemeris information and the location of ground GW. In the duration of service/feeder link switch, all UEs in the same cell will be handed over from the same serving cell to the same target cell. So, network can configure only one candidate target cell and the trigger time based on the feeder/service link switch timing and UE can access the target cell at the right time base on the time information. |
| Spreadtrum | No | The NLOS may be possible for NTN cell, so the RSRP/RSRP is also useful. |
| Thales | Yes | The specification should be able to configure the time event independently with the RSRP/Q event. |
| CATT | No | Time-based event should be used with RSRP/Q together. When the timer reaches the pre-configured threshold, and the RSRP/Q of the target cell could satisfy the CHO condition. . |
| Samsung | No | We do not recommend a standalone time trigger for handover. Without a combination trigger that uses neighbor cell RSRP, the suitability of the candidate cell for handover cannot be reliably determined. Hence, we recommend the use of RSRP while evaluating the CHO execution condition. Note that the neighbor cell RSRP measurements obtained via a Measurement Report are not suitable for CHO execution due to a significant delay between the measurement report and CHO execution. |

10 companies supported allowing an option in the specification that the “time” trigger can also be configured without RSRP/RSRQ.

8 companies think it needs to be fixed in the specification that network always configured RSRP with the “time” based trigger.

1 company commented against being able to configure CHO with a ”time” that is per “RRCReconfig->conditionalReconfig”.

Proposal 2 Support having the option to configure the ”time” trigger per “RRCReconfig->conditionalReconfig”. FFS if allowed to be configured without RSRP/RSRQ related event.

Note: this does not preclude networks always configuring time trigger with RSRP/RSRQ event if that is what the network wants to configure

Q5.4) If yes to Q5.3, should the “time” be starting time, expiry time, time range or depending on configuration?

| Company | Comments |
| --- | --- |
| Ericsson | Any combination that does the trick. For example it could expiry time and the network would ensure there is then at least one candidate target that does not have conflicting trigger conditions. |
| Sony | Starting time/expiry time/timer |
| MediaTek | A starting time or a time range would be useful for the UE to start searching for the neighbour cell. |
| InterDigital | Agree with Ericsson (e.g. open to considering multiple options) |
| Huawei, HiSilicon | If it is an absolute time, it should be the starting time, i.e. UE starts to check if a handover is triggered. If it is a timer, when the timer expires, UE checks if a handover is triggered. |
| BT | More evaluation is required to find the optimal value. |
| Apple | More evaluation is needed. |
| Rakuten | No strong opinion, any timer would work fine. |
| Lenovo | Open to consider each option. |
| OPPO | More evaluation is needed. |
| CMCC | All the solutions need to be discussed. |
| Xiaomi | We prefer the “time” is depending on configuration. Different mobility scenarios are suitable for configurations with different types of “time”. |
| Spreadtrum | Open to all the three options. |
| Thales | The timing information should configure the following information: - the stop serving time / timer of the serving cell - the start serving time / timer of the upcoming cell If the start time is before the expiry time, it is make before break (soft switch) If the start time is after the expiry time, it is break before make. (hard switch) Both options should be covered by the standard. |
| CATT | It’s related to what time should be introduced, e.g. the start/stop time of the serving cell, the start/stop time of the target cell, this is pending to the discussion of Q5.1 and Q5.2. Both absolute time or SFN should be feasible for time configuration. Details need further evaluation. |
| Samsung | Per gNB configuration. Time should be per gNB configuration to distribute handover for quasi-Earth-fixed beams. Timer and RSRP thresholds, as part of a combination trigger, should be different for CHO measurement reporting and CHO execution condition. Time is irrelevant for Earth-fixed beams. |

Based on the comments further discussion is needed which definition of the ”time” would be suitable when ”time” is configured per “RRCReconfig->conditionalReconfig”

Proposal 3: RAN2 to discuss which definition of the ”time” would be suitable when ”time” is configured per “RRCReconfig->conditionalReconfig”.

Q5.5) When configured with RSRP/RSRQ event, CHO triggers, when start time and RSRP/Q event fulfils, or only after expiry time and RSRO/Q event fulfils?

| Company | Comments |
| --- | --- |
| Ericsson | When this time is per serving cell the more natural is expiry time although it seems better to allow CHO when RSRP/Q fullfills even that would be earlier than expiry time and latest at expiry time. |
| Nokia | In principle the condition associated with the target needs to be fulfilled. It may not be optimal to wait until the last moment, i.e. when source cell expires. |
| Sony | Agree with Ericsson |
| MediaTek | It is not clear to us what is being asked in the question. When jointly configured, both the time and the RSRP event must be met. |
| InterDigital | Agree with Nokia |
| Huawei, HiSilicon | The question is not so clear. But anyway we prefer “for a timer, only after expiry time and RSRO/Q event fulfils” |
| BT | Agree with Nokia but it is important to clarify the conditions. |
| ZTE | We understand CHO will be triggered within the valid time range of the candidate target when the RSRP/RSRQ event is also fulfilled. |
| Apple | Either of the conditions should be fine. As a further comment, this criteria might be left to the UE to decide in order to ensure a handover is complete instead of waiting for network triggers. |
| Qualcomm | The time should only determine when UE should start evaluating the CHO. The existing entering and leaving condition (for example using event A4) should still be applicable for the target cell. |
| Lenovo | If the timer is configured to be expired upon serving cell expiration, we think “start time and RSRP/Q event fulfils” is more reasonable. |
| OPPO | Not sure what this question is asking. Simply, CHO is executed when all configured events are fulfilled. |
| CMCC | From our perspective, the CHO should be triggered before the expiry time with the RSRP/RSRQ is fulfilled. |
| Xiaomi | About time or timer based CHO triggering event, we should discuss how to configure the value of time or timer firstly. And then, we can discuss some details in the event, such as the question. |
| Spreadtrum | CHO is triggered when one of configured events is fulfilled. |
| Thales | Timing and RSRP/RSRQ events are not used in the same scenarios. So they are “OR” conditions Timing event is used in the following scenarios: - Earth fixed cells, fixed UE, during satellite switch-over and feeder link switch-over - Earth moving cells during feeder link switch-over only If the start time is before the expiry time, it is make before break If the start time is after the expiry time, it is break before make. Both options should be covered by the standard. RSRP/RSRQ event is used in: - Connected mode mobility for earth moving beam when the beam no longer serves the UE - Connected mode mobility for both earth moving and earth fixed beam due to UE movement If the timing/timer condition is triggered, then the UE should make the HO to the upcoming cell, regardless the RSRP/Q event. Once the UE is connected to the new cell, then it will start to monitor the RSRP/Q again (in ignoring the served cell) |
| CATT | Agree with Nokia, the UE should handover to the target cell when condition is fulfilled, no need to wait for the last moment. |
| Samsung | In our view, like a TN, one set of thresholds can be used for CHO measurement triggering and another set of thresholds can be used for CHO execution. For CHO execution, RSRP of the neighbor cell would perhaps be slightly more important than a timer. However, having a timer is valuable to distribute signaling load in case of quasi-Earth-fixed beams. |

The views and undertanding how the ”time” trigger should be defined in relation to RSRP/RSRQ event seem to vary such that almost each company had their own view.

Proposal 4: RAN2 to discuss how the ”time” based trigger is definied with respect to RSRP/RSRQ event when ”time” is configured per “RRCReconfig->conditionalReconfig”. Some options mentioned

- There is starting time per candidate cell when to UE should start evaluating an event

- There is time/timer for evaluating whether event was fullfilled

- Time range within which RSRP event needs to fullfill

- It is left to the UE

Q5.6) Should it be possible to configure UE with time(see Q5.7, Q5.8 for details) which is per candidate target cell and upon which UE makes HO? That is, the time is per “RRCReconfig->conditionalReconfig->CondReconfigID”. If yes, should the specification support option that this be configured without RSRP/Q related event? (That is, specification is not restricted to mandate related RSRP/Q event)

| Company | Yes/No | Comments (reasoning if limitation is suggested) |
| --- | --- | --- |
| Ericsson | depends | Here the need to configure without RSRP/Q is less important. It will depend how these candidate target cell times are going to be defined. |
| Nokia | No | The timing per candidate cell is OK, but jointly with the RSRP/RSRQ based condition (e.g. A3, A4). |
| Sony | Yes |  |
| MediaTek | Yes, but | See our response similar to Q 5.3. |
| InterDigital | Neutral | This scenario is less critical than Q5.3. We are open to further study, but think serving cell case is higher-priority. |
| Huawei, HiSilicon | Yes | It is reasonable to make the time per candidate cell. As the starting time should align with the beginning of serving time of a cell, therefore it is different between cells. |
| BT | Depends | Time configuration is not clear so more discussion is required. |
| ZTE | Yes for per CondReconfigID, No for configured without RSRP/RSRQ event |  |
| Apple | Depends | More discussion is needed . |
| Qualcomm | Partly yes | upon time expiry, UE does not make HO but starts evaluating CHO condition. So it has to be configured together with event like A4. But yes to configure time per candidate cell. |
| Rakuten Mobile | Yes, but | All scenarios should be considered in further discussion. |
| Lenovo | Yes to both | With network implementation the time(r) can be configured in association with candidate cell strength or not. Therefore both “Time OR RSRP/Q” and “Time AND RSRP/Q” can be considered for NTN. |
| OPPO | No | See our comments to Q 5.3. |
| CMCC |  | All possible options should be studied. |
| Xiaomi | Yes, but | More discussion is required about the detail of time or timer configuration. |
| Spreadtrum | Yes | The time based CHO shall be configured with RSRP/Q related event. |
| Thales | No | For scenarios in which the time triggering event could be used, the timing information of serving cell and candidate target cell should be indicated together. For one serving cell, we have only one candidate cell. If the UE is moving from a cell to a neighbour cell, so it should use RSRP/Q triggering event. |
| CATT | No | Share the view with Nokia, even if the RSRP/Q is not so important for NTN handover. We should guarantee the RSRP/Q of the target cell is available before handover to that cell. So the timer and the candidate cell strength should be considered. |
| Samsung | No | RSRP would be quite important to ensure an adequate signal strength in the target cell. Hence, we suggest that RAN2 make RSRP mandatory. |

9 companies supporting per “RRCReconfig->conditionalReconfig->CondReconfigID” and no company was against this option

5 ok to configure without RSRP

7 companies would prefer to configure the “time” per “RRCReconfig->conditionalReconfig->CondReconfigID” with RSRP/RSRQ event

Proposal 5: Support having the option to configure the ”time” trigger per “RRCReconfig->conditionalReconfig->CondReconfigID”. FFS if allow it to be configured without RSRP/RSRQ related event.

Q5.7) If yes to Q5.6, should the “time” be starting time, expiry time, time range or depending on configuration?

| Company | Comments |
| --- | --- |
| Ericsson | It could be at least start time that tells when the candidate target cell becomes available as well as potentially end time after which UE should not choose this candidate target. |
| Sony | Starting time/timer |
| MediaTek | Yes, but |
| Huawei, HiSilicon | If it is an absolute time, it should be the starting time, i.e. UE starts to check if a handover is triggered. If it is a timer, when the timer expires, UE checks if a handover is triggered. |
| ZTE | Time range showing availability of the candidate target cell is preferred as the candidate target cell will not be available forever considering the satellite movement. |
| Lenovo | For candidate cell configuration the “expiry time” needs to be further clarified e.g. expiry time of serving cell or candidate cell? Open to consider each option. |
| Xiaomi | Depending on configuration. |
| Spreadtrum | Starting time or time range |
| Thales | Same as 5.4 |
| Samsung | If the majority view prefers time to be per cell, we suggest that it is kept per gNB configuration so that the gNB can distribute the load in time in a given cell for all the CHO requests it has received. |

Based on the comments further discussion is needed which definition of the ”time” would be suitable when ”time” is configured per “RRCReconfig->conditionalReconfig”

Proposal 6: RAN2 to discuss which definition of the ”time” would be suitable when ”time” is configured per “RRCReconfig->conditionalReconfig->CondReconfigID”

Q5.8) When configured with RSRP/RSRQ event, CHO triggers when start time and RSRP/Q event fulfils, only after expiry time of RSRO/Q event fulfils?

| Company | Comments |
| --- | --- |
| Ericsson | Start time that tells when RSRP/RSRQ event can trigger to this candidate cell and end time after which it cannot trigger to that candidate target. |
| Nokia | It shall trigger when the timing condition associated with the target cell is met and when the RSRP/Q event is fulfilled. |
| Sony | CHO execution should be the latest by the expiry of timer and before the timer expiry, it could be based on RSRP/Q. |
| MediaTek | It is not clear to us what is being asked in the question. When jointly configured, both the time and the RSRP event must be met. |
| InterDigital | for UE with time which is per candidate target cell, agree with Ericsson (e.g. a range where cell is valid) |
| Huawei, HiSilicon | Same question as Q5.5? |
| ZTE | We understand CHO will be triggered within the valid time range of the candidate target when the RSRP/RSRQ event is also fulfilled. |
| Qualcomm | please see our response in Q5.5. |
| Lenovo | If the timer is configured to be expired upon serving cell expiration, we think “start time and RSRP/Q event fulfils” is more reasonable. |
| OPPO | See our comments to Q5.5? |
| CMCC | Pls. to see our comments to Q5.5. |
| Xiaomi | See our response in Q5.5. |
| Spreadtrum | CHO shall be triggered in a time window. |
| Thales | Same as 5.5 |
| CATT | It should be triggered when the timing condition associated with the target cell is met and when the RSRP/Q event is fulfilled. |
| Samsung | We can have one set of thresholds for combination triggers for CHO measurements to facilitate early detection of potential CHO candidate cells and another set of thresholds for combination triggers for CHO execution. |

The views and undertanding how the ”time” trigger should be defined in relation to RSRP/RSRQ event seem to vary such that almost each company had their own view.

Almost same discussion as for Q5.5

Proposal 7: RAN2 to discuss how the ”time” based trigger is definied with respect to RSRP/RSRQ event when ”time” is configured per “RRCReconfig->conditionalReconfig->CondReconfigID”.

# 6 Discussion for location trigger definition

For the location based event, it should be discussed how the location is defined and how it is used together with other events. For the location trigger definition, the following options were identified:

- a: The distance between UE and the satellite, serving or to candidate target(which may be the same for a bunch of cells).

- b: The distance between UE and the cell center of a serving cell. Defines circle around serving cell.

- c: The distance between UE and the cell center of a candidate target cell. Defines circle around target candidate cell.

- d: The distance difference between “UE and center of serving cell” and “UE and center of candidate target cell”. Defines hexagonal cell areas

- e: Configure a geographical area scope referring to the coverage of the candidate target cell.

- f: please describe

Q6.1) Please provide preference a-f, or whether a combination should be supported?

| Company | Comments |
| --- | --- |
| Ericsson | B, c, or d |
| Nokia | None of these. We actually do not think location-based trigger is needed, especially if we have radio measurements + time(r). In particular, we do not think option a) is a feasible solution. It seems complex and risky to implement UE’s and satellite’s position coordination and rely on such metric to trigger the events. |
| Sony | B and C |
| MediaTek | We prefer Option (a) for its simplicity. The UE will anyway calculate this distance for uplink synchronization. |
| InterDigital | B,C, or D. Not sure how A would work for intra-satellite mobility considering all cells within satellite coverage originate from same location (as mentioned in description). |
| Huawei, HiSilicon | B,C,E For E, actually B and C are within the scope of E, they are detailed examples how to set a geographical area scope. |
| BT | B, c, d or e |
| ZTE | - For option a), UE would be able to derive the location of a satellite based on the ephemeris but the association between satellite and cells should also be provided to UE since the CHO evaluation and execution is performed per cell level. - For option b) c) d), the location of the cell centre can either be provided to UE directly or derived by UE based on the ephemeris together with the cell deployment information. We understand that b/c/d are similar to A2/A3/A4 with the distance to cell centre as a new MeasTriggerQuantity. Down selection from them should only be done after we conclude to use distance to cell centre as a measure. - For option e), since UE is assumed with GNSS capabilities, it seems that no additional information is needed. We are open to all these options with slightly preference for b)c)d). |
| Apple | Option E and leave it up to the UE implementation. |
| Qualcomm | Agree with Mediatek. - distance between UE and satellite. For intra-satellite HO, obviously the value of distance threshold configured would be different for each cell within same satellite as elevation angle would be different. |
| Rakuten Mobile | Option A is simpler to implement. However, for intra-satellite handover should be managed differently. |
| Lenovo | a) is simpler and can reuse the calculated TA. For b)~e) we have concern on signalling overhead of frequent change in cell centre or range, and additional UE calculation/measurement for distance. |
| OPPO | B and C, or D |
| CMCC | Slightly prefer to the first four options, e) may be a bit coarse . |
| Xiaomi | We prefer option c, e and f. Option a is same for a bunch of cells, such as in the scenario of intra satellites HO. Option b may not work well. Because option b cannot indicate any information about candidate target cells, UE may not be able to find a suitable target cell to trigger CHO. Even though the RSRP/Qs of candidate target cells have been considered to trigger CHO，UE may also access the inappropriate cell due to the near-far effect which is not obvious in NTN. Option d may not work well to trigger suitable CHO, which cannot represent UE at the edge of the serving cell and in the coverage of the candidate target cell because cells in different types of satellites have different cell sizes. Option f can be described as the combination of option b and c, which can reflect whether UE is in the coverage of serving cell and candidate target cell. |
| Spreadtrum | B, C, D |
| Thales | We prefer option E Option A could not work well, for example, in satellite switch scenario. Sometimes the UE will have to select a new satellite when the serving one is leaving, not because of a closer distance to the new satellite. The distances from all UE in an area to each of 2 satellites are not the same. All options from B to D that mention only the distance between the UE and the cell center could not work because it is not sure that the cells have the same size and circular shape. For option E, cells pattern/layout should be provided. So the UE can use the GNSS location to determine its location vs the cells edges. |
| CATT | B, c, d For option a, we are not sure how it work for intra-satellite cell handover. |
| Samsung | Options “b” and “f”. Option “f”: An elliptical cell around a Reference Point in the serving cell when the serving cell-based location trigger is used. Also, an elliptical cell around a Reference Point in the neighbor cell when the neighbor cell-based location trigger is used. An elliptical cell would better match the NTN platform’s beams (e.g., satellite beams). The gNB needs to convey the center, the major axis, and the minor axis of such elliptical cell to the UE. Hexagonal cells would not be suitable for the handover purpose. They can, however, be quite useful when RAN2 considers Earth-fixed virtual cells for other purposes (e.g., location-based services). |

There was most support for option B and C. Few comments sharing a concern for B and C as the cell center may be moving. There was also support for option A but then a concern on how does it work for HO between cells that are from same satellite.

Q6.2) Is UE assumed to receive all location based information of the CHO trigger in the CHO configuration or is it assumed UE is provided with part of the info in e.g. ephemeris?

| Company | Comments |
| --- | --- |
| ericsson | Serving cell reference point can be assumed to be elsewhere in the RRC configuration. Distance to that and distance to candidate target and reference location to candidate target(which can be given as distance to serving reference point) should be in CHO event configuration. |
| Nokia | Hard to imagine the UE is provided with exact geographical coordinates linked to each CHO candidate cell. |
| Sony | Agree with Ericsson |
| MediaTek | Potentially some information could be provided in the ephemeris. However, the details of the ephemeris information needs to be addressed first. |
| InterDigital | Reference point is FFS, however agree with Ericsson that distance triggers should be in CHO event configuration. |
| Huawei, HiSilicon | The decision on ephemeris is postponed. And we think radio coverage information is needed for UE. |
| BT | FFS |
| ZTE | - For option a), UE would be able to derive the location of a satellite based on the ephemeris but the association between satellite and cells should also be provided to UE in CHO config. - For option b) c) d), the distance to cell centre can be defined as a new MeasTriggerQuantity. The location of the cell centre of the serving cell and candidate target cell can either be configured in CHO config or in MeasObjectNR while the thresholds or offsets can be configured in CondTriggerConfig. - For option e), the geographical area scope referring to the coverage of the candidate target cell can be configured in CHO config. |
| Apple | In the CHO config. But we prefer to postpone this until the reference point discussion is finalized. |
| Qualcomm | If target cell is in different satellite, the satellite position information can be provided in CHO command. |
| Rakuten Mobile | Should be postponed till decision on ephemeris. |
| Lenovo | Wait for decision on ephemeris. |
| OPPO | Should be discussed after RAN2 has clear understanding on ephemeris. |
| CMCC | Postpone the discussion until the final ephemeris format is confirmed. |
| Xiaomi | We think RAN2 can postpone the discussion of the question until the definition of location info in CHO triggering event have been agreed. |
| Spreadtrum | It depends on the details of ephemeris. |
| Thales | The UE is provided with part of the info in, for example the ephemeris, in the SIB. Other information is given in the CHO configuration. FFS if the layout could be given by UE configuration in advance and updated only when needed. |
| CATT | Agree with Ericsson. |
| Samsung | For efficiency, we suggest that the information common to all UEs be broadcast in System Information (e.g., Reference Points of the serving cell and neighbor cells and characteristics of elliptical cells) and only UE-specific parameters such as thresholds are conveyed via dedicated RRC signaling or groupcast/multicast signaling (as appropriate). |

Most companies to prefer to postpone this discussion until the ephemeris discission has progressed.

Proposal 8: define the location based CHO trigger with respect to a cell center, FFS whether serving or candidate target and FFS which information is given in ephemeris and which part of information is given in CHO configuration

Q6.3) When configuring the location based CHO event together with RSRP/Q based event, should both, or(that is AND functionality) all configured events be fulfilled per CondReconfigId (candidate target cell) or if “one of” (that is OR functionality configured events fullfill then UE executes CHO?

| Company | Supported functionality AND, OR, or both | Comments |
| --- | --- | --- |
| Ericsson | both | Different deployments may have different needs. E.g. whether UE is near country boarder or not. |
| Nokia | AND | If location-based is to be introduced, then it needs to be fulfilled jointly with the radio measurement based event. |
| Sony | OR | We think any of the event can trigger the CHO, considering that RSRP/Q difference is not that huge in NTN. |
| MediaTek | AND | This must be configured with RSRP/RSRQ event, as there is no point in attempting a handover without finding a cell, even if the location condition is met. Hence, it must be a joint configuration. |
| InterDigital | AND (baseline) OR (FFS) | AND should be baseline. We find the country border scenario an interesting use case for “OR” and are open to further study. |
| Huawei, HiSilicon | AND (if both are cofigured) | And we are ok to only have one trigger, e.g. location alone CHO. |
| BT | FFS | BT |
| ZTE | AND |  |
| Apple | OR | An OR criteria. |
| Qualcomm | AND | We think, to avoid any possible miscalculation/error, both conditions should be met. |
| Rakuten | OR/FFS | Can be defined by Network. |
| Lenovo | both | Both can be supported for different deployment or coverage scenario. The “OR” association could be used for a more relaxed HO e.g. when the serving satellite will be inevitably unavailable in a short time due to movement, while the “AND” association could be used for a stricter HO e.g. when the serving satellite will still be available for a while and user experience is important. |
| OPPO | AND |  |
| CMCC | Both | Both solutions should be considered for different scenarios. |
| Xiaomi | Both | The two functionality (AND, OR) both need to be considered for different mobility scenarios, and FFS is how to select the suitable functionality for different scenarios. |
| Spreadtrum | both | The weights of distance and RSRP/RSRQ shall be configured by gNB. |
| Thales | OR | From our perspective, if position based CHO event is implemented, it could replace RSRP/Q based events because position based event is more precise. FFS the complexity of the position trigger condition. |
| CATT | AND | RSRP/Q is the basic event connecting the UE and network. |
| Samsung | Flexible (AND, OR, Both) | Let’s say the gNB has configured a set of distinct combination triggers and that each combination trigger combines two or more individual triggers. In some cases, AND would be applicable while combining multiple combination triggers. In other cases, OR would be applicable while combining multiple combination triggers. After RAN2 determines a full set of useful combination triggers for all the key scenarios such as all the three types of beams (i.e., Earth-fixed, quasi-Earth-fixed, and Earth-moving), satellite movement, and UE movement, RAN2 can identify what combinations are most useful or relevant. At this time, there is no need to be restrictive. |

9 support AND

4 support OR

6 support both

From these there is largest support to AND functionality and second largest for supporting both

Proposal 9: When configuring the location based CHO event together with RSRP/Q based event, support having AND functionality with location based and RSRP/RSRQ related event. FFS if additionally OR functionality is supported

Q6.4) Should the specification support option that the location trigger and time based trigger can be configured per UE and/or per candidate target cell?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Not clear | If there becomes use case for such combination |
| Nokia | Not clear (but it relates to the way the question is formulated;) | The question is whether a single CHO candidate can have simultaneously time and location based events for a single CHO candidate cell? If that is the case, then we do not think that should be allowed without radio measurement based condition. |
| Sony | Yes |  |
| MediaTek | - | Both should be possible. It seems to be a choice between better performances vs. overhead. |
| InterDigital | Postpone | Assume that question is whether UE can be configured with time and location trigger simultaneously. In this case we think we should progress individually before considering both together. |
| Huawei, HiSilicon | Per UE and per candidate cell | CHO configuration should be per UE as different UE is located in different places, so the location/time condition is different. Then the serving time of each candidate cell may start at different time and in different places, so it is better to make it per candidate cell. |
| BT | FFS | BT |
| ZTE | - | We understand that this question is whether a single CHO candidate can have simultaneously time and location based events for a single CHO candidate cell. It seems to be a choice and we would like to see clear use cases first. |
| Apple | Postpone |  |
| Qualcomm | Yes per candidate cell | We think it should per candidate target cell because it may belong to different satellite. |
| Rakuten | FFS |  |
| Lenovo | FFS | It could be a choice for further case if identified (No need to preclude for now). |
| OPPO | FFS |  |
| CMCC | FFS |  |
| Xiaomi | Per candidate cell and per UE | We support that the location trigger and time based trigger can be configured per candidate target cell, which can reflect the coverage information of different candidate target cells. So, UE can access the suitable target cell. In one case that only one target cell for UE(such as the scenario of feeder link switch), the location trigger and time based trigger can be configured per UE to avoid that all connected mode UEs will execute handover at the same time which may lead to network congestion and signalling storm. |
| Spreadtrum | FFS | It depends on the relative speed of UE and SAT. |
| Thales | No | Time base trigger should not be per UE but per group of UE per serving cell. Configuration per UE is too signalling consuming. Location trigger based is configured per serving cell with the information about all neighbour cells. |
| CATT | Yes | The location and time condition is different for each UE. |
| Samsung | - | We have a preference for per UE. For consistency, the serving cell can specify CHO execution conditions or coordinate among candidate cells. In an NTN, reliability is very important due to the very high cost of an incorrect handover or handover failure due to longer delays compared to a TN. |

There is no consensus whether location based and time based trigger should or should not be configured together.

Proposal 10: RAN2 to discuss whether location based and time based trigger should or should not be configured together

Q6.5) Should the specification support option that the location trigger is configured without RSRP/Q related event, or without another event? (That is, specification is not restricted)

| Company | Yes/No | Comments (reasoning if limitation is suggested) |
| --- | --- | --- |
| Ericsson | yes |  |
| Nokia | No |  |
| Sony | Yes |  |
| MediaTek | No | This must be configured with RSRP/RSRQ event, as there is no point in attempting a handover without finding a cell, even if the location condition is met. Hence, it must be a joint configuration. |
| InterDigital | FFS | As mentioned in Q6.3, we find the country border scenario an interesting use case for location-only event and are open to further study. |
| Huawei, HiSilicon | Yes | If network ensures the RSRP/Q is good enough, a location alone CHO can be configured. |
| BT | FFS | BT |
| ZTE | No |  |
| Apple | FFS |  |
| Qualcomm | No | To avoid any possible error/miscalculation, at least event like A4 should be configured together. |
| Rakuten Mobile | Yes |  |
| Lenovo | Yes | With network implementation the location can be configured in association with candidate cell strength or not. Therefore both “location OR RSRP/Q” and “location AND RSRP/Q” can be supported for different deployment or coverage scenario. |
| OPPO | No |  |
| CMCC | FFS | Considering different scenarios, the combination solution should also be evaluated. |
| Xiaomi | Yes | Considering the deterministic movement of satellites and corresponding cells, the target cell for a UE can be predict based on its location and whether the UE is in the coverage of the serving/target cell can also be judged based on its location. So, UE can trigger suitable handover, even if it is only based on location without RSRP/Q related event. |
| Spreadtrum | FFS | The weights of location and RSRP/RSRQ shall be configured by gNB. |
| Thales | No | Location trigger event cannot operate without other events because it does not cover all the handover scenarios. However, the triggering events to be selected is up to implementation. |
| CATT | No | RSRP/Q is the basic event connecting the UE and network. UE cannot find the cell without RSRP/Q, even if the location condition is met. |
| Samsung | No | In our view, combination triggers are very important/useful for reliable handover. It is very risky if we have only time based trigger or only location based trigger. |

7 companies no

6 companies yes

5 companies FFS

There is no consensus whether location based trigger can be configured without RSRP/RSRQ based event or not.

Proposal 11: RAN2 to discuss whether location based trigger can be configured without RSRP/RSRQ based event or not.

# 7 Other

In, R2-2100744	it is proposed that UE does not discard the CHO commands provided by source even after the CHO.

Q7.1) Should the UE discard or not the CHO commands after HO if they include future candidate cells?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | yes | If UE keeps source cell CHO configuration it may become relatively messy what is the UE’s configuration after few HO events. Target cell may configure the same CHO events. One could discuss if UE should keep some measurement or event filtering in case same candidate target cells are configured |
| Nokia | No | In the baseline CHO those configurations are discarded. However, for NTN the circumstances are different, the next candidate cell could be predicted with high certainty, and the UE may be provided in advance with configurations for subsequent CHO candidates. Thus, we should consider keeping some preparations even after first successful CHO execution. |
| Sony | No | HO is mainly due to the movement of cells so keeping the CHO configuration make sense if it is relevant for next upcoming cells. |
| MediaTek | Yes, Discard | After the CHO is over, there is no reason to keep the configurations. |
| InterDigital | Yes | Considering frequency of HO in LEO it may be simpler to discard. |
| Huawei, HiSilicon | Yes, discard | If the CHO command is not discarded, it also means candidate target cell cannot end this handover preparation procedure, which leads to a waste of system resources. |
| ZTE | Yes, Discard |  |
| Apple | Yes, discard |  |
| Qualcomm | No | If the UE is configured with future candidate cells that are yet not in the coverage of UE, obviously it should not be discarded. We are not talking about the CHOs of candidate cells which will be going to disappear or become useless. |
| Rakuten | Yes | Can not understand the reason of keeping it after HO. |
| Lenovo | Yes | For now we see no reason of keeping it (and defining additional condition for keeping). |
| OPPO | Discard |  |
| CMCC | Yes | It’s fine for now, and we can revisited this issue if some benefits are foreseen. |
| Xiaomi | Yes, Discard | Considering the movement of UE, the CHO commands may become invalid as time goes on. And, if UE not discard the CHO commands, system resources will be wasted. |
| Spreadtrum | Yes |  |
| Thales | No | This is an implementation issue |
| CATT | Yes | UE need new configurations, after UE complete handover. |
| Samsung | Yes- ignore CHO commands from source after CHO. | The UE should only focus on the selected CHO target cell. A key benefit of CHO is that the overall handover delay is reduced. This is useful in an NTN, where delays are long. However, per-use resources are fewer in an NTN than in a TN. In an NTN, massive handovers are expected. Resource reservation in CHO would lead to a reduction in the amount of radio resources available for user traffic transfer, degrading cell throughput and user experience. To avoid significant performance degradation, we suggest that RAN2 consider mechanisms to minimize the period during which the radio resources are reserved (but not used!) at multiple candidate cells in CHO. We also suggest that RAN2 consider enhancing traditional handover as well (e.g., minimize user traffic interruption during handover). The good news is that many of the CHO enhancements would be applicable to traditional handover, too. |

Consensus seems to be to discard the CHO command.

Conclusion is that RAN2 to not agree that UE to keeps the CHO command after HO. Which means no change to current procedure.

# 8 Conclusion:

List of proposals for agreement (if any)

Proposal 1: support A4 event for NTN CHO

Proposal 2: Support having the option to configure the ”time” trigger per “RRCReconfig->conditionalReconfig”. FFS if allowed to be configured without RSRP/RSRQ related event.

Proposal 5: Support having the option to configure the ”time” trigger per “RRCReconfig->conditionalReconfig->CondReconfigID”. FFS if allow it to be configured without RSRP/RSRQ related event.

Proposal 8: define the location based CHO trigger with respect to a cell center, FFS whether serving or candidate target and FFS which information is given in ephemeris and which part of information is given in CHO configuration

Proposal 9: When configuring the location based CHO event together with RSRP/Q based event, support having AND functionality with location based and RSRP/RSRQ related event. FFS if additionally OR functionality is supported

List of proposals that require online discussions

Proposal 3: RAN2 to discuss which definition of the ”time” would be suitable when ”time” is configured per “RRCReconfig->conditionalReconfig”.

Proposal 4&7: RAN2 to discuss how the ”time” based trigger is definied with respect to RSRP/RSRQ event. This was similar for both when ”time” is configured per “RRCReconfig->conditionalReconfig” or when it is configured per “RRCReconfig->conditionalReconfig->CondReconfigID”. Some options mentioned

- There is starting time per candidate cell when to UE should start evaluating an event

- There is time/timer for evaluating whether event was fullfilled

- Time range within which RSRP event needs to fullfill

- It is left to the UE

Proposal 6: RAN2 to discuss which definition of the ”time” would be suitable when ”time” is configured per “RRCReconfig->conditionalReconfig->CondReconfigID”

Proposal 10: RAN2 to discuss whether location based and time based trigger should or should not be configured together

Proposal 11: RAN2 to discuss whether location based trigger can be configured without RSRP/RSRQ based event or not.

List of proposals that should not be pursued (if any)

Proposal 12: UE to keep the CHO configuration after HO

# 9 Reference

CHO

R2-2100346	Connected mode aspects for NTN	Ericsson	discussion

- P1~P10 discussed in offline 106

R2-2101197	Discussion on time(r) and location CHO triggering event configuration in NTN	ZTE corporation, Sanechips	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2101708	Discussion on CHO in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2100383	Location based measurement event and location based CHO execution condition for NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2100744	Configuration and execution of CHO	Qualcomm Incorporated	discussion	Rel-17	NR_NTN_solutions-Core	R2-2009455

- Discussed in offline 106

R2-2101129	CHO in NTN system	Lenovo, Motorola Mobility	discussion	Rel-17

- Discussed in offline 106