3GPP TSG-RAN WG2 Meeting #113 electronic	R2-2102016

Online, January 25th - February 5th, 2021

Agenda item:	8.10.3.3

Source:	Ericsson

Title:	Report of [AT113-e][106][NTN] CHO aspects (Ericsson)

Document for:	Discussion & Decision

# 1Introduction

New trigger conditions were agreed for CHO in RAN2#112-e meeting and this email discussion is to understand further the needed procedures and other details when specifying the CHO for NTN:

- [AT113-e][106][NTN] CHO aspects (Ericsson)

Scope: Discuss CHO aspects based on the proposals in R2-2100346 (P1~P10), R2-2101197, R2-2101708, R2-2100383, R2-2100744 and R2-2101129

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Monday 2021-02-01 17:00 UTC

Initial deadline (for rapporteur's summary in R2-2102016../../../../../../../Data/3GPP/archive/RAN2/RAN2%23112/Tdocs/R2-2010761.zip): Monday 2021-02-01 23:00 UTC

Proposals marked "for agreement" in R2-2102016 not challenged until Tuesday 2020-02-02 11:00 UTC will be declared as agreed by the session chair. For the rest the discussion will continue online.

# 2 Contact Information

| Company | Name | Email |
| --- | --- | --- |
| Ericsson | Helka-Liina Määttänen | Helka-liina.maattanen@ericsson.com |
| Nokia | Jedrzej Stanczak | jedrzej.stanczak[at]nokia.com |
| Sony | Vivek Sharma | Vivek.sharma@sony.com |
| MediaTek | Abhishek Roy | Abhishek.Roy@mediatek.com |
| InterDigital | Dylan Watts | Dylan.Watts@interdigital.com |
| BT | Salva Diaz | salva.diazsendra@bt.com |
| ZTE | Yuan Gao | gao.yuan66@zte.com.cn |
| Apple | Sarma Vangala | svangala@apple.com |

# 3 Background

Agreements related to CHO from RAN2#102:

Agreements:

- RAN2 to consider the case where gNB is co-located at the GW with higher priority.

- RAN2 will continue working with the assumption that service link switch implies L3 mobility (meaning that at least in case the SSBs are on the same sync raster point the PCIs need to be different). Check if an LS to RAN1 asking for feasibility of having same PCI as well can be agreed

Agreements

- Reconfiguration with sync is the baseline for connected mode mobility in NTN (the use of legacy RLF and re-establishment mechanism are not excluded)

- The CHO can be used in NTN for both moving cell and fixed cell scenarios, and the CHO procedure and execution condition defined in Rel-16 is the baseline for NTN CHO.

3.	NTN specific CHO execution condition can be further discussed.

4.	The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

5.	Legacy SSB periods (as in TN) shall be supported in NTN

Agreements via email - offline 105:

- Time or timer based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario.  FFS on how to configure the time or timer based CHO triggering event. Also FFS how to consider the feeder/service link switch timing.

- DAPS HO for NTN is de-prioritized in this release.

- Location based CHO triggering event, in combination with the existing R16 CHO measurement based event, should be introduced for both moving cell and fixed cell scenario. FFS on how to configure the location based CHO triggering event. FFS if location based CHO triggering event only (not in combination with other events) can also be considered.

- The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

# 4 Discussion for adopting A4 event for NTN CHO

The existing CHO events A3 and A5 can serve as signal strength/quality measurement event for NTN CHO. In R2-2100346 it is proposed that it should also be considered if the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event. As the RSRP/RSRQ levels of different NTN cells may be similar and/or unpredictable w.r.t. location or time, which may possibly be configured together with RSRP/RSRQ events, it is seen beneficial to have possibility to have even that compares to absolute threshold rather than event that compares relative RSRP/RSRQ of two cells.

Q4.1) Do companies agree that the A4 event (Neighbour becomes better than threshold) should be defined as an additional CHO event?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Ericsson | Yes | Proponent of the proposal |
| Nokia | Yes | The NTN scenario justifies the introduction of A4 as a triggering event for CHO due to the likely small difference in RSRP/RSRQ levels between the serving and the neighbour. |
| Sony | Yes | We are fine to discuss additional trigger for CHO in NTN and think that event A4 is reasonable. |
| MediaTek | Yes |  |
| InterDigital | Yes | This would be a natural triggering event when combining measurement with another NTN-specific trigger (e.g. time + A4 or location + A4). |
| Huawei, HiSilicon | Yes |  |
| BT | No | The radio conditions experienced by UE connected to satellites are different from UEs connected into TN. For that reason, current mechanisms shouldn’t apply directly. A minor margin may have a severe impact in the network performance. |
| ZTE | Ok to discuss | We understand that A4+time/location event might be useful in NTN but would prefer to focus on what we have agreed earlier first, i.e. A3/A5 + time/location event. |
| Apple | Maybe | Depending on how the other triggers have to be propagated to the UE. |

# 5 Discussion for time/timer trigger definition

For the time/timer based event, it should be discussed how the feeder/service link switch scenario is supported. For example, is it possible to support with one time/timer based event the case that current serving cell ceases to serve the area of the cell. There were three different proposals presented in R2-2100346, R2-2101197 and R2-2101708. It seems beneficial to try to understand the use and pros and cons for each proposal, as well as the implications for defining the time/timer event per serving cell versus per candidate target cell, or if both are needed. When event is defined per serving cell, UE needs to evaluate or monitor it “per UE”, whereas when event is defined per candidate target cell UE needs to evaluate it per monitored candidate target cell.

In R2-2100346, and the time/timer event is proposed to be defined as absolute time without TTT or hysteresis. As an option UTC time was mentioned though the absolute time can also be represented as e.g. frame number.

In R2-2101708, in Proposal 3 also an absolute time is proposed.

The difference between these two definitions for the absolute time seem to be that in R2-2101708, UE is assume to check the other associated condition for the CHO and if that condition is fulfilled by the absolute time, the CHO triggers. In R2-2100346 some options were provided but main thinking was that after the absolute time, when the associated other event triggers, UE executes the CHO.

In R2-2101708, in Proposal 2 a timer is suggested such that upon expiry of the timer UE checks if the other condition is fulfilled or not and decides on CHO. That is, if the other event had been fulfilled during the timer, the CHO triggers.

In R2-2101197, the thinking is similar that a time range is provided and if the other event is fulfilled during the time range, given by two absolute times or one absolute time and duration, the CHO triggers.

If it beneficial to understand use cases for the CHO to be able to discuss how the time related event should be defined and if other time related information is needed.

When feeder/service link switch happens, from serving cell point of view, at some point, the serving cell ceases to serve the area. Thus, connected mode UEs need to be handed over to another cell before this happens or otherwise the connection is dropped. For this, it might be beneficial to be able to configure the UE with a time by which the HO needs to happen. This time is not per candidate target cell but per serving cell. Actually, if UE is configured with timer/absolute time/time range related only to candidate target cell, the UE may unnecessarily delay the CHO and the serving cell may disappear before the UE has executed the HO.

The other issue that was identified during the SI was that it is beneficial to distribute when the HO to the target cell happens. For this, a UE specific, can also be seen as serving cell specific time can be given that is the first possible time when UE may consider HO. This time can be also given per candidate target cell to accommodate when that cell starts to be available.

Additionally, UE may be given further information on candidate target cell about when it is no longer available.

All the above may also be used for the moving beam LEO although setting the time values may be more challenging in practice.

Question 5.1

First question is related to whether UE is assumed to be configured with information when serving cell is going to stop serving the area. This can be given in CHO configuration (FFS where), or it can be given in elsewhere in the serving cell configuration. This means that it could be possible to either configure CHO with only this time, or to indicate in the configuration that by this time a HO needs to be executed. The use case is that network is assumed to have configured CHO towards the cells that are going to take over the same.

Q5.1) Do companies see beneficial that UE is assumed to be configured with information when serving cell is going to stop serving the area?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Ericsson | Yes | For conditional HO where network may not after configuration control when UE performs the HO it is beneficial for the network to give early a HO command, CHO, that network knows that by this time or at this time UE performs the HO to the upcoming cell. Use case example: Earth fixed NTN cell has appeard to serve area and has UEs connected to it. Network may configure UEs with CHO where some candidate target cells are real neighbor cells where UE may move to due to UE’s own mobility. These CHO configurations may even be without any time event. One candidate target cell is with CHO with potentially information about when that target cell becomes available and information when serving cell will disappear(or when serving cell wants that UE to make HO to the replacing cell). This option is like timed regular HO where network just decides HO for a UE but now it can be timed in advance(and different UEs may be timed differently to spread the load), hence it use be the CHO structure. |
| Nokia | In some cases | The discussion on that (in another thread) is not yet concluded and so far, based on the feedback collected before RAN2#113, slight majority was assuming such information is not needed. If A4 is considered for NTN CHO then serving cell’s availability may not be of utmost importance, but rather the timer should say when the new/CHO candidate cell is expected to be available. The information about when serving cell is going to stop serving the area could be more doable in Earth-fixed deployments. And anyway, it is likely not a part of this discussion (i.e. not a part of CHO config, but could be included in SI or ephemeris). |
| Sony | Yes |  |
| MediaTek | Not Always | The time a serving cell serves a UE will change depending on UE’s location within the beam footprint. As such measurement based events are more suitable for mobility. Timing information only makes sense for cases where the serving cell is known to change (e.g. feeder link switch or change of earth-fixed beam in LEO). |
| InterDigital | In some cases | For earth fixed cell and feeder-link switch when the time a cell changes is uniform for all UEs served by the cell this would be beneficial. For earth moving cell the time a cell appears/disappears is UE-specific and based on relative position between the UE and cell center. In this case a location or distance-based trigger (e.g. UE-cell center distance threshold) may be more suitable. |
| Huawei, HiSilicon | Partially Yes | We have agreed to introduce timer/time based CHO, and if this timer is related to “when serving cell is going to stop serving the area” is not so important. The exact value can be provided by network for each UE, e.g. for accommodating all UEs during a short handover period in feeder link switching scenario. So it’s not necessary to restrict the timer length is equal to service time, the value can be determined by network. |
| BT |  | It seems that more discussion is needed on this before we conclude it is beneficial or not. |
| ZTE | Not always | We understand the time range during which the candidate target cell is available would be more important to UE in CHO decision as it is better for UE to perform CHO to an available cell before the serving cell expires. Also for earth moving cell, the expire time of a cell would be different for UE in different areas. |
| Apple | Not always | From our understanding, the time and timer based CHOs aren’t necessarily needed to be related to the residual cell serving time. We definitely need to discuss this further. Based on the question Q5.1, the benfits of this parameter are limited. |

Question 5.2

Second question is related to whether UE is assumed to be configured with information when candidate target cell is going to start and or stop serving the area. This may be given in the CHO/HO configuration and it can be discussed further if and how this information translates into rules if and when CHO is executed. This information might also serve assisting information in case UE is moving and e.g. considering two target candidate serving cells where one of them is about to leave soon.

Q5.2) Do companies see beneficial that UE is assumed to be configured with information when candidate target cell is going to start and/or stop serving the area?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Ericsson | Yes | Especially information on when a cellcandidate target cell will start serving the area is useful. Optionally it could also have information when a given target cell is going to stop serving that area as that can also be used at UE when selecting target, e.g. in case UE itself is moving. |
| Nokia | Yes | That should be the primary focus of the timer-related CHO discussion. |
| Sony | Yes | We think multiple cells information e.g. when the corresponding target cell becomes available, the related timing information can be included in CHO command together with DRB setup message. |
| MediaTek | Not Always | Response is similar to Question 5.1 |
| InterDigital | Yes | For the scenarios listed in our Q5.1 comment this would be useful. Agree that when a target cell would start serving the area is most important information. |
| Huawei, HiSilicon | Partially Yes | Similar comments as on Q5.1. Anyway it’s a timer/time configured by network, and it’s not necessary to restrict it must be related to “when candidate target cell is going to start and/or stop serving the area.” |
| BT |  | It seems that more discussion is needed on this before we conclude it is beneficial or not. |
| ZTE | Yes | If A3/A5+time is considered for NTN CHO, the time range showing availability of the candidate target cell would be useful. For example, when A3+time range is configured, the a3-Offset can be configured as a smaller value compared to the case when only A3 is configured for CHO. In this case, the A3 event is relaxed a little bit and CHO can be triggered earlier with both the A3 and time range showing availability of the candidate target cell taken into consideration. |
| Apple | Not always | Please see response in Q5.1 |

Question 5.3

The next set of questions are related to how time trigger should be understood in CHO configuration and what options should be supported.

Q5.3) Should it be possible to configure UE with time(see Q5.4, Q5.5 for details) which is per serving cell and upon which UE makes HO? That is, the time is per “RRCReconfig->conditionalReconfig”. If yes, should the specification support option that this be configured without RSRP/Q related event? (That is, specification is not restricted to mandate related RSRP/Q event)

| Company | Yes/No | Comments (reasoning if limitation is suggested) |
| --- | --- | --- |
| Ericsson | Yes to both | For the below use case it is needed and it is needed to be able to configure it without RSRP/RSRQ event. This is for UEs conneting, e.g. from idle mode, to a cell that wants to make timed HO without waiting RSRP/RSRQ event to be fulfilled, possible for replacing cell case. Use case example copied from above: Earth fixed NTN cell has appeard to serve area and has UEs connected to it. Network may configure UEs with CHO where some candidate target cells are real neighbor cells where UE may move to due to UE’s own mobility. These CHO configurations may even be without any time event. One candidate target cell is with CHO with potentially information about when that target cell becomes available and information when serving cell will disappear(or when serving cell wants that UE to make HO to the replacing cell). This option is like timed regular HO where network just decides HO for a UE but now it can be timed in advance(and different UEs may be timed differently to spread the load), hence it use be the CHO structure. |
| Nokia | No | The RSRP/RSRQ event should be also met before the UE executes any CHO configuration associated with a timer. |
| Sony | Yes for both |  |
| MediaTek | Yes, but | However, this must be configured with RSRP/RSRQ event, as there is no point in attempting a handover without finding a cell, even if the timer is configured. Hence, it must be a joint configuration. |
| InterDigital | Yes for both | Satisfying both and RSRP/RSRQ + time trigger would be ideal, but we see benefits to having a purely time-based CHO trigger as a “fallback”-like event. As mentioned by the rapporteur this would be useful to prevent UE from delaying execution of CHO so long that serving cell disappears, or to spread out UEs in time to prevent RACH collision as mentioned in SI. Another options would be to configure time + low threshold A4 event, but this seems unnecessarily complicated. |
| Huawei, HiSilicon | Yes | Time alone CHO could be an option. If network ensure the RSRP/Q can be met, a time alone CHO can be configured by network. And we are also fine with the combination of time and RSRP/Q as a CHO trigger. |
| BT | Yes but | This requires an accurate UE position. |
| ZTE | No | As mentioned above, we understand it is better to provide the valid time range of each candidate target cell and the RSRP/RSRQ/SINR event configured together should be fulfilled when CHO is triggered. |
| Apple | Yes but | As some of the othes have suggested, this should be a joint configuration. We have provided some analysis in R2-2100882 on why individual triggers might not work for all possible configurations. |

Q5.4) If yes to Q5.3, should the “time” be starting time, expiry time, time range or depending on configuration?

| Company | Comments |
| --- | --- |
| Ericsson | Any combination that does the trick. For example it could expiry time and the network would ensure there is then at least one candidate target that does not have conflicting trigger conditions. |
| Sony | Starting time/expiry time/timer |
| MediaTek | A starting time or a time range would be useful for the UE to start searching for the neighbour cell. |
| InterDigital | Agree with Ericsson (e.g. open to considering multiple options) |
| Huawei, HiSilicon | If it is an absolute time, it should be the starting time, i.e. UE starts to check if a handover is triggered. If it is a timer, when the timer expires, UE checks if a handover is triggered. |
| BT | More evaluation is required to find the optimal value. |
| Apple | More evaluation is needed. |

Q5.5) When configured with RSRP/RSRQ event, CHO triggers, when start time and RSRP/Q event fulfils, or only after expiry time and RSRO/Q event fulfils?

| Company | Comments |
| --- | --- |
| Ericsson | When this time is per serving cell the more natural is expiry time although it seems better to allow CHO when RSRP/Q fullfills even that would be earlier than expiry time and latest at expiry time. |
| Nokia | In principle the condition associated with the target needs to be fulfilled. It may not be optimal to wait until the last moment, i.e. when source cell expires. |
| Sony | Agree with Ericsson |
| MediaTek | It is not clear to us what is being asked in the question. When jointly configured, both the time and the RSRP event must be met. |
| InterDigital | Agree with Nokia |
| Huawei, HiSilicon | The question is not so clear. But anyway we prefer “for a timer, only after expiry time and RSRO/Q event fulfils” |
| BT | Agree with Nokia but it is important to clarify the conditions. |
| ZTE | We understand CHO will be triggered within the valid time range of the candidate target when the RSRP/RSRQ event is also fulfilled. |
| Apple | Either of the conditions should be fine. As a further comment, this criteria might be left to the UE to decide in order to ensure a handover is complete instead of waiting for network triggers. |

Q5.6) Should it be possible to configure UE with time(see Q5.7, Q5.8 for details) which is per candidate target cell and upon which UE makes HO? That is, the time is per “RRCReconfig->conditionalReconfig->CondReconfigID”. If yes, should the specification support option that this be configured without RSRP/Q related event? (That is, specification is not restricted to mandate related RSRP/Q event)

| Company | Yes/No | Comments (reasoning if limitation is suggested) |
| --- | --- | --- |
| Ericsson | depends | Here the need to configure without RSRP/Q is less important. It will depend how these candidate target cell times are going to be defined. |
| Nokia | No | The timing per candidate cell is OK, but jointly with the RSRP/RSRQ based condition (e.g. A3, A4). |
| Sony | Yes |  |
| MediaTek | Yes, but | See our response similar to Q 5.3. |
| InterDigital | Neutral | This scenario is less critical than Q5.3. We are open to further study, but think serving cell case is higher-priority. |
| Huawei, HiSilicon | Yes | It is reasonable to make the time per candidate cell. As the starting time should align with the beginning of serving time of a cell, therefore it is different between cells. |
| BT | Depends | Time configuration is not clear so more discussion is required. |
| ZTE | Yes for per CondReconfigID, No for configured without RSRP/RSRQ event |  |
| Apple | Depends | More discussion is needed . |

Q5.7) If yes to Q5.6, should the “time” be starting time, expiry time, time range or depending on configuration?

| Company | Comments |
| --- | --- |
| Ericsson | It could be at least start time that tells when the candidate target cell becomes available as well as potentially end time after which UE should not choose this candidate target. |
| Sony | Starting time/timer |
| MediaTek | Yes, but |
| Huawei, HiSilicon | If it is an absolute time, it should be the starting time, i.e. UE starts to check if a handover is triggered. If it is a timer, when the timer expires, UE checks if a handover is triggered. |
| ZTE | Time range showing availability of the candidate target cell is preferred as the candidate target cell will not be available forever considering the satellite movement. |

Q5.8) When configured with RSRP/RSRQ event, CHO triggers when start time and RSRP/Q event fulfils, only after expiry time of RSRO/Q event fulfils?

| Company | Comments |
| --- | --- |
| Ericsson | Start time that tells when RSRP/RSRQ event can trigger to this candidate cell and end time after which it cannot trigger to that candidate target. |
| Nokia | It shall trigger when the timing condition associated with the target cell is met and when the RSRP/Q event is fulfilled. |
| Sony | CHO execution should be the latest by the expiry of timer and before the timer expiry, it could be based on RSRP/Q. |
| MediaTek | It is not clear to us what is being asked in the question. When jointly configured, both the time and the RSRP event must be met. |
| InterDigital | for UE with time which is per candidate target cell, agree with Ericsson (e.g. a range where cell is valid) |
| Huawei, HiSilicon | Same question as Q5.5? |
| ZTE | We understand CHO will be triggered within the valid time range of the candidate target when the RSRP/RSRQ event is also fulfilled. |

# 6 Discussion for location trigger definition

For the location based event, it should be discussed how the location is defined and how it is used together with other events. For the location trigger definition, the following options were identified:

- a: The distance between UE and the satellite, serving or to candidate target(which may be the same for a bunch of cells).

- b: The distance between UE and the cell center of a serving cell. Defines circle around serving cell.

- c: The distance between UE and the cell center of a candidate target cell. Defines circle around target candidate cell.

- d: The distance difference between “UE and center of serving cell” and “UE and center of candidate target cell”. Defines hexagonal cell areas

- e: Configure a geographical area scope referring to the coverage of the candidate target cell.

- f: please describe

Q6.1) Please provide preference a-f, or whether a combination should be supported?

| Company | Comments |
| --- | --- |
| Ericsson | B, c, or d |
| Nokia | None of these. We actually do not think location-based trigger is needed, especially if we have radio measurements + time(r). In particular, we do not think option a) is a feasible solution. It seems complex and risky to implement UE’s and satellite’s position coordination and rely on such metric to trigger the events. |
| Sony | B and C |
| MediaTek | We prefer Option (a) for its simplicity. The UE will anyway calculate this distance for uplink synchronization. |
| InterDigital | B,C, or D. Not sure how A would work for intra-satellite mobility considering all cells within satellite coverage originate from same location (as mentioned in description). |
| Huawei, HiSilicon | B,C,E For E, actually B and C are within the scope of E, they are detailed examples how to set a geographical area scope. |
| BT | B, c, d or e |
| ZTE | - For option a), UE would be able to derive the location of a satellite based on the ephemeris but the association between satellite and cells should also be provided to UE since the CHO evaluation and execution is performed per cell level. - For option b) c) d), the location of the cell centre can either be provided to UE directly or derived by UE based on the ephemeris together with the cell deployment information. We understand that b/c/d are similar to A2/A3/A4 with the distance to cell centre as a new MeasTriggerQuantity. Down selection from them should only be done after we conclude to use distance to cell centre as a measure. - For option e), since UE is assumed with GNSS capabilities, it seems that no additional information is needed. We are open to all these options with slightly preference for b)c)d). |
| Apple | Option E and leave it up to the UE implementation. |

Q6.2) Is UE assumed to receive all location based information of the CHO trigger in the CHO configuration or is it assumed UE is provided with part of the info in e.g. ephemeris?

| Company | Comments |
| --- | --- |
| ericsson | Serving cell reference point can be assumed to be elsewhere in the RRC configuration. Distance to that and distance to candidate target and reference location to candidate target(which can be given as distance to serving reference point) should be in CHO event configuration. |
| Nokia | Hard to imagine the UE is provided with exact geographical coordinates linked to each CHO candidate cell. |
| Sony | Agree with Ericsson |
| MediaTek | Potentially some information could be provided in the ephemeris. However, the details of the ephemeris information needs to be addressed first. |
| InterDigital | Reference point is FFS, however agree with Ericsson that distance triggers should be in CHO event configuration. |
| Huawei, HiSilicon | The decision on ephemeris is postponed. And we think radio coverage information is needed for UE. |
| BT | FFS |
| ZTE | - For option a), UE would be able to derive the location of a satellite based on the ephemeris but the association between satellite and cells should also be provided to UE in CHO config. - For option b) c) d), the distance to cell centre can be defined as a new MeasTriggerQuantity. The location of the cell centre of the serving cell and candidate target cell can either be configured in CHO config or in MeasObjectNR while the thresholds or offsets can be configured in CondTriggerConfig. - For option e), the geographical area scope referring to the coverage of the candidate target cell can be configured in CHO config. |
| Apple | In the CHO config. But we prefer to postpone this until the reference point discussion is finalized. |

Q6.3) When configuring the location based CHO event together with RSRP/Q based event, should both, or(that is AND functionality) all configured events be fulfilled per CondReconfigId (candidate target cell) or if “one of” (that is OR functionality configured events fullfill then UE executes CHO?

| Company | Supported functionality AND, OR, or both | Comments |
| --- | --- | --- |
| Ericsson | both | Different deployments may have different needs. E.g. whether UE is near country boarder or not. |
| Nokia | AND | If location-based is to be introduced, then it needs to be fulfilled jointly with the radio measurement based event. |
| Sony | OR | We think any of the event can trigger the CHO, considering that RSRP/Q difference is not that huge in NTN. |
| MediaTek | AND | This must be configured with RSRP/RSRQ event, as there is no point in attempting a handover without finding a cell, even if the location condition is met. Hence, it must be a joint configuration. |
| InterDigital | AND (baseline) OR (FFS) | AND should be baseline. We find the country border scenario an interesting use case for “OR” and are open to further study. |
| Huawei, HiSilicon | AND (if both are cofigured) | And we are ok to only have one trigger, e.g. location alone CHO. |
| BT | FFS | BT |
| ZTE | AND |  |
| Apple | OR | An OR criteria. |

Q6.4) Should the specification support option that the location trigger and time based trigger can be configured per UE and/or per candidate target cell?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | Not clear | If there becomes use case for such combination |
| Nokia | Not clear (but it relates to the way the question is formulated;) | The question is whether a single CHO candidate can have simultaneously time and location based events for a single CHO candidate cell? If that is the case, then we do not think that should be allowed without radio measurement based condition. |
| Sony | Yes |  |
| MediaTek | - | Both should be possible. It seems to be a choice between better performances vs. overhead. |
| InterDigital | Postpone | Assume that question is whether UE can be configured with time and location trigger simultaneously. In this case we think we should progress individually before considering both together. |
| Huawei, HiSilicon | Per UE and per candidate cell | CHO configuration should be per UE as different UE is located in different places, so the location/time condition is different. Then the serving time of each candidate cell may start at different time and in different places, so it is better to make it per candidate cell. |
| BT | FFS | BT |
| ZTE | - | We understand that this question is whether a single CHO candidate can have simultaneously time and location based events for a single CHO candidate cell. It seems to be a choice and we would like to see clear use cases first. |
| Apple | Postpone |  |

Q6.5) Should the specification support option that the location trigger is configured without RSRP/Q related event, or without another event? (That is, specification is not restricted)

| Company | Yes/No | Comments (reasoning if limitation is suggested) |
| --- | --- | --- |
| Ericsson | yes |  |
| Nokia | No |  |
| Sony | Yes |  |
| MediaTek | No | This must be configured with RSRP/RSRQ event, as there is no point in attempting a handover without finding a cell, even if the location condition is met. Hence, it must be a joint configuration. |
| InterDigital | FFS | As mentioned in Q6.3, we find the country border scenario an interesting use case for location-only event and are open to further study. |
| Huawei, HiSilicon | Yes | If network ensures the RSRP/Q is good enough, a location alone CHO can be configured. |
| BT | FFS | BT |
| ZTE | No |  |
| Apple | FFS |  |

# 7 Other

In, R2-2100744	it is proposed that UE does not discard the CHO commands provided by source even after the CHO.

Q7.1) Should the UE discard or not the CHO commands after HO?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericsson | yes | If UE keeps source cell CHO configuration it may become relatively messy what is the UE’s configuration after few HO events. Target cell may configure the same CHO events. One could discuss if UE should keep some measurement or event filtering in case same candidate target cells are configured |
| Nokia | No | In the baseline CHO those configurations are discarded. However, for NTN the circumstances are different, the next candidate cell could be predicted with high certainty, and the UE may be provided in advance with configurations for subsequent CHO candidates. Thus, we should consider keeping some preparations even after first successful CHO execution. |
| Sony | No | HO is mainly due to the movement of cells so keeping the CHO configuration make sense if it is relevant for next upcoming cells. |
| MediaTek | Yes, Discard | After the CHO is over, there is no reason to keep the configurations. |
| InterDigital | Yes | Considering frequency of HO in LEO it may be simpler to discard. |
| Huawei, HiSilicon | Yes, discard | If the CHO command is not discarded, it also means candidate target cell cannot end this handover preparation procedure, which leads to a waste of system resources. |
| ZTE | Yes, Discard |  |
| Apple | Yes, discard |  |

# 8Conclusion:

# 9 Reference

CHO

R2-2100346	Connected mode aspects for NTN	Ericsson	discussion

- P1~P10 discussed in offline 106

R2-2101197	Discussion on time(r) and location CHO triggering event configuration in NTN	ZTE corporation, Sanechips	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2101708	Discussion on CHO in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2100383	Location based measurement event and location based CHO execution condition for NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2100744	Configuration and execution of CHO	Qualcomm Incorporated	discussion	Rel-17	NR_NTN_solutions-Core	R2-2009455

- Discussed in offline 106

R2-2101129	CHO in NTN system	Lenovo, Motorola Mobility	discussion	Rel-17

- Discussed in offline 106