3GPP TSG RAN WG2 #113-e	draft R2-21xxxxx

Electronic meeting, 25th January – 5th February, 2021

Source: 	Google

Title: 	[draft] Summary of [AT113-e][114][SRVCC] Corrections (Google)

Agenda Item:	6.12

Document for:	Discussion and decision

# Introduction

This document is used to continue the following online discussion to collect feedback:

SRVCC

R2-2101891	Avoid UTRA capabilities forwarding in handover preparation	Google Inc.	CR	Rel-16	38.331	16.3.1	2448	-	F	SRVCC_NR_to_UMTS-Core

- Lenovo thinks the current table already implies that the UTRA capabilities are not sent

- HW thinks the wording can be changed to say the information may be included and the target can skip this. Qualcomm/Ericsson has the same view. Lenovo thinks this is strange.

- Intel/Samsung wonder if the source is allowed to include only if it knows this is the latest.

- continue in offline 114

- [AT113-e][114][SRVCC] Corrections (Google)

Scope: Discuss a revision of CRs in R2-2101891

Intended outcome: rapporteur's summary in R2-2102033 and corresponding CR (if agreeable)

Deadline (for companies' feedback): Tuesday 2021-02-02 11:00 UTC

Deadline (for summary and CRs): Tuesday 2021-02-02 17:00 UTC

CRs (if any) listed as "can be agreed as is" in R2-2102033 and not challenged until Wednesday 2021-02-03 11:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Companies are invited to share their views.

# Discussion

During the online discussion on R2-2101891, some companies think the source gNB can include UTRA capabilities and the target gNB can ignore the UTRA capabilities, i.e., follow the case that the source RAT is UTRAN as above. Some companies think the source gNB can include UTRA capabilities if the source gNB knows the START value in the UTRA capabilities is the latest. Then we take a look at the LTE RRC specification [1]. The following is excerpted from the LTE RRC specification:

#### – HandoverPreparationInformation This message is used to transfer the E-UTRA RRC information used by the target eNB or target ng-eNB during handover preparation or UE context retrieval, e.g. in case of resume or re-establishment, including UE capability information. Direction: source eNB/ source RAN to target eNB or target ng-eNB HandoverPreparationInformation message <unrelated part omitted> Source RAT; E-UTRA capabilites; UTRA capabilities; GERAN capabilities; MR DC capabilities; NR capabilities. UTRAN; Included; May be included, ignored by eNB if received; May be included; Excluded; Excluded. GERAN CS; Excluded; May be included, ignored by eNB if received; Included; Excluded; Excluded. GERAN PS; Excluded; May be included, ignored by eNB if received; Included; Excluded; Excluded. E-UTRAN; May be included if UE Radio Capability ID as specified in 23.502 [102] is used for the UE. Included otherwise.; May be included; May be included; May be included; May be included. NR; May be included if UE Radio Capability ID as specified in 23.502 [102] is used for the UE. Included otherwise.; Excluded; Excluded; May be included; May be included.

Based on the above, there are three different behaviors to handle UTRA capabilities depending on which source RAT. After looking at the LTE RRC specification, we did analysis as described below.

The source gNB obtains UTRA capabilities (including the START value) from the UE before performing a SRVCC handover preparation. The source gNB may proceed a SRVCC handover preparation for the UE after obtaining the UTRA capabilities, and then the source gNB sends a MobilityFromNRCommand message to the UE to trigger a SRVCC handover. The source gNB releases the UTRA capabilities after the UE hands over to the UTRAN, because the UE does not connect to the source gNB anymore. However, after obtaining the UTRA capabilities, the source gNB may prepare a NR handover instead of a SRVCC handover. In this case, the START value in the UTRA capabilities is never used, i.e., the START value is the latest.

From the analysis above, we have the observation below.

Observation: If the source gNB has the UTRA capabilities of the UE, the START value in the UTRA capabilities is the latest.

To discuss how to handle UTRA capabilities in NR, we provide the following questions by referring to LTE.

Q1: Please indicate which behaviour of the following is the correct behaviour for handover preparation and UE context retrieval within NR (i.e., both the source RAT and target RAT are NR).

- May be included, ignored by the target gNB if received

- May be included

- Excluded

| Company | (1), (2) or (3) | Comments |
| --- | --- | --- |
| Lenovo | Option (3) | Referring to the 5G SRVCC WID RP-200151 the HO preparation between gNBs was not part of the objectives, see below. Therefore, the exchange of UTRA capabilities in the HandoverPreparationInformation message was never discussed during the WI phase and considered as not supported. - Indirect Inter-RAT handover procedure to support voice service continuity from 5GS to UTRAN [RAN2, RAN3]. - Specify the procedures of SRVCC (including emergency call) in RAN, which includes handover preparation between gNB and AMF, gNB and UE, and Handover execution between UE to RNS etc. |
| Huawei, HiSIlicon | Option (1) | In last meeting we specified “the gNB always requests the UE UTRA-FDD capabilities before handover to UTRA-FDD” in TS 38.00, so it’s natural to further clarify gNB ignores this UTRA capability derived otherwhere. And the wording “may be included” also means it depends on source RAN implementation to decide whether this UTRA capability is sent to target, e.g. when it is the latest. |

Q2: Please indicate which behaviour is the correct behaviour for handover to NR from E-UTRA.

- May be included, ignored by the target gNB if received

- May be included

- Excluded

| Company | (1), (2) or (3) | Comments |
| --- | --- | --- |
| Lenovo | Option (2) | To be aligned with 36.331. |
| Huawei, HiSilicon | Option (1) |  |

# Conclusion

To be added.

# Reference

[1] 3GPP TS 36.331 v16.3.0.