3GPP TSG-RAN WG2#113-e	R2-210xxxx

Online, January 25 - February 5, 2021

Source: 	Huawei

Title: 	[AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation (Huawei)

Agenda Item:	8.2.2

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation (Huawei)

Scope:

    - Summarize main solution directions based on alternative approaches submitted to 8.2.2: Which combined solutions have the most support? What are the main solution approaches to consider in Rel-17?

Intended outcome:

    - Discussion summary in R2-2101969 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  2nd week Wed, UTC 0900

    - Initial deadline (for rapporteur's summary):  2nd week Thu, UTC 1000

# 2	Discussion

RAN2 made the following conclusions in the ongoing meeting:

Agreements

1a: 	SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

1b: 	SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

3: 	RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

and

Agreements

1: 	Confirm that there is no PUSCH transmission on deactivated SCG. FFS if any other UL is allowed towards SCG.

2: 	Confirm that there is no PDCCH monitoring on PSCell of the deactivated SCG.

3: 	Confirm that there is no support of SCell dormancy for SCG SCells within a deactivated SCG.

## 2.1	SCG activation

SCG activation can be triggered by UL or DL traffic on DRBs with an SCG RLC bearer (split or SCG bearer).

### 2.1.1	Radio interface

#### 2.1.1.1	Activation triggered by the network alone

Based on the above agreements, in case DL data for the UE arrive from the CN at the MN or the SN and, possibly after MN-SN interactions, the network decides to activate the SCG, the only solution is an indication to the UE via the MCG.

Q1: Do companies agree that, if the network decides to activate the SCG (e.g. in case of DL data arrival), the SCG activation is indicated to the UE via the MCG?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes | Since the PDCCH monitoring is out of question on PSCell, the indication has to come from MCG. |
| MediaTek | Yes |  |
| Interdigital | Yes |  |
| LG | Yes | The indication via MCG is desirable because the UE doesn’t perform PDCCH monitoring on SCG. Note that no PDCCH monitoring is main reason how to achieve the power saving of SCG deactivation, |
| Intel | Yes | Agree with Apple |
| NEC | Yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes | Only the MCG can send indication to UE when SCG is deactivated. |
| Sharp | Yes |  |

Upon reception via the MCG of the indication to activate the SCG, there are mainly two solutions:

1)	similar to reconfiguration with sync, i.e. the UE initiates random access to the PSCell.

2)	if the TA timer is still running and possibly other conditions (FFS, e.g. related to BFD/RLM):

-	the UE does not initiate random access and monitors PDCCH on the PSCell

-	the SCG can schedule data transmission on the PDCCH directly

Solution 2) has more complexity than solution 1) but can reduce the delay until data transmission via the SCG.

For solution 2), some companies propose that, in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

-	initiates random access towards the PSCell; or

-	consider the TAT to be expired and/or report something via the MCG.

Q2: Do companies agree that the above description covers the solutions for UE behaviour when the SCG activation is indicated to the UE via the MCG?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes for the most part. Pls see comments. | Opt1 is practical and solid solution Op2 is complex and not always useful. The only concern with option-1 is the delay associated with RACH. But option-2 can have considerable delay as well, as the gNB has to know for sure when the UE would be ready with PDCCH monitoring (on which beam). With option-2 the NW has to be defensive and cannot use UE-specific beams and probably has to go with wider beams (like used in broadcast) and this would require the UE report the beam measurements periodically. Associating TAT with beams is something we do not like. We cannot associate the UE mobility with a timer. In our view both options work. Opt-1 can be configured by the NW, and for Op-2, the NW can also configure periodic RRC measurements of PSCell with beam info, to be delivered via MCG and NW can provide info at the time of SCG activation which option the UE should use. If it’s direct monitoring of PDCCH, the UE can be given info on which beam to monitor PDCCH (via TCI etc..). This is also one of the reasons why RRC based SCG activation is better. |
| MediaTek | Yes | So far we have only solution 1 and 2. But we are also open to discuss the enhancement proposed by Apple |
| Interdigital | Yes | RACH should be avoided when not necessary as it can slow down the activation process, and making the SCG deactivation/activation not so more useful than SCG release/addition |
| LG | Yes |  |
| Intel | Yes | While Solution 2 is a baseline, we think we should not preclude Solution 1. NW should be able to trigger RACH when it thinks necessary. |
| NEC | Yes mostly but .. | we are wondering what “- the SCG can schedule data transmission on the PDCCH directly” in Solution 2) really mean? Some hidden assumptions are expected. For example, the SN can know when the UE is ready to receive after SCG activation. However, it is still unclear how this can be done and whether there is any difference between MN-initiated and SN-initiated activation (given both supported). |
| Ericsson | Yes, but with comments | It should be possible to activate the SCG without performing random access as indicated for solution 2. To always require a random access procedure to activate the SCG, even if TA timer is still running and there has e.g. been no BFD, would lead to additional delays and also higher power consumption in case the SCG is (de)activated frequently. For solution 2, regarding the UE actions in case it sees that “the beams are not good enough”, we do not see the need to immediately initiate a random access towards the PSCell. The UE can also wait with the random access until SCG activation. This should however not be mixed up with Timing Advance, i.e. BFD should not lead to that the TA timer is stopped. |
| Spreadtrum | Yes | Since SRS transmission is not allowed in the PSCell, the network has no idea of beam info when activating SCG. So Option 1 is suitable for any scenario. For option 2, it is more complex and will cause UE power consumption to perform RLM-like behaviour. The UE only has to maintain TA timer for PSCell and it has minimum impact on UE. If the timer expired, the UE has to perform random access procedure in the PSCell when activating SCG. |
| Sharp | Yes |  |

There was a large majority of companies proposing solution 2), so it can be checked whether it would be agreeable.

Q3: Do companies agree to go for solution 2) for UE behaviour when the SCG activation is indicated to the UE via the MCG (further details FFS)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes for solution-2 where the NW can indicate to the UE whether to RACH or to monitor PDCCH using a particular config (like TCI) after SCG activation |  |
| MediaTek | Partial | We originally think solution 2 is good way to reduce the latency. But after further thinking, it seems that TA value is not the only factor that requests RACH. The UE also has actively maintain the beam in order not to doing the RACH. We feel like a little bit more time is needed to study this (including the proposal from Apple). In additional, the configuration of TAT timer could be infinity, which we assume that this is the case we are discussing. TimeAlignmentTimer ::= ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity} As a baseline, we suggest to clarify that While SCG is activated from deactivate state, the UE shall trigger RACH to the PSCell if TAT timer is expired |
| Interdigital | Yes (for solution 2) | See comment to Q2 |
| LG | Yes | To reduced delay of SCG reactivation, SCG reactivation w/o RACH is needed at some conditions. In our view, the conditions can be TA validity and beam quality. The gain is very obvious that the UE can reduced the time of RACH procedure for SCG activation considering that RACH procedure is the longest time to take to reactivate SCG. The gain may become very increase if the network want the UE SCG state change frequently. |
| Intel | Yes | Solution 2 can be a baseline behaviour when reconfiguration with sync is not indicated upon receiving SCG activation command. |
| NEC | Yes/No | Although generally fine, it is a bit early to decide. This is because firstly, we would like to know the answer to our comment in Q2. Also, feasibility of TA maintenance during deactivated and its accuracy should be consulted with RAN1/4. As far as it is confirmed that this can work, we also support this. If companies want to agree with this now, RAN2 should make it as working assumption. |
| Ericsson | Yes |  |
| Spreadtrum | Yes for only maintaining TA timer |  |
| Sharp | Yes |  |

Q4: For solution 2), do companies see the need that in SCG deactivated state while the TA timer is not expired, the UE monitors some DL beams in order to ensure that SCG activation (i.e. reception on PDCCH is possible)?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | No | We do not see the necessary relation between TA timer and beam handling. |
| MediaTek | See comment | Yes if we want to skip RACH before TAT expired. But we are not sure it is a good solution to request UE to perform beam management with large TAT value. |
| Interdigital | FFS | Needs further discussion (e.g. solutions like Apple’s proposal to Q3) |
| LG | Yes but.. | Not only TA validity, but we think checking beam quality is also important because, when reactivating SCG, BFR can be initiated even tough TA is valid especially in the case of the FR2 deployment scenario. On the other hand, we don’t think BFR declaration is necessary during SCG deactivation because BFR declaration leads to RACH procedure. The UE may estimate how beam quality is going based on (RRM) measurement. Thus, we think the UE can just check fast whether beam quality is good enough when reactivating SCG. |
| Intel | No preference |  |
| NEC |  | no need to monitor PDCCH. but this point should be asked to RAN1/4. |
| Ericsson | Yes |  |
| Spreadtrum | No |  |
| Sharp | Yes | To consider whether UE needs to send RACH for SCG activation, monitoring of DL beams is useful. |

For the case where the SCG activation is indicated to the UE via the MCG, there are other proposals, somehow independent from solutions 1) and 2) above, to reduce the delay until data transmission via the SCG is possible:

a)	maintains DL sync while the SCG is deactivated

b)	reduced processing time for RRC reconfiguration for activating SCG with limited changes to the SCG configuration

c)	activation by MCG MAC CE (no change to the SCG configuration)

Q5: Do companies agree with the above description for possible optimizations reducing the delay until data transmission is possible on the SCG after the UE has received an SCG activation indication via the MCG? (please indicate in case another proposal related to that case is missing.)

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | - Is ok. - Not sure about this - RRC based activation has flexibility | For (a), the UE would maintain DL sync as long as the UE measures the SSB of PSCell periodically For (b), we do not any additional reduction in the processing time as the current RAN4 requirements for the PSCell addition for the PSCell known timing case is applicable here as well. RRC processing delay reduction can be discussed, and avoiding RACH is another possible reduction (though not part of RRC processing time). For (c) not much of gain from MAC CE compared to RRC based activation. In EN-DC case, LTE MAC CE needs to be modified as well, but with RRC it can be transparent. The TCI info can also be provided via RRC. |
| MediaTek | Yes | For (a), similar view as Apple, we assume that SSB based RRM could reach the “DL Sync”. We believe that this is just to say that the PSCell is still a known cell. For (b), there will be anyway some basic RRC processing time no matter some parameters are not changed. It is difficult (and time consuming) to classify which kind of RRC parameters take more time. We are not so eager to go this way. For (c), we think it is possible solution. If we allowed SCG to be reconfiguration during deactivate state, it would be useful. In that case, the latest RRC configuration already provided to the UE. Then we only need one MAC CE command to activate the SCG. |
| Interdigital | Yes to a) and c), neutral to b |  |
| LG | Yes except (b) | For (b), since the UE should apply RRC configuration no matter how the configuration can be limited, the processing time seems to be not reduced much. For (c), We think both ways of signalling should be supported for case by case. Considering the scenario that the network wants to (de)activate SCG with more, lower layer singling has a gain with flexibility than the case of RRC signalling. It helps time reduction to signal to the UE and the gain can increase as frequently as SCG deactivation is commanded. Also, lower layer signalling is beneficial for the case that there is no other RRC configuration except indicating SCG activation. That is, the network can just send the lower layer signalling to reactivate SCG if there no other reconfiguration to provide. |
| Intel |  | (a): OK (b): In general, we do not want to unnecessarily limit NW capability – what to change as part of this RRC reconfiguration should be left up to NW. If NW wants to really reduce activation delay by saving from RRC processing time, it may decide to just indicate to activate without changing any SCG configuration (if keeping the current SCG configuration is OK from NW point of view). Overall, we think RAN2 should not agree that, for fast reactivation, e.g. NW should send only activation command indication without changing SCG configuration. (c) Open but we doubt how much delay we could save by MN sending MAC CE instead of RRC message. As the door for fast reactivation directly via SN is closed (no PDCCH monitoring while SCG is deactivated), we think that via MN (regardless of RRC or MAC CE) is already slow. |
| NEC | see comments | a) should ask RAN1/4, b) should ask RAN4, if necessary, c) this depends on possible procedure. For example, if MN initiates the SCG activation (and SN accepts), MCG MAC CE can work from signaling point of view. If SN initiates (and MN accepts), SCG RRC may be directly sent to UE via MCG, where RRC is suitable. So, RAN2 should discuss overall procedure first. |
| Ericsson | Yes, see comment | - is critical for ensuring fast SCG activation - can be used to reduce delay when RRC is used - we don't see a need for MAC CE based activation, if we anyway support RRC based. |
| Spreadtrum | - Maybe - Not sure - Yes | For a), RRM measurement is supported for PSCell change and it can be used to maintain DL sync. Do not support other mechanism to maintain DL sync. For c), MAC CE can be used to activate SCG with pre-configured SCG configuration. |
| Sharp | Yes but | The above optimizations can reduce the delay. However, the gain of (c) may not so much in the whole process and (b) may be the cause of additional RRC reconfiguration. Unified solution by using RRC message is preferable. |

Q6: For which of these optimizations do companies see potential interest or not (please explain)? Note that at this stage, it is not proposed to agree anything.

| Company | Views |
| --- | --- |
| Apple | Pls see our comments for Q5. |
| MediaTek | - and c), |
| Interdigital | a and c |
| Intel | Please see above our comments for Q5. |
| Ericsson | a) Yes b) Yes c) No |
| Spreadtrum | - and c) |

#### 2.1.1.2	Uplink data on a SCG bearer

If there are uplink data to be sent on an SCG bearer while the SCG is in deactivated state, the UE cannot send the data directly because, as the SN considers that the UE is not monitoring PDCCH on the PSCell, it is probably not providing any grant for SCG transmission.

One first question is whether it is possible to deactivate the SCG while there are SCG bearers.

Q7: Do companies want to support SCG deactivated state with SCG bearers?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes | We can view this as data to be transmitted by the UE but there is no UL grant or SR -> UE can RACH on the PSCell to come out of SCG deactivation. The UE would also have the SCG C-RNTI that can be used at RACH to let the SCG know. This also provides the NW with the necessary beam info for PDCCH and data transfer. |
| MediaTek | Maybe | We do support UE triggered SCG activation but are not sure if it is good idea to trigger SCG activation small amount of UL data is arrived from SCG bearer. Using split bearer with primary path on MCG would be a better solution. However, we are okay to discuss this behavior if majorities prefer to support SCG bearer during SCG deactivation. |
| Interdigital | Yes | Changing the bearer type back and forth whenever the SCG is deactivated/activated will lead to a lot of unnecessary reconfigurations/signaling, as such it would be beneficial to have the possibility to keep the bearer configuration during the SCG deactivation. Also, keeping the bearer configuration as it is will make it easier to detect when SCG resources are needed/useful (e.g. when UL/DL data arrives that is mapped to SCG bearers). |
| LG | Yes but, | However, we wonder if the UE can send uplink data on SCG in SCG deactivation. In our understanding, there is only one scenario to send UL data is that split bearer is configured with ul-DataSplitThreshold. Assuming that all SCG bearers can be suspended in SCG deactivation like the case of SCG failure, the UE will send UL data via MCG bearers implicitly even though split bearer is configured. Thus, we think the UE may not send any UL data on SCG if all SCG bearers are suspended when SCG is deactivated. |
| Intel | Yes | NW should be able to deactivate SCG even if there are some SCG bearers. |
| NEC | Yes | The agreement “1a: SCG activation can be requested by MN/SN/UE” already imply this? If the SCG bearer is for delay tolerant or delay non-sensitive service, this may be useful. Otherwise, e.g. SN terminated MCG bearer can be configured while the SCG is deactivated instead. |
| Ericsson | Yes | Both MN- and SN-terminated SCG bearers and split bearers should be supported with deactivated SCG. There should not be a need to change bearer types just because the SCG is deactivated. |
| Spreadtrum | Yes | SCG activation can be triggered by UE if it has UL data to be transmitted in SCG. UE can perform RACH in PSCell to initiate data transmission. |
| Sharp | Yes | How to treat the SCG bearers during the deactivation should be discussed. |

If it is supported, there are mainly two solutions:

1)	The UE sends an indication via the MCG then the network can indicate SCG activation via the MCG

2)	The UE sends an indication to the SCG (random access or PUCCH)

Solution 2) can reduce the delay until data transmission via the SCG as compared with 1), but it does not allow the network to change any configuration between activation. Also, if PUCCH or contention-free RACH is used, it means the corresponding resources must remain allocated by to the UE.

Q8: Do companies agree that the above description covers the solutions for UE behaviour when the SCG is in deactivated state and the UE has uplink data to send on a SCG bearer?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Prefer (2) and not (1) | We think (1) creates more delay and does not provide the critical PHY info (beam etc) that (2) does. We do not prefer (1). |
| MediaTek | Yes |  |
| Interdigital | Prefer 2 | Activation via the MCG may take considerable time (as it includes double inter-node messaging), so direct implicit activation from the UE to the SCG is preferred. |
| LG | Yes but | We don’t see a scenario exactly that the UE should send UL data on SCG. However, we prefer solution (2) for supporting time-efficient activation if given scenario is possible. |
| Intel | Prefer (2) | But how to handle RACH failure case based on (1) can be further discussed. |
| NEC | Yes basically | 1): “via the MCG” may not be clear enough. We see two options; a) indication: UE->MN, activation: MN->UE (and MN->SN in parallel), b) indication: UE->MN(forward)->SN, activation: SN->MN->UE. 2): we prefer to remove PUCCH for now. |
| Ericsson | Yes | Both are valid depending on DRB type. 1) is already supported with BSR in MCG for split bearers, i.e. as long as buffer is below threshold, BSR is reported on primary path. |
| Spreadtrum | Yes |  |
| Sharp | Yes | The case for SCG leg of split bearer should also be discussed. |

There were several companies (but not a majority) proposing solution 2), so it can be checked how much support ther is for it.

Q9: Do companies support solution 2) for UE behaviour when the SCG is in deactivated state and the UE has uplink data to send on a SCG bearer?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes | We support RACH or PUCCH notification (like SR) on PSCell |
| MediaTek | FFS | Solution (1) could work. We also have intention to support solution (2) but maybe request more discussion. |
| Interdigital | Yes | See comment to Q8. |
| LG |  | In case of the UE indicates SCG activation, the UE can simply indicates SCG reactivation via RA procedure if given scenario is possible. Otherwise, i.e. the network indicates SCG activation, |
| Intel | Yes, but | We think it is not good to always mandate Solution (2) behaviour. It should be configurable by NW and up to NW to decide. |
| NEC | Yes but | we support only RACH and need more justification for PUCCH |
| Ericsson | Yes | 2) should be supported for at least SCG bearers. |
| Spreadtrum | Yes | CBRA can be used. |
| Sharp | FFS | For SCG bearer (not for split bearer), either 1) or 2) should be supported. |

Another question is whether the above indication could apply to other scenarios than the above one.

Q10: Do companies think that, when the SCG is in deactivated state and there are no uplink data to be transmitted on any SCG bearer, the UE should be allowed to send an indication requesting activation of the SCG? If yes, in which scenario?

| Company | Yes/No | If yes, please explain the scenario |
| --- | --- | --- |
| Apple | Yes | To move out of SCG state…and we prefer UE RACH or SR type indication… we think the main reason the UE wants to be out of SCG deactivation is for data transfer.. and RACH is faster and more effective. We also see a potential case of UE using RACH for reporting MCG recovery using SCG when the SCG is deactivated, if the SCG (PSCell) have very good channel conditions (the NW can configure a threshold that can help UE decide if SCG can be activated to recover MCG). |
| MediaTek | Yes | We already agreed to have UE triggered SCG activation so some kind of indication is needed. It may overlapped with current MCG SR, BSR, or assistance information but anyway need to be discussed. |
| Interdigital | Yes | There can be several reasons for UE triggered activation of the SCG other than arrival of UL data that is to be sent via the SCG (e.g. when UL data volume increases, SCG radio conditions improve, etc.). The details can be left FFS. |
| LG |  | See above our response at Q7 |
| Intel | Yes | A fast MCG recovery could be one scenario. |
| NEC | FFS | we understand the possible case might be MCG fast recovery. If RAN2 agrees to keep TAT running in deactivated SCG and skip RACH unless it expires, then this may give some benefit for reducing the latency in recovery. Otherwise, as anyway RACH is needed, the legacy re-establishment procedure would be enough. This can be discussed later. |
| Ericsson | Yes, but | It should be clearly defined when the UE may request activation via SCG, e.g. due to MCG failure recovery or when data is above a threshold for a split bearer. |
| Spreadtrum | FFS | UL data transmission in SCG will trigger SCG activation by UE. For fast MCG recovery scenario, it should be further discussed. |
| Sharp | Yes | If UE has SRB data to send via SCG (e.g. MCG failure information if supported), UE should be able to send an indication. |

### 2.1.2	MN-SN interactions

RAN3 already agreed that the MN-initiated SN modification procedure can be used by the MN in order to activate the SCG. This could be triggered e.g. when DL traffic towards the MCG leg of a split bearer is increasing.

In existing specifications, the SN has the possibility to reject an MN initiated modification. However, the SN can maintain a suitable SCG configuration for the UE while the SCG is deactivated (the SN can reconfigure the UE if needed), so if the MN only requests activation of the SCG, there may be no reason for the SN to reject the activation request.

Q11: Do companies think that, under normal circumstances (e.g. no network malfunction), the SN might reject a request from the MN to activate the SCG, even though the MN does not request any modification of the UE configuration?

| Company | Yes/No | If yes, please explain the scenario |
| --- | --- | --- |
| Apple | No, but we are open | We see no reason for SN to reject as MN might have other reasons to activate SCG. But we are open to views. |
| Interdigital | No strong view (but prefer “No”) | As explained by the rapporteur, if there was no SCG modification, it is not very clear why the SN may reject the activation |
| LG | No | There seem to be no reason to reject SCG reactivation. |
| Intel | Yes (but should be up to RAN3) | It is not clear in what grounds SN can reject MN’s request to activate SCG (e.g. due to MN terminated bearers using SCG resources), but we generally think that SCG should be fully in charge of SN in terms of (re)activation, and it may be better to allow SN to reject. It is typical that SN is allowed to reject unless request for release. |
| NEC | Yes | if the bearer is MN terminated or SN terminated split bearer which is using MCG bearer and the SCG (or SN) is overloaded, the SN may want to reject. |
| Ericsson | Yes | We think following the principle that the SN is responsible for its own resources, the SN should have the possibility to reject the SCG activation. This discussion is though more related to RAN3 and they should have the final say. |
| Spreadtrum | No but | But it can be decided by RAN3. |
| Sharp | No but | We think there is no reason to reject but this is up to RAN3. |

If SCG deactivated state with SCG bearers is supported (see Q7), when downlink data arrives for an SN-terminated SCG bearer while the SCG is deactivated, the SN could request the MN to activate the SCG.

Q12: Do companies think that, under normal circumstances (e.g. no network malfunction), the MN might reject a request from the SN to activate the SCG, even though the SN does not request any modification of the UE configuration?

| Company | Yes/No | If yes, please explain the scenario |
| --- | --- | --- |
| Apple | Mostly no but it depends… | We think that if SCG intends to activate the SCG, there would be good reason (data to be sent etc..). Even if the MN has “other thoughts” like SN change, we already have signalling procedures and they can be done after the SCG activation. So in most cases we do not think, but MN might decide not to, if the UE has earlier provided a preference for SCG deactivation… but we think under normal circumstances no. |
| Interdigital | No strong view ( but prefer “No”) | Not clear what happens in the case of rejection, e.g. if the request was due to DL data arrival for an SN terminated SCG bearer. |
| LG | No mostly | Agree with Apple |
| Intel | No (but should be up to RAN3) | SCG should be in charge of SN in terms of (re)activation and if SN decides to (re)activate SCG, then there is no reason for MN to reject. If SN requests to change some MCG resources for a SN terminated bearer, MN may be able to reject (via a nested MN-initiated SN modification procedure) but still MN should not reject sending SCG (re)activation command that was decided by SN. |
| NEC | Yes | In normal case, the MN has to accept. One concern is a possible race condition (even though it is rare case), where the MN has already initiated the SN change procedure, right before receiving the SCG activation request from the SN. If the MN cannot reject the request, the MN must stop the ongoing SN change procedure. This restriction is not so nice for network flexibility. Given the MN should accept the request on its own responsibility unless there is the valid reason (e.g. race condition above), it would be better to allow the MN to reject. |
| Ericsson | Yes | We think that there may be circumstances under which the MN may want to refuse the activation, same as with other SN triggered SCG modifications. One example could be the SN change, i.e. it may be necessary to change the SN before the activation. Same as in Q11, this discussion is though more related to RAN3 and they should have the final say. |
| Spreadtrum | No | PSCell change can be performed when SCG is deactivate for UE. So there seems no reason to reject the SCG activation requested by SN. |
| Sharp | No but | We think there is no reason to reject but this is up to RAN3. |

## 2.2	SCG deactivation

### 2.2.1	Radio interface

Unlike for activation, deactivation could be indicated to the UE via the MCG or via the SCG.

Q13: Do companies want to support indicating SCG deactivation to the UE via the MCG?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes |  |
| MediaTek | Yes |  |
| Interdigital | Yes |  |
| LG | Yes |  |
| Intel | Yes | It has to be supported as RRC based signalling is the baseline. |
| NEC | Yes |  |
| Ericsson | Yes |  |
| Spreadtrum | Yes |  |
| Sharp | Yes |  |

Q14: Do companies want to support indicating SCG deactivation to the UE via the SCG?

| Company | Yes/No | Comments (if any) |
| --- | --- | --- |
| Apple | Yes | We think this is also viable and faster. |
| MediaTek | No | From SPEC design or from UE implementation, single control source is more simple and straightforward. If MN anyway should be informed while SCG is deactivated, we think there is no much delay difference to use MN or SN to trigger the deactivation. |
| Interdigital | No strong view | It is not a big problem if deactivation takes more time (i.e. via the MCG), but if it can be realized without too much complexity (standardization effort), we also support direct deactivation from the SCG. |
| LG | Yes |  |
| Intel | Yes | Why not. |
| NEC | No | Our basic assumption is that the MN can know the SCG activation/deactivation. |
| Ericsson | No | We don't foresee an SCG deactivation via SRB3 and it adds complexity. Please note that SCG activation via SRB3 will not be possible since the UE does not monitor PDCCH on SCG. For a split SRB1 yes, technically it may come via SCG. |
| Spreadtrum | No | We think it’s better to inform the MN of the SCG deactivation and MN can send the SCG deactivation indication to UE. |
| Sharp |  | It should be clarified first that this "deactivation" requires the response message (e.g. RRCReconfigurationComplete). If the response message is needed, we need to consider how to send this message. |

SCG deactivation could be initiated by the MN or the SN when, for a certain time, there is low (or no) traffic on all split bearers and there is no traffic on any SCG bearer.

For split bearers, the network has its own decision criteria to decide whether the SCG RLC bearer is needed or not (e.g. the load on the MCG) that the UE is not aware of, so it seems difficult to specify any request from the UE to deactivate the SCG if there are split bearers.

If the UE is configured only with SCG bearers (i.e. no MCG or split bearer besides SRB1/2), if no data transmission is expected for a while, it is even possible to move the UE to RRC_INACTIVE or RRC_IDLE, so the indication seems to be a general purpose indication that the UE does not expect any traffic soon.

Q15: Do companies see the need that the UE provides some information to the network, which would be specifically useful for the network decide to deactivate the SCG?

| Company | Yes/No | If yes, please explain |
| --- | --- | --- |
| Apple | We didn’t fully understand the question. Pls see the comments. | If it is about UE providing it’s preference (and this is ‘some information’), then yes, we see the need for the UE to provide some info. If it about UE providing additional information along with the preference (on why the UE prefers the SCG deactivation etc..) then we do not see compelling reason to have the UE provide this info to the NW. There can be some reasons at the UE to request deactivation like power saving when the UE anticipates there is no data. The end result is a deactivation (and we do not see any need to change the RB config). So we see no compelling reason to have this info to the NW. But we are open to views. |
| MediaTek | Not so far, but open for discussion | We think the major motivation is for overheating or power saving (low traffic) purpose, but seems already covered by current assistance information. But we are open for discussion. |
| Interdigital | Yes | Though network have some information about the conditions at the UE (e.g. based on measurements, BSR, etc), the more information the network gets about the latest conditions at the UE, the more informed decision the network can make. Considerations could be made also regarding UE preferences. |
| LG |  | Providing UE input may be for optimisation. Need to take some more time to clarify in which scenario the UE can provide usefully. |
| Intel | Yes, but | Regardless of which information is provided from the UE, the final decision to deactivate SCG should be up to NW. |
| NEC | Yes | We understand this is a kind of UE assistance information (UAI) discussed online. This way should be taken, rather than explicit indication for deactivation request from the UE |
| Ericsson | Maybe | Not fully clear what the question is, but in general we are open to discuss enhancements to UE assistance information, in order to provide input to network. |
| Spreadtrum | Not sure | The UE can use legacy procedure to provide its preference to the network and it can be used to make the decision of SCG deactivation by network. |
| Sharp | No, but open for UAI discussion | We think BSR is enough to consider UE's situation. Additionally, existing semi-static indications (like preference indication or overheating indication) can be considered for this purpose. However, we are open to discuss the enhance of UAI. |

### 2.2.2	MN-SN interactions

As mentioned before, SCG deactivation could be initiated by the MN or the SN when, for a certain time, there is low (or no) traffic on all split bearers and there is no traffic on any SCG bearer.

For MN-terminated split bearers, only the MN can know whether the traffic is low enough so that the SCG can be deactivated. For SN-terminated split bearers, only the SN can know how much traffic is conveyed via the SCG.

Supposing there is a "SCG deactivation request" from the MN to the SN or from the SN to the MN, it seems rather likely that the MN and t

Q16: Do companies agree that, if the MN (or respectively the SN) requests SCG deactivation to the SN (respectively to the MN), it could be very common that the SCG deactivation is actually not possible for the node that receives the request?

| Company | Yes/No | If yes, please explain |
| --- | --- | --- |
| Apple | It depends | At least SCG, we think a deactivation request made by the SCG is valid to be not rejected. But this needs to be discussed case-by-case. |
| Interdigital | No strong view |  |
| LG | Maybe No | Regardless of the node that request, it would be very common that SCG deactivation is actually possible for the node that receives the request. But this needs to be discussed case-by-case. |
| Intel | No | For decision to deactivate SCG, the baseline assumption should be that a node who requests to deactivate SCG to the other node is requesting because it should be confirmed by the other node. From this sense, the node that receives the request is possible to make final decision based on its own situation. For example, if there is a MN terminated bearer using SCG resources, then SN (who detects no traffic on SN terminated bearers) has to request SCG deactivation to the MN as it is oblivious of traffic activity on that MN terminated bearer using SCG resources. MN can make final decision based on its situation. But, MN cannot decide to deactivate SCG on its own even if there is no traffic activity in MN terminated side as MN is oblivious of how SN manages SN terminated DRBs (unless using MCG resources). On the other hand, if there is no MN terminated bearer using SCG resources, then SN should be able to decide SCG deactivation and sends to the UE. In this case, MN who receives such request from SN should not reject sending SCG deactivation command that was decided by SN. |
| NEC | No (not very common) | if the MN requests SCG deactivation to the SN when there is no SN terminated bearer, the SN can accept most likely. Similarly, if the SN request the SCG deactivation to the MN when there is no MN terminated bearer using SCG, the MN can accept most likely. Otherwise, the receiving node may reject the request for SCG deactivation. |
| Ericsson | Not clear | It is not clear what is meant here with “very common”. This will depend on network implementation and how well MN and SN are coordinated. Note that there are already activity notification signalling between nodes to inform about traffic activity, or lack of it. But we agree that this may be one reason to reject SCG deactivation. Ultimately, similar as Q11 and Q12, we believe this is more of a RAN3 topic. |
| Spreadtrum | Not sure |  |
| Sharp | Yes if | If the using of MN terminated bearers which use SCG resources is very common, it may be very common because SN may not be able to take MN terminated bearers into account correctly. |

# 3	Conclusion

…