3GPP TSG-RAN2 Meeting #113-e	draft_R2-2102061

- Omline, 25th January – 5th February 2021

Agenda Item:	4.2

Source: 	Huawei

Title:  	Offline 401 – Applicability of CRS muting configuration

Document for:	Discussion and decision

# Introduction

This document is the summary of the offline email discussion “[AT113-e][401][ eMTC R15] Applicability of CRS muting configuration (Huawei)”, as indicated below:

- [AT113-e][401][ eMTC R15] Applicability of CRS muting configuration (Huawei)

Status: Started

Scope:

Week 1: Check whether the intention is agreeable and there is sufficient support in principle; collect initial comments.

Week 2: Agreeable CRs, if there is sufficient support

Intended outcome:

Week 1: Report in R2-2102061

Week 2: Agreeable 36.331 CRs in R2-210XXXX and R2-210XXXX (to be announced later)

Deadline: Wednesday 2021-01-27 4:45 UTC

# Discussion

This document discusses the CRs below:

R2-2101041	Correction to the applicability of CRS muting configuration	Huawei, HiSilicon	CR	Rel-15	36.331	15.12.0	4565	-	F	LTE_eMTC4-Core

R2-2101042	Correction to the applicability of CRS muting configuration	Huawei, HiSilicon	CR	Rel-16	36.331	16.3.0	4566	-	A	LTE_eMTC4-Core

The intention is to check whether the intention of the CR is agreeable and whether there are comments on the actual proposed changes.

Companies are requested to provide comments in the table below (one row for each new comment to better keep track of the discussion – please don’t edit the previous comments).

| Company | Do you agree with the intention? yes/no | Detailed comments |
| --- | --- | --- |
| Qualcomm | Yes | CR to TS 36.331 is fine. Generally, most CE capabilities are applicable to BL UE and UE in CE but this is one case where ce-CRS-IntfMitig-r15 is only applicable to BL UE. Therefore should TS 36.306 be also updated to clarify the corresponding capability is only applicable to BL UE? #### 4.3.4.125 ce-CRS-IntfMitig-r15 This field indicates whether the BL UE supports CRS interference mitigation, i.e., value supported indicates UE does not rely on the CRS outside certain PRBs and subframes as defined in TS 36.133 [16], clauses 3.6.1.2 and 3.6.1.3 and TS 36.213 [23] when operating in coverage enhancement mode. A UE indicating support of ce-CRS-IntfMitig-r15 shall also indicate support of ce-ModeA-r13. |

# Conclusion

TBD

# Annex – Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Huawei (Rapporteur) | Brian Martin | brian.alexander.martin@huawei.com |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |