3GPP TSG-RAN WG2 Meeting #113 electronic	                                                                             R2-21xxxxx

25th January – 5th February 2021

Agenda Item:	7.2.2

Source: 	ZTE (email discussion rapporteur)

Title:  	                  Report of [AT113-e][402][eMTC R16] SIB change notification in RRC_INACTIVE (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT113-e][402][eMTC R16] System information change notification in RRC_INACTIVE”, as indicated below:

- [AT113-e][402][eMTC R16] System information change notification in RRC_INACTIVE (ZTE)

Status: Started

Scope:

Week 1: Check whether the intention is agreeable and there is sufficient support in principle; collect initial comments.

Week 2: Agreeable CR, if there is sufficient support

Intended outcome:

Week 1: Report in R2-2102062

Week 2: Agreeable 36.331 CR in R2-210XXXX (to be announced later)

Deadline: Wednesday 2021-01-27 4:45 UTC

# Offline email discussion

## 2.1 Solutions

In last RAN2#112-e meeting, based on the contributions, the issue of system information change notification for UE in RRC_INACTIVE was discussed but no agreement was achieved.

In the current RRC specification, the SIB acquisition/update procedure only mentions the UEs in RRC_IDLE and RRC_CONNECTED state, no description for UE in RRC_INACTIVE. As there is a high level description that the UE in RRC_INACTIVE applies RRC _IDLE procedures unless specified otherwise, one possible assumption is that SIB acquisition/update for UE in RRC_INACTIVE would follow the current procedures for UE in RRC_IDLE. That means, if eDRX is configured and the Paging eDRX cycle is longer than BCCH modification period, the UE in RRC_INACTIVE would always detect system information change with systemInfoModification-eDRX IE during eDRX acquisition period, if transmitted.

Considering that the maximal DRX cycle for eMTC UE in RRC_INACTIVE would be the maximum value of the RAN paging cycle, e.g., rf1024, which can be far less than the eDRX acquisition period (e.g. hf256), if simply reusing the system information acquisition principles as that for UE in RRC_IDLE, the system information acquisition will be not timely when DRX cycle is longer than the BCCH modification period. However, during the discussion in RAN2#112-e meeting [1], some companies thought this issue doesn’t matter much. The only need is to clarify what does the “DRX cycle” in RRC spec which is used to compare with modification period mean to UE in RRC_INACTIVE.

In current RAN2#113 e-meeting, several contributions are submitted with the following proposals:

| Contribution (Company) | Proposals |
| --- | --- |
| R2-2100932 (ZTE) | Proposal 1: It’s suggested to clarify that RRC_INACTIVE UE should detect system information change via Paging message with the same indicator as used in RRC_IDLE mode. |
| R2-2101038 (Huawei) | Proposal 1: UEs in RRC_INACTIVE follow systemInfoModification-eDRX notification if the used DRX Cycle is larger than the BCCH modification period. Proposal 2: UEs in RRC_INACTIVE use the eDRX cycle for monitoring system information change. |
| R2-2101155 (Qualcomm) | Proposal 1: UE in RRC_INACTIVE state follows same system modification indication as it does in RRC_IDLE. |
| R2-2101467 (Ericsson) | Addition of a NOTE in 5.2.1.3 to clarify that UE in RRC_INACTIVE compares the configured RAN paging cycle length to the configured modification period, to determine whether it follows the eDRX acquisition period. Addition of RRC_INACTIVE in 5.2.1.3 so that UE in RRC_INACTIVE monitors for system information update notification, and compares the paging cycle length to modication period. |

In [R2-2101038], even company has the Observation 4 “In RRC_INACTIVE, if the UEs is not configured with idle eDRX or if the UE is configured with idle eDRX and its RAN paging cyle is shorter than or equal to the BCCH modification period, following systemInfoModification notification is more efficient for power consumption.”, based on the following further observation:

Observation 7: Having to read the MIB before resuming an RRC connection in RRC_INACTIVE may be an acceptable trade off to keep the UE behaviour identical in RRC_IDLE and RRC_INACTIVE.

They still suggest UEs in RRC_INACTIVE to follow systemInfoModification-eDRX notification.

In [R2-2101155], based on the following observations, company gives a similar proposal as that in [R2-2100932] and [R2-2101038].

- Observation 4: All UEs will use the new system information before access regardless of whether UE follows systemInfoModification or systemInfoModification-eDRX.

- Observation 5: UE in RRC_IDLE or RRC_INACTIVE state can receive either of the two system information modification indications regardless of whether UE follows systemInfoModification or systemInfoModification-eDRX.

It seems more companies prefer that UE in RRC_INACTIVE follows same system modification indication as it does in RRC_IDLE. But there is still different opinion. Therefore, we have the following question:

Q1: Companies are invited to indicate which option is preferred for clarification on SIB change notification for UE in RRC_INACTIVE?

- Option 1: UE in RRC_INACTIVE follows same system modification indication as it does in RRC_IDLE. E.g., to clarify that the “DRX cycle” in TS 36.331 section 5.2 which is used to compare with modification period is the idle mode eDRX cycle.

- Option 2: To clarify that, for UE in RRC_INACTIVE, the “DRX cycle” in TS 36.331 section 5.2 which is used to compare with modification period is the configured RAN paging cycle.

| Company | Preference | Detailed comments |
| --- | --- | --- |
| ZTE | Option 1 |  |
| Qualcomm | Option 1 |  |
| Huawei, HiSilicon | Option 1 | No strong option. We are also fine with option 2 as we think it does not really matter system wise and power consumption wise. |

Conclusion:

Proposal:

## 2.2 Possible changes for solutions

Rapporteur quote the main changes from different CRs as following for comparison:

| R2-2100936 (ZTE) | TS 36.331 (5.2.1, 5.2.2) , For example: 5.2.1.3 System information validity and notification of changes |
| --- | --- |
| R2-2101036 (Huawei) | TS 36.331 (5.2.1.3, 5.2.2.4), For example: 5.2.1.3 System information validity and notification of changes |
| R2-2101467 (Ericsson) | TS 36.331 (5.2.1.3) 5.2.1.3 System information validity and notification of changes |

Q2: Companies are invited to provide their detailed suggestions or comments for the corresponding CRs.

| Company | Preferred CR changes | Comments |
| --- | --- | --- |
| ZTE | Slightly prefer changes in R2-2100936. | It’s clearer to mention RRC_INACTIVE and to use the wording “idle mode eDRX cycle”. |
| Qualcomm | None | - Changing DRX to eDRX is not right as this may not be backwards compatible. This has been in specification since Release 13. - We don’t think it is right to add clarification as to which modification period applies to RRC_INACTIVE because this will only diminish the purpose of the following sentence in the existing specification: - The UE: - Applies RRC_IDLE procedures unless specified otherwise; |
| Huawei, HiSilicon | R2-2100936 | We think a clarification is needed as, per the discussion, companies have different understanding of which “configured” DRX cycle is used to monitor system information, as it is specified as the eMTC UE in RRC_INACTIVE is configured with the RAN paging cycle. We think R2-2100936 is simple and we do not think there is any backward compatibility issues as there is only one configured DRX cycle in RRC_IDLE. We are open to other way to clarify. |

Conclusion:

Proposal:

Q3: Is there any other issue or comments, e.g., whether the CR needs to be approved in LTE main session? (with consideration that idle mode eDRX is not supported for eLTE)

| Company | Comments |
| --- | --- |

Conclusion:

Proposal:

# Summary

Conclusion:

Updated CRs:

# References

[1] 3GPP, R2-2010816, Report of R16 SIB acquisition for UEs in RRC_INACTIVE, RAN2#112-e, November 2020

[2] 3GPP, R2-2100932	Discussion for clarification on SIB acquisition for UE in RRC_INACTIVE	ZTE Corporation, Sanechips, discussion, RAN2#113-e, January 2021

[3] 3GPP, R2-2100936	Clarification on SIB acquisition for UE in RRC_INACTIVE	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.3.0	4555, RAN2#113-e, January 2021

[4] 3GPP, R2-2101038	System information change notification in RRC_INACTIVE	Huawei, HiSilicon, discussion, RAN2#113-e, January 2021

[5] 3GPP, R2-2101036 Clarification to the DRX cycle in RRC_IDLE and RRC_INACTIVE, Huawei, HiSilicon, RAN2#113-e, January 2021

[6] 3GPP, R2-2101155	SIB acquisition by eMTC UE in RRC-INACTIVE	Qualcomm Incorporated, discussion, RAN2#113-e, January 2021

[7] 3GPP, R2-2101467	Clarification of SI acquisition for UEs configured with eDRX in RRC_INACTIVE	Ericsson LM	CR	Rel-16 	36.331 	16.3.0 4578, RAN2#113-e, January 2021

# Annex – Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| ZTE (Rapporteur) | Ting Lu | lu.ting@zte.com.cn |
| Qualcomm | Mungal Dhanda | mdhanda@qti.qualcomm.com |