3GPP TSG-RAN WG2 Meeting #113e                                                 draft-R2-2102064

E-Meeting: January 25- February 05, 2021

Agenda item:	7.2.3

Source:	Qualcomm Incorporated

Title:	[AT113-e][404][eMTC R16] PDCCH-based HARQ-ACK for multi-TB scheduling

Document for:	Discussion and Decision

# Introduction

RAN1 has agreed a change in PDCCH-based HARQ ACK when using multiple TBs transmission. The corresponding RAN2 CR is provided in [1]. Based on [1], the following email discussion is started.

- [AT113-e][404][eMTC R16] PDCCH-based HARQ-ACK for multi-TB scheduling (Qualcomm)

Status: Started

Scope:

Week 1: Check whether the intention is agreeable and there is sufficient support in principle; collect initial comments.

Week 2: Agreeable CR, if there is sufficient support

Intended outcome:

Week 1: Report in R2-2102064

Week 2: Agreeable 36.321 CR in R2-210XXXX (to be announced later)

Deadline: Wednesday 2021-01-27 4:45 UTC

This document is to provide summary on this email discussion.

# Discussion

RAN1 has agreed following text in TS 36.212 (R1-2009613, CR#0353) such that HARQ specific PDCCH-based HARQ ACK is used for transmissions with scheduling of multiple TBs. The new change is shown in color font.

If ce-PUSCH-MultiTB-Config is not enabled and the Resource block assignment in format 6-0A is set to all ones, or ce-PUSCH-MultiTB-Config is enabled and mpdcch-UL-HARQ-ACK-FeedbackConfig is configured and the 6 MSB bits of the Scheduling TBs for Unicast Field are set to '110111', format 6-0A is used for the indication of ACK feedback. 8 bits including the 6 LSB bits of the Scheduling TBs for Unicast Field and 2 MSB bits of Repetition number are used to indicate HARQ-ACK by bitmap, where the order of the bitmap to HARQ process index mapping is such that HARQ process indices are mapped in ascending order from MSB to LSB of the bitmap. For each bit of the bitmap, value 1 indicates ACK, and value 0 is reserved. And all the remaining bits except Flag format 6-0A/format 6-1A differentiation and DCI subframe repetition number are set to zero.

However, in TS 36.321, the PDCCH-based HARQ-ACK is assumed to be the acknowledgement for all HARQ processes for which PUSCH transmission is completed regardless of multiple TBs transmission as shown below.

- if the PDCCH indicates an UL HARQ-ACK feedback for an asynchronous UL HARQ process for a UE configured with mpdcch-UL-HARQ-ACK-FeedbackConfig; and - if the PUSCH transmission is completed: - stop drx-ULRetransmissionTimer for all UL HARQ processes.

To correct this, a draft CR for TS 36.321 is provided in [1].

Question 1: Do you agree with the intent of the change in [1]? Please elaborate if further change/clarification is needed in the comments.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Ericson | Yes | We agree with the intention, however we should check that the resulting change covers all of the cases: multi-TB and single-TB and implicit and explicit HARQ-ACKs. The existing text (before suggested case) seems to cover the explicit HARQ-ACK cases for both multi-TB and single-TB case, but without the specific bit field introduced in the referred RAN1 changes. With the proposed change in the CR, the resulting “else” clause seems not to be correct as it refers to “all UL HARQ processes” although this would be only for the single-TB case? We think this should at least be clarified to refer to a single-TB case, if this is the intention. We would further like to check whether for the implicit case (when a specific HARQ-ACK is not sent but a DCI scheduling new data is sent and received) is correctly captured for the multi-TB case and whether clarifications are needed. |
| Huawei, HiSilicon | Yes | RAN1 agreed to support HARQ-ACK for individual TB. Since the HARQ-ACK feedback is provided using a bitmap corresponding to multiple HARQ processes, the text should refer to plural e.g. - stop drx-ULRetransmissionTimer for the corresponding UL HARQ processes. This can potentially also address the Ericsson concern above. In addition, the consequence if not approved on the cover page seems more severe than misalignment, based on impact analysis. This appears to functionally be a NBC. |
| Qualcomm | Yes | In our understanding, this section particularly is not applicable for implicit case (DCI scheduling new transmission). DCI can either indicate PDCCH-based HARQ ACK or UL grant for new transmission but not both together. Suggestion from Huawei is also fine to change process to process(es). We think this is not the case of NBC for the December version specifications because it would be just alignment to RAN1 text. |

Question 2: Is the any change needed in the specification other than TS 36.321? if yes, please elaborate.

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | No other impact identified |

# Conclusion

TBD..

# References

[1] R2-2100735,	PDCCH-based HARQ-ACK for a specific HARQ process with multi-TB scheduling, Qualcomm Incorporated, RAN2#113e.