3GPP RAN WG2 Meeting #113-e	R2-201xxxx
January 25th– February 5th, 2021

Agenda Item:	8.5.3

Source:	InterDigital

Title:	Summary of URLLC over unlicensed controlled environment

Document for:	Discussion, Decision

# Introduction

This is the summary report for tdocs submitted to the IIoT unlicensed controlled environment agenda item and for offline discussion 505:

- [AT113-e][505][IIoT] Summary of URLLC in unlicensed (InterDigital) Scope: - Identify set of open issues for UCE that need to be addressed based on company contributions and identify any agreeable aspects to be discussed in the first week session - Get company inputs on opens issues (to be kicked off after first session) Intended outcome: - Set of issues that should be discussed in the first session and any proposals that could be agreeable - Set of additional issues that should be addressed but with lower priority Deadline for providing comments: - Companies comments on the summary: January 29th 12:00 UTC (TBC)

| Company | Contact Name, Email |
| --- | --- |
| Huawei,Hisilicon | Li Zhao (zhaoli8@huawei.com) |
| MediaTek | Pradeep Jose (pradeep[dot]jose[at]mediatek[dot]com) |
| CATT | Pierre Bertrand (pierrebertrand@catt.cn) |
| Yassin Awad | Yassin.Awad@sony.com |
| Lenovo | Joachim Löhr (jlohr@lenovo.com) |
| ZTE | Dong Fei (dong.fei@zte.com.cn) |
| LG | SunYoung LEE (ssunyoung.lee@lge.com) |
| Sharp | Fangying.xiao@cn.sharp-world.com |
| III | Yen Chih Kuo (jasonkuo@iii.org.tw) |
| APT | Henry Chin, HenryChin@fginnov.com |
| OPPO | Zhe Fu(fuzhe@OPPO.com) |
| Nokia | Ping-Heng Wallace Kuo (Ping-Heng.Kuo@nokia.com) |
| Ericsson | Zhenhua Zou (Zhenhua.Zou@ericsson.com) |
| Qualcomm | Sherif ElAzzouni (selazzou@qti.qualcomm.com) |
| vivo | Lihui Wang(wanglihui@vivo.com) |
| Apple | Ralf Rossbach (rrossbach@apple.com) |
| Samsung | Sangkyu Baek (sangkyu.baek@samsung.com) |
| Intel | Yujian Zhang (yujian.zhang@intel.com) |
| Xiaomi | Yumin Wu (wuyumin@xiaomi.com) |
| TCL | Hejun Wang (hejun.wang@tcl.com) |
| Sequans | Olivier Marco (omarco at sequans.com) |

# Open issues

## Configuration of cg-RetransmissionTimer and LCH-based prioritization

Release-16 introduced intra-UE prioritization to handle overlapping uplink grants, which is enabled by configuring LCH-based prioritization. The feature is a key feature of IIoT and is already supported in R16, and it is beneficial regardless of spectrum type (licensed or unlicensed). It’s still, however, FFS if LCH based prioritization can be configured with cg-RetransmissionTimer (CGRT).

It is therefore proposed by [2, 3, 4, 5, 9, 10, 11, 13, 14, 15, 17, 19, 20] to support configuring both LCH-based prioritization and CGRT. In the last email discussion [22], all companies agreed that LCH-based prioritization should be supported for R17 URLLC in an unlicensed controlled environment (UCE). The following is therefore proposed:

Proposal 1: 	LCH based prioritization and cg-RetransmissionTimer can be configured together in Rel-17

Question 1: Do you support the option to configure both LCH based prioritization and cg-RetransmissionTimer together in Rel-17? 
Inputs requested after completion of the first session.

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | Yes | LCH based prioritization has been introduced for MAC layer in the Intra-UE prioritization feature to handle collision among uplink grants while cg-RetransmissionTimer is used to control autonomous retransmission on the configured uplink grants on unlicensed spectrum. These two are designed from different dimensions and for resolving issues in different situations. So we support simultaneous configuration of these two parameters. |
| MediaTek | Y | They serve different purposes and should be supported together. |
| CATT | Yes | As correctly mentioned by the rapporteur, this issue was discussed in offline 501 in RAN2#112-e meeting. All companies shared the same view that LCH-based prioritization can be supported for Rel-17 URLLC in (UCE) in question 7, at least for prioritization among initial transmissions. The only issue is related to the retransmission case which, for Rel-17, is addressed in this email discussion in Q7, so we see no problem to apply the LCH based prioritization together with cg-RetransmissionTimer in R17. |
| Sony | Yes | It is obviously beneficial feature also when cg-RetransmissionTimer is configured. |
| Lenovo | Yes | We think that LCH-based prioritization is a key feature for ensuring the QoS support of URLLC traffic. Therefore, IIoT intra-UE prioritization mechanism with LCH-based prioritization should be also supported in an unlicensed controlled environment (UCE) when cg-RetransmissionTimer is configured. |
| ZTE | Yes | Since the LCH priority handling is one of the features of the URLLC configured grant, if WI is including the harmonization of the URLLC configured grant and NRU configured grant ,we think LCH based prioiritization shall be naturally configured with the CGRT. |
| LG | Yes | LCH-basedPrioritization is a key feature in support of URLLC. |
| Sharp | Yes | The two features are for different purposes. cg-RetransmissionTimer is already supported in Rel-16 NR-U and if UL Grant overlapping still happens in UCE, the benefit of supporting LCH based prioritization is same as in IIoT. Therefore, It is obviously we should support to configure both LCH based prioritization and cg-RetransmissionTimer together in Rel-17. |
| III | Yes | The LCH-based prioritization is a key feature of the URLLC to handle collision among different uplink grants. Therefore, for the URLLC in UCE, the LCH-based prioritization should be also supported even when cg-Retransmission timer is configured. |
| APT | Y | Intra-UE prioritization and LBT handling should both be supported in UCE. Hence, we should enable the network to configure both LCH-based prioritization introduced in Rel16 IIoT and cg-RetransmissionTimer in Rel16 NR-U to a UE. |
| OPPO | Yes | The intentions and the functionalities for the LCH-BasedPrioritization and CGRT are different, so, there is no issue to support simultaneous configuration. |
| Nokia | Yes | As now we are considering URLLC, the key feature such as LCH-based prioritization has to be applied in order to make sure URLLC data can be transmitted with higher priority. |
| Ericsson | Yes | LCH-based prioritization is beneficial for multiplexing different priority services. This is independent of shared spectrum and cg-RetransmissionTimer usage. |
| Qualcomm | Yes | Agree with Nokia. |
| vivo | Yes | LCH based prioritization and cg-RetransmissionTimer are introduced for different purposes. No technical reasons to prevent configuring the two features together, rather configuring both are beneficial to transmission in UCE. |
| Apple | Yes | If NR-U is meant to support URLLC type of services, then prioritization between eMBB and URLLC, including overlapping grants, will be required. LCH-based prioritization can help achieve that goal. |
| Samsung | Yes | LCH-based prioritization is one of the main features of Rel-16 IIOT for URLLC traffic. It should be adopted for URLLC in UCE. |
| Intel | Yes |  |
| Xiaomi | Yes | The CG configuration requiring the UE autonomous retransmission due to LBT-failure would require the configuration of the cg-RetransmissionTimer. The URLLC service would require the LCH-based prioritization. |
| TCL | Yes | LCH-based prioritization is to handle collision among different UL grants in MAC layer, including initial transmission while cg-RetransmissionTimer is used to control autonomous retransmission on the configured UL grants on unlicensed spectrum. As is mentioned by HW, they are designed from different dimensions for resolving issues in different situations. So we think LCH-based prioritization and cg-RetransmissionTimer can be configured together, e.g., LCH-based prioritization can be configured in UCE even when cg-ReTransmissionTimer is configured, at least for the initial transmission. |
| Sequans | Yes |  |

## Autonomous re-transmissions

In R16, the configuration of autonomousTx or CGRT allows the UE to autonomously (re)-transmit a PDU dropped due to intra-UE prioritization or LBT failure, respectively. One difference is that autonomous transmission in IIoT is restricted to use the next CG occasion of same CG configuration, unlike NR-U. Per the assumption in RAN2#112e, the network will not configure autonomousTx and CGRT simultaneously per cell, at least for R16. For release 17, CGRT operation can be harmonized with autonomous re-transmission; it was agreed last meeting that a deprioritized PDU on a configured grant and/or dropped PDU due to LBT failure or tx failure should be able to be autonomously re-transmitted.

### With cg-RetransmissionTimer configured

With CGRT configured, how does the UE retransmit a TB dropped either due to intra-UE prioritization, due to LBT failure, or due to bad radio conditions? It’s already specified in MAC that a TB dropped due to LBT failure can be retransmitted if the relevant CGRT is not running, using the NR-U framework. The question is whether the same behavior should apply for a deprioritized TB when both LCH-based prioritization and CGRT are configured. This is suggested in [6, 10，11, 15, 16, 19]. The gNB can figure whether the transmission is a new transmission or a retransmission by decoding the NDI value in the CG UCI.

References [4, 9, 15] also mention that both CGRT and autonomousTx can be configured together and work without necessary specification changes, as autonomousTx only controls new transmissions in the HARQ entity while CGRT controls the retransmissions independently. References [5, 6, 13, 14] agree, but propose an enhancement for CGRT, explained in the next section. [16] proposes to mandate the configuration of autonomousTx and LCH-based prioritization for all CGs in a cell group. References [2, 12, 18, 21] on the other hand propose to forbid configuring both together, as there could be conflicting behaviors whereby with CGRT configured the UE behavior is to treat the dropped PDU as a retransmission while with autonomousTx configured the UE treats it like a new transmission.

Question 2-a: Can cg-RetransmissionTimer and autonomousTx be configured together in R-17?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | No |  |
| MediaTek | Y | They serve different purposes and can be configured together. |
| CATT | No | 1) Configuring both autonomousTx and cg-RetransmissionTimer in current specification does not work and leads to abnormal behaviour: Current specification mandates CGRT to stop when CGT expires (5.4.2): If the configuredGrantTimer expires for a HARQ process, the HARQ process shall: 1> stop the cg-RetransmissionTimer, if running. But latest R16 IIOT MAC change in R2-2011075 CR now stops the CGT upon CG deprioritization when autonomousTx is configured. Since this is not an expiration of the CGT, the above condition is not met and the CGRT keeps running. But this case (CGRT running and CGT not running) corresponds to none of the conditions for processing a CG as either a new transmission or a retransmission in 5.4.1. Hence the CG is simply filtered out, which is obviously not the expected behaviour. Therefore configuring both autonomousTx and cg-RetransmissionTimer requires some specification enhancements to make it work properly. 2) One obvious enhancement would consist is also stopping the CGRT upon CG deprioritization, as proposed by several companies, including us. But in that case, the pending PDU of the deprioritized CG can very well be taken care of by the NRU’s autonomous retransmission if the HARQ process is switched to pending upon the deprioritization. In such case, the deprioritization is treated the same way as an LBT failure and there is no need to involve IIOT’s autonomous transmissions. 3) We think it is a wrong approach to address URLLC on UCE by requiring a UE to support both NR-U and IIOT CG protocols. For example, R16 UEs developed for IIOT market, which are not NR-U capable should be able to benefit from a simple upgrade (see Section 2.2.2) to be able to operate CGs in a UCE environment in R17. Specifically they should not be required to support the cg-RetransmissionTimer, UE-selected HPID and HARQ sharing across CGs, CG-UCI and CG-DFI features, which add significant complexity. Inversely, a R16 UE supporting NR-U but not IIOT capable should be able to benefit from a simple upgrade to be able to operate CGs in a UCE environment in R17 without the need to support IIOT’s autonomous transmissions. |
| Sony | No | We already agreed that: - When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. - When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection. Each mechanism (NR-U or URLLC) can support to autonomously (re)-transmit of a dropped/pending PDU. Hence, no need to mix them. |
| Lenovo | Yes | We agreed that LBT failure may still happen in an unlicensed controlled environment. Conclusion was that for cases that a LBT failure occurred for a CG transmission that UE should be able to autonomously retransmit the TB. Similarly, there was an agreement that if a CG is deprioritized then UE should be able to autonomously re-transmit it. We didn’t discover any major problems when configuring both cg-RetransmissionTimer (taking care of LBT failures) and autonomousTx (covering deprioritized UL grants) together. Therefore we don’t think that it is necessary to restrict network configuration. |
| ZTE | Yes | We understand that the CGRT is to represent which workframe of configured grant transmission and autonomous retransmission shall be used (i.e NRU or URLLC), and the autonomousTX is to represent whether the related configured grant configuration support the autonomous transmission for deprioritization. For example of one configured grant: (1) CGRT is configured but autonomousTX is not, which means this configured grant shall be running as NRU no matter for normal transmission or autonomous retransmission and this configured grant configuration does not support autonomous (re)transmission for deprioritization. - Both CGRT and autonomousTX is configured , which means this configured grant shall be running as NRU no matter for normal transmission or autonomous retransmission and this configured grant configuration support autonomous (re)transmission for deprioritization - CGRT is not configured while autonomousTX is, which means this configured grant shall be running as URLLC no matter for normal transmission or autonomous retransmission and this configured grant configuration support autonomous transmission for depiroritization. - Neither CGRT nor autonomousTX is configured, legacy behavior will be adopted as R15. |
| LG | Yes | autonomousTx and cg-RetransmissionTimer handles different failure case. Unless significant problem is identified, there is no reason not to have them together. We don’t think it is a real technical problem to stop CGRT upon stopping CGT as our basic assumption is that there is no case where CGRT is running while CGT is not running. |
| Sharp | No | They are two different mechanics. We do not think there is any necessary to configure both of them together. |
| III | No | Agree with CATT. When the cg-Retransmission timer is configured, stopping the cg-Retransmission timer upon CG deprioritized is better than using autonomousTX. |
| APT | Y | cg-RetransmissionTimer and autonomousTx can be configured together to handle pending PDU and deprioritized PDU, respectively. |
| OPPO | Yes | In our understanding, the functionalities are already there and we do not find essential issue to support both together, except some small modification in current spec. thus, there is no need to restrict simultaneous configuration. |
| Nokia | No | We think this is the best to avoid configuring these two features together and keep it simple. To allow autonomous transmission, we can simply rely on cg-RetransmissionTimer by considering de-prioritized MAC PDU as pending in HARQ process as well, so it can be transmitted again. |
| Ericsson | Yes | There should be two parameters for handling independently - Autonomous retransmissions regarding LBT/transmission failure, if operator would like to enable this, i.e. by cg-RetransmissionTimer, which is agreed to be optional in shared spectrum. - Autonomous transmission of de-prioritized data resulting from LCH-based prioritization, i.e. by autonomousTx. If not configured, network can take care of providing additional grants for de-prioritized data, which network can anticipate when issuing overlapping grants (with LCH-based prioritization). Our understanding is that both mechanisms for transmission/retransmission work together, without any significant specification changes. The alternate proposal, i.e. enhancing cg-RetransmissionTimer handle also de-prioritized data, is new behavior and requires specification changes. It would also lead to coupling of LBT/transmission failure and de-prioritization handling in the UE, which may be unwanted in network implementations that handle de-prioritized retransmission without autonomous retransmissions. |
| Qualcomm | Yes | CGRT and auto-Tx can work together without modification. However, some optimizations can be added (like Q3) to further harmonize the system. We think there is utility for both features that warrants supporting their simultaneous configuration. We don’t think simultaneous configuration would cause any issues. |
| vivo | Yes | They serve different purposes and there are clear requirements to configure them simultaneously. Firstly the WID says “Harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum”, the autonomousTransmission is used to recover the deprioritized initial transmission and the cg-RetransmissionTimer is used for recovery the pending PDU including the prioritized but LBT failed MAC PDUs and transmission failed, e.g.’NACKed’ PDUs. The autonomousTransmission and cg-RetransmissionTimer are already harmonized well with no/marginal specification impacts. So we against to enhance autonomous retransmission based R16 NR-U to cover autonomous transmission based R16 IIoT, or vice versa. We already have the tools to configure them simultaneously and recover the TB dropped either due to intra-UE prioritization, due to LBT failure/transmission failure. We do not need to spend efforts to duplicate/extend the functions for either autonomousTransmission or cg-RetransmissionTimer. |
| Apple | No | For Rel-17, it was agreed that HARQ process ID and RV selection is based on NR-U when the cg-RetransmissionTimer is configured. It was also agreed that HARQ process sharing is allowed in Rel-17 when the cg-RetransmissionTimer is configured. At this point in time, it is safer not to allow simultaneous config as some inconsistencies are expected. Having said this, RAN2 can study simultaneous operation further, identify issues, and propose ways to fix it. Given that companies’ views are quite divided we would be OK to postpone the decision and allow for more time to check. |
| Samsung | No | AutonomousTx can be replaced by autonomous retransmission with CGRT. We do not need to have two similar functionality with potential specification impacts. |
| Intel | No | For Rel-17 URLLC in shared spectrum, a deprioritized MAC PDU can be eventually transmitted by NR-U autonomous retransmission scheme. Therefore, autonomous transmission in IIoT is a subset of autonomous retransmission in NR-U. Therefore it is sufficient to only configure cg-RetransmisisonTimer. |
| Xiaomi | Yes | If cg-RetransmissionTimer and LCH-based prioritization are configured together, the de-prioritized MAC PDU would need to be retransmitted. We understand that some specification change would anyway be needed if we configured cg-RetransmissionTimer (i.e. for LBT failure) and autonomousTx (i.e. for the deprioritized MAC PDU) together, as the Rel-16 IIOT UE autonomous retransmission does not allow the UE to retransmit the deprioritized MAC PDU cross CG configurations. |
| TCL | Yes with enhancement | autonomousTx and cg-RetransmissionTimer are two mechanisms to handle transmission failures, autonomousTx support autonomous (re)transmission for deprioritization while the CGRT works well for both new transmission and retransmissions. So we think they can be configured together. However, there are abnormal behaviors while both the autonomousTx and CGRT configured as is mentioned by CATT which means specification enhancements are needed. |
| Sequans | Yes | In our view both can be handled independently. |

Question 2-b: With LCH-based prioritization and CGRT configured, do you agree that MAC entity can retransmit a deprioritized TB the same way as a TB dropped due to LBT failure (e.g. if CGRT isn’t running)?

| Company | autonomousTx configured | autonomousTx not configured | Additional comments if yes, clarify if a spec changes is need and what needs to be changed |
| --- | --- | --- | --- |
|  | Reply (y/n) | Reply (y/n) |  |
| HW | Depends on whether the CG for the deprioritized TB is new transmission or retransmission | Yes | Rely on the the pending/not-pending status as well as the running state of the configuredGrantTimer of a HARQ process to realize. If autonomousTx is not configured, then - if the CG for the TB is new transmission and de-prioritized before its transmission, the configuredGrantTimer and cg-RetransmissionTimer is not started/re-started, and the HARQ process is considered as pending - if the CG for the TB is retransmission and de-prioritized before its transmission, the configuredGrantTimer and cg-RetransmissionTimer keeps their current running states, and the HARQ process keeps its current pending/not-pending status - if the CG for the TB is deprioritized after the transmission starts, the configuredGrantTimer and the cg-RetransmissionTimer shall keep running, and the HARQ process is still considered as not pending If autonomousTx is configured, then whether the deprioritized TB can be treated the same way as a TB dropped due to LBT failure depends on whether the CG for the deprioritized TB is new transmission or retransmission. - If the CG for the deprioritized TB is new transmission, then IoT autonomous transmission should be performed - If the CG for the deprioritized TB is retransmission, then NR-U autonomous retransmission should be performed, same handling as LBT failure happens. |
| MediaTek | N | N | When autonomousTx is configured, we can follow the Rel-16 IIoT procedure and retransmit at the next available CG occasion. But of course, because CGRT is configured, the UE is able to select the HARQ PID (for the next available CG occasion) as in Rel-16 NR-U. When autonomousTx is not configured, deprioritized TB should not be retransmitted, as in Rel-16. It will be dropped as in Rel-16 IIoT case. This gives the network the flexibility to enable/disable autonomousTx. This is up to network configuration. |
| CATT | No | Yes | When autonomousTx is not configured, when a CG is deprioritized, and gNB can’t decode the partial transmission, NR-U autonomous retransmission is triggered when CGRT expires since no DFI-ACK was received. So for a deprioritized CG to be treated like an LBT failure, it requires, as suggested by the rapporteur in the question and discussed in Q2-a, that CGRT is stopped. But this is not sufficient, the HARQ process must also be switched to “pending”. When autonomousTx is configured: strictly speaking the question asks about handling deprioritized PDUs “the same way as a TB dropped due to LBT failure”. We understand this in NR-U sense i.e. the TB is sent via autonomous retransmission, as described above. In that sense it is useless to configure autonomousTx. And the 1st column has no object. The case where autonomousTx is configured (if allowed) means in our understanding, that deprioritized TBs are expected to be sent via autonomous transmissions, as in IIOT, which is different from LBT failure. This is enabled by stopping the CGRT and keeping the HARQ process pending. In any case we do not support configuring autonomousTx and cg-RetransmissionTimer together because it is the most complex solution and for the reasons explained in Q2-a. |
| Sony | cg-RetransmissionTimer and autonomousTx are not configured together in R-17. | Yes | If CG is deprioritised grant due to LCH-based prioritisation, then cg-RetransmissionTimer is not started, the HARQ process can be considered pending, and a UE must be able to retransmit the TB immediately at the next available CG occasion. This behaviour can be treated similar to LBT failure. |
| Lenovo | No | No | Not sure that we understand the question correctly. If autonomousTx is not configured, and TB is deprioritized, HARQ process is “not pending”, CGRT is not started. Therefore no autonomous retransmission (according to NR-U) is performed. In order to treat a deprioritized UL grant similar to a LBT failure the HARQ process needs to be switched to “pending” if a UL grant is deprioritized. If autonomousTx is configured then IIOT autonomous transmission mechanism kicks in. |
| ZTE | Yes by addressing Lenovo’s concern | No | As we mentioned above, when autonomousTX is not configured, it means the autonomous transmission for deprioritiation is not supported. When autonomousTX is configured, we’d like to admit the issue raised by lenovo is existing based on the current spec, but in our understanding, a Configured grant is deprirotized and the relevant MAC PDU is generated have a same situation with the LBT failure of configured grant from which the MAC PDU have been generated and cannot be transmitted as well. Therefore, we can provide a simple solution where the configure grant is deprioritized and the relevant MAC PDU have been generated can be treated as LBT failure when CGRT is configured. |
| LG | No | No | If LCH-basedPrioritization, CGRT, and autonomousTx are all configured, deprioritized TB is to be retransmit under IIOT autonomousTx handling. If LCH-basedPrioritization and CGRT are configured but autonomousTx is not, we don’t think deprioritized TB is to be retransmitted by any mechanism, which should be network’s decision. |
| Sharp | We prefer CGRT and autonomousTx are not configured together as in Rel-16 | Yes | if both LCH priority and CGRT are configured but the CG is deprioritized, the retransmission mechanic defined in Rel-16 for NR-U can be reused. The only spec change is just to consider the corresponding HARQ process as pending when the CG is deprioritized. |
| III | No | Yes | If autonomousTx is configured, the deprioritized TB can use the autonomousTx, just like Rel-16 URLLC, but HARQ PID selected should follow the Rel-16 NR-U rule. If autonomousTx is not configured, and the CGRT is not started, the HARQ PID can be considered pending. This situation just like LBT failure, and it should also transmission at the very next CG duration. |
| APT | N | Y | If autonomousTx is configured, a deprioritized TB could be handled by autonomousTx. And no enhancement (i.e., consider a deprioritized TB as a pending TB) is needed. If autonomousTx is not configured, the deprioritized TB could be retransmitted on a CG resource after CGRT expires. |
| OPPO | See comments. | Yes | Firstly, in our understanding, if the CG is deprioritized before LBT detection, PHY layer will not do LBT detect for this CG. Accordingly, HARQ process status will keep as is. So, our analysis only focusses on the case that the CG is deprioritized after LBT detection. The result of LBT detection can be success or failure. In the case that autonomousTX is not configured, and if companies agree to autonomously transmit the deprioritized CG to avoid data lost, NRU autonomous mechanism for LBT failure is reused. Accordingly, when the original CG timer status is not running and CG is deprioritized after LBT succeeds, it is better to consider the identified HARQ process associated to the deprioritized grant as pending, to also cover the case CGT equals to CGRT (otherwise, the HARQ of the de-prioritized MAC PDU will be flushed, and the MAC PDU is discarded before it is transmitted completely). In the case that autonomousTX is configured and CG is deprioritized after LBT succeeds, how to (re)transmit the deprioritized MAC PDU depending on HARQ process status and whether CGT is stopped when the CG is deprioritized. Regarding the support on the deprioritized MAC PDU, we consider the following, - If CG timer is already running, UE enters retransmission branch and considers the identified HARQ process as not pending. After the CG is deprioritized, to choose which branch, i.e. new transmission or retransmission on the subsequent CG depends on CG timer status, - If CG timer is stopped, UE can enter new transmission branch and generate a new MAC PDU, since the deprioritized MAC PDU has been transmitted before. - If CG timer is not stopped, UE can enter retransmission branch and follow NRU autonomous retransmission. - Else if the identified HARQ process is pending, UE follows current NRU mechanism and considers the identified HARQ process as not pending. Similarly, after the CG is deprioritized, if the CG timer is stopped, UE can follow IIoT autonomous transmission mechanism, otherwise, UE can follow NRU autonomous retransmission mechanism. - Otherwise, UE enters new transmission branch and considers the identified HARQ process as not pending. Similarly, after the CG is deprioritized, if the CG timer is stopped, UE can follow IIoT autonomous transmission mechanism, otherwise, UE can follow NRU autonomous retransmission mechanism. In the last meeting, it is agreed in R16 IIoT session that CGT will be stopped once the CG is deprioritized in case that autonomousTX is configured. However, it is also agreed in R17 IIoT session that autonomousTx and CGRT are not configured simultaneously. To us, it is unclear whether the agreement on stopping CGT for the deprioritized CG is also valid for all cases when autonomousTX and CGRT are configured together. Thus, we suggest RAN2 further confirms this issue. |
| Nokia | No | Yes | If autonomousTX is not configured and CGRT is configured, then any MAC PDU that is deprioritized or cancelled may be deemed as “HARQ process pending” as the TB is generated and stored in a HARQ buffer, while the gNB may not be aware of it. Hence, in specification we could simply clarify that grant deprioritization can be considered as “HARQ process pending” that allows autonomous re-transmission based on CGRT. |
| Ericsson | No | No | See comment on 2a) Specification changes/enhancements would be required for this. There is no way to handle de-prioritized TB “the same way” as TB of LBT/transmission failure. |
| Qualcomm | No | No | If autonomoustTx is configured, then autonomousTx is specifically designated to handle that, no need to involve CGRT with deprioritization. If autonomousTx is not configured, we still think that we should not mix LBT failures with deprioritization. LBT failures must wait the duration of CG re-tx timer before attempting a “re-transmission”. Auto-tx transmits in the next available CG with the same HARQ ID occasion. There is no point to make a de-prioritized CG wait for the duration of a re-tx timer. Furthermore, a low priority CG can exhaust re-tx timer attempts (bounded by the CG timer limit) without actually ever attempting a transmission (this erroneously counts as an ACK) Finally, it is expected that the length of re-tx timer would be different from the CG periodicity in case of deprioritization. If auto-tx is not configured, we prefer to not apply Intra-UE prioritization. |

| vivo | Depends on whether the CG for the deprioritized TB is new transmission or retransmission | No | We share the views ith HW on following: If autonomousTx is configured, then whether the deprioritized TB can be treated the same way as a TB dropped due to LBT failure depends on whether the CG for the deprioritized TB is new transmission or retransmission. - If the CG for the deprioritized TB is new transmission, then IoT autonomous transmission should be performed - If the CG for the deprioritized TB is retransmission, then NR-U autonomous retransmission should be performed, same handling as LBT failure happens. But for the case of autonomousTx not configured, this is similar case as in Rel-16: - In R16, autonomousTx is an independent UE capability, which may be not supported by one UE supporting LCH-based prioritization. - We want to keep the similar flexible in R17, i.e. one UE supporting CGRT based retransmission is not mandated to support autonomous transmission of deprioritized TB. NOTE that if NW would like to recover the CG for which the deprioritized TB is a new transmission, then simplest way is just to configure autonomousTx together with cg-RetransmissionTimer. |
| --- | --- | --- | --- |
| Apple | No | No | We think same small changes will be needed to ensure autonomous (re)transmission, which is beneficial in both scenarios, can happen. Similar view as Lenovo and Nokia. |
| Samsung | TBD | Yes | If autonomousTx and CGRT are jointly configured, RAN2 should decide how to prioritize one of the autonomous mechanisms. |
| Intel | As in our reply to Q2-a, we don’t think it is necessary to configure autonomousTx. | No | There is no need to treat deprioritized TB the same way as LBT failure. Deprioritized TB can be automatically retransmitted upon expiry of cg-RetransmissionTimer, according to TS 38.321 clause 5.4.1, as highlighted below: 2> else if the cg-RetransmissionTimer for the corresponding HARQ process is configured and not running, then for the corresponding HARQ process: 3> if the configuredGrantTimer is not running, and the HARQ process is not pending (i.e. new transmission): 4> consider the NDI bit to have been toggled; 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. 3> else if the previous uplink grant delivered to the HARQ entity for the same HARQ process was a configured uplink grant (i.e. retransmission on configured grant): 4> deliver the configured uplink grant and the associated HARQ information to the HARQ entity. |
| Xiaomi | No | No | When the autonmousTx is configured, the Rel-16 IIOT UE autonomous retransmission does not allow the deprioritized MAC PDU retransmission across CG configurations. When the autnomousTx is not configured, the MAC specification cannot retransmit the deprioritized MAC PDU. |
| TCL | Depends on whether the CG for the deprioritized TB is new transmission or retransmission | Yes | Agree with HW. |
| Sequans | No | No | When autonomousTx is configured, UE transmits the deprioritized TB according to this feature, which is different from NR-U. When autonomousTx is not configured, the deprioritized TB would not be retransmitted. |

Impact on CGRT:

References [5, 13, 14] think CGRT and autonomousTx can be configured together, but mention if deprioritization happens after the first symbol of PUSCH transmission on the CG, there may be conflicting behavior specified whereby, autonomousTx instructs the retransmission again on the next CG occasion while CGRT mandates the retransmission after CGRT expiry. This is caused by:

    - CGRT starts at the first symbol of a PUSCH transmission on the corresponding CG.

    - When the UE is configured with autonomousTx, it was agreed in RAN2#112e to stop the CG timer when a TB on the corresponding CG is deprioritized, to allow for a deprioritized PDU for a configured grant to be autonomously transmission immediately.

To enable immediate retransmission when CGRT is configured, [3, 5, 6, 13, 14] propose to stop CGRT when the CG resource associated with the CGRT is deprioritized. Reference [14] also supports the transmission of a deprioritized PDU on the next available CG occasion, including the case where CGRT was never started, though that is already up to UE implementation in such case to select the HARQ process ID. Reference [2] argues that it is not favorable to stop the CGRT in such case, as the gNB can decode the transmitted TB successfully even if the transmission is not completely finished or the gNB can issue a dynamic grant for retransmission after decoding the CG-UCI.

Question 3: Do you support stopping cg-RetransmissionTimer when the CG resource associated with the CGRT is deprioritized due to LCH-based prioritization?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | No | Autonomous retransmission can be performed when the cg-RetransmissionTimer expires. If the network configures a suitable cg-RetransmissionTimer, the delay is not that critical. |
| MediaTek | See comment | To clarify, in our paper [14] we suggested that the CGRT should not be running. We think that this can be achieved by not starting CGRT when the UL grant is deprioritized, rather than starting and then stopping it. |
| CATT | Yes, as an optimization | We think it is low probability that network can decode the packet or UCI successfully. Hence, most likely the UE will perform autonomous retransmission after CGRT expires. So stopping the CGRT allows treating the pending PDU faster, in the same way as for an LBT failure. But the HARQ process needs to be switched to pending as well. |
| Sony | Yes | It is better to stop the timer to allow for a deprioritized PDU for a configured grant to be autonomously transmitted immediately. This makes aligned with the behavior of autonomousTx. |
| Lenovo | Yes | Stopping CGRT allows for faster autonomous retransmissions. Also it seems like an unintended UE behaviour if cg-RetransmissionTimer is running while the configuredGrantTimer is not running. |
| ZTE | Yes | Agree with CATT |
| LG | Yes | Our baseline assumption is that there should be no case cg-RetransmissionTimer is running while configuredGrantTimer is not running. |
| Sharp | Yes | The benefit why stop cg-RetransmissionTimer for a deprioritized CG we did in Rel-16 is still valid. |
| III | Yes | Agree with CATT |
| APT | Y | This removes the conflict due to both autonomousTx and CGRT being configured at a UE. The deprioritized TB can also be retransmitted quicker because the CGRT is stopped. |
| OPPO | Yes | It is better to stop the CGRT to allow the PDU is treated faster. In addition, as we mentioned above, if autonomousTX is not configured, the HARQ process status needs to be set as pending as well. |
| Nokia | Yes | This allows more rapid autonomous retransmission, which is desirable for URLLC data. The case mentioned by [2] is a specific case where the gNB can decode CG-UCI even when the PUSCH is not completely transmitted, we cannot assume this is always the case. |
| Ericsson | Yes |  |
| Qualcomm | Yes but clarification is needed.. | It is not favorable to run re-tx timer for a deprioritized grant. However, it is not clear what circumstances may lead to this scenario as interrupting an ongoing PUSCH Tx (due to high priority traffic arriving) is not applicable. On the other hand, it is not clear how a HARQ process with a running CG re-tx timer can be deprioritized, as a running timer means the PDU will not be transmitted and thus, not subject to deprioritization. |

| vivo | Maybe | CGRT is introudced to control autonomous retransmision when CG timer is running. No autonomous retransmision therefore will be triggered when CGRT expires but CG timer is not running. We do not see benefit to keep a CGRT running while a CG Timer is stopped. However, keep the CGRT running leads to not big drawback, only the deprirotized TB needs to wait CGRT to expire to perform autonoums transmisison. As it is a deprirotized(i.e. delay-tolerant) TB, not a big issue. |
| --- | --- | --- |
| Apple | Yes |  |
| Samsung | No | It can be retransmitted after CGRT expiry. |
| Intel | No | Deprioritized TB can be automatically retransmitted upon expiry of cg-RetransmissionTimer. |
| Xiaomi | Yes | We should avoid redundant retransmission. |
| TCL | Yes | The configuration of cg-RetransmissionTimer means that the autonomous retransmission after CGRT expired can meet the delay requirement. Although stop the timer and perform retransmission immediately allows handle the pending PDU faster which can be an optimization. And in addition this avoided the conflicts of UE behaviors. |
| Sequans | Yes |  |

How to perform autonomous re-tx and impact on HARQ process status

To support the IIoT autonomous re-transmission of a deprioritized TB without autonomousTx configured, [2, 3, 10, 11] propose that the HARQ status to should be switched to pending (similar handling to when a CG transmission fails LBT). Reference [13] proposes considering the HARQ process as not pending upon deprioritization, but with autonomousTx configured. The following proposals were made for the HARQ process status:

- Option 1 - To enable transmitting a deprioritized PDU at the very next CG occasion: upon deprioritization of an UL grant, the corresponding HARQ process switches to “pending” [2, 3, 10, 11]. This can be used to support configuring CGRT without autonomousTx

- Option 2 – When both CGRT and autonomousTx are configured, perform IIoT autonomous framework if the PDU was deprioritized and no indication of LBT failure was received, and perform NRU autonomous retransmission framework for the MAC PDU if LBT fails [4, 10, 16]. HARQ process status remains as is, unless it’s changed per existing R16 specification.

- Option 3 – No further enhancement is needed. Current specification can handle configuration of LCH-based prioritization and CGRT as is to perform the autonomous retransmission of a deprioritized TB.

- Option 4 – No further enhancement is needed. Do not support retransmission of de-prioritized TB.

- Option 5 - When both CGRT and autonomousTx are configured, perform NRU autonomous framework if the PDU was deprioritized and no indication of LBT failure was received, and perform NRU autonomous retransmission framework for the MAC PDU if LBT fails. HARQ process status remains as is, unless it’s changed per existing R16 specification.

Question 4: With LCH-based prioritization and CGRT configured, do you support any of the above options for the HARQ process status?

| Company | autonomousTx configured | autonomousTx not configured | Additional comments |
| --- | --- | --- | --- |
|  | Preferred option(s) | Preferred option(s) |  |
| HW | Option 2 but | Option 1 but | We think option 1 and option 2 only apply to new transmission and option 1 only applies to the case that new transmission is deprioritized before the transmission. There are some other cases need to be considered as well. - If autonomousTx is not configured and the CG is for retransmission and de-prioritized before its transmission, the HARQ process keeps its current pending/not-pending status - If autonomousTx is not configured and the CG is de-prioritized after the transmission starts (either new transmission or retransmission), the HARQ process is still considered as not pending. - If autonomousTx is configured, if the CG is for retransmission and deprioritized, no matter before transmission or after transmission, perform NRU autonomous retransmission framework, HARQ process status remains as is. |
| MediaTek | Option 2/3 | Option 4 | When autonomousTx is configured, any deprioritized or cancelled CG should use the autonomousTx framework. If the CG was prioritized but LBT failed we use the NR-U framework. When autonomousTx is not configured, the network has has explicitly informed the UE not to retransmit a deprioritized UL grant. The Rel-16 spec can handle it without any changes to the HARQ process status. |
| CATT | No support | Option 1 | The HARQ process status (pending/not pending) only exists (and plays a role) when cg-RetransmissionTimer is configured. In such case, switching the HARQ process to pending (as well as stopping cg-RetransmissionTimer as discussed in Q3) upon CG deprioritization allows UE to perform autonomous retransmission on the subsequent CG resource, in the same manner as for an LBT failure. As correctly mentioned by the rapporteur, this only involves NR-U mechanisms and does not require configuring autonomousTx. It is clean as it does not require to discuss and decide, as is the case with Option 2 and as explained in [13], which of NR-U retransmission or IIOT autonomous transmission will take place when a CG experiences consecutively an LBT failure and (on the subsequent CGO where takes place the autonomous retransmission) a deprioritization. And vice versa. |
| Sony | Not supported | Option 1 but | We agree case 1 and 2 described by Huawei. Case 3 is not needed as we preferred already that cg-RetransmissionTimer and autonomousTx are not configured together in R-17. |
| Lenovo | Option 2 but | Option 1 | When autonomousTx is configured, any deprioritized /cancelled CG should use the IIOT autonomous transmission framework. For the case of LBT failure NR-U autonomous retransmission functionality should be applied. However we think it would be cleaner if HARQ process status is switched to “not pending” for cases when CG is deprioritized, i.e. for the case that autonomous retransmission is deprioritized. Since MAC spec is already quite complex when it comes to NR-U autonomous ReTx and IIOT autonomousTx it would be a cleaner solution in our understanding. When autonomousTx is not configured, switching the HARQ process status to “pending” upon CG deprioritization enables UE to perform an autonomous retransmission of the deprioritized TB on the subsequent CG resource, similar to LBT failure. |
| ZTE | Yes, go for improved option 2 (option 5) | Option 4 | We think no matter normal configured grant transmission or autonomous (re)transmission shall use the framework of NRU if CGRT is configured, thus we think option 5 is a better solution since the NRU framework can provide more prompt retransmission. |
| LG | Option 2 | Option 4 | If LCH-basedPrioritization, CGRT, and autonomousTx are configured, LBT failed transmission is to be covered by NRU autonomous retransmission whereas deprioritized TB is to be retransmitted by IIOT autonomous transmission. For deprioritized TB, we assume that there is no LBT failure indication received because LBT is not performed. If LCH-basedPrioritization and CGRT are configured but autonomousTx is not, we don’t think deprioritized TB is to be retransmitted by any mechanism, which should be network’s decision. It is not clear to us, what are the difference between option 2 and 3 as no reference is shown for option 3 at all. |
| Sharp | Not support | Option 1 | See our comments in Q2-b. |
| III | Not supported | Option 1 | As we preferred the cg-RetransmissionTimer and autonomousTx are not configured together in R-17. For deprioritized uplink grant, switching the corresponding HARQ process to “pending” is necessary. |
| APT | Option 2 | Option 1 | If autonomousTx is configured, a deproiritized PDU could be handled by autonomousTx, i.e., does not consider the HARQ process of a deprioritized PDU as pending. If autonomousTx is not configured, a deproiritized PDU could be handled by CGRT, i.e., consider the HARQ process of a deprioritized PDU as pending. |
| OPPO | Option 2 | Option 1 | In our paper, it is clearly indicated that the proposals are related to the case that original CGT status is not running, which means they focus on the case that the MAC PDU is not transmitted at least once (i.e. CGT is not running and HARQ process is as pending/not pending). For the retransmission case, when CG is considered as deprioritized after LBT succeeds, - If autonomousTX is not configured and the original CGT status is already running, HARQ process status keeps as not pending, to assure CGT is not re-started when the next CG is available and CG timer is still running. - If autonomousTX is not configured and the original CGT status is not running, HARQ process status needs to set as pending, for the reason we mentioned in Question 2-b. - If autonomousTX is configured and the original CGT status is already running, we are open to generate a new MAC PDU or follow NRU auto retransmission mechanism. Accordingly, it will introduce another issue whether we need to stop CGT for retransmission when the CG is deprioritized. On the other hand, for the case that autonomousTX is not configured, we can also accept Option 4, if majorities consider no need to retransmit the deprioritized TB. Note that autonomousTX is also an optional IE for R16 IIoT. It means that there is the case that the deprioritized MAC PDU exists but the gNB does not want UE autonomous transmission. |
| Nokia | Option 2 | Option 1 | We prefer not to configure AutonomusTX and CGRT together. But if that is the case, then perhaps Option 2 ensures each of the configured functionality is best utilized. When autonomusTx is not configured, then it should be more straightforward if we allow autonomous re-tx of the deprioritized MAC PDU by treating it as a pending HARQ process as well. |
| Ericsson | Option 2 | Option 4 | Make use of the existing specification branches, i.e. NR-IIoT handling of de-prioritized data with autonomousTx, and NR-U branch for handling LBT/transmission failures. |
| Qualcomm | Option 2 | Option 4 | If auto-Tx is not configured we assume that Intra UE prioritization is not applicable |
| vivo | Option3 | Option4 | When both CGRT and autonomousTx are configured, perform IIoT autonomous framework if the PDU was deprioritized for new transmission and no matter indication of LBT failure was received or not. Otherwise, perform NRU autonomous retransmission framework for the MAC PDU if LBT fails. HARQ process status remains as is, unless it’s changed per existing R16 specification. We think the behaviour can be carried out without Rel-16 specification impact. When autonomousTx is not configured, we share the same view with MediaTek. |
| Apple | Option 2 | Option 1 | We prefer not to support simultaneous config of CGRT and autonomousTx, but if that was the case then Option 2 seems acceptable. When autonomousTx is not configured then Option 1 could be an assumption for the further investigation going forward. |
| Samsung | Option 3 | Option 1 | Regarding joint configuration, the conditions of autonomousTx and autonomous retransmission are mutually exclusive. We don’t need to do anything. But it’s not an efficient way to configure two similar mechanisms without harmonization. |
| Intel | Not supported | Option 3 | As in our reply in Q2-b, Deprioritized TB can be automatically retransmitted upon expiry of cg-RetransmissionTimer. Specification change is not needed. |
| Xiaomi | Option 2 | Option 4 | If autonmousTx is configured, we should follow the Rel-16 IIOT to switch the HARQ status to pending. If autonmousTx is not configured, then the UE does not perform the retransmission for the deprioritized MAC PDU as the Rel-16 IIOT. |
| TCL | Option 2 | Option 1 | Agree with HW. |
| Sequans | Option 2 | Option 4 | We prefer to keep features separated. |

### With cg-RetransmissionTimer not configured

An open issue is whether CG retransmissions due to LBT-failure should also be handled when cg-RetransmissionTimer is not configured, e.g. using the autonomousTx framework. There are two expressed opinions:

Option 1 - no enhancements needed: if an operator would like to handle LBT-failures and UE autonomous retransmission, it can simply configure the CGRT [4, 5, 15, 16].

Option 2 - consider a grant dropped due LBT failure as a “deprioritized grant” to enable the autonomous retransmission of the PUSCH that failed LBT [2, 3, 13, 10, 11, 19, 20].

Question 5: Which option do you prefer for handling autonomous retransmission due to LBT failure when cg-RetransmissionTimer is not configured?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| HW | Option 2 |  |
| MediaTek | Option 1 | No need to make any enhancements. If the network has not configured CGRT, it is informing the UE that it should not attempt to retransmit data if LBT fails. The network choice should be respected. |
| CATT | Option 2 | We have not identified a technical issue with this simple solution. So we think it should be supported, even if other solutions are also adopted with the NR-U road, i.e. when cg-RetransmissionTimer is configured (Section 2.2.1). We think providing the industry with a set of solutions UE vendors can chose from depending on their R16 implementations and roadmap increases development flexibility, allows reducing costs, and is future-proof. |
| Sony | Option 2 | It is better to be consistent for both mechanisms (NR-U and URLLC) that due to LBT failure the PDU is deprioritized and then it will have immediate autonomous transmission. |
| Lenovo | Option 2 | Seems like a simple solution. However no strong opinion here. |
| ZTE | FFS | We think it is a little earlier to discuss how to deal with the LBT failure when CGRT is not configured even we still have no idea whether CGRT can be configured with autonomousTX or not. We tend to agree with MTK, if CGRT is not configured by NW which means NW have a confidence in which no LBT failure can be occurred. |
| LG | Option 1 | It should be network’s decision to configured CGRT to cope with failed transmission due to LBT failure. |
| Sharp | Option 2 | If autonomousTx is configured, autonomous transmission should be allowed. |
| III | Option 2 |  |
| APT | Option 2 | If CGRT is not configured, the PDU that cannot be transmitted due to LBT failure should be considered as a deprioritized PDU. |
| OPPO | Option 2 | As we understood, UCE means sporadic LBT failures existing. If we want to cover LBT failure case, Option 2 seems a simple way. |
| Nokia | Option 1 | If gNB decides not to configure cg-Retransmission timer, essentially it means the network is very confident that LBT failure does not happen in UCE. In other words, LBT failure can be seen as an extremely rare corner case, and it could be resolved via certain implementation. |
| Ericsson | Option 1 | Handling of LBT-failures only when CGRT is configured. No need for a further branch of solutions beyond the CGRT for LBT-handling needed. |
| Qualcomm | Option 1 | Option 2 means that auto-tx is by default running for every Tx until a DFI-Ack is received, possibly unnecessarily transmitting auto-Tx retransmissons until a DFI-Ack is received. Nominally, auto-tx would treat deprioritization and gNB failure to decode/LBT failure exactly the same, any improvements would need spec change for auto-tx feature. No good reasoning on treating LBT failures and deprioritization the same. |
| vivo | Option 1 | Agree with MediaTek |
| Apple | FFS |  |
| Samsung | Option 2 |  |
| Intel | Option 1 | There is no need to define two mechanisms to handle the same scenario. Configuration of cg-RetransmissionTimer is sufficient to handle the UCE scenario. |
| Xiaomi | Option 2 |  |
| TCL | Option 2 | Option 2 is a simpler and it aligns the UE behavior for both NR-U framework and URLLC framework. |
| Sequans | Option 1 | In our view if LBT failures need to be handled, NW should configure CGRT. |

## CG selection for autonomous retransmissions

When CGRT is configured, the UE still selects the HARQ process on its own and HARQ process sharing between multiple CGs are allowed as in Rel-16 NR-U. Given autonomous (re)transmission mechanism after CGRT expiry can be performed across the different Configured grant configurations sharing the same HARQ process, [14, 17] proposes to avoid using other CG configurations not suitable for the service – compared to the CG initially used. However, [4] questions whether HARQ sharing should still be allowed in R17 when LCH-based prioritization is configured, as it is more suited for same priority data. The following options are possible:

Option 1 - Restrict that the configured grant used for autonomous (re)transmission to be from the same CG configuration used initially, e.g. when LCG-based prioritization is configured [4]

Option 2 - LCH restriction is considered when selecting a configured grant for autonomous (re)transmission from a different CG configuration (consider all restrictions, including allowedCG-List) [14, 17]

Option 3 – No enhancement needed, e.g. rely on the network to configure HARQ sharing for CG configurations that can meet the same type of services.

Question 6: Which of the options above do you support to ensure the CG selected for autonomous retransmission is selected properly to meet the service?

| Company | Reply (y/n) | Additional comments if yes, which enhancement is preferred? |
| --- | --- | --- |
| HW | Option 3, | We don’t think any enhancement is needed. According to the mapping between LCH and CG lists as well as the mapping between CG and HARQ ID, we can totally rely on NW implementation to ensure the CG selected for autonomous retransmission is selected properly to meet the service. |
| MediaTek (proponent) | Option 2 | The LCH restrictions are critical for the IIoT operation and should be met for retransmissions as well. Different CG configurations are used for different QoS requirements. Therefore, an appropriate CG configuration must always be picked for retransmission. For example, allowedCG-List maps data to an appropriate CG configuration. LCH restrictions such as these must be satisified for retransmissions. Option 3 would limit the network flexibility and can reduce the number of HARQ processes per CG configuration. |
| CATT | Option 3 | In RAN2#112-e meeting, it was agreed that 5 As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U. We think if CGRT is configured, the autonomous retransmission among different CG configurations should be allowed like NR-U. This is one benefit of NR-U. |
| Sony | Option 2 | Because we support different services with different requirements (i.e. reliability, latency), it would make sense to comply LCH restriction if configured. |
| Lenovo | Option 3 | We don’t see a need for any enhancement. |
| ZTE ((proponents) | Option 2 | This option is good for guarantee the TSN service won’t interrupted by the autonomous retransmission from other TSN services. |
| LG | Option 3 | It should be networks responsibility. |
| Sharp | Option 3 | Agree with CATT. |
| III | Option 3 | Agree with CATT. |
| APT | Option 2 | Follow Rel-16 NRU behavior when CGRT is configured. Hence, we should allow a deprioritized TB to be retransmitted on another CG configuration if it satisfies the LCH restriction of the TB. As such, the service requirement could be ensured. |
| OPPO | Option 3 | We think we can rely on gNB configuration to assure it. |
| Nokia | Option 3 | For Option 1, we are losing the benefits of HARQ PID sharing. For Option 2, we think LCH mapping restriction should be considered for initial transmission only. Basically network implementation should be sufficient to handle the issue. |
| Ericsson | Option 1 | We assume different CG configurations are configured to handle different traffic (different periodicity, size, data), therefore we think it makes sense to apply LCH-restrictions for transmissions and retransmissions to certain CG alike. This would mean that autonomous retransmissions are also only send on the CG where the initial transmission was done. This option does not have any specification impact. On the other hand, without these restrictions, the same HARQ processes for transmission and autonomous transmission may overlap between the CGs and lead to conflicts. It would be okay if RAN2 agrees no spec enhancements are needed when network configures HARQ sharing. |
| Qualcomm | Option 3 | Agree with Nokia |
| vivo | Option 3 | To keep UE simple and leave it to network implementation. |
| Apple | Option 2 | As per the NR-U WI agreements captured in RP-201141, in Rel-16 LCH restrictions are applicable to the first transmission only. LCH restrictions are an essential to ensure stringent QoS requirements for IIoT/URLLC. In order to utilize logical channel mapping restrictions for all transmissions, Rel-17 should be allowed to use LCH restrictions also for retransmissions. To lift restrictions on retransmissions is not preferable in our view. |
| Samsung | Option 3 | Agree with Huawei |
| Intel | Option 3 | Network configuration is sufficient. |
| Xiaomi | Option 3 | We prefer to rely on the gNB implementation to handle this. |
| TCL | Option 3 | Agree with HW, it can be left to NW implementation. |
| Sequans | Option 3 | Agree with Nokia |

## Prioritization between CG initial transmissions and retransmissions

In Rel-16 NR-U, the UE selects the HARQ process ID for CG and the UE prioritizes retransmissions of PDUs (e.g. due to failed LBT) over new transmissions, while for IIoT on licensed band the HARQ process is strictly associated to the CG occasion time. The NR-U behavior however can result in added latency for newly arrived URLLC data. For CG operation in a controlled unlicensed environment (IIoT and NR-U), the UE can benefit from prioritization between Initial transmission (which may contain higher priority data/control) and retransmissions (due to UL LBT failure, CGRT expiring, or intra-UE de-prioritization), as proposed in [2, 3, 5, 6, 13, 15]. This can require changes, as explained in [4, 5], given currently UE prioritizes retransmission within each CG with CGRT and LCH-based prioritization overrides the retransmission choice.

Question 7: With cg-RetransmissionTimer and LCH-based prioritization configured, do you support prioritization between initial transmissions and retransmissions on a CG based on the LCH priority multiplexed -or to be multiplexed-?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | Yes | Considering URLLC service is supported, it seems not reasonable to always prioritize retransmission than new transmission. If a MAC PDU for retransmission contains data from a LCH with low priority, while there are some data with higher LCH priorities awaiting for transmission, it is better to allow a subsequent CG occasion to perform new transmission |
| MediaTek | Y | For IIoT use cases, it is better to prioritize the initial transmission and retransmission based on the Rel-16 IIoT principles (priorities of LCH that are multiplexed or that can be multiplexed in the TB). |
| CATT | Yes | We see no issue in supporting prioritization between initial transmission and retransmission when CGRT and LCH-based prioritization are configured. The legacy NR-U behavior was not intended for URLLC traffic and would benefit here from this small upgrade. |
| Sony | Yes | In general, it is important that a low priority re-transmission should not prevent another initial transmission with high priority. |
| Lenovo | Yes | UE should perform the UL grant prioritization functionality defined for Rel-16 I-IOT also for retransmissions, e.g. retransmission triggered by LBT failure. Otherwise transmission of high priority data might get delayed. |
| ZTE | No for now | The prioritization of selection of HARQ process ID from HARQ process poor for one CG occasion is totally different with our current LCH based prioritization rule from which only determine the prioritized UL grant from conflict UL grants. This is a new function we shall be careful with that. |
| LG | No | The CGT should be set to a value by considering the delay requirement. Thus, there should be no problem in transmission of data after CGT expiry. CGRT only gives additional chance of retransmission within CGT. Furthermore, even today there is a case that all HARQ processes are in use but we don’t stop any HARQ when receiving a higher priority data but wait until CGT expires to transmit such data. |
| Sharp | Yes | To fufil the QoS requirement of data with higher priority is more important than data with lower priority. |
| III | Yes | If a retransmission data has lower priority then initial data, the retransmission data should not be transmitted. |
| APT | Y | In UCE, some of the incoming data to be multiplexed may be more important (e.g., URLLC data) than the pending data. Hence, the UE should conditionally prioritize new transmission over retransmission when selecting a HARQ ID. |
| OPPO | Y | It is good way to support URLLC traffic in the case that the new data is related to a higher LCH priority. |
| Nokia | Yes | This is needed to make sure higher priority traffic can be transmitted quickly to fulfil QoS requirement. |
| Ericsson | No | For the same CG, we don’t believe enhancements are needed. This would go beyond the scope of Rel-16 intra-UE multiplexing where LCH-based prioritizations are considered only between overlapping CGs which typically have different BLER targets. For different priority traffic restricted to different CGs, such enhancement would be redundant/has no effect. There are issues for network if it is allowed. - Suppose the retransmission is due to LBT failure, i.e., network is not aware of the initial transmission. This retransmission data can be de-prioritized numerous times so that the CG timer expires and then the retransmission data is flushed and lost. Note that the MAC PDU may contain only MAC CEs and it has the lowest LCH-priority. - Network configure the CGTimer and CGReTxtimer values assuming a certain number of re-transmissions to fulfil the PER target for all traffic restricted in that CG configuration. If this operation is impacted by the arrival of the new data so that the actual retransmission times is smaller, then network cannot configure correct timer values and satisfy the QoS requirement. |
| Qualcomm | Yes | Avoid reversal of logical channel in the case when a low priority re-tx is prioritized over a high priority new transmission |
| vivo | No | On one hand, we agree there are some benefits to support prioritization between initial transmission and retransmission. On the other hand, we think it is simple for UE to follow the R16 NR-U mechanism and a smart network can configure LCHs with similar priorities to CGs sharing a subset of HARQ processes. Therefore, we prefer to leave it to UE implementation, i.e. when a HARQ associated with retransmission and a HARQ associated with initial transmission compete for one CG, it is UE to choose which one should be transmitted with the CG. |
| Apple | Yes | For UCE, usage of LCH-based prioritization for overlapping grants should follow similar rules as in IIoT, this helps ensure support for URLLC services requiring low latency. In other words, retransmissions should not generally prioritize over new transmissions. |
| Samsung | No | Logical channels sharing a CG will have similar QoS characteristic. Otherwise, NW should avoid the scenario that high-priority new transmission and low-priority retransmission collide. |
| Intel | Yes | It is reasonable to allow initial transmission of high LCH priority to be performed on available CG occasion. |
| Xiaomi | Yes | To support the URLLC service via NR-U, the UE should be allowed to prioritized the MAC PDU retransmission or initial transmission based on the LCH priority. |
| TCL | Yes | The legacy NR-U mechanism to prioritize retransmission over new transmission may not meet the delay requirement of the URLLC service. |
| Sequans | Yes | This would make sense, as this is expected behavior when LCH based prioritization is configured. |

## HARQ Process ID selection when CGRT is not configured

It was agreed in R2#112e that Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection when CGRT is not configured, i.e. nothing changes in that case and the UE uses the formula to determine the HARQ PID according to timing of the CG occasion, as in R16. RAN1 is however discussing the configuration options for the CG-UCI, including configuring it independently of the CGRT. References [3, 5] argue that CG-UCI can be configured independently of CGRT, and when it is configured the UE can select the HARQ PID and send it on the UCI. This issue boils down to whether configuring CG-UCI without CGRT can be common. The following options are possible:

Option 1 - When CG-UCI is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. CG-UCI can be optionally configured without CGRT configured [3, 5]. LS to RAN1 can be sent if needed.

Option 2 – Wait for RAN1 to conclude their discussions [21].

Option 3 - no enhancements needed: if CGRT is not configured, the UE uses the existing formula to determine the HARQ PID according to timing of the CG occasion, as in R16. This is in line with the agreements from last meeting.

Question 8: Which option do you prefer for HARQ process and RV selection when CGRT is not configured?

| Company | Preferred option | Additional comments |
| --- | --- | --- |
| HW | Option 3 |  |
| MediaTek | Option 3 |  |
| CATT | Option 1 | It is obvious the CG-UCI only needs be configured to inform the gNB which HPID and RV UE selected for a given CGO. On the contrary, when the HPID is determined based on CGO timing, the CG-UCI is totally useless. |
| Sony | Option 1 or 2 | It is reasonable to wait RAN1 outcome if RAN2 cannot make a decision. |
| Lenovo | Option 3 |  |
| ZTE | Option 3 | From RAN2 perspective , we would like to insist on our decision made in the previous meeting. |
| LG | Option 3 |  |
| Sharp | Option 3 |  |
| III | Option 3 |  |
| APT | Option 1 | If CG-UCI is not configured, there is no way a UE could inform the network the HARQ ID that is selects. Hence, the UE should select a HARQ ID itself only when CG-UCI is configured. |
| OPPO | Option 3 |  |
| Nokia | Option 2 | We tend to wait and see what RAN1 concludes, to avoid potential misalignment between RAN1 and RAN2. |
| Ericsson | Option 2 |  |
| Qualcomm | Option 1 or 2 | Higher flexibility in the system. Allows for rapid retransmission for high priority traffic with no need to wait for the full CG period. But we are willing to wait for RAN1 to conclude their discussions on the issue. |
| vivo | Option 3 |  |
| Apple | Option 2 | It is better to take RAN1’s conclusion into account before finalizing this. |
| Samsung | 1 or 2 | Irrespective of CGRT configuration, configuration of CG-UCI is beneficial to reduce the latency. Since RAN1 is currently discussing this issue, we are ok to wait for RAN1 to conclude. |
| Intel | Option 2 | We can wait for RAN1 conclusion regarding how CG-UCI is configured. |
| Xiaomi | Option 3 |  |
| TCL | Option 3 |  |
| Sequans | Option 2 |  |

# Other issues

Issues not previously discussed or not widely discussed.

## Intra-UE Prioritization considering LBT Failure

References [6, 13] mention that intra-UE prioritization in some case can select a grant outside of an ongoing COT (or a grant for which LBT failure is more likely), even though the deprioritized grant can be selected with a higher probability of succeeding LBT. The following enhancements are proposed:

- Enh 1: [6] proposes that MAC takes the likelihood of succeeding LBT into account in addition to LCH priority, when determiening which grants to (de)-prioritize.

- Enh 2: [13] proposes to transmit a PDU on a deprioritized grant if the transmission on the overlapping prioritized grant failed LBT, to reduce potential waste of resources.

Question 9: Do you support considering LBT failure when determining a grant priority for intra-UE prioritization?

| Company | Reply (y/n) | Additional comments if yes, which enhancement is preferred? |
| --- | --- | --- |
| HW | No | As we are talking about UCE, the LBT failure can almost be ignored and happens quite rare, so it is not necessary to consider the LBT failure for intra-UE prioritization. In addition, the current intra-UE prioritization mechanism is already quite complicated, we don’t support to introduce any additional condition on top of the current scheme unless significant problem is foreseen. |
| MediaTek | N | We do not see any need for these enhancements. |
| CATT | No | The LBT failure is assumed to be low probability in UCE. If LBT is taken into account, MAC also needs the information from PHY, e.g. whether the grants are in the same COT. This brings complexities but not great benefits. Regarding enh2, we think this may already be executed by current spec, because if a grant failed LBT the associated PUSCH is not transmitted hence does not collide anymore with the deprioritized grant which is therefore prioritized by default (no other overlapping PUSCH). |
| Sony | No | We don’t think it is necessary to add an extra burden to the prioritization procedure. Anyway, if LBT succeeds/fails, there is a clear procedure how to handle it based on previous questions/solutions. |
| Lenovo | Yes | In our understanding LBT is only performed for the prioritized UL grant in case of colliding UL grants. It may happen though that UE cannot perform transmission of the prioritized grant due to LBT failures. Therefore we think that it’s beneficial when UE also performs LBT procedure for a deprioritized grant, i.e. UE has generated TB for the deprioritized grant, in order to increase the likelihood that PUSCH transmission can be performed. It will be beneficial from resource point of view when UE is allowed to perform the transmission on the deprioritized UL grant for cases when LBT fails for prioritized grant and LBT is successful for deprioritized grant. |
| ZTE | Yes | We are open for this issue. |
| LG | No |  |
| Sharp | No |  |
| III | No | We do not think it is necessary, the LBT failure is happens rarely in UCE. |
| APT | N | LBT is not likely to happen in UCE. Hence, we do not have to consider the likelihood of LBT failure when determining a grant priority. |
| OPPO |  | We are open for this issue. |
| Nokia | Yes | There could be a risk where the prioritized grant cannot be transmitted eventually due to LBT failure, which would diminish the benefits of LCH-based prioritization. Therefore, it could be beneficial if LBT failure could be taken into account in order to select the prioritized grant more appropriately. |
| Ericsson | No |  |
| Qualcomm | No | Enh 1: Can be ensured by implementation. No spec impact Enh 2: UE can choose any CG occasion with CG-UCI. Use case seems overly specific |
| vivo | No | Same view as Huawei. In UCE, LBT failure occurs rare. No need to complex the intra-UE prioritization mechanism for optimization, given the combination of LCH based prioritization and CGRT has been complicated enough. |
| Apple | No | Enh 1: MAC needs to be fully deterministic. It is better not to mandate these things in the spec. UE implementation can take care of it. Enh 2: We agree with the problem and think Lenovo’s concerns are valuable. Though the complexity-add is not low, it means the UE has to provisionally process multiple hypothetical routes in parallel. This seems quite some effort for a failure expected to happen rather occasionally. |
| Samsung | No, but | Enh 2 can be realized by de-prioritizing uplink grant for which LBT failure is indicated. Then, overlapped other grant can be re-evaluated and may become a prioritized uplink grant to transmit. |
| Intel | No | Agree with Huawei. |
| Xiaomi | No |  |
| TCL | No | The LBT failure is of low probability in UCE and take it into account shall bring complexities. |

## LCH-dependent HARQ Process Pool

A concern expressed in [6] is that all CG HARQ processes can at times be occupied with (re)-transmission, especially when the channel occupancy is higher, and that could potentially block transmission of newly arrived URLLC/high priority data. Reference [6] therefore proposes to configure a subset of HARQ processes as “restricted processes” for transmission of data from higher priority LCHs, whereby the UE can use a grant from a restricted process if the PDU contains at least one bit from higher priority LCHs associated with the restricted HARQ process. R16 already allows for a CG type-1 based LCP LCH selection restriction, whereby the UE only fills the CG with data from LCHs configured with configuredGrantType1Allowed. This however cannot be configured per CG or per HARQ process, and thus does not forbid the UE form occupying such CGs for transmission of lower priority data, and thus can block the transmission of higher priority data (due to retransmissions, higher channel occupancy etc).

Question 10: Do you support configuring a subset of HARQ processes as “restricted processes”, whereby the UE can multiplex data on a grant from associated restricted process if the PDU contains at least one bit from higher priority LCHs associated with the restricted HARQ process?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | No | According to the mapping between LCH and CG lists as well as the mapping between CG and HARQ ID, we can totally rely on NW implementation to avoid the mentioned case. |
| MediaTek | N | We think there is already enough flexibility with Rel-16 LCH restrictions to achieve similar results. |
| CATT | No | This brings restrictions on HARQ process assignment and resource allocation. For example, when there is resource left in grant with higher priority, the data of low priority can’t be multiplexed into the HARQ process of high priority. We agree with HW that there already is quite a tool box allowing “reserving” resources for high-priority traffic. |
| Sony | FFS | More discussion is needed to understand the issue. |
| Lenovo | No | Agree with Huawei |
| ZTE | No | Agree with HW |
| LG | No | We believe network can handle this. |
| Sharp | No | We can rely on NW implementation. |
| III | No | Agree with HW. |
| APT | Y | We should always ensure that there is a HARQ process for selection when high priority data arrives. |
| OPPO | No |  |
| Nokia | Yes | This is to avoid the situation where HARQ processes are occupied by low priority data and there is no HARQ processes available for high priority data when it arrives. Although we could rely on association among LCH, CG and HARQ PIDs, it somehow loses the flexibility that NR-U mechanisms was developed for. |
| Ericsson | No | Instead LCH-restrictions can be configured to CGs, and HARQ process pools (with help of offset) can be configured per CG. |
| Qualcomm | No | LCH restrictions already available to handle the issue. |
| vivo | No | Agree with Huawei , MediaTek. |
| Apple | No | New restrictions would increase complexity. Network configuration can ensure distribution of priorities and HARQ ID pools already in Rel-16. |
| Samsung | No | HP allocation is totally up to NW. We think NW implementation can do it or not do it. |
| Intel | No | Agree with Huawei that the issue can be handled by proper configuration. |
| Xiaomi | No | We think the gNB implementation can provide a proper HARQ process for the high priority data. |
| TCL | No | Agree with HW |

## DG and CG with the same HARQ PID

Overlapping DG and CG are assumed to be on different HARQ process IDs in R16 IIoT, as the network knows a priori the HARQ process ID associated with the CG at that CG occasion. However, given the UE selects the HARQ process ID when CGRT and LCH-based prioritization configured, a scheduled dynamic grant can have the same HARQ process ID as a PDU already generated for transmission on a CG. The CG and DG transmission may or may not overlap in time. Reference [15] thus proposes to consider handling conflicting DG-CG transmissions of the same HARQ process ID. The alternative is to rely on the network to partition the HARQ PID space to avoid such collisions, which is possible if there are many HARQ processes.

Question 11: Do you support handling conflicting DG-CG transmissions of the same HARQ process?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | Yes |  |
| MediaTek | N | This can be handled by the network implementation, by proper selection of HARQ PID for the DG. |
| CATT | No | Even with NR-U, UE selects the HPID but that is only among the set of HPIDs it is allowed to use, as defined by both RRC parameters nrofHARQ-Processes and harq-procID-offset. Therefore NW can use other HPIDs for dynamic grant without ambiguity. When, as in R15, a DG overrides a CG with same HPID we think it is a rare case where NW is short of HPIDs to schedule an urgent transmission. And R16 prioritization should apply even the HARQ processes are the same. Of course NW should setup the priorities consistently so that the DG overrides the CG in that case. |
| Sony | No | We rely on the network to partition the HARQ PID space to avoid such collisions. Follow NR-U procedure. |
| Lenovo | No | Can be handled by Network implementation |
| ZTE | No |  |
| LG | No | Network should handle this. |
| Sharp | Yes |  |
| III | No |  |
| APT | N |  |
| OPPO | No |  |
| Nokia | No | Network implementation should be sufficient |
| Ericsson | No | Up to network configuration. |
| Qualcomm | No | No issue detected |
| vivo | No | Leave to network implementation |
| Apple | No |  |
| Samsung | No | We think NW can avoid this collision, even though CG selection is done by UE. |
| Intel | No | We can still rely on the network to partition HARQ process ID space to avoid the collision. |
| Xiaomi | No |  |
| TCL | No | It can be up to NW implementation. |

## UE-initiated COT for FBE

Reference [13] proposes to investigate prioritization mechanisms aiming to decrease the likelihood of a collision of transmissions from different UEs for UE initiated CO in FBE. A limited set of UEs under certain conditions to initiate channel occupancy procedure can be allowed. For example, the UE is only allowed to access the channel if the determined transmission priority exceeds the threshold level.

Question 12: Do you support investigating L2 means to decrease the likelihood of a collision of transmissions from different UEs in FBE?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |
| HW | No | We think this should be discussed in RAN1. |
| MediaTek | N | We can follow RAN1 behavior with regards to COT in FBE. |
| CATT | No | UE-initiated COT is in RAN1’s scope. We can wait for RAN1’s design to be more mature. We would prefer to avoid MAC/PHY tight interactions. |
| Sony | No | It is RAN1 issue. |
| Lenovo |  | Fine to wait for RAN1 outcome. |
| ZTE | No | It is RAN1 issue as sony mentioned. |
| LG | No | Up to RAN1. |
| Sharp | No |  |
| III | No |  |
| APT | N | This is a RAN1 issue |
| OPPO | No | It is more like RAN1 issue. |
| Nokia | No | We think this is RAN1 issue |
| Ericsson | No |  |
| Qualcomm | No | RAN1 scope |
| vivo | No | Can be discussed in RAN1 |
| Apple |  | It’s a potential enhancement. We can wait for the outcome of the RAN1 discussions. |
| Samsung | No | Up to RAN1 |
| Intel | No | This issue should be discussed by RAN1. |
| Xiaomi | No | This is up to RAN1. |
| TCL | No | It belongs to RAN1 scope. |

## Dependency of configuring DFI and CGRT

Reference [17] proposes the CG-DFI should not be configured for a configured grant configuration if its CGRT is not configured. It’s worth noting that RAN1 is considering a number of options how to implement the configured grant transmission for URLLC on shared spectrum channel, including whether to configure CG-DFI procedures and CG-UCI procedures jointly or independently

Question 13: Do you support not allowing to configure CG-DFI for a configured grant configuration if its CGRT is not configured? Or we wait for RAN1?

| Company | Resolve now or wait for RAN1? | Additional comments |
| --- | --- | --- |
| HW | Wait for RAN1 |  |
| MediaTek | Wait for RAN1 |  |
| CATT | Wait for RAN1 | When the cg-RetransmissionTimer is configured, CG-DFI must be enabled/configured, but the contrary is not necessarily true. It is too early to rule out configuring CG-DFI without cg-RetransmissionTimer. |
| Sony | Wait for RAN1 |  |
| Lenovo | Wait for RAN1 |  |
| ZTE (proponent) | Wait for RAN1 | This proposal is to provide our opinion if RAN1 have achieved some interesting conclusion. |
| LG | Wait for RAN1 |  |
| Sharp | Wait for RAN1 |  |
| III | Wait for RAN1 |  |
| APT | Wait for RAN1 |  |
| OPPO | Wait for RAN1 |  |
| Nokia | Wait for RAN1 | To avoid inconsistency between RAN1 and RAN2 later on, we prefer to wait until we have a more clear picture of RAN1’s decision on CG-DFI configuration. |
| Ericsson | Wait for RAN1 |  |
| Qualcomm | Wait for RAN1 |  |
| vivo | Wait for RAN1 |  |
| Apple | Wait for RAN1 |  |
| Samsung | No/Wait for RAN1 | We see there is a benefit of separate configuration of CG-DFI. CG-DFI supports early termination for repetition upon received ACK, auto retransmission upon received NACK in DFI, and reduced PDCCH overhead by all HARQ-ACK in one DCI (DFI) rather than separate UL grant for each HARQ process. We think all these benefits can be achieved without CGRT configuration. Since RAN1 discussion is ongoing, we are fine to wait for RAN1. |
| Intel | Wait for RAN1. |  |
| Xiaomi | Wait for RAN1 |  |
| TCL | Wait for RAN1 |  |

# Conclusion

RAN2 should discuss the above and agree to the following:

TBA

# References

- R2-2100001, Report of 3GPP TSG RAN WG2 meeting #112-e Online

- R2-2100214	Uplink enhancements for URLLC in UCE	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2100222	Analysis on IIoT in Unlicensed Spectrum	CATT	discussion	NR_IIOT_URLLC_enh-Core

- R2-2100233	Harmonizing UL CG enhancements in NR-U and URLLC	Ericsson	discussion	Rel-17

- R2-2100268	CG Harmonization for Unlicensed Controlled Environment	QUALCOMM Europe Inc. - Italy	discussion	Rel-17

- R2-2100717	Support of URLLC in Unlicensed Spectrum	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh

- R2-2100758	Transmission Handling in UCE	Sharp	discussion

- R2-2100759	Autonomous transmission/Retransmission in Unlicensed Controlled Environments	Sharp	discussion

- R2-2100830	Simultaneous configuration of LCH based prioritization and CGRT	vivo	discussion

- R2-2100891	Consideration on URLLC over NR-U	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2100905	Prioritization of UL transmissions in unlicensed URLLC	Sony	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2100920	CG Harmonization for NR-U and IIoT/URLLC in Unlicensed Controlled Environments	III	discussion	NR_IIOT_URLLC_enh

- R2-2100921	Enhancements for URLLC in unlicensed controlled environments	Lenovo, Motorola Mobility	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101321	Remaining issues on configured grant harmonization	MediaTek Inc.	discussion	Rel-17	NR_IIOT_URLLC_enh-Core	Late

- R2-2101508	IIoT operation in unlicensed controlled environments	InterDigital	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101520	IIOT CG operation on shared spectrum	LG Electronics UK	discussion	NR_IIOT_URLLC_enh-Core

- R2-2101531	Considerations on UL Enhancement on the shared spectrum Channel	ZTE Corporation, Sanechips	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101614	Discussion on uplink enhancements for URLLC in unlicensed controlled environments	CMCC	discussion	Rel-17	NR_IIOT_URLLC_enh

- R2-2101667	LCH based Prioritization in UCE	Samsung	discussion	Rel-17

- R2-2101672	LBT failure and LCH based priority	Beijing Xiaomi Mobile Software	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101757	Uplink enhancements for URLLC in unlicensed controlled environments	Intel Corporation	discussion	NR_IIOT_URLLC_enh-Core

- R2-2010836, [AT112e][501][IIoT]: Summary of URLLC in unlicensed controlled environment, Qualcomm Incorporated