3GPP RAN WG2 Meeting #113-e	R2-201xxxx
January 25th– February 5th, 2021

Agenda Item:	8.5.3

Source:	InterDigital

Title:	Summary of URLLC over unlicensed controlled environment

Document for:	Discussion, Decision

# Introduction

This is the summary report for tdocs submitted to the IIoT unlicensed controlled environment agenda item and for offline discussion 505:

- [AT113-e][505][IIoT] Summary of URLLC in unlicensed (InterDigital) Scope: - Identify set of open issues for UCE that need to be addressed based on company contributions and identify any agreeable aspects to be discussed in the first week session - Get company inputs on opens issues (to be kicked off after first session) Intended outcome: - Set of issues that should be discussed in the first session and any proposals that could be agreeable - Set of additional issues that should be addressed but with lower priority Deadline for providing comments: - Companies comments on the summary: January 29th 12:00 UTC (TBC)

| Company | Contact Name, Email |
| --- | --- |

# Open issues

## Configuration of cg-RetransmissionTimer and LCH-based prioritization

Release-16 introduced intra-UE prioritization to handle overlapping uplink grants, which is enabled by configuring LCH-based prioritization. The feature is a key feature of IIoT and is already supported in R16, and it is beneficial regardless of spectrum type (licensed or unlicensed). It’s still, however, FFS if LCH based prioritization can be configured with cg-RetransmissionTimer (CGRT).

It is therefore proposed by [2, 3, 4, 5, 9, 10, 11, 13, 14, 15, 17, 19, 20] to support configuring both LCH-based prioritization and CGRT. In the last email discussion [22], all companies agreed that LCH-based prioritization should be supported for R17 URLLC in an unlicensed controlled environment (UCE). The following is therefore proposed:

Proposal 1: 	LCH based prioritization and cg-RetransmissionTimer can be configured together in Rel-17

Question 1: Do you support the option to configure both LCH based prioritization and cg-RetransmissionTimer together in Rel-17? 
Inputs requested after completion of the first session.

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## Autonomous re-transmissions

In R16, the configuration of autonomousTx or CGRT allows the UE to autonomously (re)-transmit a PDU dropped due to intra-UE prioritization or LBT failure, respectively. One difference is that autonomous transmission in IIoT is restricted to use the next CG occasion of same CG configuration, unlike NR-U. Per the assumption in RAN2#112e, the network will not configure autonomousTx and CGRT simultaneously per cell, at least for R16. For release 17, CGRT operation can be harmonized with autonomous re-transmission; it was agreed last meeting that a deprioritized PDU on a configured grant and/or dropped PDU due to LBT failure or tx failure should be able to be autonomously re-transmitted.

### With cg-RetransmissionTimer configured

With CGRT configured, how does the UE retransmit a TB dropped either due to intra-UE prioritization, due to LBT failure, or due to bad radio conditions? It’s already specified in MAC that a TB dropped due to LBT failure can be retransmitted if the relevant CGRT is not running, using the NR-U framework. The question is whether the same behavior should apply for a deprioritized TB when both LCH-based prioritization and CGRT are configured. This is suggested in [6, 10，11, 15, 16, 19]. The gNB can figure whether the transmission is a new transmission or a retransmission by decoding the NDI value in the CG UCI.

References [4, 9, 15] also mention that both CGRT and autonomousTx can be configured together and work without necessary specification changes, as autonomousTx only controls new transmissions in the HARQ entity while CGRT controls the retransmissions independently. References [5, 6, 13, 14] agree, but propose an enhancement for CGRT, explained in the next section. [16] proposes to mandate the configuration of autonomousTx and LCH-based prioritization for all CGs in a cell group. References [2, 12, 18, 21] on the other hand propose to forbid configuring both together, as there could be conflicting behaviors whereby with CGRT configured the UE behavior is to treat the dropped PDU as a retransmission while with autonomousTx configured the UE treats it like a new transmission.

Question 2-a: Can cg-RetransmissionTimer and autonomousTx be configured together in R-17?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

Question 2-b: With LCH-based prioritization and CGRT configured, do you agree that MAC entity can retransmit a deprioritized TB the same way as a TB dropped due to LBT failure (e.g. if CGRT isn’t running)?

| Company | autonomousTx configured | autonomousTx not configured | Additional comments if yes, clarify if a spec changes is need and what needs to be changed |
| --- | --- | --- | --- |
|  | Reply (y/n) | Reply (y/n) |  |

Impact on CGRT:

References [5, 13, 14] think CGRT and autonomousTx can be configured together, but mention if deprioritization happens after the first symbol of PUSCH transmission on the CG, there may be conflicting behavior specified whereby, autonomousTx instructs the retransmission again on the next CG occasion while CGRT mandates the retransmission after CGRT expiry. This is caused by:

    - CGRT starts at the first symbol of a PUSCH transmission on the corresponding CG.

    - When the UE is configured with autonomousTx, it was agreed in RAN2#112e to stop the CG timer when a TB on the corresponding CG is deprioritized, to allow for a deprioritized PDU for a configured grant to be autonomously transmission immediately.

To enable immediate retransmission when CGRT is configured, [3, 5, 6, 13, 14] propose to stop CGRT when the CG resource associated with the CGRT is deprioritized. Reference [14] also supports the transmission of a deprioritized PDU on the next available CG occasion, including the case where CGRT was never started, though that is already up to UE implementation in such case to select the HARQ process ID. Reference [2] argues that it is not favorable to stop the CGRT in such case, as the gNB can decode the transmitted TB successfully even if the transmission is not completely finished or the gNB can issue a dynamic grant for retransmission after decoding the CG-UCI.

Question 3: Do you support stopping cg-RetransmissionTimer when the CG resource associated with the CGRT is deprioritized due to LCH-based prioritization?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

How to perform autonomous re-tx and impact on HARQ process status

To support the IIoT autonomous re-transmission of a deprioritized TB without autonomousTx configured, [2, 3, 10, 11] propose that the HARQ status to should be switched to pending (similar handling to when a CG transmission fails LBT). Reference [13] proposes considering the HARQ process as not pending upon deprioritization, but with autonomousTx configured. The following proposals were made for the HARQ process status:

- Option 1 - To enable transmitting a deprioritized PDU at the very next CG occasion: upon deprioritization of an UL grant, the corresponding HARQ process switches to “pending” [2, 3, 10, 11]. This can be used to support configuring CGRT without autonomousTx

- Option 2 – When both CGRT and autonomousTx are configured, perform IIoT autonomous framework if the PDU was deprioritized and no indication of LBT failure was received, and perform NRU autonomous retransmission framework for the MAC PDU if LBT fails [4, 10, 16]. HARQ process status remains as is, unless it’s changed per existing R16 specification.

- Option 3 - When an UL grant is de-prioritized or cancelled, the corresponding HARQ process is considered as not pending since the IIoT autonomous transmission functionality will be applied. When a HARQ process is pending due to LBT failure the corresponding UL grant is considered as prioritized, since autonomous retransmission functionality is applied [13]

- Option 4 – No further enhancement is needed. Current specification can handle configuration of LCH-based prioritization and CGRT as is to perform the autonomous retransmission of a deprioritized TB.

- Option 5 – No further enhancement is needed. Do not support retransmission of de-prioritized TB.

Question 4: With LCH-based prioritization and CGRT configured, do you support any of the above options for the HARQ process status?

| Company | autonomousTx configured | autonomousTx not configured | Additional comments |
| --- | --- | --- | --- |
|  | Preferred option(s) | Preferred option(s) |  |

### With cg-RetransmissionTimer not configured

An open issue is whether CG retransmissions due to LBT-failure should also be handled when cg-RetransmissionTimer is not configured, e.g. using the autonomousTx framework. There are two expressed opinions:

Option 1 - no enhancements needed: if an operator would like to handle LBT-failures and UE autonomous retransmission, it can simply configure the CGRT [4, 5, 15, 16].

Option 2 - consider a grant dropped due LBT failure as a “deprioritized grant” to enable the autonomous retransmission of the PUSCH that failed LBT [2, 3, 13, 10, 11, 19, 20].

Question 5: Which option do you prefer for handling autonomous retransmission due to LBT failure when cg-RetransmissionTimer is not configured?

| Company | Preferred option | Additional comments |
| --- | --- | --- |

## CG selection for autonomous retransmissions

When CGRT is configured, the UE still selects the HARQ process on its own and HARQ process sharing between multiple CGs are allowed as in Rel-16 NR-U. Given autonomous (re)transmission mechanism after CGRT expiry can be performed across the different Configured grant configurations sharing the same HARQ process, [14, 17] proposes to avoid using other CG configurations not suitable for the service – compared to the CG initially used. However, [4] questions whether HARQ sharing should still be allowed in R17 when LCH-based prioritization is configured, as it is more suited for same priority data. The following options are possible:

Option 1 - Restrict that the configured grant used for autonomous (re)transmission to be from the same CG configuration used initially, e.g. when LCG-based prioritization is configured [4]

Option 2 - LCH restriction is considered when selecting a configured grant for autonomous (re)transmission from a different CG configuration (consider all restrictions, including allowedCG-List) [14, 17]

Option 3 – No enhancement needed, e.g. rely on the network to configure HARQ sharing for CG configurations that can meet the same type of services.

Question 6: Which of the options above do you support to ensure the CG selected for autonomous retransmission is selected properly to meet the service?

| Company | Reply (y/n) | Additional comments if yes, which enhancement is preferred? |
| --- | --- | --- |

## Prioritization between CG initial transmissions and retransmissions

In Rel-16 NR-U, the UE selects the HARQ process ID for CG and the UE prioritizes retransmissions of PDUs (e.g. due to failed LBT) over new transmissions, while for IIoT on licensed band the HARQ process is strictly associated to the CG occasion time. The NR-U behavior however can result in added latency for newly arrived URLLC data. For CG operation in a controlled unlicensed environment (IIoT and NR-U), the UE can benefit from prioritization between Initial transmission (which may contain higher priority data/control) and retransmissions (due to UL LBT failure, CGRT expiring, or intra-UE de-prioritization), as proposed in [2, 3, 5, 6, 13, 15]. This can require changes, as explained in [4, 5], given currently UE prioritizes retransmission within each CG with CGRT and LCH-based prioritization overrides the retransmission choice.

Question 7: With cg-RetransmissionTimer and LCH-based prioritization configured, do you support prioritization between initial transmissions and retransmissions on a CG based on the LCH priority multiplexed -or to be multiplexed-?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## HARQ Process ID selection when CGRT is not configured

It was agreed in R2#112e that Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection when CGRT is not configured, i.e. nothing changes in that case and the UE uses the formula to determine the HARQ PID according to timing of the CG occasion, as in R16. RAN1 is however discussing the configuration options for the CG-UCI, including configuring it independently of the CGRT. References [3, 5] argue that CG-UCI can be configured independently of CGRT, and when it is configured the UE can select the HARQ PID and send it on the UCI. This issue boils down to whether configuring CG-UCI without CGRT can be common. The following options are possible:

Option 1 - When CG-UCI is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. CG-UCI can be optionally configured without CGRT configured [3, 5]. LS to RAN1 can be sent if needed.

Option 2 – Wait for RAN1 to conclude their discussions [21].

Option 3 - no enhancements needed: if CGRT is not configured, the UE uses the existing formula to determine the HARQ PID according to timing of the CG occasion, as in R16. This is in line with the agreements from last meeting.

Question 8: Which option do you prefer for HARQ process and RV selection when CGRT is not configured?

| Company | Preferred option | Additional comments |
| --- | --- | --- |

# Other issues

Issues not previously discussed or not widely discussed.

## Intra-UE Prioritization considering LBT Failure

References [6, 13] mention that intra-UE prioritization in some case can select a grant outside of an ongoing COT (or a grant for which LBT failure is more likely), even though the deprioritized grant can be selected with a higher probability of succeeding LBT. The following enhancements are proposed:

- Enh 1: [6] proposes that MAC takes the likelihood of succeeding LBT into account in addition to LCH priority, when determiening which grants to (de)-prioritize.

- Enh 2: [13] proposes to transmit a PDU on a deprioritized grant if the transmission on the overlapping prioritized grant failed LBT, to reduce potential waste of resources.

Question 9: Do you support considering LBT failure when determining a grant priority for intra-UE prioritization?

| Company | Reply (y/n) | Additional comments if yes, which enhancement is preferred? |
| --- | --- | --- |

## LCH-dependent HARQ Process Pool

A concern expressed in [6] is that all CG HARQ processes can at times be occupied with (re)-transmission, especially when the channel occupancy is higher, and that could potentially block transmission of newly arrived URLLC/high priority data. Reference [6] therefore proposes to configure a subset of HARQ processes as “restricted processes” for transmission of data from higher priority LCHs, whereby the UE can use a grant from a restricted process if the PDU contains at least one bit from higher priority LCHs associated with the restricted HARQ process. R16 already allows for a CG type-1 based LCP LCH selection restriction, whereby the UE only fills the CG with data from LCHs configured with configuredGrantType1Allowed. This however cannot be configured per CG or per HARQ process, and thus does not forbid the UE form occupying such CGs for transmission of lower priority data, and thus can block the transmission of higher priority data (due to retransmissions, higher channel occupancy etc).

Question 10: Do you support configuring a subset of HARQ processes as “restricted processes”, whereby the UE can multiplex data on a grant from associated restricted process if the PDU contains at least one bit from higher priority LCHs associated with the restricted HARQ process?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## DG and CG with the same HARQ PID

Overlapping DG and CG are assumed to be on different HARQ process IDs in R16 IIoT, as the network knows a priori the HARQ process ID associated with the CG at that CG occasion. However, given the UE selects the HARQ process ID when CGRT and LCH-based prioritization configured, a scheduled dynamic grant can have the same HARQ process ID as a PDU already generated for transmission on a CG. The CG and DG transmission may or may not overlap in time. Reference [15] thus proposes to consider handling conflicting DG-CG transmissions of the same HARQ process ID. The alternative is to rely on the network to partition the HARQ PID space to avoid such collisions, which is possible if there are many HARQ processes.

Question 11: Do you support handling conflicting DG-CG transmissions of the same HARQ process?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## UE-initiated COT for FBE

Reference [13] proposes to investigate prioritization mechanisms aiming to decrease the likelihood of a collision of transmissions from different UEs for UE initiated CO in FBE. A limited set of UEs under certain conditions to initiate channel occupancy procedure can be allowed. For example, the UE is only allowed to access the channel if the determined transmission priority exceeds the threshold level.

Question 12: Do you support investigating L2 means to decrease the likelihood of a collision of transmissions from different UEs in FBE?

| Company | Reply (y/n) | Additional comments |
| --- | --- | --- |

## Dependency of configuring DFI and CGRT

Reference [17] proposes the CG-DFI should not be configured for a configured grant configuration if its CGRT is not configured. It’s worth noting that RAN1 is considering a number of options how to implement the configured grant transmission for URLLC on shared spectrum channel, including whether to configure CG-DFI procedures and CG-UCI procedures jointly or independently

Question 13: Do you support not allowing to configure CG-DFI for a configured grant configuration if its CGRT is not configured? Or we wait for RAN1?

| Company | Resolve now or wait for RAN1? | Additional comments |
| --- | --- | --- |

# Conclusion

RAN2 should discuss the above and agree to the following:

TBA

# References

- R2-2100001, Report of 3GPP TSG RAN WG2 meeting #112-e Online

- R2-2100214	Uplink enhancements for URLLC in UCE	Huawei, HiSilicon	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2100222	Analysis on IIoT in Unlicensed Spectrum	CATT	discussion	NR_IIOT_URLLC_enh-Core

- R2-2100233	Harmonizing UL CG enhancements in NR-U and URLLC	Ericsson	discussion	Rel-17

- R2-2100268	CG Harmonization for Unlicensed Controlled Environment	QUALCOMM Europe Inc. - Italy	discussion	Rel-17

- R2-2100717	Support of URLLC in Unlicensed Spectrum	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_IIOT_URLLC_enh

- R2-2100758	Transmission Handling in UCE	Sharp	discussion

- R2-2100759	Autonomous transmission/Retransmission in Unlicensed Controlled Environments	Sharp	discussion

- R2-2100830	Simultaneous configuration of LCH based prioritization and CGRT	vivo	discussion

- R2-2100891	Consideration on URLLC over NR-U	OPPO	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2100905	Prioritization of UL transmissions in unlicensed URLLC	Sony	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2100920	CG Harmonization for NR-U and IIoT/URLLC in Unlicensed Controlled Environments	III	discussion	NR_IIOT_URLLC_enh

- R2-2100921	Enhancements for URLLC in unlicensed controlled environments	Lenovo, Motorola Mobility	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101321	Remaining issues on configured grant harmonization	MediaTek Inc.	discussion	Rel-17	NR_IIOT_URLLC_enh-Core	Late

- R2-2101508	IIoT operation in unlicensed controlled environments	InterDigital	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101520	IIOT CG operation on shared spectrum	LG Electronics UK	discussion	NR_IIOT_URLLC_enh-Core

- R2-2101531	Considerations on UL Enhancement on the shared spectrum Channel	ZTE Corporation, Sanechips	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101614	Discussion on uplink enhancements for URLLC in unlicensed controlled environments	CMCC	discussion	Rel-17	NR_IIOT_URLLC_enh

- R2-2101667	LCH based Prioritization in UCE	Samsung	discussion	Rel-17

- R2-2101672	LBT failure and LCH based priority	Beijing Xiaomi Mobile Software	discussion	Rel-17	NR_IIOT_URLLC_enh-Core

- R2-2101757	Uplink enhancements for URLLC in unlicensed controlled environments	Intel Corporation	discussion	NR_IIOT_URLLC_enh-Core

- R2-2010836, [AT112e][501][IIoT]: Summary of URLLC in unlicensed controlled environment, Qualcomm Incorporated