3GPP TSG-RAN WG2 Meeting #113bis-e	R2-2103399

Online, April 12th – 20th 2021

Agenda item:	   8.2.2.3

Source:		Lenovo, Motorola Mobility

Title:		Discussion on SCG activation

Document for:		Discussion and Decision

1	Introduction

In the last RAN2#113e meeting, the following agreements have been made for SCG activation.

RAN2#113e Agreements: 1 NW-triggered SCG activation is indicated to the UE via the MCG. 2 The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options: option 1) similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell. option 2) in certain cases: - the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time). - the SCG can schedule data transmission on the PDCCH The UE decides not to perform random access (one option to be selected): option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts) option 2b) based on the contents of the SCG activation indication FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected): - will perform random access upon reception of the next SCG activation indication from the MCG - reports measurement results (details FFS) via the MCG and wait for reconfiguration. 7 Further discuss the format and content of the SCG activation indication from the MCG to the UE after there is more progress on solution 2. 8 Further discuss the comparison between - define a mechanism for SCG activation upon UL data arrival on SCG bearers - use split bearer with primary path on MCG (network sees UL data and can initiate activation) 1a SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used. 3 RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

In this paper, we further discuss issues related to SCG activation.

2	Discussion

2.1	Issues related to random access

One issue remaining from previous meeting is what happens to the timeAlignmentTimer if SCG is deactivated. In our understanding, there are two sub-cases:

- if MCG and SCG belong to the same TAG, then UE can rely on the TAT in the MCG to determine if it’s still synced in SCG UL.

- If SCG belongs to a different TAG than MCG, the maintenance of SCG related TAT could be tricky. When SCG is deactivated, UE is not expected to tranmit anything in the SCG UL, therefore there is no mean to maintain time alignment when SCG is deactivated. On the other hand, it does not harm to keep the current TAT timer running. As long as TAT does not expire, UE can assume it’s still synced with the current TAG. One benefit of keeping current TAT running is that, when the SCG is activated and if TAT is not expired, UE does not need to perform random access since UE is still synced in the UL. Of course, upon SCG activation and TAT expiry, UE needs to perform random access as in legacy.

TS 38.300 The random access procedure is triggered by a number of events: - Initial access from RRC_IDLE; - RRC Connection Re-establishment procedure; - DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised"; - UL data arrival during RRC_CONNECTED when there are no PUCCH resources for SR available; - SR failure; - Request by RRC upon synchronous reconfiguration (e.g. handover); - Transition from RRC_INACTIVE; - To establish time alignment for a secondary TAG; - Request for Other SI (see clause 7.3); - Beam failure recovery; - Consistent UL LBT failure on SpCell.

At this stage, time alignment towards SCG when SCG is deactivated is not supported. Random access helps to achieve time alignment towards SCG when SCG is activated.

Upon SCG deactivation, the relevant TAT continues running.

Upon SCG activation, UE performs random access towards PSCell if the relevant TAT expires.

Another issue left from RAN2#112e meeting is that if UE does RACH towards the target PSCell during PSCell change. In our view, it is needed, otherwise the NW does not know if the PSCell change is successful or not.  Besides, since the SCG RRC reconfiguration can select the SCG activation state at PSCell change, UE can determine if it needs to stay in SCG activation state or enter SCG deactivation state after random access.

RAN2#112e Agreements: - SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell addition/change, RRC resume or HO. - The SCG RRCReconfiguration can change the PSCell. FFS if the UE does RACH towards the target PSCell, in that case.

SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell change

UE performs random access towards the target PSCell during PSCell change in case the target SCG is configured as deactivated state.

UE enters SCG deactivated state after random access towards the target PSCell if the SCG state is set to be deactivated during PSCell change.

2.2	Issues related to transmission power reallocation

When SCG is deactivated, UE can in principle allocate more power to MCG transmission. When SCG is activated again, the transmissions in MCG and SCG are constrained by the maximum power in MCG and maximum power in SCG respectively. In this case, it could be beneficial to trigger a power headroom report sent from UE to MN. Then MN is aware of the estimated transmission power over SCG and can control the transmission power over MCG accordingly. Similar as the legacy, wherein the PHR will be triggered if a PSCell is added or changed, RAN2 could support PHR triggered by SCG activation.

TS 38.321 5.4.6 Power Headroom Reporting The Power Headroom reporting procedure is used to provide the serving gNB with the following information: - Type 1 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH transmission per activated Serving Cell; - Type 2 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for UL-SCH and PUCCH transmission on SpCell of the other MAC entity (i.e. E-UTRA MAC entity in EN-DC, NE-DC, and NGEN-DC cases); - Type 3 power headroom: the difference between the nominal UE maximum transmit power and the estimated power for SRS transmission per activated Serving Cell; - MPE P-MPR: the power backoff applied by the UE to meet the MPE FR2 requirements for a Serving Cell. ……. A Power Headroom Report (PHR) shall be triggered if any of the following events occur: …… - activation of an SCell of any MAC entity with configured uplink of which firstActiveDownlinkBWP-Id is not set to dormant BWP; - addition of the PSCell (i.e. PSCell is newly added or changed); - phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink: - there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell. - Upon switching of activated BWP from dormant BWP to non-dormant DL BWP of an SCell of any MAC entity with configured uplink;

PHR is triggered if PSCell is added or changed.

Upon SCG activation, MCG PHR is triggered.

2.3	Issues related to NW triggered SCG activation

Regarding the issue if MN or SN can reject the SCG activation triggered by the peer SN or MN, RAN3 is currently discussing it, RAN2 can simply wait for RAN3 conclusion.

RAN3#111-e Agreements: Open issue 1: During SN addition procedure, if the request of SCG (de)activation is rejected: 1) SN uses the response message including “SCG deactivation” result is sufficient ; 2) or SN allows to use the reject message including new Cause value; 3) or SN allows to uses the reject message as legacy (without new Cause) Open issue 2: During SN modification procedure, if the request of SCG (de)activation is rejected: 1) SN uses the response message including “SCG (de)activation” is sufficient ; 2) or SN allows to use the reject message including new Cause value; 3) or SN allows to use the reject message as legacy (without new Cause).

It is upon RAN3 discussion whether MN/SN triggered SCG activation can be rejected by the peer SN/MN.

Besides, we don’t see any reason for UE to reject the SCG activation triggered by the NW. which might lead to extra signaling overhead too.

UE does not reject the SCG activation triggered by NW.

In the legacy, for an activated SCG, the SCell can be either in activated state, deactivated state, or dormant state. NW may also configure the SCell states when activating SCG reusing the legacy SCell Addition/Modification procedure

TS 38.331 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated; 2> if the sCellState is included: 3> configure lower layers to consider the SCell to be in activated state; 2> else: 3> configure lower layers to consider the SCell to be in deactivated state; …… 3> if the sCellState is included: 4> configure lower layers to consider the SCell to be in activated state; 3> else: 4> configure lower layers to consider the SCell to be in deactivated state.

When activating the SCG, NW can also configure the SCells states, e.g. if SCells enters activated state, deactivated state or dormant state.

In the last meeting, RAN2 agreed to support RRC signalling to activate the SCG. Whether an explicit SCG activation indicator is needed in the RRC message to activate the SCG, we believe it depends on whether any SCell is set to activation or dormant state. As agreed in the last meeting, when SCG is deactivated, no SCell is in activated state or dormant state. Thus, when setting a single SCell, that belongs to the deactivated SCG, to either activated or dormant state, it implicitly means the whole SCG is activated.

Considering above, an explicit SCG activation indicator might be not needed if any SCell is set to activated or dormant state. In another word, the explicit SCG activation indicator is needed if only PSCell is activated.

RAN2 #112e Agreements 7a: While the SCG is deactivated: - there can be SCG SCells in deactivated state - there cannot be SCG SCells in activated state - it is FFS whether there can be SCells in SCG dormant state. RAN2#113e Agreements 3 Confirm that there is no support of SCell dormancy for SCG SCells within a deactivated SCG.

All SCells are in deactivated state when SCG is deactivated

When activating the SCG, explicit SCG activation indicator is not needed if any SCell is configured to enter activated or dormant state.

From UE perspective, when UE receives a RRC message to configure a SCell with activated state while the associated SCG has been deactivated, UE follow the procedures to activate the SCG.

When UE receives a RRC message to configure a SCell with activated or dormant state while the associated SCG has been deactivated, UE follow the procedures to activate the associated SCG.

2.4	Issues related to UE triggered SCG activation

When SCG is deactivated, there is no UL and DL transmission in SCG radio bearers either terminated in MN or SN. When there is DL data arrival at SCG radio bearer, depending on whether it’s MN or SN terminated, MN or SN could initiate the SCG activation for the DL data arrival. Similarly, when there is UL data arrival at SCG radio bearer, UE shall initiate the SCG activation.

In another scenario, if UE is configured to support fast MCG link recovery, UE is supposed to send a MCG link failure information via SCG and forwarded to MN via SN. If the SCG is deactivated at the time when MCG link fails, UE has to first activate the SCG before sending the MCG link failure information.

When fast MCG link recovery is configured, UE sends a MCG failure information to MN via SCG.

UE may trigger SCG activation in the following two scenarios:

  - Upon UL data arrival at SCG radio bearer and SCG is deactivated

  - Upon MCG failure and fast MCG link recovery is configured

There are two alternatives in our view to support UE triggered SCG activation with their own pros and cons:

Option 1: MCG based. UE first informs MN about the request to activate the SCG via e.g. RRC signalling. After receiving the acknowledge from MN, UE either triggers random access to PSCell, or sends SR to SN directly.

Option 1 works if MCG works. After receiving the SCG activation request from UE, MN could first handshake with SN about the UE’s request to activate SCG. After both MN and SN are fine with the SCG activation request, MN/SN can provide the UE with RRC reconfiguration to activate SCG including possible SCell configurations.

Option 2: SCG based. If SCG TAT has expired, UE triggers random access to PSCell. If SCG TAT is still running, UE sends SR to SN requests UL transmission.

Option 2 can work in both data arrival and MCG failure case. Besides, option2 is considered a faster procedure compared to option 1. However, it’s a bit unclear how MN and SN handshake about the UE triggered SCG activation in this case. For example, shall SN notify MN about the UE triggered SCG activation via random access, and shall the notification happen before SN replies UE and complete the random access or shall the notification happen after the completion of random access?

In fact, Option 1 does not seem much different from the NW-triggered SCG activation based on UE provided information, thus shall be supported. While option 2 can be considered as a faster method to activate the SCG and to handle MCG failure scenario, which is beneficial to be supported as well.

RAN2 supports two ways for UE to trigger SCG activation

  - UE provides assisting information to MN requesting SCG activation.

  - UE triggers random access to the PSCell if SCG TAT expires, or sends SR to SCG if SCG TAT is still running.

3	Conclusion

Based on the discussion above, we observe:

Observation 1	At this stage, time alignment towards SCG when SCG is deactivated is not supported. Random access helps to achieve time alignment towards SCG when SCG is activated.

Observation 2	SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell change

Observation 3	PHR is triggered if PSCell is added or changed.

Observation 4	When activating the SCG, NW can also configure the SCells states, e.g. if SCells enters activated state, deactivated state or dormant state.

Observation 5	All SCells are in deactivated state when SCG is deactivated

Observation 6	When fast MCG link recovery is configured, UE sends a MCG failure information to MN via SCG.

Based on the discussion above, we propose:

Proposal 1	Upon SCG deactivation, the relevant TAT continues running.

Proposal 2	Upon SCG activation, UE performs random access towards PSCell if the relevant TAT expires.

Proposal 3	UE performs random access towards the target PSCell during PSCell change in case the target SCG is configured as deactivated state.

Proposal 4	UE enters SCG deactivated state after random access towards the target PSCell if the SCG state is set to be deactivated during PSCell change.

Proposal 5	Upon SCG activation, MCG PHR is triggered.

Proposal 6	It is upon RAN3 discussion whether MN/SN triggered SCG activation can be rejected by the peer SN/MN.

Proposal 7	UE does not reject the SCG activation triggered by NW.

Proposal 8	When activating the SCG, explicit SCG activation indicator is not needed if any SCell is configured to enter activated or dormant state.

Proposal 9	When UE receives a RRC message to configure a SCell with activated or dormant state while the associated SCG has been deactivated, UE follow the procedures to activate the associated SCG.

Proposal 10	UE may trigger SCG activation in the following two scenarios:

a.	Upon UL data arrival at SCG radio bearer and SCG is deactivated

b.	Upon MCG failure and fast MCG link recovery is configured

Proposal 11	RAN2 supports two ways for UE to trigger SCG activation

a.	UE provides assisting information to MN requesting SCG activation.

b.	UE triggers random access to the PSCell if SCG TAT expires, or sends SR to SCG if SCG TAT is still running.

References