3GPP TSG RAN WG2 Meeting #113bis-e                              	R2-2103888
Electronic Meeting, 16th Mar – 26th Mar 2021

Source: 			Apple Inc

Title: 	RRM Relaxation down selection of options for RedCap

Agenda item:	8.12.3.2

Document for: 	Discussion and Decision

# Background

RAN-91e has added the below as RedCap WI objectives [1] and this document focuses on selecting the RRM relaxation solutions that are listed in the SI TR [2].

- Specify support for the following Extended DRX enhancements for RedCap UEs [RAN2, RAN3, RAN4]: - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10.24 s, without using PTW and PH, and with common design (e.g. common set of eDRX values) between RRC Inactive and Idle - Extended DRX for RRC Inactive and Idle with eDRX cycles up to 10485.76 s; the details of mechanisms and feasibility regarding maximum length of the extended DRX cycles for RRC Inactive and Idle need to be checked by SA2, CT1 and/or RAN4. - RAN2 to decide which Node(s) configure eDRX in RRC_Idle and RRC_Inactive. - RRM relaxations for neighbouring cells for RedCap devices: for RRC_Idle/Inactive/Connected, considering the alternatives identified in the RedCap SI: - Study until RAN#92e, and, if agreed, specify RRM measurement relaxation criteria (where, for RRC_Idle/Inactive the Rel-16 mechanism is the baseline, and for RRC_Connected the mechanism reuses the Rel-16 RRM relaxation criteria from RRC_Idle/Inactive so as to maximize the commonality with Idle/Inactive UEs) [RAN2] - Enabling/disabling of RRM relaxation should be under the network’s control. Specify both broadcast and dedicated signalling for enabling/disabling of RRM relaxation. - After RAN#92e, if agreed in RAN2, specify RRM measurement relaxation [RAN4] - No RRM relaxations are specified for the serving cell.

# Discussion

## 2.1 View from the RedCap use-cases perspective

Based on the RedCap WI/SI description [2], it can be said that the majority of the use-cases that RedCap UEs are expected to execute fall into the below categories of mobility:

- Surveillance/Industry sensors

  - Low mobility or stationery

  - Confined mobility

- Wearables

  - Mobility compared to legacy

  - Potential stationery/low mobility periods in time

Based on the above, it can be inferred that the kind of mobility the UE is in, while this can be controlled by the NW, some of it ( confined mobility etc..) is known at the UE better than the NW configured detection. So we think that RedCap UEs should be able to report the type of mobility it is expected to be in.

As captured in the TR, we propose that RAN2 allows the UEs to report the mobility state it is in, atleast for the cases of stationary/confined mobility.

Observation 1: Certain RedCap UEs inherently are confined to specific usecases and it is beneficial to the NW to know about this.

Observation 2: The usage and scope of this capability is limited to RAN, and so it’s better to have this reported using the RAN UE capability.

For the industry sensors or surveillance type of devices, it is already expected that their mobility is very restricted or in other words confined. However, there could be other sensors which are actively mobile within an industrial complex with speeds comparable to the legacy UEs, but still their mobility is confined to a geographical area.

Observation 3: Confined mobility is inherent to surveillance/industry sensors. The additional aspect is whether this mobility within a confined location is low mobility/stationary or comparable to legacy mobility.

Proposal 1: RAN2 to agree that for surveillance/industry sensors, the mobility of these type of UEs is confined. This confined mobility can be slow/stationary, or the mobility can be comparable to legacy mobility while still being confined.

Proposal 2: RedCap UEs can inform the NW of the type of the mobility it is expected to follow using existing UE capability procedure. The allowable types are : {stationary/low confined mobility, confined mobility}

## 2.2 NW control of configuration

For all of the RedCap UEs types, it is agreed that the NW controls the states/actions at the UE for RRM relaxation. The TR [2] discussed enhancements 1-5 on the triggering of the relaxation. It is in our view that both the time domain as well as the signal quality measurement are a good mix of indicators of the UE mobility. So we propose that enhancements 1 and 2 are needed with the thresholds tuned to the type of RedCap mobility, so that the NW has the option to control the triggering of the conditions.

Proposal 3: Enhancement 1 and enhancement 2 are agreed to be used by the NW, where RedCap specific thresholds are configured by the NW by taking into consideration the RedCap mobility type from proposal 2.

## 2.3 Allowed UE relaxations

Observation 4: The number of potential Ncells/frequencies the RedCap UE is heavily influenced by the whether the RedCap UE is locally confined in mobility or not.

In this aspect, we think enhancements 3 and 4 of the RRM relaxation methods are very useful.

Proposal 4: Enhancements 3 and 4 of the RRM relaxation methods are agreed to be used for RedCap devices, with the option of allowing the NW to apply these based in the mobility type the RedCap UE reports.

## 2.3 CONNECTED mode operation

All of views so far do not distinguish between RRC_IDLE/INACTIVE and RRC_CONNECTED, as the NW is the one that controls the configuration and the UE just reports its mobility type in the UE capability.

It is our view to keep the above proposal common between IDLE and CONNECTED, with the added benefit that the UE can inform the NW of additional changes (if any) during the CONNECTED mode.

Proposal 5: The RedCap RRM relaxation triggering framework is applicable to both IDLE and CONNECTED modes, and RRM relaxation methods are also common between IDLE and CONNECTED modes.

# Proposals

# Reference

- RP-210918	RedCap WID

- TR 38.875