3GPP TSG RAN WG2#114-e	R2-2104710

e-Meeting, 19th - 27th May, 2021

3GPP TSG RAN WG 1 Meeting #104b-e		R1-2104045

e-Meeting, April 12th – 20th, 2021

Title:	LS on G-RNTI and G-CS-RNTI for MBS

Response to:

Release:	Rel-17

Work Item:	NR_MBS

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Fei Wang

wangfei@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN1 discussed whether a UE can be configured with multiple G-RNTIs/G-CS-RNTIs or not. Regarding G-RNTI (group-common RNTI), the following is RAN1 common understanding:

- G-RNTI is used for PTM scheme 1 based (re)transmission of dynamic group-common PDSCH and the corresponding group-common PDCCH.

- PTM transmission scheme 1: For RRC_CONNECTED UEs in the same MBS group, use group-common PDCCH with CRC scrambled by group-common RNTI to schedule group-common PDSCH which is scrambled with the same group-common RNTI.

Regarding G-CS-RNTI, RAN1 made the following agreement:

Agreement:

Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

- G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH

- FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH.

- FFS: Number of G-CS-RNTI.

RAN1 seeks answers from RAN2 regarding the following questions:

Question 1: Whether RAN1 should consider the case of UE supporting multiple G-RNTIs?

Question 2: Whether RAN1 should consider the case of UE supporting multiple G-CS-RNTIs?

# 2	Actions

To: RAN2

ACTION:	RAN1 respectfully asks RAN2 to provide answers for the questions and additional details that RAN1 shall further consider.

# 3	Dates of next TSG RAN WG1 meetings

TSG RAN WG1 Meeting #105	10th May – 27th May, 2021	E-meeting

TSG RAN WG1 Meeting #106	16th Aug – 27th Aug, 2021	E-meeting