3GPP TSG-RAN WG2 Meeting #114 electronic				R2-2104966
Online,  May 19 – May 27, 2021

Agenda item:		8.10.2.1

Source:			Asia Pacific Telecom, FGI

Title:		Discussion on UE-specific TA report

Document for:	 	Discussion and decision

Introduction

RAN1 has agreed that the UE-specific TA report is beneficial in RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED state.

RAN1 #103 Agreement - An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. - An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. RAN1#104 Agreement - An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control - For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. FFS: Details of the combination of open and closed loop TA control

In last RAN2 meeting, we agreed that the UE may report UE-specific TA. The exact information and frequency of reporting is determined by RAN1, while RAN2 should further discuss when/how to report the UE-specific TA.

RAN2 #113bis Agreement - At least for uplink scheduling adaptations, the UE may report information about the UE specific TA pre-compensation. The exact information and frequency of reports depend on RAN1 outcome. FFS on when/how to report. - The UE reports the UE specific TA pre-compensation during RACH procedure using MAC CE (FFS if this needs to be configured). Actual content is FFS and also depends on further RAN1 input.

However, the second agreement, i.e., the UE reports the UE specific TA pre-compensation during RACH procedure using MAC CE, turned to FFS due to objected by a company.

In this contribution, we proposed our views on the format of UE-specific TA report and when/how to trigger the UE-specific TA report.

Discussion

Format of UE-specific TA report

In [1], two signalling formats have been discussed on the table, i.e., MAC CE or RRC signalling. The majority view (16 out of 24) preferred to use MAC CE to carry UE-specific TA report. Nonetheless, a few companies raised the concern on security for MAC CE since there is no integrity protection on MAC CE.

From our perspective, we think the integrity protection is not needed for UE-specific TA report since the legacy TA command (either in MAC CE or in RAR) does not have integrity protection as well. In addition, the UE should report the UE-calculated TA information instead of UE location in the UE-specific TA report since there is privacy concern on UE location, so the UE-specific TA report is not a sensitive information. Therefore, it’s better to align the legacy TA mechanism to use MAC CE format, which is also more dynamic than RRC message.

Proposal 1: UE-specific TA is reported by MAC CE.

UE-specific TA report for a RA procedure

In NR, the gNB is responsible for maintaining the TA to keep the L1 synchronized, i.e., to ensure the DL/UL frame boundary alignment at the NW side. In the current mechanism, gNB can transmit an absolute TA via the RAR and a delta TA via TA command MAC CE to maintain the TA of the UE. It is beneficial for the subsequent scheduling efficiency if the UE could moderately report the UE-specific TA to gNB for assistance as soon as possible.

For initial access or when UL synchronisation status is "non-synchronised", the RA procedure is firstly applied to make the connection with the gNB, thus it is profitable that the UE can trigger the UE-specific TA report and report the UE-specific TA report MAC CE via the RA procedure. In the RA procedure, Msg3 has the first PUSCH resource for the 4-step RA type, and MsgA has the first PUSCH resource for the 2-step RA type. Thus, the UE-specific TA report should be triggered before the Msg3/MsgA transmission. In our view, two timings are feasible to trigger the UE-specific TA report:

- Option (1): Upon RA procedure initialization

- Option (2): Upon transmission of the RA preamble.

The difference is that the UE only performs the RA procedure initialization once but may perform the preamble transmission several times, e.g., in case that the UE suffers from the RAR reception error or the contention resolution failure, the UE may need to re-attempt the preamble transmission for the same RA procedure. Therefore, it should be considered that whether the UE needs to trigger the UE-specific TA report again when re-attempting the preamble transmission in the same RA procedure.

In the current mechanism, the UE will store the MAC PDU for Msg3/MsgA in the Msg3/MsgA buffer. If the UE needs to re-attempt the RA procedure, the UE can obtain the same MAC PDU from the Msg3/MsgA buffer to re-transmit. In other words, there is no need to generate a new UE-specific TA report MAC CE if it has been included in the MAC PDU for Msg3/MsgA, i.e., the UE only needs to trigger the UE-specific TA report once for a RA procedure even if the UE suffers from the RAR reception error or the contention resolution failure. In that sense, option (1) is preferred.

Proposal 2: UE triggers the UE-specific TA report when the RA procedure is initiated.

In NR, the RA procedure can be triggered by a number of events. It can be known that the UE-specific TA report is helpful for some events, e.g., initial access from RRC_IDLE, RRC Connection Re-establishment procedure, DL or UL data arrival during RRC_CONNECTED when UL synchronisation status is "non-synchronised, etc., since the gNB does not maintain TA information for these events. However, for some other events, e.g., Request for Other SI, BFR, Consistent UL LBT failure, etc., it’s not clear whether the UE-specific TA report is also needed from the gNB perspective, since the UE is in RRC_CONNECT state and the TA is under control by the gNB.

Proposal 3: RAN2 discusses the UE should trigger the UE-specific TA report when the RA procedure is initiated by which of the RA events.

UE-specific TA report by NW control

Based on [2], the majority of companies think that NW can request UE to report UE-specific TA. There may be a different understanding on the meaning of NW request. One option is that the NW can configure (e.g., by SIB or dedicated RRC configuration) whether or not the UE should trigger/report the UE-specific TA. In some cases, e.g., the cell size is small enough to limit all UE’s differential RTT, or when the UE has no time-critical service, the function of UE-specific TA report can be disabled by NW. The other option is that the NW can trigger the UE-specific TA report aperiodically by DCI, e.g., like aperiodic CSI report, when the NW requires it. Enable/disable-based function is beneficial to reduce the unnecessary signalling overhead if the UE-specific TA report is redundant in some scenarios from gNB point of view. On the other hand, aperiodic triggering-based function is beneficial to provide the UE-specific TA report more precisely with less misalignment opportunity.

Proposal 4: NW can control the triggering of the UE-specific TA report by the following options:

- Option 1: Enabling/disabling the UE-specific TA report (e.g., by SIB or dedicated RRC configuration)

- Option 2: Aperiodically triggering the UE-specific TA report (e.g., by DCI)

Triggering the UE-specific TA report periodically

Based on [2], most companies also have sympathy that NW can request UE to report UE-specific TA periodically. This concept can be realized by reusing the mechanism of periodic BSR. For BSR, a periodic BSR timer is (re-)started upon the UE generates the BSR MAC CE. When the periodic BSR timer expires, the UE should trigger the BSR. Similarly, for the UE-specific TA report, a periodic BSR timer is (re-)started upon the UE generates the UE-specific TA report MAC CE. When the periodic timer for the UE-specific TA report expires, the UE should trigger the UE-specific TA report.

Proposal 5: A period timer for UE-specific TA report is introduced.

- The period timer for UE-specific TA report is (re-)started upon the UE generates the UE-specific TA report MAC CE.

- When the periodic timer for the UE-specific TA report expires, the UE should trigger the UE-specific TA report.

UE-specific TA report based on some criteria

In [2], some companies proposed that the UE specific TA report can be triggered by some criteria/rules detected by the UE itself. For example, if the difference between the current TA used by the UE and the value last reported by the UE exceeds a threshold, the UE should trigger the UE specific TA report. It is noted that the UE-specific TA is calculated by UE itself. Without receiving the UE-specific TA report up-to-date, the gNB cannot acquire the timely information of TA. In some situations, the UE may move too fast to not trigger the UE-specific TA report on time, e.g., if the period timer is too long or if the NW miss triggering the UE to report UE-specific TA. In that sense, if the difference between the current TA used by the UE and the value last reported is too large, the misalignment of TA between the UE and gNB would occur, which is not favourable. To reduce the chance of TA misalignment, event-based triggering by UE itself can be supported.

Proposal 6: UE triggers the UE-specific TA report when the difference between the current TA value calculated by the UE and the last TA value reported by the UE is above a threshold.

Conclusion

In summary, we have the following proposals.

Proposal 1: UE-specific TA is reported by MAC CE.

Proposal 2: UE triggers the UE-specific TA report when the RA procedure is initiated.

Proposal 3: RAN2 discusses the UE should trigger the UE-specific TA report when the RA procedure is initiated by which of the RA events.

Proposal 4: NW can control the triggering of the UE-specific TA report by the following options:

- Option 1: Enabling/disabling the UE-specific TA report (e.g., by SIB or dedicated RRC configuration)

- Option 2: Aperiodically triggering the UE-specific TA report (e.g., by DCI)

Proposal 5: A period timer for UE-specific TA report is introduced.

- The period timer for UE-specific TA report is (re-)started upon the UE generates the UE-specific TA report MAC CE.

- When the periodic timer for the UE-specific TA report expires, the UE should trigger the UE-specific TA report.

Proposal 6: UE triggers the UE-specific TA report when the difference between the current TA value calculated by the UE and the last TA value reported by the UE is above a threshold.

Reference

[1] R2-2104370	Summary of offline 103 - [NTN] RACH aspects - second round (Oppo)

[2] R2-2103630 Report of [POST113-e][106][NTN] MAC aspects (Huawei)