3GPP TSG-RAN WG2 Meeting #114-e                                                     R2-2105194

Online, May 19 – May 27, 2021

Source:	CATT

Title:	Further Consideration on Paging Collision Avoidance

Agenda Item:	8.3.2

Document for:	Discussion and Decision

Introduction

At RAN2#113bis-e meeting, only some agreements about EPS have been achieved for paging collision avoidance [1]:

- Agreements

- 1: For the EPS PO/PF calculation, include the UE_offset to the UE_ID calculation formula.

- 2: No additional modification for the EPS eDRX case.

Other issues have not been discussed adequately. So in this contribution, we first intend to discuss which solution we support to enhance the 5GS paging collision avoidance, and then to discuss the predictability of paging collision avoidance.

Discussion

Support Solution for 5GS Paging Collision Avoidance

Even there is some support for solution 1 (for 5GS), it is not finally decided which solution is preferred by RAN2 at last. Therefore we have to further discuss and then decide the solution. As we mentioned in [2], since the detailed advantage and the disadvantage of each option have already been discussed before, from the analysis of above sections, we listed the key points and the RAN impact of each option in the table below:

Table 1: Key point and RAN impact of options 1, 2a, 2b, 2c, 3

| Option | Key point | RAN impact |
| --- | --- | --- |
| Option 1 | Paging collisions may occur after cell reselection; Simplest way for 5GS. | No impact. |
| Option 2a | Paging collisions may occur after cell reselection; More complex than option1. | Change the legacy way to calculate PF/PO. |
| Option 2b | Paging collisions may occur after cell reselection; Simplest way for EPS. | Change the legacy way to calculate PF/PO. |
| Option 2c | Paging collisions may occur after cell reselection; Increase the power consumption | Depend on other options |
| Option 3 | The signal overhead on Uu may be significantly increased in the RAN. | Increase numbers of UE Paging monitoring. |

First, the effectiveness ‎of option 3 is much lower compared with other options, so we do not think option 3 should be used, no matter with or without other solution else. Second, since the Paging collisions may occur after cell reselection for all the other options, the result of the enhancement cannot be inherited by other cells, we think the simplest way is enough and should be adopted in a single cell.

Hence based on the discussion in section 2.1, for 5GS, option 1 without other enhancement is the effective and simplest way:

Proposal 1: ‎ Option 1 (UE-requested 5G-GUTI reassignment) is used for 5GS, no other assistant information from UE is needed.

Predictability of Paging Collision Avoidance

Paging collision may have a low probability within all the Pagings. After UE being distributed an ID (e.g. 5G-GUTI) and receiving the Paging configuration from the network, the Paging occasion is fixed. At this time the UE could detect whether a paging collision may be occurred by comparing the Paging occasion of different USIMs, and then ask the network to adjust if needed. The adjustment of PO can be a passive method to solve the potential paging collision, and it is not necessary to define a new UE behavior predictable for paging collision avoidance.

Proposal 2: No UE behavior predictable for paging collision avoidance is needed.

Conclusion

According to the above discussion, the proposals for the left issues about paging collision avoidance are as follows:

Proposal 1: ‎ Option 1 (UE-requested 5G-GUTI reassignment) is used for 5GS, no other assistant information from UE is needed.

Proposal 2: No UE behavior predictable for paging collision avoidance is needed.

Reference

- R2-113bise Chairman notes 2021-04-21 0700 UTC

- R2-2100280 Further Consideration on Paging Collision Avoidance CATT