3GPP TSG-RAN WG2 Meeting #114 electronic	R2-2105384

Online, May 19 – 27, 2021

Agenda Item:		8.10.3.3

Source:				ASUSTeK

Title:		Discussion on measurement event triggering in NTN

Document for: 	Discussion and Decision

- Introduction

In RAN2 #113bis meeting, the issue about the combination of time/location-based event and radio-based event has been discussed in e-mail discussion and be summarized as postponed [1]. In this contribution, we further discuss the combination of location-based and radio-based event for measurement reporting triggering.

- Discussion

For measurement report triggering, it has been agreed in RAN2 #112 meeting [2] that location-based measurement event in combination with existing measurement event (i.e., the radio-based measurement) should be supported in NTN. The related configuration is still need discussion.

Agreements via email - offline 105:

4.	The Location-based measurement event, in combination with the existing measurement event in NR, should be supported in NTN for both moving cell and fixed cell scenarios. FFS on how to configure the location based measurement event.

According to the report from the e-mail discussion in RAN2 #113bis meeting [1], it has been proposed to postpone that if the time/location-based event shall be always used with radio-based event for CHO triggering or measurement report triggering in NTN. Some companies mentioned that the combination is always needed to make sure the robustness for CHO, while the concern may be less critical for measurement report. Considering the radio condition (e.g., RSRP) is more unreliable in NTN and the criteria of location-based measurement event can represent the UE location within a cell (e.g. whether the UE is at the cell edge), the location-based measurement event could also be configured standalone to trigger a measurement report. From network point of view, whether to configure the location-based measurement event combined with a radio-based measurement event or to configure the location-based measurement event solely can be decided by NW. Therefore, location-based measurement event should be able to be configured as independent or combined event with the radio-based measurement event to the UE to trigger a measurement report.

Proposal 1: A location-based measurement event could be configured independently, or be configured to combine with a radio-based measurement event by the network.

Moreover, according to the agreement in RAN2 #112 meeting, all the existing radio-based measurement events can be used in NTN. For the NW flexibility, all the radio-based measurement events can be discussed to combine with the location-based measurement events if applicable.

Agreements

4.	The existing measurement framework (e.g. measurement configuration, execution and reporting) is the baseline, and all the existing measurement criteria and event can be used in NTN. Support for new measurement is not excluded.

Proposal 2: It can be further discussed that the location-based measurement event can be configured to combine with all the existing radio-based measurement events.

- Conclusion

In this contribution, we discuss configuration of the location-based measurement event, and have the following proposals:

Proposal 1: A location-based measurement event could be configured independently, or be configured to combine with a radio-based measurement event by the network.

Proposal 2: It can be further discussed that the location-based measurement event can be configured to combine with all the existing radio-based measurement events.

- Reference

- R2-2104373, “Report from [113bis-e][107][NTN] CHO aspects (Nokia) – Phase 2”

- R2-2100001, “Report of 3GPP TSG RAN2#112-e meeting, Online.”