3GPP TSG-RAN WG2 Meeting #114-e	R2-2105501

# Electronic Meeting, May 19th – 27th 2021                   Revision of R2-2104339

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.340 | CR | 0262 | rev | 2 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous corrections to 37.340 on mobility enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation (Rapporteur), Sanechips, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh-Core |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At RAN2#113e meeting, it’s agreed that CHO/CPC in LTE/5GC is not support in Rel-16. Some normative texts in section 10.8.2 and Annex B needs to be updated to preclude CHO in LTE/5GC. The CPC part on non-SRB3 and SRB3 case needs to be updated with some separate description and figures in section 10.3.1 for EN-DC case, as that in section 10.3.2. Some editorial changes are needed in section 10.3.2, 10.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The CPC part on non-SRB3 and SRB3 case is updated with some separate description and figures in section 10.3.1; - Simplify the sentence for CPC as “The procedure is not supported for NE-DC and NGEN-DC” and make some editorial changes in section 10.3.2; - Some editorial changes in section 10.6; - Remove “ng-eNB/” in NOTE 1a in section 10.8.2 to preclude CHO in LTE/5GC; - Update the NOTE 3 in Annex B to “NOTE 3: Only intra-RAT conditional handover is supported except for E-UTRA with 5GC scenario”. Impact analysis Impacted 5G architecture options: (NG)EN-DC, NR-DC Impacted functionality: CHO, CPC Inter-operability: If the network implements the CR and the UE does not, there is no inter-operability issue. If the UE implements the CR and the network does not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description in section 10.8.2 and Annex B may cause the confusion that CHO can be supported in LTE/5GC. The description of CPC for SRB3 and non-SRB3 case in section 10.3.1 is confusing and blended with the non-conditional reconfiguration. Figures are not depicting the actual steps described in the procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.1, 10.3.2, 10.6, 10.8.2, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE START

10.3	Secondary Node Modification (MN/SN initiated)

10.3.1	EN-DC

The Secondary Node Modification procedure may be initiated either by the MN or by the SN and be used to modify, establish or release bearer contexts, to transfer bearer contexts to and from the SN or to modify other properties of the UE context within the same SN. It may also be used to transfer an NR RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (e.g. when SRB3 is not used). In case of CPC, this procedure is used to configure or modify CPC configuration within the same SN.

The Secondary Node modification procedure does not necessarily need to involve signalling towards the UE.

MN initiated SN Modification

Figure 10.3.1-1: SN Modification procedure - MN initiated

The MN uses the procedure to initiate configuration changes of the SCG within the same SN, e.g. the addition, modification or release of SCG bearer(s) and the SCG RLC bearer of split bearer(s), as well as configuration changes for SN terminated MCG bearers. Bearer termination point change is realized by adding the new bearer configuration and releasing the old bearer configuration within a single MN initiated SN Modification procedure for the respective E-RAB. The MN uses this procedure to perform handover within the same MN while keeping the SN. The MN also uses the procedure to query the current SCG configuration, e.g. when delta configuration is applied in an MN initiated SN change. The MN also uses the procedure to provide the S-RLF related information to the SN. The MN may not use the procedure to initiate the addition, modification or release of SCG SCells. The SN may reject the request, except if it concerns the release of SN terminated bearer(s) or the SCG RLC bearer of MN terminated bearer(s), or if it is used to perform handover within the same MN while keeping the SN. Figure 10.3.1-1 shows an example signalling flow for an MN initiated SN Modification procedure.

1.	The MN sends the SgNB Modification Request message, which may contain bearer context related or other UE context related information, data forwarding address information (if applicable) and the requested SCG configuration information, including the UE capability coordination result to be used as basis for the reconfiguration by the SN. In case a security key update in the SN is required, a new SgNB Security Key is included. In case of SCG RLC re-establishment for E-RABs configured with an MN terminated bearer with an SCG RLC bearer for which no bearer type change is performed, the MN provides a new UL GTP tunnel endpoint to the SN. The SN shall continue sending UL PDCP PDUs to the MN with the previous UL GTP tunnel endpoint until it re-establishes the RLC and use the new UL GTP tunnel endpoint after re-establishment. In case of PDCP re-establishment for E-RABs configured with an SN terminated bearer with an MCG RLC bearer for which no bearer type change is performed, the MN provides a new DL GTP tunnel endpoint to the SN. The SN shall continue sending DL PDCP PDUs to the MN with the previous DL GTP tunnel endpoint until it performs PDCP re-establishment and use the new DL GTP tunnel endpoint starting with the PDCP re-establishment.

2.	The SN responds with the SgNB Modification Request Acknowledge message, which may contain SCG radio resource configuration information within a NR RRC configuration message and data forwarding address information (if applicable). In case of a security key update (with or without PSCell change), for E-RABs configured with the MN terminated bearer option that require X2-U resources between the MN and the SN, for which no bearer type change is performed, the SN provides a new DL GTP tunnel endpoint to the MN. The MN shall continue sending DL PDCP PDUs to the SN with the previous DL GTP tunnel endpoint until it performs PDCP re-establishment or PDCP data recovery, and use the new DL GTP tunnel endpoint starting with the PDCP re-establishment or data recovery. In case of a security key update (with or without PSCell change), for E-RABs configured with the SN terminated bearer option that require X2-U resources between the MN and the SN, for which no bearer type change is performed, the SN provides a new UL GTP tunnel endpoint to the MN. The MN shall continue sending UL PDCP PDUs to the SN with the previous UL GTP tunnel endpoint until it re-establishes the RLC and use the new UL GTP tunnel endpoint after re-establishment.

3-5.	The MN initiates the RRC connection reconfiguration procedure, including the NR RRC configuration message. The UE applies the new configuration, synchronizes to the MN (if instructed, in case of intra-MN handover) and replies with RRCConnectionReconfigurationComplete, including a NR RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SgNB Reconfiguration Complete message.

7.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SgNB addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN Status Transfer takes place between the MN and the SN (Figure 10.3.1-1 depicts the case where a bearer context is transferred from the MN to the SN).

NOTE 0:	The SN may not be aware that a SN terminated bearer requested to be released is reconfigured to a MN terminated bearer. The SN Status for the released SN terminated bearers with RLC AM may also be transferred to the MN.

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.1-1 depicts the case where a bearer context is transferred from the MN to the SN).

10.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the E-RABs to be released and for the E-RABs for which the S1 UL GTP Tunnel endpoint was requested to be modified.

NOTE 1:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

11.	If applicable, a path update is performed.

SN initiated SN Modification with MN involvement

Figure 10.3.1-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the release of SCG bearer(s) and the SCG RLC bearer of split bearer(s) (upon which the MN may release the bearer or maintain current bearer type or reconfigure it to an MCG bearer, either MN terminated or SN terminated), and to trigger PSCell change (e.g. when a new security key is required or when the MN needs to perform PDCP data recovery). The MN cannot reject the release request of SCG bearer and the SCG RLC bearer of a split bearer. Figure 10.3.1-2 shows an example signalling flow for an SN initiated SgNB Modification procedure, with MN involvement.

1.	The SN sends the SgNB Modification Required message including a NR RRC configuration message, which may contain bearer context related, other UE context related information and the new SCG radio resource configuration. For bearer release or modification, a corresponding E-RAB list is included in the SgNB Modification Required message. In case of change of security key, the PDCP Change Indication indicates that a S-KgNB update is required. In case the MN needs to perform PDCP data recovery, the PDCP Change Indication indicates that PDCP data recovery is required.

The SN can decide whether the change of security key is required.

2/3.	The MN initiated SN Modification procedure may be triggered by the SN Modification Required message (e.g. to provide information such as data forwarding addresses, new SN security key, measurement gap, etc...)

NOTE 2:	If only SN security key is provided in step 2, the MN does not need to wait for the reception of step 3 to initiate the RRC connection reconfiguration procedure.

4.	The MN sends the RRCConnectionReconfiguration message including a NR RRC configuration message to the UE including the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the RRCConnectionReconfigurationComplete message, including an encoded NR RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the RRCConnectionReconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SgNB Modification Confirm message containing the encoded NR RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN Status Transfer takes place between the MN and the SN (Figure 10.3.1-2 depicts the case where a bearer context is transferred from the SN to the MN).

NOTE 2a:	The SN may not be aware that a SN terminated bearer requesting to release is reconfigured to a MN terminated bearer. The SN Status for the released SN terminated bearers with RLC AM may also be transferred to the MN.

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.1-2 depicts the case where a bearer context is transferred from the SN to the MN).

10.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE over the NR radio for the E-RABs to be released.

NOTE 3:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related bearer is stopped.

11.	If applicable, a path update is performed.

SN initiated SN Modification without MN involvement

Figure 10.3.1-3: SN modification - SN initiated without MN involvement

The SN initiated modification without MN involved procedure is used to modify the configuration within SN in case no coordination with MN is required, including the addition/modification/release of SCG SCell and PSCell change (e.g. when the security key does not need to be changed and the MN does not need to be involved in PDCP recovery). The SN may initiate the procedure to configure or modify CPC configuration within the same SN. Figure 10.3.1-3 shows an example signalling flow for SN initiated SN modification procedure, without MN involvement. The SN can decide whether the Random Access procedure is required.

1.	The SN sends the RRCReconfiguration message to the UE through SRB3. The UE applies the new configuration. In case the UE is unable to comply with (part of) the configuration included in the RRCReconfiguration message, it performs the reconfiguration failure procedure.

2.	If instructed, the UE performs synchronisation towards the PSCell of the SN.

3.	The UE replies with the RRCReconfigurationComplete message.

SN initiated Conditional SN Modification (CPC) without MN involvement (SRB3 is used)

Figure 10.3.1-3a: SN Modification - SN-initiated without MN involvement and SRB3 is used to configure CPC.

The SN initiates the procedure when it needs to transfer an NR RRC message to the UE and SRB3 is used to configure CPC.

1.	The SN sends the RRCReconfiguration message including CPC configuration to the UE through SRB3.

2.	The UE applies the new configuration. The UE starts evaluating the CPC execution conditions for the candidate PSCell(s). The UE maintains connection with the source PSCell and replies with the RRCReconfigurationComplete message to the SN via SRB3.

3.	If at least one CPC candidate PSCell satisfies the corresponding CPC execution condition, the UE detaches from the source PSCell, applies the stored configuration corresponding to the selected candidate PSCell and synchronises to the candidate PSCell.

4.	The UE completes the CPC execution procedure by sending an RRCReconfigurationComplete message to the new PSCell.

Transfer of an NR RRC message to/from the UE (when SRB3 is not used)

Figure 10.3.1-4: Transfer of an NR RRC message to/from the UE

The SN initiates the procedure when it needs to transfer an NR RRC message to the UE and SRB3 is not used.

1.	The SN initiates the procedure by sending the SgNB Modification Required to the MN.

2.	The MN forwards the NR RRC message to the UE in the RRCConnectionReconfiguration message.

3.	The UE applies the new configuration and replies with the RRCConnectionReconfigurationComplete message.

4.	The MN forwards the NR RRC response message, if received from the UE, to the SN in the SgNB Modification Confirm message.

5.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SgNB Addition procedure. Otherwise the UE may perform UL transmission after having applied the new configuration.

SN initiated Conditional SN Modification (CPC) without MN involvement (SRB3 is not used)

Figure 10.3.1-4a: SN Modification - SN-initated without MN involvement and SRB3 is not used to configure CPC

The SN initiates the procedure when it needs to transfer an NR RRC message to the UE and SRB3 is not used to configure CPC.

1.	The SN initiates the procedure by sending the SgNB Modification Required to the MN including the SN RRC reconfiguration message with CPC configuration.

2.	The MN forwards the SN RRC reconfiguration message to the UE including it in the RRCConnectionReconfiguration message.

3.	The UE replies with the RRCConnectionReconfigurationComplete message by including the SN RRC reconfiguration complete message. The UE maintains connection with source PSCell after receiving CPC configuration, and starts evaluating the CPC execution conditions for the candidate PSCell(s).

4.	The MN forwards the SN RRC response message, if received from the UE, to the SN by including it in the SgNB Modification Confirm message.

5.	If at least one CPC candidate PSCell satisfies the corresponding CPC execution condition, the UE completes the CPC execution procedure by an ULInformationTransferMRDC message to the MN which includes an embedded RRCReconfigurationComplete message to the selected target PSCell.

6.	The RRCReconfigurationComplete is forwarded to the SN embedded in RRC Transfer.

7.	The UE detaches from the source PSCell, applies the stored corresponding configuration and synchronises to the selected candidate PSCell.

NEXT CHANGE

10.3.2	MR-DC with 5GC

The SN Modification procedure may be initiated either by the MN or by the SN and be used to modify the current user plane resource configuration (e.g. related to PDU session, QoS flow or DRB) or to modify other properties of the UE context within the same SN. It may also be used to transfer an RRC message from the SN to the UE via the MN and the response from the UE via MN to the SN (e.g. when SRB3 is not used). In NGEN-DC and NR-DC, the RRC message is an NR message (i.e., RRCReconfiguration) whereas in NE-DC it is an E-UTRA message (i.e., RRCConnectionReconfiguration). In case of CPC, this procedure is used to configure or modify CPC configuration within the same SN. The CPC configuration cannot be used to configure target PSCell in NE-DC or in NGEN-DC.

The SN modification procedure does not necessarily need to involve signalling towards the UE.

MN initiated SN Modification

Figure 10.3.2-1: SN Modification procedure - MN initiated

The MN uses the procedure to initiate configuration changes of the SCG within the same SN, including addition, modification or release of the user plane resource configuration. The MN uses this procedure to perform handover within the same MN while keeping the SN, when the SN needs to be involved (i.e. in NGEN-DC). The MN also uses the procedure to query the current SCG configuration, e.g. when delta configuration is applied in an MN initiated SN change. The MN also uses the procedure to provide the S-RLF related information to the SN or to provide additional available DRB IDs to be used for SN terminated bearers. The MN may not use the procedure to initiate the addition, modification or release of SCG SCells. The SN may reject the request, except if it concerns the release of the user plane resource configuration, or if it is used to perform handover within the same MN while keeping the SN. Figure 10.3.2-1 shows an example signalling flow for an MN initiated SN Modification procedure.

1.	The MN sends the SN Modification Request message, which may contain user plane resource configuration related or other UE context related information, PDU session level Network Slice info and the requested SCG configuration information, including the UE capabilities coordination result to be used as basis for the reconfiguration by the SN. In case a security key update in the SN is required, a new SN Security Key is included. In case the PDCP data recovery in the SN is required, the PDCP Change Indication is included which indicates that PDCP data recovery is required in SN.

2.	The SN responds with the SN Modification Request Acknowledge message, which may contain new SCG radio configuration information within an SN RRC reconfiguration message, and data forwarding address information (if applicable).

NOTE 1:	For MN terminated bearers to be setup for which PDCP duplication with CA is configured in NR SCG side, the MN allocates up to 4 separate Xn-U bearers and the SN provides a logical channel ID for primary or split secondary path to the MN.

For SN terminated bearers to be setup for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via an additional MN-initiated SN modification procedure.

2a.	When applicable, the MN provides data forwarding address information to the SN. For SN terminated bearers using MCG resources, the MN provides Xn-U DL TNL address information in the Xn-U Address Indication message.

3/4.	The MN initiates the RRC reconfiguration procedure, including an SN RRC reconfiguration message. The UE applies the new configuration, synchronizes to the MN (if instructed, in case of intra-MN handover) and replies with MN RRC reconfiguration complete message, including an SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

5.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Reconfiguration Complete message.

6.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN addition procedure. Otherwise, the UE may perform UL transmission after having applied the new configuration.

7.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN Status Transfer takes place between the MN and the SN (Figure 10.3.2-1 depicts the case where a bearer context is transferred from the MN to the SN).

8.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-1 depicts the case where a user plane resource configuration related context is transferred from the MN to the SN).

9.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 2:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

10.	If applicable, a PDU Session path update procedure is performed.

SN initiated SN Modification with MN involvement

Figure 10.3.2-2: SN Modification procedure - SN initiated with MN involvement

The SN uses the procedure to perform configuration changes of the SCG within the same SN, e.g. to trigger the modification/release of the user plane resource configuration and to trigger PSCell changes (e.g. when a new security key is required or when the MN needs to perform PDCP data recovery). The MN cannot reject the release request of PDU session/QoS flows. The SN also uses the procedure to request the MN to provide more DRB IDs to be used for SN terminated bearers or to return DRB IDs used for SN terminated bearers that are not needed any longer. Figure 10.3.2-2 shows an example signalling flow for SN initiated SN Modification procedure.

1.	The SN sends the SN Modification Required message including an SN RRC reconfiguration message, which may contain user plane resource configuration related context, other UE context related information and the new radio resource configuration of SCG. In case of change of security key, the PDCP Change Indication indicates that an SN security key update is required. In case the MN needs to perform PDCP data recovery, the PDCP Change Indication indicates that PDCP data recovery is required.

The SN can decide whether the change of security key is required.

2/3.	The MN initiated SN Modification procedure may be triggered by SN Modification Required message, e.g. when an SN security key change needs to be applied.

NOTE 3:	For SN terminated bearers to be setup for which PDCP duplication with CA is configured in NR MCG side, the SN allocates up to 4 separate Xn-U bearers and the MN provides a logical channel ID for primary or split secondary path to the SN via the nested MN-initiated SN modification procedure.

4.	The MN sends the MN RRC reconfiguration message to the UE including the SN RRC reconfiguration message with the new SCG radio resource configuration.

5.	The UE applies the new configuration and sends the MN RRC reconfiguration complete message, including an SN RRC response message, if needed. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

6.	Upon successful completion of the reconfiguration, the success of the procedure is indicated in the SN Modification Confirm message including the SN RRC response message, if received from the UE.

7.	If instructed, the UE performs synchronisation towards the PSCell configured by the SN as described in SN Addition procedure. Otherwise, the UE may perform UL transmission directly after having applied the new configuration.

8.	If PDCP termination point is changed for bearers using RLC AM, and when RRC full configuration is not used, the SN Status Transfer takes place between the MN and the SN (Figure 10.3.2-2 depicts the case where a bearer context is transferred from the SN to the MN).

9.	If applicable, data forwarding between MN and the SN takes place (Figure 10.3.2-2 depicts the case where a user plane resource configuration related context is transferred from the SN to the MN).

10.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 4:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

11.	If applicable, a PDU Session path update procedure is performed.

SN initiated SN Modification without MN involvement

This procedure is not supported for NE-DC.

Figure 10.3.2-3: SN Modification – SN initiated without MN involvement

The SN initiated SN modification procedure without MN involvement is used to modify the configuration within SN in case no coordination with MN is required, including the addition/modification/release of SCG SCell and PSCell change (e.g. when the security key does not need to be changed and the MN does not need to be involved in PDCP recovery). The SN may initiate the procedure to configure or modify CPC configuration within the same SN. Figure 10.3.2-3 shows an example signalling flow for SN initiated SN modification procedure without MN involvement. The SN can decide whether the Random Access procedure is required.

1.	The SN sends the SN RRC reconfiguration message to the UE through SRB3.

2.	The UE applies the new configuration and replies with the SN RRC reconfiguration complete message. In case the UE is unable to comply with (part of) the configuration included in the SN RRC reconfiguration message, it performs the reconfiguration failure procedure.

3.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN Addition procedure. Otherwise the UE may perform UL transmission after having applied the new configuration.

SN initiated Conditional SN Modification (CPC) without MN involvement (SRB3 is used)

This procedure is not supported for NE-DC and NGEN-DC.

Figure 10.3.2-3a: SN Modification – SN-initiated without MN involvement and SRB3 is used to configure CPC.

The SN initiates the procedure when it needs to transfer an NR RRC message to the UE and SRB3 is used to configure CPC.

1.	The SN sends the SN RRC reconfiguration including CPC configuration to the UE through SRB3.

2.	The UE applies the new configuration. The UE starts evaluating the CPC execution conditions for the candidate PSCell(s). The UE maintains connection with the source PSCell and replies with the RRCReconfigurationComplete message to the SN via SRB3.

3.	If at least one CPC candidate PSCell satisfies the corresponding CPC execution condition, the UE detaches from the source PSCell, applies the stored configuration corresponding to the selected candidate PSCell and synchronises to the candidate PSCell.

4.	The UE completes the CPC execution procedure by sending an RRCReconfigurationComplete message to the new PSCell.

Transfer of an NR RRC message to/from the UE (when SRB3 is not used)

This procedure is supported for all the MR-DC options.

Figure 10.3.2-4: Transfer of an NR RRC message to/from the UE

The SN initiates the procedure when it needs to transfer an NR RRC message to the UE and SRB3 is not used.

1.	The SN initiates the procedure by sending the SN Modification Required to the MN including the SN RRC reconfiguration message.

2.	The MN forwards the SN RRC reconfiguration message to the UE including it in the RRC reconfiguration message.

3.	The UE applies the new configuration and replies with the RRC reconfiguration complete message by including the SN RRC reconfiguration complete message.

4.	The MN forwards the SN RRC response message, if received from the UE, to the SN by including it in the SN Modification Confirm message.

5.	If instructed, the UE performs synchronisation towards the PSCell of the SN as described in SN Addition procedure. Otherwise the UE may perform UL transmission after having applied the new configuration.

SN initiated Conditional SN Modification (CPC) without MN involvement (SRB3 is not used)

This procedure is not supported for NE-DC and NGEN-DC.

Figure 10.3.2-4a: SN Modification – SN-initated without MN involvement and SRB3 is not used to configure CPC

The SN initiates the procedure when it needs to transfer an NR RRC message to the UE and SRB3 is not used to configure CPC.

1.	The SN initiates the procedure by sending the SN Modification Required to the MN including the SN RRC reconfiguration message with CPC configuration.

2.	The MN forwards the SN RRC reconfiguration message to the UE including it in the RRCReconfiguration message.

3.	The UE replies with the RRCReconfigurationComplete message by including the SN RRC reconfiguration complete message. The UE maintains connection with source PSCell after receiving CPC configuration, and starts evaluating the CPC execution conditions for the candidate PSCell(s).

4.	The MN forwards the SN RRC response message, if received from the UE, to the SN by including it in the SN Modification Confirm message.

5.	If at least one CPC candidate PSCell satisfies the corresponding CPC execution condition, the UE completes the CPC execution procedure by an ULInformationTransferMRDC message to the MN which includes an embedded RRCReconfigurationComplete message to the selected target PSCell.

6.	The RRCReconfigurationComplete is forwarded to the SN embedded in RRC Transfer.

7.	The UE detaches from the source PSCell, applies the stored corresponding configuration and synchronises to the selected candidate PSCell.

NEXT CHANGE

10.6	PSCell change

In MR-DC, a PSCell change does not always require a security key change.

If a security key change is required, this is performed through a synchronous SCG reconfiguration procedure towards the UE involving random access on PSCell and a security key change, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For SN terminated bearers, PDCP is re-established. In all MR-DC options, to perform this procedure within the same SN, the SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a S-KgNB (for EN-DC, NGEN-DC and NR-DC) or S-KeNB (for NE-DC) update is required when the procedure is initiated by the SN or including the SgNB Security Key / SN Security Key when the procedure is initiated by the MN. In all MR-DC options, to perform a PSCell change between different SN nodes, the SN Change procedure as described in clause 10.5 is used.

If a security key change is not required (only possible in EN-DC, NGEN-DC and NR-DC), this is performed through a synchronous SCG reconfiguration procedure without security key change towards the UE involving random access on PSCell, during which the MAC entity configured for SCG is reset and RLC configured for SCG is re-established regardless of the bearer type(s) established on SCG. For bearers using RLC AM mode PDCP data recovery applies, for bearers using RLC UM no action is performed in PDCP while for SRBs PDCP discards all stored SDUs and PDUs. Unless MN terminated SCG or split bearers are configured, this does not require MN involvement. In this case, if location information was requested for the UE, the SN informs the MN about the PSCell change (as part of location information) using the SN initiated SN modification procedure independently from the reconfiguration of the UE. In case of MN terminated SCG or split bearers, the SN initiated SN Modification procedure as described in clause 10.3 is used, setting the PDCP Change Indication to indicate that a PDCP data recovery is required.

A Conditional PSCell Change (CPC) is defined as a PSCell change that is executed by the UE when execution condition(s) is met. The UE starts evaluating the execution condition(s) upon receiving the CPC configuration, and stops evaluating the execution condition(s) once PSCell change is triggered. Only intra-SN CPC without MN involvement is supported.

The following principles apply to CPC:

-	The CPC configuration contains the configuration of CPC candidate PSCell(s) and execution condition(s) generated by the SN.

-	An execution condition may consist of one or two trigger condition(s) (CPC events A3/A5, as defined in TS 38.331[4]). Only single RS type is supported and at most two different trigger quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously for the evalution of CPC execution condition of a single candidate PSCell.

-	Before any CPC execution condition is satisfied, upon reception of PSCell change command or PCell change command, the UE executes the PSCell change procedure as described in clause 10.3 and 10.5 or the PCell change procedure as described in clause 9.2.3.2 in TS 38.300[3] or clause 10.1.2.1 in TS 36.300 [2], regardless of any previously received CPC configuration. Upon the successful completion of PSCell change procedure or PCell change procedure, the UE releases all stored CPC configurations.

-	While executing CPC, the UE is not required to continue evaluating the execution condition of other candidate PSCell(s).

-	Once the CPC procedure is executed successfully, the UE releases all stored CPC configurations.

-	Upon the release of SCG, the UE releases the stored CPC configurations.

CPC configuration in HO command, PSCell change command or conditional reconfiguration is not supported.

NEXT CHANGE

10.8.2	MR-DC with 5GC

The MN to ng-eNB/gNB Change procedure is used to transfer UE context data from a source MN/SN to a target ng-eNB/gNB. Both the cases where the source MN and the target node belong to the same RAT (i.e. they are both ng-eNBs or both gNBs) and the cases where the source MN and the target node belong to different RATs are supported.

NOTE 0:	Inter-system HO from ng-eNB/gNB MN to eNB is also supported.

Figure 10.8.2-1: MN to ng-eNB/gNB Change procedure

Figure 10.8.2-1 shows an example signalling flow for the MN to ng-eNB/gNB Change procedure:

1.	The source MN starts the MN to ng-eNB/gNB Change procedure by initiating the Xn Handover Preparation procedure, including both MCG and SCG configuration.

NOTE 1:	The source MN may trigger the MN-initiated SN Modification procedure (to the source SN) to retrieve the current SCG configuration and to allow provision of data forwarding related information before step 1.

2.	The target ng-eNB/gNB includes the field in HO command which releases the SCG configuration, and may also provide forwarding addresses to the source MN.

3.	If the resource allocation of target ng-eNB/gNB was successful, the MN initiates the release of the source SN resources towards the source SN including a Cause indicating MCG mobility. The SN acknowledges the release request. If data forwarding is needed, the MN provides data forwarding addresses to the source SN. Reception of the SN Release Request message triggers the source SN to stop providing user data to the UE and, if applicable, to start data forwarding.

NOTE 1a:	In case the handover is a conditional handover, step 3a and step 3b are performed after the source MN receives an indication that the UE has successfully accessed one of the potential target gNB as described in TS 38.300 [3], (i.e. after step 6)

NOTE 1b:	In case the handover is a conditional handover, the step 3c is executed right after step 2. The Xn-U Address Indication message notifies conditional handover to the source SN, for which it may decide to perform, if applicable, early data forwarding for SN-terminated bearers, together with the sending of an EARLY STATUS TRANSFER message to the source MN. If applicable, the normal data forwarding and SN STATUS TRANSFER message would follow from the source SN once it receives the SN Release Request message of the step 3a that is performed after step 6. In case the step 3c Xn-U Address Indication procedure is rejected by the source SN, the source MN re-sends it after the step 3b that is performed after step 6.

4.	The MN triggers the UE to perform HO and apply the new configuration. Upon receiving the new configuration, the UE releases the entire SCG configuration.

5/6.	The UE synchronizes to the target ng-eNB/gNB.

7.	If PDCP termination point is changed for bearers using RLC AM, the SN sends the SN Status Transfer, which the source MN sends then to the target ng-eNB/gNB.

8.	If applicable, data forwarding takes place from the source side.

9a.	The source SN sends the Secondary RAT Data Usage Report message to the source MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 2:	The order the SN sends the Secondary RAT Data Usage Report message and performs data forwarding with MN is not defined. The SN may send the report when the transmission of the related QoS flow is stopped.

9b.	The source MN sends the Secondary RAT Report message to AMF to provide information on the used NR/E-UTRA resource.

10-14.	The target ng-eNB/gNB initiates the Path Switch procedure.

15.	The target ng-eNB/gNB initiates the UE Context Release procedure towards the source MN.

16.	Upon reception of the UE Context Release message from the MN, the source SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

NEXT CHANGE

Annex B (informative):
Supported MR-DC Handover Scenarios

Table B-1 summarizes the supported handover scenarios involving MR-DC configurations.

Table B-1: Supported MR-DC handover scenarios.

| To (column) HO from (row) | E-UTRA with EPC | E-UTRA with 5GC | NR | GERAN or UTRAN | EN-DC | NGEN-DC | NE-DC | NR-DC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA with EPC | YES | YES | YES | YES | YES | NO | NO | NO |
| E-UTRA with 5GC | YES | YES | YES | NO | NO | YES | NO | NO |
| NR | YES | YES | YES | NOTE 1 | YES | NO | YES | YES |
| GERAN or UTRAN | YES | NO | NO | YES | NO | NO | NO | NO |
| EN-DC | YES | YES | YES | YES | YES | NO | NO | NO |
| NGEN-DC | YES | YES | YES | NO | NO | YES | NO | NO |
| NE-DC | YES | YES | YES | NOTE 1 | NO | NO | YES | NO |
| NR-DC | YES | YES | YES | NOTE 1 | NO | NO | NO | YES |

NOTE 1:	Only SRVCC handover of IMS voice bearer to UTRAN is supported.

NOTE 2:	All handover scenarios according to Table B-1 that have a DC option in the column "from" are supported during fast MCG failure recovery.

NOTE 3:	Only intra-RAT conditional handover is supported except for E-UTRA with 5GC scenario.

NOTE 4:	DAPS handover is only supported from E-UTRA with EPC to E-UTRA with EPC and from NR to NR.

CHANGE END