3GPP TSG-RAN WG2 Meeting #114 Electronic	R2-2105878

Elbonia, 19 – 27 May 2021

Agenda item:	8.6.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Details of RACH specific schemes

WID/SID:	NR_SmallData_INACTIVE - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In the previous RAN2-113Bis-e meeting, the following points were left FFS:

11 UE switches from SDT to non-SDT in following cases: - Case 1 (27/0): UE receive indication from network to switch to non-SDT procedure. - Network can send RRCResume. FFS whether network can send indication in RAR/fallbackRAR/DCI to switch to non-SDT procedure. - FFS Case 2 (18/9): Initial UL transmission (in msgA/Msg3/CG resources) fails configured number of times

In this TDoc, we discuss these issues from RA-SDT point of view.

# 2	Discussion

First of all, the Case 2 in the above FFS where initial UL transmission fails configured number of times naturally shall be supported for RA-SDT which can rely on the legacy means in the RA procedure, ie., PREAMBLE_TRANSMISSION_COUNTER hits preambleTransMax +1 based on which Random Access problem is indicated to upper layers by MAC.

Proposal 1: Support switching from SDT to non-SDT procedure for RA-SDT if initial UL transmission fails configured number of times (ie., PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax +1).

Secondly, about the other FFS point, ie., whether network can send indication in RAR/fallbackRAR to switch to non-SDT procedure, it seems to become an issue in case the UE is re-transmitting Msg1 or MsgA when either Msg3 or MsgA, respectively, was previously transmitted. In such a case, the UE built the Msg3/MsgA already and has such MAC PDU stored in the buffer. However, if the UE switches to normal RA procedure, it would seem impossible to avoid different size of Msg3/MsgA grants between SDT and non-SDT in which case the rebuilding might be needed. On the other hand, as the backoff mechanism is supported for both 2-step RACH and 4-step RACH, it seems the loading of the RA-SDT resources could be controlled by legacy means. Only meaningful case to consider could be to allow NW to switch number UEs from 2-step RACH to 4-step RACH in case the 2-step RACH for SDT gets loaded.

Proposal 2: Indication to switch from SDT to non-SDT over RAR/fallbackRAR is not supported.

Proposal 3: Discuss if the NW should be able to direct number of UEs from 2-step RACH to 4-step RACH in case the 2-step RACH for SDT is loaded.

# 3	Conclusion

Proposal 1: Support switching from SDT to non-SDT procedure for RA-SDT if initial UL transmission fails configured number of times (ie., PREAMBLE_TRANSMISSION_COUNTER = preambleTransMax +1).

Proposal 2: Indication to switch from SDT to non-SDT over RAR/fallbackRAR is not supported.

Proposal 3: Discuss if the NW should be able to direct number of UEs from 2-step RACH to 4-step RACH in case the 2-step RACH for SDT is loaded.