3GPP TSG-RAN WG2 Meeting #114-e	R2-2105947

# Electronic Meeting, 19th May – 27th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 2655 | rev | - | Current version: | 16.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Abortion of RRC connection resume procedure by upper layers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI16 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Similar to abortion of RRC Connection establishment, UE shall abort the RRC Connectioon Resume procedure at indication by upper (NAS) layer, and remain in RRC_Inactive state. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduced new section for upper layer abortion of RRC Connection Resume procedure. According to the procedure text, UE MAC is reset and UE will hence not receive a potential RRC Resume message after the resume procedure is aborted. Impact Analysis Impacted 5G architecture options: NR SA, NE-DC,NR-DC Impacted functionality: RRC Connection resume Inter-operability: There are no interoperability issue, since CR only impact UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE might enter RRC_Connected, but the trigger for entering RRC_Connected no longer remains. Eventually, network will release the RRC Connection. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.13.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### 5.3.13.x	Abortion of RRC connection resume

If upper layers abort the RRC connection resume procedure, due to a NAS procedure being aborted as specified in TS 24.501 [23], while the UE has not yet entered RRC_CONNECTED, the UE shall:

1>	stop timer T319, if running;

1>	reset MAC, release the MAC configuration and re-establish RLC for all RBs that are established;

END OF CHANGE