s3GPP TSG-RAN WG2 Meeting #114 electronic	               _R2-2106522

Online, May 19 – May 27, 2021

Agenda Item:	8.12.2.2

Source: 	Huawei, HiSilicon

Title:	 Summary of [AT114-e][106][RedCap] Identification and access restrictions

Document for:	Discussion and Decision

# Introduction

This paper aims at capturing the summary of offline discussion.

- [AT114-e][106][RedCap] Identification and access restrictions (Huawei)

Initial scope: Discuss the proposals from R2-2106487

Initial intended outcome: Summary of the offline discussion with e.g.:

    - List of proposals for agreement (if any)

    - List of proposals that require online discussions

    - List of proposals that should not be pursued (if any)

Initial deadline (for companies' feedback): Thursday 2021-05-20 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2106522): Thursday 2021-05-20 09:00 UTC

# Open issue to discuss

- 2.1: Early identification

- 2.1.1 Msg1 early identification

Issue A: whether there is need to support Msg1 early identification

The proposal in R2-2106487 is

Draft_Proposal 1: [To discuss][12/17] Msg1 early identification is needed from RAN2 perspective.

Option 1: Yes, it is needed from R2 perspective (with detailed solution as FFS)

Option 2: No, it is not needed from R2 perspective

Option 3: Just let RAN1 to discuss this

Question 1: Which option do you prefer on the need of Msg1 early identification from RAN2 perspective?

| Companies | Preferred option? | Comments |
| --- | --- | --- |
| OPPO | Option 3 |  |
| Ericsson | Option 2/3 | There is no need from RAN2 perspective so RAN1 should indicate whether such is needed. In our understanding there are some deployments where such can be beneficial. We are OK to have this as an optional, configurable, indication. |
| Apple | Option 3/1 | If RAN2 wants to identiy RedCap UE at RACH, then we see this as option -1. Also differentiation based on Rx ports can be useful (but this is also RAN1 related). |
| Qualcomm | Option 1 | As discussed during the SI phase, there are multiple use cases for “early identification during Msg1”. Some of them, e.g. coverage enhancements for Msg2/3 transmission, maybe should be discussed by RAN1. But there are also issues such as initial BWP configuration and PRACH configuration that clearly should be discussed by RAN2. |
| MediaTek | 3 (and not needed from R2 perspective) | The reasons for an early indication are R1 centric (BWP operation, coverage recovery and so on). We prefer to let R1 discuss this and conclude on the need. If a need is identified by R1, R2 can introduce this indication in msg1 or msg3 as required. |
| Futurewei | Option 3 |  |
| Sequans | Option 3 |  |
| Intel | Option 1 | The main motivation for early identification is for selection of proper PDSCH MCS/PDCCH ALs (link adaptation) for non-RedCap UEs instead of always using the most conservative scheduling assuming that any UE is a RedCap UE. The impact due to conservative scheduling for non-RedCap UEs can be significant (increased system OH, increased CP latency for non-RedCap UEs, etc.) since the link performance gap between a non-RedCap and RedCap UE in the DL can even be ~6 dB (4Rx vs. 1Rx in certain FR1 “TDD bands”). Thus, with early identification during Msg1, the benefits can be realized for Msg2 PDCCH/PDSCH, PDCCH for Msg3 reTx, and Msg4 PDCCH/PDSCH as well as for UL transmissions (Msg3, PUCCH-for-Msg4, Msg5). |
| China Telecom | Option 1/3 | If early identification of Redcap UE is needed, we prefer as option 1. But it is better to let RAN1 discuss. |
| Samsung | Option 3 |  |
| Lenovo | Option.3 | Similar view as MTK, R1 is discussing this early indication, leave it to be determined by RAN1. |
| LGE | Option 3 | Same comment as MediaTek |
| Huawei, HiSilicon | Option 1/3 |  |
| Sierra Wireless | Option1 |  |
| NEC | Option 3 | We have the same understanding as Intel above but consider it as RAN1 perspective (although scheduling is in MAC). |
| ZTE | Option 2 and Option 3 | The necessities of Msg 1 based early identification captured in TR are RAN1 related issues. We don’t think it is needed from RAN2 point of view. We can wait for RAN1 decision. |
| Sharp | Option 3 |  |
| Fujitsu | Option 3 | We think it’s depending on RAN1. As discussed by RAN1, Msg1 early identification of RedCap UEs has been accepted for 4-step RACH and the identification in Msg1 can be configured to be enabled/disabled. |
| DENSO | Option 1/3 | The early indication by Msg.1 or Msg.3 can be used for compensating the coverage loss and the BWP operation, as commented by the other companies. The BWP aspects are relevant to RAN2 specs, as well as RAN1. Nonetheless, it can be up to RAN1 to make the final decision. |
| Spreadtrum | Option 3 |  |
| CATT | Option 1/3 |  |
| BT | Option1/3 |  |
| Nokia, Nokia Shanghai Bell | Option 1/3 | It is our understanding the WID already requires specifying such support which is optionally configurable by the NW: Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1] |

- 2.1.2 Msg3 early identification

Issue A: whether there is need to support Msg3 early identification

The proposal in R2-2106487 is

Draft_Proposal 2: [To discuss][9/14] RAN2 to discuss the need of Msg3 early identification from RAN2 perspective (in case Msg1 early identification is optionally configured or not supported).

Option 1: Yes, it is needed from R2 perspective, in case Msg1 early identification is optionally configured or not supported (with detailed solution as FFS)

Option 2: No, it is not needed from R2 perspective

Option 3: Just let RAN1 to discuss this

Question 2: Which option do you prefer on the need of Msg3 early identification from RAN2 perspective?

| Companies | Preferred option? | Comments |
| --- | --- | --- |
| OPPO | Option 2 | Since SA1 don’t agree on the need to differentiate access control for RedCap UEs and non-RedCap UEs, we don’t think Msg3 identification is needed. |
| Ericsson | Option 1 | Prefer to have Msg3 indication, e.g. when Msg1 indication is not configured. |
| Apple | Option 2 | No need to differentiatiate, similar view as Oppo. |
| Qualcomm | Option 1 | In our view, if Msg1 indication is not configured, early identification in Msg3 is needed for subscription validation. |
| MediaTek | 3 (and not needed from R2 perspective) | Similar to Q1, we prefer to let R1 determine the need for such an indication. If the need is identified by R1, R2 can introduce necessary signaling for this. |
| Futurewei | Option 3 |  |
| Sequans | Option 3 |  |
| Intel | Option 2 | MSG 3 early identification can only cover limited scenario, and therefore it is not needed. |
| China Telecom | Option 1 | When Msg1 indication is not configured, it can be optionally accepted. |
| Samsung | Option 3 |  |
| Lenovo | Option 3 | Same view as Q1, R1 is discussing this early indication, leave it to be determined by RAN1. |
| LGE | Option 3 |  |
| Huawei, HiSilicon | Option 1 |  |
| Sierra Wireless | Option 1 |  |
| NEC | Option 2 | From RAN2 point of view, the purpose of this Msg3-based is to let gNB to reject the request only from RedCap. However we do not think it is really necessary. |
| ZTE | Option 2 and Option 3 | We don’t think Msg3 identification is needed from RAN2 perspective. However, in case Msg1 identification is not configured or not supported, it may be needed if physical layer configuration in Msg 4 for RedCap UE should be different from non-RedCap UE. Thus we prefer to let RAN1 to discuss it. |
| Sharp | Option 1 |  |
| Fujitsu | Option 1 | If Msg1 indication is not configured, early identification in Msg3 is needed to inform gNB the UE is RedCap type so as for gNB to control the access of UE. |
| DENSO | Option 1 | Early indication via Msg.3 cannot cover the case that the coverage extension is required from Msg.2. On the other hand, the Msg.3 based early indication can be used to determine the bandwidth of the initial BWP, especially for Option 1 of the BWP operation, as defined in TS 38.331, Annex B2 (i.e. the bandwidth of BWP #1). Of course, if the Msg.1 indication is supported, both the potential usages (i.e. coverage extension and BWP operations) can be supported. The necessity of the Msg.3 based indication can be considered if RAN1 don’t agree on the Msg.1 based indication. If the Msg.1 based indication is supported, it is sufficient to support the potential usages. |
| Spreadtrum | Option 2 |  |
| CATT | Option 3 |  |
| BT | Option 1 |  |
| Nokia, Nokia Shanghai Bell | Option 1 | It is our understanding the WID already requires specifying such support and NW can optionally configure also the Msg1 indication: Specify functionality that will enable RedCap UEs to be explicitly identifiable to networks through an early indication in Msg1 and/or Msg3, and Msg A if supported, including the ability for the early indication to be configurable by the network. [RAN2, RAN1] |

- 2.1.2 Rx braches specific early identification

Issue A: whether there is a need for Rx branches specific early identification

The proposal in R2-2106487 is

Draft_Proposal 3: There is no need to support Rx branches specific early identification from RAN2 perceptive.

Option 1: Yes, early identification of RedCap UE’s Rx capabilities

Option 2: No, only early identification of RedCap UE (not to differentiate 1Rx and 2Rx)

Option 3: Just let RAN1 to discuss this.

Question 3: Which option do you prefer for Rx branches specific early identification?

| Companies | Preferred option? | Comments |
| --- | --- | --- |
| OPPO | Option 3 |  |
| Ericsson | Option 2 |  |
| Apple | Option 3 |  |
| Qualcomm | Option 2 | Binary indication of RedCap is sufficient for initial access. We are also fine with leaving its discussion to RAN1. |
| MediaTek | 2 | While this is again a R1 centric discussion, we do not see a need for an early indication of UE’s RX capabilities at this point. This information will anyways be provided to the NW at UE capability exchange. |
| Futurewei | Option 2 |  |
| Sequans | Option 2/3 | Option 2 should be agreed from RAN2 perspective |
| Intel | Option 2 | Do not see the need to have Rx branches specific early identification from RAN2 perspective. |
| China Telecom | Option 2 | We think there is no need for an early indication of UE’s RX capabilities. |
| Samsung | Option 3 |  |
| Lenovo | Option 2/3 |  |
| LGE | Option 3 |  |
| Huawei, HiSilicon | Option 2/3 |  |
| Sierra Wireless | Option 1 | If the number of Rx branches is known by the network, then the network can differentiate based on this. However we are also fine with leaving it for RAN1. |
| NEC | Option 2 | The need of differentiation of 1 Rx and 2 Rx is coming from the request from some operators in RANP who do not support 1 Rx RedCap at all. |
| ZTE | Option 3 |  |
| Sharp | Option 2 |  |
| Fujitsu | Option 2/3 | We think Rx number is not necessary from RAN2 perspective, but it can depends on RAN1 decision. |
| DENSO | Option 3 | The Rx branch specific early indication is relevant to the coverage recovery, and so it is O.K to let RAN1 decide the necessity. |
| Spreadtrum | Option 2 |  |
| CATT | Option 3 |  |
| BT | Option 1 or 3 |  |
| Nokia, Nokia Shanghai Bell | Option 3 |  |

- 2.2: Access and Camping Restriction

- 2.2.1 Cell barring

Issue A: Cell barring specific to the number of Rx branches of the UE

The proposal in R2-2106487 is

Draft_Proposal 4: [Easy][17/18] SIB1 indicates cell barring for 1 Rx branch and 2 Rx branches separately for RedCap UEs.

The proposal above is aligned with WID objective and clear majority view. Rapporteur would like to ask if this proposal is agreeable. Silence means agreement in this case.

Question 4: Do you agree above draft proposal 4? If not, please indicate whether you have any strong concern on the proposal.

| Companies | Agree or not? | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Ericsson | Agree |  |
| Apple | agree |  |
| Qualcomm | Agree |  |
| MediaTek | Agree | This is aligned with the WID objective |
| Futurewei | Agree |  |
| Sequans | Agree |  |
| Intel | Agree |  |
| China Telecom | Agree |  |
| Samsung | Disagree | Even though we are quite minority, we would like to understand what this actually means. As described in our paper R2-2105472, to indicate separate bits has no meaning since to acquire this bit already means that UE is able to receive SIB1 (e.g. with the 1RX branch). If UE is able to receive SIB1 (which means that the signal strength is good enough), then no reason to block those UEs as network should be able to accommodate those UE. We agree that network has to send some additional parameters in SIB1/2/4 (e.g. Qrxlevmin/Qqualmin) for UE to decide whether to camp on the cell, but it is a bit weird to us that network that supports RedCap UEs bars the access of 1RX branch UE even if the signal strnegh is good enough. |
| Lenovo | Agree |  |
| LGE | Agree |  |
| Huawei, HiSilicon | Agree | For Samsung’s comments, the cell barring is not to check if UE is able to access, but is to indicate if UE is allowed to access. This is more like operator’s flexibility. |
| Sierra Wireless | Agree |  |
| NEC | Agree |  |
| ZTE | Agree |  |
| Sharp | Agree |  |
| Fujitsu | Agree |  |
| DENSO | Agree |  |
| Spreadtrum | Agree |  |
| CATT | Agree |  |
| BT | Agree |  |
| Nokia, Nokia Shanghai Bell | Agree | This is what WID requires us to specify. Additionally, we shall specify barring/allowance per frequency for REDCAP UE as per the WID. |

Issue B: whether cell barring for RedCap UE is per PLMN

The issue is discussed in R2-2105161.

Companies propose that the cell barring for RedCap UE can be per PLMN [ZTE].

Rapporteur understands the baseline is cell specifci cell barrring. Therefore, below quesiton is asked to check if companies want to enhance this for RedCap as per PLMN cell barring.

Question 5: Do you prefer the cell barring is per cell or per PLMN for RedCap?

| Companies | Per cell, or per PLMN? | Comments |
| --- | --- | --- |
| OPPO | Per cell | Stick to the baseline where current cell barring is cell specific. Don’ t see the need for optimization. |
| Ericsson | Per cell | Also prefer baseline as OPPO explains. |
| Apple | Per cell |  |
| Qualcomm | Per cell | Cell barring for RedCap UEs could depend on cell loading. So per-cell barring makes more sense. |
| MediaTek | Per cell | Agree with Oppo |
| Futurewei | Per cell |  |
| Sequans | Per cell | Agree with QC. No need to optimize further |
| Intel | Per cell | We assume ZTE’s proposal aims to clarify whether cell barring for RedCap UEs is enabled per cell and/or per PLMN in a given cell. We agree per cell should be baseline. FFS on per cell per PLMN. |
| China Telecom | Per cell |  |
| Samsung | Per cell |  |
| Lenovo | Per cell | The cell barring per cell is the baseline. Whether it is per PLMN can be further discussion. |
| LGE | Per cell |  |
| Huawei, HiSilicon | Per cell |  |
| NEC | Per cell |  |
| ZTE | Per PLMN | When deciding whether to bar 1 Rx and/or 2 Rx RedCap UE, besides network capability/deployment, the impact on resource efficiency may also be taken into consideration. If different operators have different preference in barring 1 Rx and/or 2 Rx RedCap UE. Then per PLMN configuration provides this flexibility. |
| Sharp | Per cell |  |
| Fujitsu | Per cell |  |
| DENSO | Per cell | Agree that the per cell barring is a baseline. There might be a potential scenario of per PLMN in the network sharing. However, such a demand should come from operators. |
| Spreadtrum | Per cell | Agree with QC. |
| CATT | Per cell |  |
| BT | Per cell |  |
| Nokia, Nokia Shanghai Bell | Both |  |

Issue C: whether to ignore the cellbarred in MIB

The proposal in R2-2106487 is

Draft_Proposal 6: [To discuss] RAN2 to discuss whether RedCap UE should ignore or apply the legacy cellbarred in MIB.

Option 1: RedCap UE ingores the cellBarred in MIB (i.e. supporting the case of barring non-RedCap UE but allowing RedCap UE to camp)

Option 2: RedCap UE applies the cellBarred in MIB (i.e. not supporting the case of barring non-RedCap UE but allowing RedCap UE to camp)

Rapporteur understands this depends on whether RAN2 consider the case of barring non-RedCap UE but allowing RedCap UE to camp as valid.

Question 6: Do you prefer whether RedCap UEs ignore or apply the legacy cellbarred in MIB (in addition to its RedCap specific indication)?

| Companies | Ignore, or apply? | Comments |
| --- | --- | --- |
| OPPO | Ignore | This is most flexible from network control’s perspective. |
| Ericsson | Apply | The question is whether we want to support “RedCap-only” cells. We don’t see this necessary at the moment but can discuss further. |
| Apple | Apply | Same view as Ericsson. |
| Qualcomm | Ignore | Option 1 supports all four cases (only non-RedCap is barred, only RedCap is barred, both are barred, no barring for both). Option 2 does not support the case only non-RedCap is barred. |
| MediaTek | Ignore | This option is more future proof, as it allows the case of barring non-RedCap UEs while allowing RedCap UEs to camp onto the cell. |
| Futurewei | No strong view | Agree with rapporteur that it all depends on if we can establish valid use cases for “RedCap-only” cell. |
| Sequans | Ignore |  |
| Intel | Ignore |  |
| China Telecom | Ignore | The option is more flexible. |
| Samsung | Apply | As described in our paper R2-2105472. |
| Lenovo | Ignore | Option 1. offers more flexibility to control the access from the view of system. |
| LGE | Ignore |  |
| Huawei, HiSilicon | Ignore | We want to give the NW more flexibility. |
| Sierra Wireless | Ignore |  |
| NEC | No strong view | at this moment, we are open to discuss further the need of such use case |
| ZTE | Ignore | This allows operator to deploy RedCap dedicated cell, e.g. for industrial and video Surveillance use cases. |
| Sharp | No strong view | For “RedCap-only” cell (if supported), the cellbarring in MIB should be ignored. For “both RedCap and non-RedCap” cell, applying the cellbarring in MIB allows the RedCap UEs not to read SIB1 if the cellbarring in MIB is set. |
| Fujitsu | Ignore | We support to have separate camping restriction for non-RedCap UE and RedCap UE. |
| DENSO | Apply | If the existing field of cellBarred in MIB is applied for RedCap UEs, it can be used for the NW to bar all of the UEs, no matter if the UE is RedCap UEs or not. It is not clear if there is a NPN/CSG-like use case for RedCap UEs. Such a scenario could be supported by utilising the NPN functionality to RedCap UEs, and could be discussed in later releases. |
| Spreadtrum | Ignore |  |
| CATT | Ignore |  |
| BT | Apply | At this stage, we don’t see the use case to support RedCap-only cells. |
| Nokia, Nokia Shanghai Bell | Ignore | NW can anyway set the RedCap specific barring bits according to cellBarred bit. |

- 2.2.2 IFRI

Issue A: whether to introduce RedCap specific IFRI

The proposal in R2-2106487 is

Draft_Proposal 7a: [Easy] RedCap UE supports the Intra Frequency Reselection Indicator.

Draft_Proposal 7b: [To discuss] RAN2 to discuss whether RedCap UEs reuse the legacy IFRI in MIB or use new RedCap specific IFRI in SIB1.

Rapporteur understands there is no clear majority view on the solution to support IFRI for RedCap UE in the contributions. But, it seems companies indeed support the application of IFRI to RedCap UE. Therefore, rapporteur would like to ask if companies are fine to agree on the high level P7a.

Question 7: do you agree above draft proposal 7a (i.e. RedCap UE supports the Intra Frequency Reselection Indicator)? If not, please indicate whether you have any strong concern on the proposal.

| Companies | Agree or not? | Comments |
| --- | --- | --- |
| OPPO | Agree |  |
| Ericsson | Agree |  |
| Apple | Agree to 7a |  |
| Qualcomm | Agree |  |
| MediaTek | Agree |  |
| Futurewei | Agree |  |
| Sequans | Agree |  |
| Intel | Agree |  |
| China Telecom | Agree |  |
| Samsung | Agree |  |
| Lenovo | Agree |  |
| LGE | Agree |  |
| Huawei, HiSilicon | Agree |  |
| NEC | Agree |  |
| ZTE | Agree |  |
| Sharp | Agree |  |
| Fujitsu | Agree |  |
| DENSO | Agree |  |
| Spreadtrum | Agree |  |
| CATT | Agree |  |
| BT | Agree |  |
| Nokia, Nokia Shanghai Bell | Agree | Additionally, WID requires such indication to be per Rx branch (ie., separate for 1Rx and 2Rx REDCAP UEs): Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1] |

As for the support of Intra Freq Reselection indicator for RedCap UE, companies propose the following options:

Option 1: Reuse the legacy IFRI in MIB (i.e. no RedCap specific IFRI)

Option 2: Introduce RedCap specific IFRI in SIB1

- Option 2a: not to differentiate 1Rx and 2Rx

- Option 2b: specific to the number of Rx branches

Question 8: Which option do you prefer on Intra Freq Reselection Indicator for RedCap UE?

| Companies | Preferred option | Comments |
| --- | --- | --- |
| OPPO | Option 2 | Whether 2a or 2b can be further discussed. |
| Ericsson | Option 1 | We don’t currently see use case to differentiate this between RedCap and non-RedCap. |
| Apple | Option 1 |  |
| Qualcomm | Option 2 | If Rx branch specific barring is supported, the same reasoning applies to IFRI (i.e. Option 2b). |
| MediaTek | 2a | In case all cells in the frequency do not support RedCap operation, it is useful to indicate this information to the UE to avoid unnecessarily searching for cells that do not support RedCap on the frequency. |
| Futurewei | Option 1 |  |
| Sequans | Option 2b | This seems linked to both Q4 (Rx branch barring) and Q6, as a similar reasoning applies. If we are not consistent there should be a good reason. |
| Intel | Option 1 | Normally the cells in the same frequency should be upgraded simultaneously, and therefore legacy IFRI can be reused; |
| China Telecom | Option 2 | In case there is a cell do not support both Redcap UE and non-Redcap UE , introduce RedCap specific IFRI in SIB1 is needed. |
| Samsung | Option 1 |  |
| Lenovo | Option 1 |  |
| LGE | Option 2b |  |
| Huawei, HiSilicon | No strong view |  |
| NEC | Option 2 | we prefer option 2a, as differentiation of Rx branches is coming from the request from some operators who do not want to support 1 Rx at all and thus no need to have separate IFRI. |
| ZTE | Option 1 |  |
| Sharp | Option 1 |  |
| Fujitsu | Option 2 | We prefer to introduce RedCap specific IFRI in SIB1, so that non-RedCap UEs and RedCap UEs may reselect different cells on one frequency. |
| DENSO | Option 2a | The policy of setting the value of intraFreqReselection could be different between RedCap UEs and the others. The necessity of further discrimination is questionable, unless there is the difference of supporting frequency bands between 1Rx UEs and 2 Rx UEs. |
| CATT | Option 2 | 2a or 2b can be further discussed. |
| Nokia, Nokia Shanghai Bell | Option 2b | This is only option according to the WID requirements: Specify a system information indication to indicate whether a RedCap UE can camp on the cell/frequency or not; it shall be possible for the indication to be specific to the number of Rx branches of the UE. [RAN2, RAN1] |

- 2.2.3 Neighbor cell RedCap support indication

Issue A: For cell reselection

The issue is discussed in R2-2104775, R2-2105137, R2-2105161, R2-2105399.

Knowing whether a neighbour cell accepts access by Redcap can avoid RedCap UEs’ unnecessary RRM measurements and save power. Companies propose to include an indication in system information on whether a neighbour cell accepts (support and/or allow) access by RedCap UEs. [QC, Apple, ZTE, Fujitsu]

The proposal in R2-2106487 is

Draft_Proposal 8: [2nd batch] [To discuss] RAN2 to discuss the need for an indication in system information on whether a 13eighbor cell accepts access by RedCap UEs.

Question 9: Do you support to indicate in system information on whether a 13eighbor cell accepts the access by RedCap UEs?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| OPPO | No | We don’t think such optimization is needed. |
| Ericsson | No |  |
| Apple | Yes | It would be a waste of UE power to perform re-selection actions only to realize that the NCell does not support RedCap or bars certain RedCap UE. If NW broadcasts the support/no-support of RedCap for cell selection, it helps for the NW to also broadcasts this for re-selection. |
| Qualcomm | Yes | We do not consider it an optimization, because cell barring for RedCap can be more dynamic than in legacy (e.g. barring can depend on cell loading, which can be dynamic). So a RedCap UE can’t check its neighbor cells for only once and then know whether it can skip RRM measurements on a neighbor cell forever. It still needs to check periodically on all neighbor cells to see if they have changed their barring status. Having this indication in the system information of its serving cell can help a RedCap UE avoid those periodic check on its neighbor cells. |
| MediaTek | Yes | It is likely that not all basestations will be upgraded to Rel-17 and support RedCap operation. In this scenario, it is beneficial to provide information the UE as to which 13eighbor cells allow RedCap UE access. |
| Futurewei | No |  |
| Sequans | Yes | Agree with QC |
| Intel | No | We believe existing frequency priority should be enough. |
| China Telecom | Yes | Agree with QC |
| Samsung | Yes | We share the view with Qualcomm, and moreover, a separate priority of the frequency can be provided for RedCap UEs, as RedCap UEs may not be supported in the specific frequency. |
| Lenovo | No | It is an optimization and will introduce complexity from the view of signaling. |
| LGE | No |  |
| Sierra Wireless | Yes |  |
| NEC | No |  |
| ZTE | Yes |  |
| Sharp | No | If RAN2 think it is helpful for the further power saving of RedCap UEs, we are also open to discuss further. |
| Fujitsu | Yes | Indicating whether a neighbour cell allows the camping of RedCap UEs can save RedCap UE’s power consumption on RRM measurements and cell reselection. |
| DENSO | Yes | Agree with QC. From the viewpoint of power saving and reduction of unnecessary of UE processing, RedCap UE should not perform RRM measurement on neighbor cells that do not support RedCap UE. Therefore, the access availability information should be indicated to RedCapUE. |
| Spreadtrum | Yes | Agree with QC |
| BT | Yes |  |
| Nokia, Nokia Shanghai Bell | No | We think providing a list of cells would be quite an overkill solution in terms of overhead. It seems providing the information about supporting frequencies as Samsung pointed out could suffice. |

Issue B: For handover

The issue is discussed in R2-2104775.

the companies proposes than knowing whether a neighbour cell accepts access by Redcap or not can help gNB ensures not to handover a RedCap UE to a target cell that it can’t access. [QC]

The proposal in R2-2106487 is

Draft_Proposal 9: [2nd batch] [To discuss] RAN2 to discuss the need for coordination between gNBs on whether a neighbour/target gNB supports RedCap UEs. (Send LS to RAN3, if agreed).

Question 10: Is it needed of the coordination between gNBs on whether a neighbour/target gNB supports RedCap UEs, to avoid handover RedCap to a target cell that it can’t access?

| Companies | Yes or No | Comments |
| --- | --- | --- |
| OPPO | No | Existing handover preparation procedure already allows such, e.g. if target gNB does not support RedCap UEs, it can reject the handover request. |
| Ericsson | Yes | But not clear whether anything is needed in RAN2 – there could be RAN3 impact though. |
| Apple | Co-ordination is needed across gNBs. | The signaling details can be discussed if RAN2 or RAN3 needs to handle and if existing signaling is enough. |
| Qualcomm | Yes | Without this enhancement, a source cell has to reply on “trial and error” to select a target cell that supports RedCap, e.g. it picks a target cell based on measurement report, sends handover request to the selected target cell. If the request is denied, the source cell has to try another target cell. This increases likelihood of handover failure and handover latency. On the other hand, if the source cell is able to know whether a neighbor cell supports RedCap, it can select a target cell with much better chance of success. |
| MediaTek | Yes | The NW should ensure that a RedCap UE should only do a handover to a cell that supports RedCap operation. To enable this, inter-gNB coordination is needed. We can send an LS to RAN3 to trigger this discussion. |
| Futurewei | Yes |  |
| Sequans | Yes | If not, it should be clear whether the UE gets handed over to a non-supporting cell or will there be trial-and-error by gNB beforehand. Either option seems undesirable. An LS to RAN3 is needed |
| Intel | No | It can be left to network implementation or leave the discussion to RAN3. |
| China Telecom | Yes | If not, UE might hand over to a non-supporting cell. |
| Samsung | - | As many others commented, target can determine based on UE's capability, but we tend to agree that it can be optimized to avoid trial-and-error. So, perhaps it would be beneficial to indicate it to RAN3 so that they can decide the issue. |
| Lenovo | Yes | It needs to send a LS to RAN3. |
| LGE | - | This should be discussed in RAN3. |
| Huawei, HiSilicon | Yes | For the comment from OPPO, the legacy target gNB can not understand the HO request is for RedCap, since even if the capability container includes the RedCap indication, legacy gNB cannot understand this. |
| Sierra Wireless | Yes |  |
| NEC | No | agree with Intel |
| ZTE | - | It is up to RAN3 to decide whether coordination between gNB is needed. |
| Sharp |  | Depends on RAN3 |
| Fujitsu |  | We think this can be discussed by RAN3. |
| DENSO | Yes | Agree with Qualcomm. By sharing the accessibility to RedCap UE between gNBs, it is possible to prevent the source cell from sending a handover request to a neighbor gNB that does not support RedCap. |
| Spreadtrum |  | Suggest to leave the discussion to RAN3. |
| CATT |  | No strong view. OK to consult R3. |
| BT | Yes | This is under RAN3 domain |
| Nokia, Nokia Shanghai Bell | Yes | But this is RAN3 discussion. |

- 2.2.4 RedCap specific cell (re)selection configurations

Issue A: whether to support the RedCap specific cell reselection priorities

The issue is discussed in R2-2105161, R2-2105472, R2-2105399.

Companies propose that the network may provide different cell reselection priority (or other parameters) for non-RedCap UE and RedCap UEs or for RedCap UEs with 1Rx branch. [ZTE, Samsung, Fujitsu]

Issue B: whether to support the RedCap specific cell selection parameter

The issue is discussed in R2-2105472, R2-2104790, R2-2105399, R2-2105443, R2-2106243.

Companies propose that the network may provide separate S-criteria parameters for a RedCap UE with 1 Rx branch. [Samsung, Thales, Fujitsu, DENSO, CMCC]

The proposal in R2-2106487 is

Draft_Proposal 10: [2nd batch] [To discuss] RAN2 to discuss whether to support RedCap specific Cell (re)selection parameters. (FFS only for 1 RX branches RedCap UE or all RedCap UEs; FFS on which parameters e.g. cell reselection priorities, cell reselection parameters and cell selection parameters)

Question 11: Do you prefer to support RedCap specific cell (re)selection parameters?

Please also comment on the details, if you support this: FFS only for 1 RX branches RedCap UE or all RedCap UEs; FFS on which parameters e.g. cell reselection priorities, cell reselection parameters and cell selection parameters.

| Companies | Yes or No | Comments |
| --- | --- | --- |
| OPPO | No | For the need of separate S-criteria parameters for a RedCap UE with 1 Rx branch, this should be discussed and decided by RAN1. Regarding Redcap specific reselection parameters, we think this is a general issue for every new feature, and we don’t think they are needed. |
| Ericsson | No |  |
| Apple | No strong view. |  |
| Qualcomm | Yes | For RedCap UEs which have reduced Rx capabilities, their Qrxlevmin and Qqualmin should be different from those for non-RedCap UEs to compensate the loss in their received power. |
| MediaTek | No | Given that the cell layout doesn’t change with the introduction of RedCap, we do not see a strong need to enhance cell reselection procedures. |
| Futurewei | No |  |
| Sequans | Yes for reselection | Don’t see a stron need for selection, but are OK to discuss further |
| Intel |  | For cell reselection priority used during cell reselection, we believe existing frequency priority can be reused. For 1Rx/2Rx specific threshold used during cell (re)selection, we see the benefits on this considering the coverage difference for 1Rx and 2Rx UEs.. |
| China Telecom | Yes for reselection |  |
| Samsung | Yes | As commented earlier, UE with 1RX branch should be able to access the network that supports the RedCap UE in general, and in that sense, to provide RedCap specific Cell (re)selection parameters would have the minimum change to the specification. |
| Lenovo | No |  |
| LGE | No |  |
| Huawei | No, at least for now | We need more RAN1 input on the 1 RX braches coverage. |
| NEC | No | similar view as MediaTek |
| ZTE | Yes for reselection | For the neighboring frequency that supporting both non-RedCap and RedCap UE, we think it is more flexible for operator and network to provide different cell reselection priorities for non-RedCap and RedCap UE. E.g. let RedCap UEs to camp on low-frequency cells, or small bandwidth cells. On different thresholds, we are open to discuss it further, and maybe we need to check with RAN4 about the RSRP/RSRQ accuracy requirement for RedCap 1Rx UEs? E.g. whether different requirements will be defined for RedCap and non-RedCap. |
| Sharp | No |  |
| Fujitsu | Yes | Since 1 RX branch RedCap UE may have coverage shrink compared with 2 RX branches RedCap UE and non-RedCap UE, the RedCap specific cell (re)selection parameters can be configured only for 1 RX branch RedCap UE. Regarding the parameters, we think that the cell reselection priorities, cell selection parameters such as Qrxlevmin or Qqualmin, and cell reselection parameters such as Qoffsets,n, can be separately configured for RedCap UEs. |
| DENSO | Yes | When focusing on the continuity of communication apps and services, 1Rx RedCap UE should reselect cells according to the reception sensitivity of the UE, in which case the provision of cell (re) selection parameters is useful. Also, on the NW side, it should be more beneficial for the UE to (re) select according to the reception sensitivity of the UE than the behavior of the UE when the parameter is not provided. |
| Spreadtrum | No |  |
| CATT |  | Maybe this could be discussed in a later stage. |
| BT | No | A network deployment won’t be modified therefore there is no need for extra complexity. As an operator, it is completely out of scope to configure different reselection conditions for UE, RedCap UEs with 1Rx, RedCap UEs with 2Rx. |
| Nokia, Nokia Shanghai Bell | No strong view | This could be discussed further, e.g., whether 1Rx UEs would require different parameters. |

# Conclusion and proposals

Based on the above summary, following proposals are given.

- 3.1 Proposals in number order

TBD

- 3.2 For chair notes (proposals in priority order)

TBD

# Reference

- R2-2104775	Access and camping restrictions for RedCap UEs	Qualcomm Incorporated

- R2-2104777	Discussion on early identification and SI indication	CAICT

- R2-2104790	NR-REDCAP identification and SI indication	THALES

- R2-2104809	Discussion on RedCap UE’s access control	OPPO

- R2-2104911	Identification and access restrictions for RedCap UEs	vivo, Guangdong Genius

- R2-2104928	Early identification and camping restrictions  for RedCap UE	Intel Corporation

- R2-2105014	Methods for barring and for capability reporting	Sierra Wireless, S.A.

- R2-2105071	Discussion on UAC for Redcap devices	Xiaomi Communications

- R2-2105072	Discussion on Identification and UE access restrictions for Redcap devices	Xiaomi Communications

- R2-2105137	Power-saving aspects from cell access and camping of RedCap UEs	Apple

- R2-2105161	Identification and Access Restriction for RedCap	ZTE Corporation, Sanechips

- R2-2105235	Early indication & access restriction for RedCap UEs	Ericsson

- R2-2105320	Early Identification and camping restrictions for Redcap UEs	CATT

- R2-2105399	Camping restrictions of RedCap UE	Fujitsu

- R2-2105443	Camping restriction and cell selection criterion	DENSO CORPORATION

- R2-2105472	Access control for RedCap UEs	Samsung

- R2-2105540	Discussion on early indication design for Redcap UE	Spreadtrum Communications

- R2-2105635	Identification and access restriction of RedCap UE	Huawei, HiSilicon

- R2-2105793	Early identification and SI indication	NEC

- R2-2105814	Cell barring for REDCAP UEs	Lenovo, Motorola Mobility

- R2-2105879	Access for REDCAP UE	Nokia, Nokia Shanghai Bell

- R2-2105883	Identification and access restrictions of RedCap Ues	LG Electronics UK

- R2-2105957	Discussion on access and camping restrictions for RedCap UEs	Futurewei Technologies

- R2-2106052	Identification and restriction of RedCap UE	InterDigital

- R2-2106243	Access control for RedCap UEs	cmcc

- R2-2106244	Discussion on early identification	cmcc

- R2-2106274	Early identification and camping restrictions of RedCap UE	China Telecommunications

- R2-2106487	[Pre114-e][106][RedCap] Summary 8.12.2.2 - Identification and access restrictions (Huawei)