3GPP TSG-RAN WG2 Meeting #114 electronic	                                      R2-210xxxx

19th May – 27th May 2021

Agenda item:	7.3.3

Source:	ZTE (email discussion rapporteur)

Title:	Report of [AT114-e][303][NBIOT/eMTC R16] PUR Corrections (ZTE)

Document for:	Discussion and Decision

# Introduction

This document is the summary of the offline email discussion “[AT114-e][303][NBIOT/eMTC R16] PUR Corrections”, as indicated below:

- [AT114-e][303][NBIOT/eMTC R16] PUR Corrections (ZTE)

Scope: Discussion of CRs in R2-2106214 and R2-2106277. Poll for support and initial comments to CRs.

Intended outcome: Report in R2-2106604

Deadline: Monday May 24 1200 UTC

# Contact information

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Qualcomm | Umesh Phuyal | uphuyal at qti.qualcomm.com |

# Discussion

This offline is to check whether the intention of the following CRs is agreeable and whether there are comments on the actual proposed changes.

[1] R2-2106214	Add ack-NACK-NumRepetitions for PUR-Config-NB	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.4.0	4679	-	F	NB_IOTenh3-Core

Q1: Do you agree with the intent of the change in [1]?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | No comments. |
| vivo | Yes with comments | We generally agree with the intention. It seems we need to figure out the raised issue. But we think this issue shall be handled in RAN1 at first since the repetition transmission of NPUSCH carrying ACK/NACK is out of RAN2 scope. Thus, we may better wait for RAN1 input for now. |
| MediaTek | No | If ack-NACK-NumRepetitions is not configured in NPUSCH-ConfigDedicated-NB-r13, the value used for reception of Msg4 is used. As for the PUR configuration, the Tdoc explains why the value used for reception of MSG4 can not be used, that is “ack-NACK-NumRepetitions-Msg4 is configured per CEL. UE cannot determine which CEL’s value can be used for PUR procedure.” However, as the timing of PUR configuration would not be much later than the connection establishment, can UE assume the same CE level is used for PUR configuration? |
| Qualcomm | Yes. See comments | While the ACK/NACK repetition value used for Release message could work, it should be ok to add the field for the case when eNB wants to provide a different repetiton level for PUR, e.g. be conservative and provide higher repetition for PUR. That possiblity is missing today. However, name of the field should be updated to include “pur-” (consistent with other field names e.g. pur-NumOccasions). And to avoid overhead if so chosen by network, UE should use the value used for Release message where the PUR config is provided. So, the following changes (yellow highlighted) are needed to the proposed field description. pur-ACK-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH for PUR, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured, the value of repetitions for the ACK NACK response to NPDSCH containing this RRCConnectionRelease-NB applies. Another minor comment: comma after “OPTIONAL,” should be removed. |

Conclusion:

Proposal:

[2] R2-2106277	MAC clarifications for PUR	ZTE Corporation, Sanechips, MediaTek Inc. 	CR	Rel-16	36.321	16.4.0	1524	-	F	LTE_eMTC5-Core, NB_IOTenh3-Core

Q2a: Do you agree with the intent of the first change in [2], e.g., if contention resolution in RA procedure fails, the NTA updated upon reception of RAR carrying a TAC would be incorrect for later PUR?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | Here are some additional thinking when drafting the CR: - Setting the stored temporary NTA to NTA (e.g., restoring the previous NTA) only occurs at Random Access procedure unsuccessfully completed, no need at each time Contention Resolution not successful. - Based on above point, the temporary NTA value is only stored once when the RAR is received for the first access attempt, not upon RAR for each access attempt. |
| vivo | Yes | For simplicity, we agree that the TAC received in RAR can be used for Msg3 transmission. Then if contention resolution is not successful, the original NTA (i.e. the restored temp NTA mentioned in the CR) for PUR transmission should be re-used. |
| MediaTek | Yes | The TAC in RAR should update the TA for PUR, and it should be done after successful contention resolution. |
| Qualcomm | Intent ok, see comments | We agree with the issue that if TA is updated but RA ultimately fails, then the UE may erronously update TA that is not intended for it. So, we assume the intent is that NTA should be preserved and in case of unsuccessful RA, PUR TAT should not restart, upper layer should not be indicated and NTA should be restored. If this is the intent, we agree. However, the changes in CR have several issues. - According to the CR, when TA MAC CE is received, TA command would have already been applied according to existing text, before reaching the new text. At this point, there is no “temporary NTA” to be stored anymore. - The existing text already “start or restart” the PUR TAT whenever TA MAC CE is recevied. The new text repeats it for sucessful RA (resets timer value), but does not prevent it for failed RA. - “indicate to upper layers” happens whenever TA MAC CE is received due to existing text. The new text repeats it for sucessful RA, but does not prevent it for failed RA. This indication is used by RRC for (N)RSRP based validation (to update the reference (N)RSRP). - There is potential confusion on “temporary NTA” and “NTA has been stored or maintained”. For a new reader, would having a temporary NTA stored (according to new text) also mean NTA has been stored or maintained (satisfying existing text)? - The behaviour that needs correction is upon unsuccessful completion of RA procedure what to do with MAC CE “already” received in RAR earlier, however the proposed text is adding behaviour for each RAR reception. In current MAC specification, there seems to be no case where a RA is considered unsuccessful at the time of RAR reception. The “unsuccessfully completed” RA in 5.1.4 RAR reception is for the case where the UE actually doesn’t get RAR intended for it (specifically “the Random Access Response reception is considered not successful”). The other case of “unsuccesfully completed” RA is in 5.1.5 contention resolution phase, i.e., RAR is not actually received at that time (but received earlier). So, strictkly speaking, some text added in the CR cannot be applicable at any time. So, further discussion is needed how to clearly catpure the exact text. First, RAN2 should agree to the intent as explained above. |

Conclusion:

Proposal:

Q2b: Do you agree with the intent of the second change in [2], e.g., UE starts the timeAlignmentTimer but not restarts pur-TimeAlignmentTimer when receiving RAR carrying a TAC?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | Here are some additional thinking when drafting the CR: - In order to keep alignment between timeAlignmentTimer and pur-TimeAlignmentTimer, the pur-TimeAlignmentTimer is implicitly restarted by setting the elapsed value (e.g., from reception of RAR to Random Access procedure successfully completed) of timeAlignmentTimer to it. |
| vivo | Yes with comments | We agree with the raised issue and think a solution is needed. Regarding the proposed solution in the CR, we can fully understand that it allows the PUR-TAT to be operated in a perfectly precise state. However, from the UE implementation of view, we are not sure how to realize the operation that setting the value of TimeAlignmentTimer to pur-TimeAlignmentTimer (i.e. it might be difficult to reset the remaining running time of a timer when it is running). For simplicity, we think the UE can restart the PUR-TAT upon successful contention resolution. |
| MediaTek | Yes | Restarting PUR-TA timer would be more clear. |
| Qualcomm | See comments in previous question | Intent to not (re)start PUR TAT when the contention resolution cannot be successful makes sense. In case of RACH contention between say UE1 and UE2, the network may detect UE2 and provide the RAR with new TA, but UE1 with PUR may wrongly apply the TA and reset the PUR TAT. Therefore, it makes sense to (re)start PUR TAT (as well as indicate to upper layer that new TA is applied) only after successful contention resolution. |

Conclusion:

Proposal:

Q2c: Do you agree with the intent of the third change in [2], e.g., the HARQ process ID for the only HARQ process in PUR is unclear?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | No comments. |
| vivo | Yes | This clarification is useful. |
| MediaTek | Yes |  |
| Qualcomm | Yes | Ok with the change. |

Conclusion:

Proposal:

# Conclusion

TBD