3GPP TSG-RAN WG2 Meeting #114 electronic	                                draft R2-2106604

19th May – 27th May 2021

Agenda item:	7.3.3

Source:	ZTE (email discussion rapporteur)

Title:	Report of [AT114-e][303][NBIOT/eMTC R16] PUR Corrections (ZTE)

Document for:	Discussion and Decision

# Introduction

This document is the summary of the offline email discussion “[AT114-e][303][NBIOT/eMTC R16] PUR Corrections”, as indicated below:

- [AT114-e][303][NBIOT/eMTC R16] PUR Corrections (ZTE)

Scope: Discussion of CRs in R2-2106214 and R2-2106277. Poll for support and initial comments to CRs.

Intended outcome: Report in R2-2106604

Deadline: Monday May 24 1200 UTC

# Contact information

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| ZTE | Ting Lu | lu.ting@zte.com.cn |
| vivo | Yitao Mo (Stephen) | yitao.mo@vivo.com |
| Qualcomm | Umesh Phuyal | uphuyal at qti.qualcomm.com |
| LGE | HyunJung Choe | stella.choe@lge.com |
| Ericsson | Tuomas Tirronen | tuomas.tirronen@ericsson.com |

# Discussion

This offline is to check whether the intention of the following CRs is agreeable and whether there are comments on the actual proposed changes.

[1] R2-2106214	Add ack-NACK-NumRepetitions for PUR-Config-NB	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.4.0	4679	-	F	NB_IOTenh3-Core

Q1: Do you agree with the intent of the change in [1]?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | No comments. |
| vivo | Yes with comments | We generally agree with the intention. It seems we need to figure out the raised issue. But we think this issue shall be handled in RAN1 at first since the repetition transmission of NPUSCH carrying ACK/NACK is out of RAN2 scope. Thus, we may better wait for RAN1 input for now. |
| MediaTek | No | If ack-NACK-NumRepetitions is not configured in NPUSCH-ConfigDedicated-NB-r13, the value used for reception of Msg4 is used. As for the PUR configuration, the Tdoc explains why the value used for reception of MSG4 can not be used, that is “ack-NACK-NumRepetitions-Msg4 is configured per CEL. UE cannot determine which CEL’s value can be used for PUR procedure.” However, as the timing of PUR configuration would not be much later than the connection establishment, can UE assume the same CE level is used for PUR configuration? |
| Qualcomm | Yes. See comments | While the ACK/NACK repetition value used for Release message could work, it should be ok to add the field for the case when eNB wants to provide a different repetiton level for PUR, e.g. be conservative and provide higher repetition for PUR. That possiblity is missing today. However, name of the field should be updated to include “pur-” (consistent with other field names e.g. pur-NumOccasions). And to avoid overhead if so chosen by network, UE should use the value used for Release message where the PUR config is provided. So, the following changes (yellow highlighted) are needed to the proposed field description. pur-ACK-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH for PUR, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured, the value of repetitions for the ACK NACK response to NPDSCH containing this RRCConnectionRelease-NB applies. Another minor comment: comma after “OPTIONAL,” should be removed. |
| Huawei, HiSilicon | Yes wit comments. | We think that the value used for RRCConnectionRelease message (similar to the TA) works and was the intention. But we are also fine with introducing signalling for the value as this is possible in eMTC. Now, PUR physical layer parameters have been defined by RAN1 and we should check whether there is any impact on their specification. The cover page (reason for change and consequences if not approved) should be rewritten as we don’t think the description is correct (e.g. we . If it is not changed, then the CR is NBC functioanly and this should be indiacted on the cover page. We agree with the comments from Qualcomm that the prefx pur- needs to be added to the field name and that the signalling should not be made mandatory. ASN.1 will not compile as pointed out by Qualcomm. Companies are supposed to compile their CR before submission. |
| ZTE2 |  | - Response to MTK: As we cannot know how long a connection would maintain, we cannot assume the timing of PUR configuration (e.g., the connection release) would not be much later than the connection establishment. Also UE cannot assume the same CE level is used for PUR configuration. - Response to vivo/HW: We have checked with RAN1 before submitting the CR. We understand in RAN1 there is a common description for reporting ACK/NACK (as following cited text) which can cover the PUR ACK/NACK transmission: ### TS 36.213 v16.5.0 ### 16.4.2 UE procedure for reporting ACK/NACK The UE shall upon detection of a NPDSCH transmission ending in NB-IoT subframe n intended for the UE and for which an ACK/NACK shall be provided, start, after the end of - DL subframe for FDD, - NB-IoT UL subframes following the end of n+12 subframe for TDD, transmission of the NPUSCH carrying ACK/NACK response, and SR (if any) if the serving cell is FDD and the UE is configured with higher layer parameter sr-with-HARQ-ACK-Config, using NPUSCH format 2 in N consecutive NB-IoT UL slots, where - , where - the value of NRepANis given by the higher layer parameter ack-NACK-NumRepetitions-Msg4 configured for the associated NPRACH resource for Msg4 NPDSCH transmission, and higher layer parameter ack-NACK-NumRepetitions otherwise, - the value of is the number of slots of the resource unit (defined in clause 10.1.2.3 of [3]), and …………. This is also response to one of QC’s comment. Previously we have considered the naming like pur-ACK-NACK-NumRepetitions but later decide to use the current naming, e.g., the one same as the legacy IE name (since RAN1 spec explicitly quotes the name of high layer parameter). We hope it can be helpful to avoid any change in RAN1. - Response to QC/HW: Previously we just assume anyway the network would configure this parameter at least once in PUR configuration. Technically we are fine with QC’s suggestion (HW also agree), e.g., the value used for RRCConnectionRelease message could work when this field is absent from PUR configruraton. But with reference to the above RAN1 specification, we a bit worry about this may be not exactly aligned with RAN1 description. Can we consider another wording like this: PUR-Config-NB field descriptions. ack-NACK-NumRepetitions Number of repetitions for the ACK NACK resource unit carrying HARQ response to NPDSCH, see TS 36.213 [23], clause 16.4.2. If this field is absent and no value was configured via pur-Config, the latest used ack-NACK-NumRepetitions value is used.. Moreover, thanks a lot for spotting the ASN.1 error. |
| LGE | Yes | We agree with the intention of the CR. Draft CR change for ack-NACK-NumRepetitions description suggested by Huawei is fine with us. |
| Ericsson | Yes in principle, please see the comments. | Both options, i.e., the number of repetitions required for transmitting the ACK/NACK response to NPDSCH transmission containing the RRCConnectionRelease-NB message or specifying explicit signaling for configuration, are fine. We agree with QC regarding the naming convention and HW regarding the wording on the coverpage and the need to check with RAN1 in case there is any impact on their specifications. |
| Qualcomm2 |  | See minor comments in updated draft CR in the drafts folder. In the field description, it is better to state what configuration/value “applies” instead of “is used”. |

Conclusion:

6 companies among all the 7 companies who provide responses can agree with the intention of CR in [1], e.g., to introduce ack-NACK-NumRepetitions for PUR-Config-NB. Companies have provided the following detailed comments:

- Some companies think previously there is intention that the number of repetitions required for transmitting the ACK/NACK response to NPDSCH transmission containing the RRCConnectionRelease-NB message can be used. But companies fine with introducing signalling in case eNB wants to provide a different repetiton level for PUR. This already can be done for eMTC but the possiblity is missing for NB-IoT. This needs to be reflected in the cover page. Clarification in the field description is also needed.

- Considering the naming convention, it may need to add prefix “pur-” for this parameter ack-NACK-NumRepetitions. But as in RAN1 spec, the name of high layer parameter ack-NACK-NumRepetitions is explicitly quoted, companies suggest to use a new parameter group to include this parameter. The naming of such parameter group can be pur-PhysicalConfig-v16xy or pur-NPUSCH-config-r16. The former is preferred.

Proposal:

Proposal 1: To introduce parameter ack-NACK-NumRepetitions for PUR-Config-NB.

Proposal 1a: The CR in [R2-2106214] is revised with taking into account the following comments:

- For the change in ASN.1, the comma after “OPTIONAL,” should be removed.

- To introduce pur-PhysicalConfig-v16xy to include the new parameter ack-NACK-NumRepetitions-r16 for PUR-Config-NB.

- In the field description of ack-NACK-NumRepetitions, besides the definition of the parameter, to add “If this field is absent and no value was configured via pur-Config, the value of ack-NACK-NumRepetitions used for ACK/NACK HARQ response to NPDSCH containing this RRCConnectionRelease-NB applies” to address the case that this parameter is absent.

- The cover page needs to be improved accordingly.

[2] R2-2106277	MAC clarifications for PUR	ZTE Corporation, Sanechips, MediaTek Inc. 	CR	Rel-16	36.321	16.4.0	1524	-	F	LTE_eMTC5-Core, NB_IOTenh3-Core

Q2a: Do you agree with the intent of the first change in [2], e.g., if contention resolution in RA procedure fails, the NTA updated upon reception of RAR carrying a TAC would be incorrect for later PUR?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | Here are some additional thinking when drafting the CR: - Setting the stored temporary NTA to NTA (e.g., restoring the previous NTA) only occurs at Random Access procedure unsuccessfully completed, no need at each time Contention Resolution not successful. - Based on above point, the temporary NTA value is only stored once when the RAR is received for the first access attempt, not upon RAR for each access attempt. |
| vivo | Yes | For simplicity, we agree that the TAC received in RAR can be used for Msg3 transmission. Then if contention resolution is not successful, the original NTA (i.e. the restored temp NTA mentioned in the CR) for PUR transmission should be re-used. |
| MediaTek | Yes | The TAC in RAR should update the TA for PUR, and it should be done after successful contention resolution. |
| Qualcomm | Intent ok, see comments | We agree with the issue that if TA is updated but RA ultimately fails, then the UE may erronously update TA that is not intended for it. So, we assume the intent is that NTA should be preserved and in case of unsuccessful RA, PUR TAT should not restart, upper layer should not be indicated and NTA should be restored. If this is the intent, we agree. However, the changes in CR have several issues. - According to the CR, when TA MAC CE is received, TA command would have already been applied according to existing text, before reaching the new text. At this point, there is no “temporary NTA” to be stored anymore. - The existing text already “start or restart” the PUR TAT whenever TA MAC CE is recevied. The new text repeats it for sucessful RA (resets timer value), but does not prevent it for failed RA. - “indicate to upper layers” happens whenever TA MAC CE is received due to existing text. The new text repeats it for sucessful RA, but does not prevent it for failed RA. This indication is used by RRC for (N)RSRP based validation (to update the reference (N)RSRP). - There is potential confusion on “temporary NTA” and “NTA has been stored or maintained”. For a new reader, would having a temporary NTA stored (according to new text) also mean NTA has been stored or maintained (satisfying existing text)? - The behaviour that needs correction is upon unsuccessful completion of RA procedure what to do with MAC CE “already” received in RAR earlier, however the proposed text is adding behaviour for each RAR reception. In current MAC specification, there seems to be no case where a RA is considered unsuccessful at the time of RAR reception. The “unsuccessfully completed” RA in 5.1.4 RAR reception is for the case where the UE actually doesn’t get RAR intended for it (specifically “the Random Access Response reception is considered not successful”). The other case of “unsuccesfully completed” RA is in 5.1.5 contention resolution phase, i.e., RAR is not actually received at that time (but received earlier). So, strictkly speaking, some text added in the CR cannot be applicable at any time. So, further discussion is needed how to clearly catpure the exact text. First, RAN2 should agree to the intent as explained above. |
| Huawei, HiSilicon | maybe | Like usual, we are confused with the question about the first change and second change, as there is only one change in 5.4.7.2 We agree with the intent that upon a successful RA procedure, the PUR TA should be updated and pur-TimeAlignmentTimer restarted. Now pur-TimeAlignmentTimer granularity is in number of periodicity of PUR, i.e. very coarse, and the timer duration very long. It should be enough to update the TA value with the current TA value and restart the timer upon successful RA completion. |
| ZTE2 |  | - Response to HW: In this questionnaire, the first/second/third change corresponds to the changes listed in the "Summary of change" in the cover page of CR R2-2106277. Yes, as the first/second actural changes are merged in only one section 5.4.7.2. we agree such saying is a bit confused. The intention is just to efficiently collect companies’s view for each issue and related change as we assume it may be possible companies agree with first change for NTA but disagree with the cahnge for PUR TAT, or wise versa. - Response to QC: See above response comments for the issues mentioned by QC. |
| LGE | Yes | We agree that incorrect NTA may be used and correction is needed. Clarifcation of HARQ process ID for PUR transmission is fine for us. |
| Ericsson | Agree with the intention, please see the comments. | We agree that TA should be updated only if random access procedure is successfully completed since otherwise only the UE would update the pur-TimeAlignmentTimer, i.e., restart the timer, but not the network. Regarding the CR: - Agree with QC that temporary NTA should be stored when RAR message is received otherwise it would be too late. - The proposed text is not clear regarding how temporary NTA is defined. - Agree with QC that existing text prior to the proposed change already indicates the following actions: “start or restart the pur-TimeAlignmentTimer, if configured” and ; “indicate to upper layers that the Timing Advance value has been adjusted.” - Why is the following change needed “set the value of TimeAlignmentTimer to pur-TimeAlignmentTimer;” upon considering that Random Access procedure successfully completed? Agree with HW that the granularity for pur-TimeAlignmentTimer is in number of periodicity of PUR, which is coarse; it should be sufficient to update the TA value and restart the timer upon successful RA completion. If RAN2 agrees with the intention, further discussion is needed regarding how the required changes would be captured. |
| Qualcomm2 | Additional comments | Before going into the exact details of the MAC CR, here are the items that need to be handled. - There are two TA timers (legacy TA timer, and PUR TA timer), but there is a single NTA value at any given time. So if 5.2 updates the NTA value for the UE, that also applies in case of PUR (section 5.4.7.2). In that sense, 5.4.7.2 is in addition to 5.2, not instead of 5.2. - Section 5.2 has the following (existing text). This is not specific to PUR or non-PUR case. So, it seems there is already duplication in 5.2 and 5.4.7.2 for yellow highlighted part. The example shown is for MAC CE. There is similar duplication for even reception of TAC in RAR. These duplications should be handled - when a Timing Advance Command MAC control element is received and if a NTA has been stored or maintained with the indicated TAG: - apply the Timing Advance Command for the indicated TAG; - For PUR-specific handling in 5.2 or in new text in section 5.4.7.2, the conditions cannot entirely be simply based on “if PUR TAT is running”. Consider the case when PUR TAT is not configured, but (N)RSRP based TA is used for PUR. This is a valid configuration explicitly confirmed by RAN2 earlier. In that case PUR TAT will not be running. But NTA should be updated if other conditions are met and “indicate to upper layer” should happen upon updating of TA even though PUR TAT may be not running (for the UE to update (N)RSRP reference). - Now with these changes, it looks like temporary NTA may be clear enough. (We don’t think “fallback” NTA is a right term anyway.) - In the new change upon considering the Random Access procedure unsuccessfully completed, it should be set the NTA to the stored temporary NTA, not the other way around. Updated comments and suggestions to MAC CR (v2) can be found in the drafts folder. In any case, as indicated in earlier response, we can continue to discuss further after the principles are agreed. |

Q2b: Do you agree with the intent of the second change in [2], e.g., UE starts the timeAlignmentTimer but not restarts pur-TimeAlignmentTimer when receiving RAR carrying a TAC?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | Here are some additional thinking when drafting the CR: - In order to keep alignment between timeAlignmentTimer and pur-TimeAlignmentTimer, the pur-TimeAlignmentTimer is implicitly restarted by setting the elapsed value (e.g., from reception of RAR to Random Access procedure successfully completed) of timeAlignmentTimer to it. |
| vivo | Yes with comments | We agree with the raised issue and think a solution is needed. Regarding the proposed solution in the CR, we can fully understand that it allows the PUR-TAT to be operated in a perfectly precise state. However, from the UE implementation of view, we are not sure how to realize the operation that setting the value of TimeAlignmentTimer to pur-TimeAlignmentTimer (i.e. it might be difficult to reset the remaining running time of a timer when it is running). For simplicity, we think the UE can restart the PUR-TAT upon successful contention resolution. |
| MediaTek | Yes | Restarting PUR-TA timer would be more clear. |
| Qualcomm | See comments in previous question | Intent to not (re)start PUR TAT when the contention resolution cannot be successful makes sense. In case of RACH contention between say UE1 and UE2, the network may detect UE2 and provide the RAR with new TA, but UE1 with PUR may wrongly apply the TA and reset the PUR TAT. Therefore, it makes sense to (re)start PUR TAT (as well as indicate to upper layer that new TA is applied) only after successful contention resolution. |
| Huwei, HiSilicon | see answer to Q2a |  |
| ZTE2 |  | We respect and thank for clarification from all the UE/chipset vendors. So we are fine with directly restart PUR-TA timer, instead of implicitly restart PUR-TA timer by setting the elapsed value of timeAlignmentTimer to PUR-TA timer. |
| LGE | Yes |  |
| Ericsson | Please see our reply in Q2a |  |
| Qualcomm2 |  | Similar to the case of MAC CE and L1 indication, PUR TAT should start or restart if RAR provides TA and the RA procedure is successful. |

Q2c: Do you agree with the intent of the third change in [2], e.g., the HARQ process ID for the only HARQ process in PUR is unclear?

| Company | Yes/no | Detailed comments |
| --- | --- | --- |
| ZTE | Yes | No comments. |
| vivo | Yes | This clarification is useful. |
| MediaTek | Yes |  |
| Qualcomm | Yes | Ok with the change. |
| LGE | Yes |  |
| Ericsson | Yes |  |

Conclusion:

All the 7 companies who provide responses can agree with the intentions of CR in [2]. Companies have provided the following detailed comments:

- From the UE implementation of view, it may be infeasible to set the value of TimeAlignmentTimer to pur-TimeAlignmentTimer). For simplicity, the UE can restart the PUR-TAT upon successful contention resolution..

- In 5.2 and 5.4.7.2 existing text, there is duplicated description of “apply the Timing Advance Command…” upon reception of TAC MAC CE. This duplication should be handled.

- The temporary NTA should be stored when RAR message is received otherwise it would be too late. Therefore, companies suggest to move the text of storing NTA as temporary NTA from 5.4.7.2 to 5.2. This can also be helpful to avoid duplication description in 5.2 and 5.4.7.2.

- In the new-added change, the condition “if PUR TAT is running” is not complete. Companies suggest to use “if the UE is configured with pur-Config (see TS 36.331 [8]) and if a NTA has been stored or maintained” to cover both cases of TAT based or (N)RSRP based TA validation.

- In the new change upon considering the Random Access procedure unsuccessfully completed, it should be set the NTA to the stored temporary NTA, not the other way around.

Proposal:

Proposal 2: The following corrections/clarifications can be agreed:

- To clarify that before NTA is updated upon reception of RAR with TAC, it should be stored as a temporary NTA. Such temporary NTA should be restored if the RA procedure ultimately fails.

- To clarify PUR TAT should directly start or restart if RAR provides TAC and the RA procedure is successful.

- To handle the duplicated description of “apply the Timing Advance Command…” upon reception of TAC MAC CE in both 5.2 and 5.4.7.2 existing text.

- To clarify HARQ process ID in PUR is zero.

Proposal 2a: The CR in [R2-2106277] is revised with taking into account the following comments:

- UE can directly restart the PUR TAT when RA procedure is successful.

- In 5.2, to exclude PUR case from the description of “apply the Timing Advance Command…” upon reception of TAC MAC CE.

- To move the new change of storing NTA as temporary NTA from 5.4.7.2 to 5.2. And the condition is changed to “if the UE is configured with pur-Config (see TS 36.331 [8]) and if a NTA has been stored or maintained”.

- In the new change upon considering the Random Access procedure unsuccessfully completed, it should be set the NTA to the stored temporary NTA, not the other way around.

- The cover page needs to be improved accordingly.

# Conclusion

Proposal 1: To introduce parameter ack-NACK-NumRepetitions for PUR-Config-NB.

Proposal 1a: The CR in [R2-2106214] is revised with taking into account the following comments:

- For the change in ASN.1, the comma after “OPTIONAL,” should be removed.

- To introduce pur-PhysicalConfig-v16xy to include the new parameter ack-NACK-NumRepetitions-r16 for PUR-Config-NB.

- In the field description of ack-NACK-NumRepetitions, besides the definition of the parameter, to add “If this field is absent and no value was configured via pur-Config, the value of ack-NACK-NumRepetitions used for ACK/NACK HARQ response to NPDSCH containing this RRCConnectionRelease-NB applies” to address the case that this parameter is absent.

- The cover page needs to be improved accordingly.

Proposal 2: The following corrections/clarifications can be agreed:

- To clarify that before NTA is updated upon reception of RAR with TAC, it should be stored as a temporary NTA. Such temporary NTA should be restored if the RA procedure ultimately fails.

- To clarify PUR TAT should directly start or restart if RAR provides TAC and the RA procedure is successful.

- To handle the duplicated description of “apply the Timing Advance Command…” upon reception of TAC MAC CE in both 5.2 and 5.4.7.2 existing text.

- To clarify HARQ process ID in PUR is zero.

Proposal 2a: The CR in [R2-2106277] is revised with taking into account the following comments:

- UE can directly restart the PUR TAT when RA procedure is successful.

- In 5.2, to exclude PUR case from the description of “apply the Timing Advance Command…” upon reception of TAC MAC CE.

- To move the new change of storing NTA as temporary NTA from 5.4.7.2 to 5.2. And the condition is changed to “if the UE is configured with pur-Config (see TS 36.331 [8]) and if a NTA has been stored or maintained”.

- In the new change upon considering the Random Access procedure unsuccessfully completed, it should be set the NTA to the stored temporary NTA, not the other way around.

- The cover page needs to be improved accordingly.