3GPP TSG-RAN WG2 Meeting #114 electronic	                                                                                R2-210xxxx

19th May – 27th May 2021

Agenda Item:	7.2.2

Source: 	ZTE (email discussion rapporteur)

Title:  	                  Report of [AT114-e][401][eMTC R16] Paging DRX cycle (ZTE)

Document for:	Discussion and Decision

# Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT114-e][401][eMTC R16] Paging DRX cycle”, as indicated below:

- [AT114-e][401][eMTC R16] Paging DRX cycle (ZTE)

Status: Started

Scope: Check whether the intention is agreeable and there is sufficient support in principle;

collect initial comments.

Intended outcome: Report in R2-2106546

Deadline: Monday 2021-05-24 12:00 UTC

# Contact information

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| ZTE | Ting Lu | lu.ting@zte.com.cn |

# Offline email discussion

In RAN2#113bis e-meeting, based on contributions, RAN2 has had some discussion on the issues related to DRX cycle (T) and paging resources determination for eMTC. RAN2 has agreed some intentions but some issues are still open.

## 3.1 Issue#1: Inconsistent paging resources determination between UE and NW

In last meeting, some companies have had common understanding on the issue that paging resources determination may be different between UE in RRC_INACTIVE and eNB due to different DRX cycle(T) determination in RRC_INACTIVE and in RRC_IDLE. But companies have no consensus on the proposed solutions. Some other companies still need more check on the issues.

In [1], company re-confirm there is the issue that UE in RRC_INACTIVE uses different PNB from that used by network and this may further cause paging failure. And similar issue also exists for i_s.

As the PFs calculation corresponding to RRC_IDLE mode do overlap with PFs for RRC_INACTIVE state, there is no issue for PF calculation. Moreover, after further check, company mentions it’s also no issue for wg as WUS is not supported for RRC_INACTIVE.

Q1a: Companies are invited to indicate whether you agree there is the issue that PNB and i_s determination are inconsistent between eMTC UE in RRC_INACTIVE and NW?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |

In RAN2#113bis e-meeting discussion, companies have mentioned that the issue with i_s calculation is from Release 15 hence impacts eLTE/NR. Therefore, correction to i_s calculation needs to be discussed in LTE/NR session first and such correction would also apply to eMTC. In this meeting, the related discussion paper and CRs are submitted in [7]~[13]. As mentioned in [9], the main issue in eLTE/NR is:

……As a result, the index of the PO (i.e. the i_s) would be different for inactive state and idle state as the N is a value related to the T while the T has different value in idle and inactive state, which deviates from the intention that the POs of a UE for CN-initiated and RAN-initiated paging should be overlapped and inactive UE can monitor CN paging and RAN paging in the overlapped POs. Under this circumstance, it is worth considering what is the expected UE behavior for UE in RRC_INACTIVE. If UE in RRC_INACTIVE only monitors the RAN paging PO, CN paging failure would happen.…….

And the suggested solution is:

Proposal 1: For both NR and EUTRA connected to 5GC, UE in RRC_INACTIVE state should use the same i_s as for RRC_IDLE state in PO determination so that the POs of a UE for CN-initiated and RAN-initiated paging are overlapped.

Moreover, since there have been a lot of R15 UEs on the market, for eLTE/NR, a UE capability needs to be introduced to indicate UE support for using the same i_s in PO determination in RRC_INACTIVE as in RRC_IDLE state so that NW can identify such UE and send both CN paging and RAN paging in the same POs.

As mentioned in [1], company think in NR and eLTE, only i_s calculation has the issue. Even if no correction is agreed for eLTE/NR, the issue may be tolerable with some special configurations. However, for eMTC, the PNB calculation have more serious problem, e.g., no matter what the nB value is, the PNB for UE in RRC_INACTIVE may be different from that used by NW. So, proponent company suggests that eMTC should deal with this paging resources determination issue even eLTE/NR does not agree with any correction.

Q1b: If answer for Q1a is Yes, companies are invited to provide comments on whether and how to address the issue for eMTC:

- Option 1: Wait and follow the eLTE/NR decision. If no correction can be agreed for eLTE/NR, the related changes for eMTC are not pursued.

- Option 2: To have a specific solution from Rel-16 eMTC:

  - Option 2-1: eMTC UE monitor the paging resources for both RAN paging and CN paging, in case RAN and CN paging resources are not overlapped.

  - Option 2-2: eMTC UE in RRC_INACTIVE should use the same rules as for RRC_IDLE to determine the PNB and i_s.

- Other option

| Company | Preferred Option | Detailed comments |
| --- | --- | --- |
| ZTE | Option 2-2 |  |

Q1c: If answer for Q1b is Option 2-2, companies are further invited to provide comments on the following suggested alternatives:

- Alt1: The related change focus on clarification for paging resources calculation [3].

- Alt2: The related change focus on clarification for DRX cycle (T) determination [4].

- Other Alternative

| Company | Preferred Alternative | Detailed comments |
| --- | --- | --- |
| ZTE | Slightly prefer Alt2 |  |

Q1d: If answer for Q1b is Option 2-2, companies are further invited to provide comments on whether UE capability is needed?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | No | As we can assume there is no R16 eMTC UEs on the market, it’s no need to differentiate UE’s capability for eMTC UEs. |

Conclusion:

Proposal:

## 3.2 Issue#2: Optional RAN paging cycle in TS 36.331

In RAN2#113bis e-meeting discussion, companies have agreed the intention that for DRX cycle determination in RRC_INACTIVE, “if allocated by upper layers” would be applied to the UE specific paging cycle rather than to default paging cycle. But company further mentioned RAN paging cycle is also optional. If “if allocated by upper layers” would not be applied to RAN paging cycle, we may need some clarification, e.g., to add “if configured” for RAN paging cycle.

However, during phase 2 draft CR review, it’s further identified that the related parameter is mandatory in XnAP interface. The absence of RAN paging cycle may cause ambiguity in RAN3 specification.

Furthermore, in the current TS 36.304, for eMTC, the DRX cycle(T) is determined by only RAN paging cycle outside the PTW. If RAN paging cycle is not configured by RRC, how to determine the DRX cycle(T) outside the PTW is unclear. Two options have been suggested in the last meeting during phase 2 CR review:

- Option 1: if the RAN paging cycle is not configured, outside the PTW, T is determined by the shortest of the UE specific paging cycle, if allocated by upper layers, and the default paging cycle.

- Option 2: if the RAN paging cycle is not configured, outside the PTW, UE does not monitor paging.

In [1], company analyze that if RAN paging cycle is not configured by RRC, none of the options are suitable to determine the DRX cycle(T) outside the PTW. Company therefore give the following proposals:

Proposal 1a: It is specified in the ran-PagingCycle field description in TS 36.331 that “This field should be present in this release”.

Proposal 1b: In TS 36.304, it’s no need to consider the case that RAN paging cycle is not configured.

Q2: Companies are invited to indicate whether you agree with the intent of the Proposal 1a and Proposal 1b in [1] and also the changes in [2]?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |

Conclusion:

Proposal:

## 3.3 Issue#3: Clarification on general description for paging monitoring

As mentioned in [1], in TS 36.300, the paging monitoring description does not explicitly differentiate UE in RRC_IDLE and UE in RRC_INACTIVE, which may lead misunderstanding that UE in RRC_INACTIVE does not monitor paging outside the PTW.

Moreover, in section 7.3 in TS 36.304, only NAS identity has been mentioned, that may also lead misunderstanding that UE in RRC_INACTIVE only monitor CN paging.

Q3: Companies are invited to indicate whether you agree with the intent of changes in [5] and in section 7.3 in [3] or [4]?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |

Conclusion:

Proposal:

## 3.4 Issue#4: LS to RAN3

Based on the discussion on above Q1~Q3, companies are invited to give suggestion on whether LS to RAN3 is needed.

Q4: Companies are invited to indicate whether you agree with the intent of sending LS to RAN3?

| Company | Yes/No | Detailed comments |
| --- | --- | --- |
| ZTE | Yes |  |

Conclusion:

Proposal:

## 3.5 Other issue

Q5: Is there any other related issue for discussion?

| Company | Detailed comments |
| --- | --- |

Conclusion:

Proposal:

# Summary

Conclusion:

Updated CRs and LS:

# References

- R2-2106285, Discussion on paging resources determination for eMTC, ZTE Corporation, Sanechips

- R2-2106307, 36331_(R16)_Clarification on paging DRX cycle, ZTE Corporation, Sanechips CR, 36.331,16.4.0

- R2-2106313, 36304_(R16)_Correction on paging resources determination-Alt1, ZTE Corporation, Sanechips, CR, 36.304

- R2-2106320, 36304_(R16)_Correction on paging resources determination-Alt2, ZTE Corporation, Sanechips, CR, 36.304

- R2-2106322, 36300_(R16)_Clarification on paging in RRC_INACTIVE, ZTE Corporation, Sanechips, CR, 36.300

- R2-2106326, draft LS to RAN3 to clarify paging DRX cycle, ZTE Corporation, Sanechips, LS out   To:RAN3

- R2-2105744, Correction on PO determination for UE in inactive state-38.304, ZTE corporation, Sanechips, Ericsson

- R2-2105745, Correction on PO determination for UE in inactive state-38.306, ZTE corporation, Sanechips, Ericsson

- R2-2105751, Discussion on PO determination for UE in inactive state, ZTE corporation, Sanechips, Ericsson

- R2-2105752, Correction on PO determination for UE in inactive state-38.331, ZTE corporation, Sanechips, Ericsson

- R2-2105753, Correction on PO determination for UE in inactive state-36.331, ZTE corporation, Sanechips, Ericsson

- R2-2105754, Correction on PO determination for UE in inactive state-36.304, ZTE corporation, Sanechips, Ericsson

- R2-2105755, Correction on PO determination for UE in inactive state-36.306, ZTE corporation, Sanechips, Ericsson