3GPP TSG-RAN WG2 #115-e	R2-2107239

Electronic meeting, 9th – 27th August, 2021

Agenda Item:	8.15.2

Source:	NEC

Title:	Discussion on DRX suspend/resume mechanism

Document for:	Discussion, Decision

Introduction

In the last RAN2#114-e meeting, regarding SL DRX configuration determination, RAN2 agreed to adopt Tx-UE centric mechanism based on the assistance information from Rx-UE as a baseline.

Agreements on TX-UE centric or RX-UE centric DRX configuration determination 1: In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, support signalling exchange including both 1) Signaling-1: signalling from RX-UE to TX-UE, and 2) Signaling-2: signalling from TX-UE to RX-UE. 2: For SL unicast, TX-UE centric DRX configuration based on the assistance information from RX-UE is agreed as baseline. 2a: In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, signaling-1 (Rx->Tx) is carried via a new PC5-RRC message, from Rx-UE to Tx-UE 2b: In SL unicast, for DRX configuration of the direction where one UE as Tx-UE and the other as Rx-UE, signaling-2 (Tx->Rx) is carried via RRCReconfigurationSidelink, to deliver DRX configuration from Tx-UE to Rx-UE 3: In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other UE as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may report the information received in signaling-1 (Rx->Tx) to the serving network 4: In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Tx-UE is in-coverage and in RRC_CONNECTED state, Tx-UE may obtain DRX configuration from dedicated RRC to generate signalling-2 (Tx->Rx) 5: In SL unicast, for DRX configuration of each direction where one UE as Tx-UE and the other as Rx-UE, when Rx-UE is in-coverage and in RRC_CONNECTED state, Rx-UE report the DRX configuration received in signalling-2 (Tx->Rx) to the serving network

For SL unicast communication between a peer TX-UE and RX-UE, the above agreements provide a basic flow on how to negotiate DRX operation. Several leftover issues are discussed in [1]. This contribution focuses on following scenarios and discusses the DRX suspend/resume mechanism.

- a RX-UE is involved into more than two SL unicast communications in which individual TX-UEs deliver contradictory DRX configurations

- a RX-UE is involved into SL unicast and groupcast/broadcast communication having contradictory DRX configurations

Discussion

The two scenarios focused in this contribution are shown in Fig.1 and Fig.2. Here we assume that two contradictory DRX configurations are serially delivered to the RX-UE.

Figure 1 A RX-UE is involved into two unicast communications in which TX-UEs deliver contradictory DRX configurations

Figure 2 A RX-UE is involved into SL unicast and groupcast/broadcast communication having contradictory DRX configurations

The wording “contradictory” here means that the RX-UE cann’t harmonize DRX configuration 1 and 2 to find out a compromised configuration (e.g., from Uu and sidelink DRX configuration alignment pespectives). In Fig.1 and Fig.2, the only thing RX-UE can do is to quit from sidelink DRX operation and monitor PSCCH continuously. Considering that sidelink broadcast/groupcast communications don’t support individual control for a certain RX-UE, there is no need to inform broadcast/groupcast TX-UE2 the fact that RX-UE is monitoring PSCCH continuously. Moreover, assuming that RX-UE can reject TX-UE2 delivered SL DRX configuration via basic signaling flow of the TX-UE centric SL DRX configuration determination in Fig.1, the point we want to discuss here is whether a DRX configuration suspend indication from RX-UE to TX-UE1 is necessary or not.

Issue 1 Whether a DRX configuration suspend indication from RX-UE to TX-UE1 is necessary or not?

With the TX-UE centric SL DRX configuration determination, the DRX configuration delivered via RRCConfigurationSidelink from TX-UE to RX-UE is mainly based on the information known at TX-UE (e.g., traffic pattern, QoS requirements) whereas the desirable DRX configuration transmitted via the new PC5-RRC message from RX-UE to TX-UE is mainly based on power saving requirements. Generally speaking, a higher QoS always needs a much more frequent transmission/reception, then it may result in a poor power saving performance at RX-UE. It is reasonable to assume that the DRX configuration applied at TX-UE1 and RX-UE finally is well balanced between QoS requirement and power saving performance.

When RX-UE silently quits from DRX operation (i.e., monitors PSCCH continuously), the well balanced DRX configuration is still applied at TX-UE1 even though there is no power saving gain at RX-UE side at all. Although there seems no big issue from keeping the sidelink unicast communication perspective because RX-UE will not miss any sidelink transmission from TX-UE1, the chance of pursuing QoS (i.e., service performance) improvement is lost. In other words, a DRX configuration suspend indication from RX-UE to TX-UE1 is necessary to improve QoS (i.e., service performance). Additionally, upon releasing of subsequent SL unicast/broadcast/groupcast connections, it is essential to instantaneously resume the well balanced DRX configuration between TX-UE1 and RX-UE. Therefore we observe that a DRX configuration suspend/resume indication from RX-UE to TX-UE is necessary for optimization of both QoS(i.e., service performance) and power saving performance.

Observation 1 A DRX configuration suspend/resume indication from RX-UE to TX-UE1 is necessary for optimization of both QoS (i.e., service performance) and power saving performance.

The illustration of DRX operation suspend/resume indication in scenario 2 is show in Fig.3. (The procedure is common for scenario 1 and 2.)

Fig.3. Procedure with DRX operation suspend/resume indication

Issue 2 How to send the DRX configuration suspend/resume indication from RX-UE to TX-UE?

Basically, we have two candidate solutions to send the DRX operation suspend/resume indication, via PC5-RRC message or MAC CEs. Given that RRC message may lead to a longer delay, we slightly prefer to have a MAC layer indication. When considering MAC layer indication, one question might be whether or not Uu link DRX Command MAC CE mechanism can be reused. Comparison of UE behaviors upon receiving DRX Command MAC CE and this DRX suspend/resume MAC indication are concluded in Table 1.

Table 1 UE behaviors upon receiving DRX Command MAC CE and DRX suspend/resume MAC indication

|  | UE behavior |
| --- | --- |
| Upon receiving DRX Command MAC CE | Stop drx-onDurationTimer and drx-InactivityTimer (i.e., enter into off duration immediately) |
| Upon receiving DRX suspend indication | Stop DRX operation and transmit/receive data continuously |
| Upon receiving DRX resume indication | Restart DRX operation and transit data/receive data continuously (based on DRX related timer) |

It is obviously that UE behaviors upon receiving DRX Command MAC CE and this DRX suspend/resume MAC indication are different. Therefore we proposed to introduce a new MAC CE to indicate DRX operation suspend/resume.

Proposal 1 RAN2 to discuss to introduce a new MAC CE to indicate DRX operation suspend/resume.

Summary

This contribution focuses on following scenarios and discuss the DRX suspend/resume mechanism

Observation 1 A DRX configuration suspend/resume indication from RX-UE to TX-UE is necessary for optimization of both QoS (i.e., service performance) and power saving performance.

Proposal 1 RAN2 to discuss to introduce a new MAC CE to indicate DRX operation suspend/resume.

- References

[1] R2-2107238 Leftover issues on overall flow of unicast TX-UE centric mechanism, NEC