3GPP TSG-RAN WG2 Meeting #115 electronic	 R2-2107241

Online, August 09th- 27th, 2021

Agenda Item:    8.8.3

Source:         Xiaomi

Title:         	Considerations on slice based RACH configuration

Document for:	Discussion and Decision

- Introduction

Here are agreements on slice based RACH configuration made at RAN2 114-e meeting as follows.[1]

RAN2 confirm for a slice group, separated RO and/or separate preamble can be configured within the existing RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA

Same as NR Rel-15 conclusion, RAN2 conclude that there is no RA-RNTI collision between slice specific RACH and legacy RACH in shared RO

Same as NR Rel-15 conclusion, RAN2 conclude that the RA-RNTI collision between slice specific RACH and legacy RACH may happen in separate RO.

Working assumption: this can be left to network implementation to resolve it (e.g. network configure RO in different time)

FFS how many slice groups we can have and how they are indicated.

In this contribution, we share our considerations on slice based RACH configuration.

- Discussion

Slice grouping

When slice number is large, it will cause issues for solution1 and solution2, i.e. resource fragment for RACH resource isolation and too many prioritized parameters for the UE. Therefore, slice grouping is necessary to be introduced. As we analyze in the contribution on slice based cell reselection[2], SST can be used as slice group to reduce the payload size of SIB, in order to reduce the impact on spec, we think the same slice grouping mechanism can be reused in RACH configuration.

Furtherly, as we agree before, the solution 1 and solution 2 can work independently in a complementary way to provide more flexible configuration[3]. To provide differentiated RACH configuration for more slices, we provide a feasible solution which is to configure RACH resource per SST and can further configure RA prioritization per SD to slices sharing the same SST-specific RACH resource.

- For slice based RACH configuration, SST can be considered as slice group.

- Separated RO or preamble in shared RO can be configured per SST and RA prioritization can be further configured per SD sharing the same SST-specific RACH resource.

Collision of slice-specific RA prioritization and legacy RA prioritization

RAN2 has confirmed at last meeting that the issue of prioritization parameter collision with MPS/MCS need to be resolved. And there are some candidate solutions as follows

Option1: UE based solution (fixed rule)

Option 1a: slice specific RA prioritization parameter should override MPS/MCS specific RA prioritization parameter.

Option 1b: MPS/MCS specific RA prioritization parameter should override slice specific RA prioritization parameter.

Option2: Network based solution (configurable rule)

Option3: Both option1 and option2.

For the collision of slice-specific RA prioritization and legacy RA prioritization, as we have discussed before, there are some cases that the RA prioritization parameters for MPS and MCS should be higher priority than that of the slice-based RA prioritization as it is configured for specific UE. But there may be some corner cases some companies raised where the MPS/MCS has the lower priority. As the priority of the two sets of RA prioritization parameters is not fixed in different cases, we prefer option2 which is more flexible to cover all cases.

- The collision of slice-specific RA prioritization and legacy RA prioritization should be resolved by network configuration.

| Cases | RACH resource configuration in one BWP | RACH type selection for slice triggered access | Fallback after MSGA or MSG1 attempt number beyond threshold |
| --- | --- | --- | --- |
| Case 1 | 2-step slice specific RACH 4-step common RACH | FFS Always perform 2-step slice specific RACH | Fallback to 4-step common RACH |
| Case 2 | 2-step slice specific RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH. FFS Fallback from 4-step slice specific RACH to 4-step common RACH |
| FFS Case 3 is valid | 4-step slice specific RACH 2-step common RACH | FFS Always perform 4-step slice specific RACH | FFS: No fallback vs. Fallback to common RACH |
| Case 4 | 4-step slice specific RACH 4-step common RACH | Always perform 4-step slice specific RACH | FFS: No fallback vs. Fallback to common RACH |
| Case 5 | 2-step slice specific RACH 2-step common RACH 4-step slice specific RACH 4-step common RACH | RACH type selection based on RSRP threshold | Fallback to 4-step slice specific RACH. FFS Fallback from 4-step slice specific RACH to 4-step common RACH. |
| FFS Case 6 is valid | 2-step slice specific RACH 2-step common RACH | Always perform 2-step slice specific RACH | FFS: No fallback vs. Fallback to common RACH |
| Case 7 | 2-step slice specific RACH 2-step common RACH 4-step common RACH | FFS Always perform 2-step slice specific RACH | Fallback to 2-step common RACH. Fallback to 4-step common RACH. |
| FFS Case 8 is valid | 4-step slice specific RACH 2-step common RACH 4-step common RACH | FFS Always perform 4-step slice specific RACH | FFS Fallback from 4-step slice specific RACH to 4-step common RACH. |

Table 2.1 Cases of slice based RACH configuration[4]

- RACH type selection

For RACH type selection (e.g., slice-specific and common, 2-step and 4-step), there are two options:

Option 1: UE first selects between slice-specific and common RACH, then selects between 2-step and 4-step

Option 2: UE first selects between 2-step and 4-step, then selects between slice-specific and common RACH

For option1, in the case of 2-step slice-specific RACH resource and 4-step common RACH resource configured, UE will first select 2-step slice-specific RACH resource without RSRP checking. However, in this case, it is quite possible that UE failed to access due to bad radio condition and fallback to 4-step common RACH, in this case, it can introduce extra procedure and access delay. It can be resolved by proper network configuration, i.e. 2-step slice-specific RACH resource is always configured together with 4-step slice-specific RACH resource, but it will cause extra RACH resource partitions.

Observation 1: If UE first selects between slice-specific and common RACH, to avoid extra access delay in bad radio condition, it requires 2-step slice-specific RACH resource is always configured together with 4-step slice-specific RACH resource which introduces extra RACH resource partitions.

- If UE first selects between slice-specific and common RACH, case 1 and 7 are invalid as 2-step slice-specific RACH should be configured together with 4-step slice-specific RACH resource to avoid extra access delay in bad radio condition.

In legacy RA procedure, RACH type selection between 2-step RACH and 4-step RACH should be performed firstly based on RSRP threshold if both 2-step and 4-step RACH resource are configured.

Compared with option1, option 2 has minor impacts on current spec, and can avoid the extra access delay in bad radio condition as 2-step slice-specific RACH resource is selected only if configured RSRP threshold is met.

Observation 2: In legacy RA procedure, UE should perform RACH type selection between 2-step RACH and 4-step RACH based on RSRP threshold at first if both 2-step and 4-step RACH resource are configured.

- For slice based RACH type selection, UE should firstly select RACH type between 2-step RACH and 4-step RACH to reduce the impacts on current spec and avoid access delay in bad radio condition.

Fallback mechanism

For the fallback from slice-specific RACH to common RACH, as the fundamental intention of slice based RACH configuration is to guarantee UE fast access, thus we think if UE has failed to access on slice-specific RACH resource, it should be allowed to use common RACH resource to initiate access attempt other than just wait.

- UE should not be prevented to initiate access attempt based on common RACH resource if it failed to get access based on slice-specific RACH resource.

- Conclusions

- During the discussion above, we have the following observations:

Observation 1: If UE first selects between slice-specific and common RACH, to avoid extra access delay in bad radio condition, it requires 2-step slice-specific RACH resource is always configured together with 4-step slice-specific RACH resource which introduces extra RACH resource partitions.

Observation 2: In legacy RA procedure, UE should perform RACH type selection between 2-step RACH and 4-step RACH based on RSRP threshold at first if both 2-step and 4-step RACH resource are configured.

- During the discussion above, we have the following proposals:

Slice grouping

- For slice based RACH configuration, SST can be considered as slice group.

- Separated RO or preamble in shared RO can be configured per SST and RA prioritization can be further configured per SD sharing the same SST-specific RACH resource.

Collision of slice-specific RA prioritization and legacy RA prioritization

- The collision of slice-specific RA prioritization and legacy RA prioritization should be resolved by network configuration.

- RACH type selection

- If UE first selects between slice-specific and common RACH, case 1 and 7 are invalid as 2-step slice-specific RACH should be configured together with 4-step slice-specific RACH resource to avoid extra access delay in bad radio condition.

- For slice based RACH type selection, UE should firstly select RACH type between 2-step RACH and 4-step RACH to reduce the impacts on current spec and avoid access delay in bad radio condition.

- Fallback mechanism

- UE should not be prevented to initiate access attempt based on common RACH resource if it failed to get access based on slice-specific RACH resource.

- References

- RAN2-114-e LTE DCCA Mobility RAN slicing and Multi-SIM (Tero)_2021-05-27-EOM

- R2-2104782 Considerations on slice based cell reselection

- RAN2-113e LTE DCCA Mobility RAN slicing and Multi-SIM (Tero)_2021_02_05_EOM

- R2-2104322 Summary of [AT113bis-e][252][NR] Slice-specific RACH (CMCC)