3GPP TSG-RAN WG2 Meeting #115 electronic	R2-2109024

Online, 9th – 27th August 2021

Agenda item:       6.2.2

Source: 	Huawei, HiSilicon

Title: 	Review report on RRC CRs

Document for:	Discussion and Decision

# Introduction

This document summarizes the contributions submitted to AI 6.2.2. Rapporteur’s recommendations on how to treat them are also provided.

# Correction CRs to be discussed together with Rapporteur’s miscellaneous CRs offline

The following correction CRs could be discussed together in an offline email discussion. The discussion would be mostly on editorial errors or straightforward clarifications. The agreed changes could be merged into Rapporteur’s miscellaneous correction CR(s).

Table 1: Correction CRs to be discussed together offline

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2107166 | Miscelleneous CR on 38.331 | Huawei, HiSilicon | - Change IE sl-ConfigDedicatedNR to sl-ConfigDedicatedForNR in clauses 5.3.1.1, 5.5.3, and 5.8.1. - In clause 5.8.2, modify the UE RRC state in the conditions for NR sidelink communication operation in limited service state. - Change “NR sidelink communication transmission’ to ‘NR sidelink communication reception” in clause 5.8.7. - Modify the description of sidelink DRB release condition in clause 5.8.9.1a.1.1. - Fix the editorial errors. |  |
| R2-2107167 | Miscelleneous CR on 36.331 | Huawei, HiSilicon | - Change SIB19 to SystemInformationBlockTypex19, SIB21 to SystemInformationBlockTypex21, SIB26 to SystemInformationBlockTypex26, SIB28 to SystemInformationBlockTypex28 in clauses 5.2.2.36, 6.2.2 and 6.3.1. - Change sl-ConfigDedicatedNR to sl-ConfigDedicatedForNR in clause 5.5.3. - Fix the editorial errors. |  |
| R2-2107437 | Correction on TS 38.331 from the latest RAN1 decisions | ZTE Corporation, Sanechips | Add the description of Value 1 indicates the corresponding RB is used for PSFCH transmission and reception: sl-PSFCH-RB-Set Indicates the set of PRBs that are actually used for PSFCH transmission and reception. The leftmost bit of the bitmap refers to the lowest RB index in the resource pool, and so on. Value 1 indicates the corresponding RB is used for PSFCH transmission and reception. | This CR is to implement the agreement and recommendation based on RAN 1 LS [1] and could be agreeable. |
| R2-2108178 | Corrections on RRC parameter PSFCH RB set | CATT | In field description of SL-PSFCH-Config: sl-PSFCH-RB-Set Indicates the set of PRBs that are actually used for PSFCH transmission and reception. The leftmost bit of the bitmap refers to the lowest RB index in the resource pool, and so on. Value 0 in the bitmap indicates that the corresponding PRB is not used for PSFCH transmission and reception while value 1 indicates that the corresponding PRB is used for PSFCH transmission and reception (see TS 38.213 [13]). | This CR is also to implement the agreement and recommendation based on RAN 1 LS [1] and could be agreeable. Rapporteur prefers this implementation for its completeness of the description. |
| R2-2108219 | CR on SL-SRB1 integrity check failure | vivo, Ericsson | In 5.8.1, add a NOTE to clarify the UE AS behavior upon detecting the integrity check failure on SL-SRB1: “NOTEX: When integrity check failure concerning SL-SRB1 for a specific destination is detected, the UE sends an indication to the upper layers [57].” | In last meeting, the intention was agreeable and the main concern was on the wording. With the improved wording, Rapporteur thinks this can be agreed. |

Recommendation 1: Discuss the CRs listed in Table 1 in an offline discussion, the agreed changes could be merged into Rapporteur’s miscellaneous correction CR(s).

# Correction CRs to be discussed separately

The following correction CRs could be discussed separately, maybe online first.

Table 2: Correction CRs could be discussed separately

| Tdoc | Title | Company | Summary of changes/proposals | Rapp’s remark |
| --- | --- | --- | --- | --- |
| R2-2107012 | Corrections to usage of dynamic SL grants when T310 is running | Samsung | Clarified that the UE should not use dynamic sidelink grants while T310 is running: NOTE 1: The UE continues to use resources configured in rrc-ConfiguredSidelinkGrant (while T310 is running) until it is released (i.e. until T310 has expired). The UE does not use sidelink configured grant type 2 resources while T310 is running. The UE does not use dynamic sidelink grants (i.e. UE does not monitor PDCCH for sidelink grant) while T310 is running. | It is questionable that it shall be specified UE does not monitor PDCCH for sidelink grant while T310 is running. Monitoring PDCCH is a common Uu behaviour regardless of the grant type (sidelink grant or Uu grant) and there seems no Uu specification on UE not monitoring PDCCH while T310 is running. Further, “UE does not use dynamic sidelink grants” is not equivalent to “UE does not monitor PDCCH for sidelink grant”, strictly speaking. |
| R2-2108218 | Discussion on SL PDCP out-of-order delivery configuration | vivo | Proposal 1： RAN2 to confirm the issue that the parameter sl-OutOfOrderDelivery on the relevant SL DRB configured to Rx UE may not be reasonable given the following unexpected cases. - The Rx UE is configured to execute the out-of-order delivery functionality but the Rx UE’s capability doesn’t support SL PDCP out-of-order delivery. - The Rx UE is configured to execute the out-of-order delivery functionality, but the related Rx UE’s App/Service type cannot use out-of-order delivery Proposal 2： RAN2 to fix the above issue by clarifying that how to set the value of the parameter sl-OutOfOrderDelivery in PC5 RRC message is up to TX UE implementation, if the sl-OutOfOrderDelivery is configured in pre-configuration or network configuration. Proposal 3： If Proposal 1 and 2 are agreeable, adopt the CR in [R2-2108741]. | When RX UE capability related to out-of-order delivery is available to gNB and gNB makes related configuration accordingly, it could be discussed whether Tx UE needs to modify parameter sl-OutOfOrderDelivery in this case. When RX UE capability related to out-of-order delivery is not available to gNB, if the sl-OutOfOrderDelivery is configured in pre-configuration or network configuration, normally such pre-configuration or network configuration shall target the basic (not optional) UE capability, it could be discussed whether Tx UE needs to modify parameter sl-OutOfOrderDelivery in this case. |
| R2-2108741 | Correction on SL PDCP out-of-order delivery configuration | vivo | In 5.8.9.1.2, add a NOTE to clarify that how to set the value of the parameter sl-OutOfOrderDelivery via PC5 RRC is up to UE implementation if the sl-OutOfOrderDelivery is configured in pre-configuration or network configuration: NOTEX: How to set the parameter sl-OutOfOrderDelivery, in case the sl-OutOfOrderDelivery is received in sl-ConfigDedicatedNR, SIB12 or in SidelinkPreconfigNR, is up to UE implementation. | If it is agreed that Tx UE needs to set the parameter sl-OutOfOrderDelivery, it could be discussed on the implementation with this NOTE. |

Recommendation 1: Discuss the contributions/CRs listed in Table 2 separately, maybe online first.

# Conclusions

The contributions submitted to AI 6.2.2 are summarized above, Rapporteur’s recommendations are as follows.

Recommendation 1: Discuss the CRs listed in Table 1 in an offline discussion, the agreed changes could be merged into Rapporteur’s miscellaneous correction CR(s).

Recommendation 2: Discuss the contributions/CRs listed in Table 2 separately, maybe online first.

# Reference

- R2-2106912, LS on RRC parameter for PSFCH RB set, RAN1.

- R2-2107012, Corrections to usage of dynamic SL grants when T310 is running, Samsung Electronics Co., Ltd.

- R2-2107166, Miscelleneous CR on 38.331, Huawei, HiSilicon.

- R2-2107167, Miscelleneous CR on 36.331, Huawei, HiSilicon.

- R2-2107437, Correction on TS 38.331 from the latest RAN1 decisions, ZTE Corporation, Sanechips.

- R2-2108178, Corrections on RRC parameter PSFCH RB set, CATT.

- R2-2108218, Discussion on SL PDCP out-of-order delivery configuration, vivo.

- R2-2108219, CR on SL-SRB1 integrity check failure, vivo, Ericsson.

- R2-2108741, Correction on SL PDCP out-of-order delivery configuration, vivo.