3GPP TSG-RAN WG2#115-e	R2-210xxxx

e-Meeting, 16th – 27th August 2021

Title:	[draft] LS to RAN1 on RAN2 Agreements Related to Resource Selection

Response to:

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	InterDigital (to be RAN WG2)

To:	RAN WG1

Cc:

Contact Person:

#### Name:	Martino Freda

E-mail Address:	martino.freda@interdigital.com

1. Overall Description:

RAN2 has made the following agreements related to resource selection at the TX UE when communicating with RX UE(s) in DRX and would like to inform RAN1 of these agreements.

Firstly, based on RAN2 agreements, RAN2 have the following understandings about the TX UE:

- For unicast, the TX UE maintains a set of timers per pair of source Layer-2 ID and destination Layer-2 ID corresponding to the SL DRX timers in the RX UE.  For groupcast/broadcast, the TX UE maintains a set of timers per destination Layer-2 ID corresponding to the SL DRX timers in the RX UE.  The TX UE uses the timers as part of the criterion for determining the allowable transmission time target for each RX UE.

- For transmissions to RX UE(s) using SL DRX operation, LCP restrictions ensure that a TX UE transmits data in the active time of the RX UE(s).

Furthermore, in RAN2#115-e, RAN2 made the following agreements:

Agreements:

When data is available for transmission to one or more RX UE in DRX, TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) determined by the timers maintained at the TX UE.  Details are FFS. FFS whether RAN1 or RAN2 implement this restriction.

For unicast, the TX UE selects the resources for the initial transmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE. How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact.

For unicast, the TX UE can select the resources for the retransmission associated with any active time (e.g. on duration timer or inactivity timer, or retransmission timer) at the RX UE.  How to handle cases when a transmission may cause these timers to be running at the RX UE is FFS. FFS on groupcast. FFS on whether any spec impact.

For broadcast, the TX UE can select the resources for the initial transmission associated with any active time supported by broadcast (i.e. on duration) at the RX UE.

For broadcast, the TX UE can select the resources for the retransmission associated with any active time supported by broadcast (i.e. on duration) at the RX UE.

2. Actions:

To RAN WG1:

ACTION: 	RAN1 of any issues. asks RAN1 to take h above agreements into account and inform RAN2 whether/how RAN1 intends to reflect the restriction in the following RAN2 agreement in the physical layer procedure of report“When data is available for transmission to one or more RX UE in DRX, TX UE selects the resources taking into account the active time (current or future) of the RX UE(s) determined by the timers maintained at the TX UE.  Details are FFS. FFS whether RAN1 or RAN2 implement this restriction.”

3. Date of Next TSG-RAN WG2 Meetings:

TSG-RAN WG2 #116-e 			Nov 01 – 12 2021	        Electronic Meeting

TSG-RAN WG2 #117			Feb 21 – 25 2022	        Athens