3GPP TSG RAN WG2#115-e	R2-2106956

e-Meeting, 9th - 27th August, 2021

3GPP TSG-RAN WG4 Meeting #99e	R4-2107907

Electronic Meeting, 19th - 27th May, 2021

Title:	Reply LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC Capabilities

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN4

To:	RAN2

Cc: 	RAN1

Contact Person:

Name:	Aijun CAO

E-mail Address:	cao.aijun@zte.com.cn

# 1. Overall Description

RAN4 kindly thanks RAN2 for the LS on the Intra-band and Inter-band (NG)EN-DC/NE-DC capabilities, and RAN4 reaches a common understanding on the three concerned RAN4 capability IEs, i.e., dualPA-Architecture, simultaneousRxTxInterBandENDC, and asyncIntraBandENDC, in the questions. The detailed answers to the RAN2 questions are shown below:

Question 1: For which (NG)EN-DC/NE-DC BC types the above capabilities (ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ /simultaneousRxTxInterBandENDC /asyncIntraBandENDC ) are applicable?

Answer: From RAN4 perspective,

- dualPA-Architecture is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2 and Type 5,

- simultaneousRxTxInterBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 2, Type 3 and Type 4,

- asyncIntraBandENDC is applicable to (NG)EN-DC/NE-DC BC Type 1, Type 2, Type 3 and Type 5.

Question 2: If the capability ul-TimingAlignmentEUTRA-NR/ pa-PhaseDiscontinuityImpacts /dualPA-Architecture/ asyncIntraBandENDC are applicable to the (NG)EN-DC/NE-DC BC Type 1/2/3, whether they are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part?

Answer: Yes, dualPA-Architecture/ asyncIntraBandENDC are used to indicate the restriction to the intra-band (NG)EN-DC/NE-DC BC part.

# 2. Actions:

ACTION: 	RAN4 kindly asks RAN2 to consider answers of above for RAN2 future work.

3. Date of Next TSG-RAN4 Meetings:

3GPP  RAN4#100-e 	16 – 27 Aug 2021	Electronic Meeting

3GPP  RAN4#101-e	1 – 12 Nov 2021	Electronic Meeting